APPENDIX C

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS

NOTES

o  Values  for  new  products are assigned by the Storage Systems

Architecture Group.  Requests for new  assignments  should  be

addressed to SSAG::SSAG.

o  Values  for unannounced products are given in Appendix H, "Un-

announced Products."

+----------+----------------------------------------------+

|Class Byte|                                              |

| (decimal)|     Subsystem Type                           |

+----------+----------------------------------------------+

Table table(): Controller and Unit "Class" Values (cont.)

+----------+----------------------------------------------+

|Class Byte|                                              |

| (decimal)|     Subsystem Type                           |

+----------+----------------------------------------------+

|     0    | reserved -- must not be assigned             |

+----------+----------------------------------------------+

|     1    | Mass storage controllers                     |

+----------+----------------------------------------------+

|     2    | Disk class devices -- DEC Standard 166 disks |

+----------+----------------------------------------------+

|     3    | Tape class devices                           |

+----------+----------------------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                       Page C-2

|     4    | Disk class devices -- DEC Standard 144 disks |

+----------+----------------------------------------------+

|     5    | Loader class devices (see Note below)        |

+----------+----------------------------------------------+

|     6    | SCSI class devices                           |

+----------+----------------------------------------------+

| Note: The  Media  Loader  device  class  is  defined in |

|       anticipation of the adoption of  a  formal  media |

|       loader  architecture.   This  class  value  is in |

|       current  use  as dictated by the temporary waiver |

|       in Section section(medldrerrlog).                                |

+---------------------------------------------------------+

+--------------+------------------------------------------+

|  Model Byte  |                                          |

|  (decimal)   |                                          |

| (see Note 1) |   Controller Type                        |

+--------------+------------------------------------------+

Table table(): Mass Storage Controller "Model" Values (cont.)

+--------------+------------------------------------------+

|  Model Byte  |                                          |

|  (decimal)   |                                          |

| (see Note 1) |   Controller Type                        |

+--------------+------------------------------------------+

|       0      | reserved -- must not be assigned         |

+--------------+------------------------------------------+

|       1      | HSC50                                    |

+--------------+------------------------------------------+

|       2      | UDA50                                    |

+--------------+------------------------------------------+

|       3      | RC25 integrated controller               |

+--------------+------------------------------------------+

|       4      | VMS (software MSCP server)               |

+--------------+------------------------------------------+

|       5      | TU81 integrated controller               |

+--------------+------------------------------------------+

|       6      | UDA5OA                                   |

+--------------+------------------------------------------+

|       7      | RQDX1/RQDX2                              |

+--------------+------------------------------------------+

|       8      | TOPS 10/20 (software MSCP server)        |

+--------------+------------------------------------------+

|       9      | TQK50                                    |

+--------------+------------------------------------------+

|      10      | RUX50                                    |

+--------------+------------------------------------------+

|      11      | unassigned                               |

+--------------+------------------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                       Page C-3

|      12      | KFBTA                                    |

+--------------+------------------------------------------+

|      13      | KDA50-Q                                  |

+--------------+------------------------------------------+

|      14      | TQK70                                    |

+--------------+------------------------------------------+

|      15      | RV20                                     |

+--------------+------------------------------------------+

|      16      | KRQ50                                    |

+--------------+------------------------------------------+

|      17      | unassigned                               |

+--------------+------------------------------------------+

|      18      | KDB50                                    |

+--------------+------------------------------------------+

|      19      | RQDX3                                    |

+--------------+------------------------------------------+

|      20      | RQDX4  (CSS, Reading Special)            |

+--------------+------------------------------------------+

|      21      | SSP/DSSI Disk (see Note 2)               |

+--------------+------------------------------------------+

|      22      | SSP/DSSI Tape (see Note 2)               |

+--------------+------------------------------------------+

|      23      | SSP/DSSI Disk and Tape(see Note 2)       |

+--------------+------------------------------------------+

|      24      | SSP/DSSI Other (see Note 2)              |

+--------------+------------------------------------------+

|      25      | TUK50                                    |

+--------------+------------------------------------------+

|      26      | unassigned                               |

+--------------+------------------------------------------+

|      27      | KDM70                                    |

+--------------+------------------------------------------+

|      28      | reserved                                 |

+--------------+------------------------------------------+

|      29      | TM32                                     |

+--------------+------------------------------------------+

|      30      | RQZX1 (See note 2)                       |

+--------------+------------------------------------------+

|      31      | KCM44 (Polecat)                          |

+--------------+------------------------------------------+

|      32      | HSC70                                    |

+--------------+------------------------------------------+

|      33      | HSC40                                    |

+--------------+------------------------------------------+

|      34      | HSC60                                    |

+--------------+------------------------------------------+

|      35      | HSC90                                    |

+--------------+------------------------------------------+

|    36 - 38   | unassigned                               |

+--------------+------------------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                       Page C-4

|      39      | HSJ30                                    |

+--------------+------------------------------------------+

|      40      | HSJ40                                    |

+--------------+------------------------------------------+

|      41      | HSC65                                    |

+--------------+------------------------------------------+

|      42      | HSC95                                    |

+--------------+------------------------------------------+

|    43 - 44   | unassigned                               |

+--------------+------------------------------------------+

|      45      | HSJ50                                    |

+--------------+------------------------------------------+

|      46      | HSJ80                                    |

+--------------+------------------------------------------+

|    47 - 64   | unassigned                               |

+--------------+------------------------------------------+

|      65      | TK50-DEBNT                               |

+--------------+------------------------------------------+

|      66      | TBK70                                    |

+--------------+------------------------------------------+

|    67 - 95   | unassigned                               |

+--------------+------------------------------------------+

|      96      | RF30                                     |

+--------------+------------------------------------------+

|      97      | RF71                                     |

+--------------+------------------------------------------+

|      98      | TF85                                     |

+--------------+------------------------------------------+

|      99      | TF70                                     |

+--------------+------------------------------------------+

|     100      | RF31                                     |

+--------------+------------------------------------------+

|     101      | RF72                                     |

+--------------+------------------------------------------+

|     102      | RF73                                     |

+--------------+------------------------------------------+

|     103      | TF837                                    |

+--------------+------------------------------------------+

|     104      | RF35                                     |

+--------------+------------------------------------------+

|     105      | EF51                                     |

+--------------+------------------------------------------+

|     106      | HSD30                                    |

+--------------+------------------------------------------+

|     107      | see Appendix appendix(unannouncedprd), Table table(uncntrlrmodel)

|

+--------------+------------------------------------------+

|     108      | RF36                                     |

+--------------+------------------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                       Page C-5

|     109      | RF74                                     |

+--------------+------------------------------------------+

|     110      | TF86                                     |

+--------------+------------------------------------------+

|  111 - 112   | see Appendix appendix(unannouncedprd), Table table(uncntrlrmodel)

|

+--------------+------------------------------------------+

|     113      | unassigned                               |

+--------------+------------------------------------------+

|     114      | RF37                                     |

+--------------+------------------------------------------+

|  115 - 116   | unassigned                               |

+--------------+------------------------------------------+

|     117      | HSD10                                    |

+--------------+------------------------------------------+

|     118      | HSD50                                    |

+--------------+------------------------------------------+

|  119 - 127   | unassigned                               |

+--------------+------------------------------------------+

|  128 - 159   | reserved for future use                  |

+--------------+------------------------------------------+

|  160 - 165   | see Appendix appendix(unannouncedprd), Table table(uncntrlrmodel)

|

+--------------+------------------------------------------+

|  166 - 191   | unassigned                               |

+--------------+------------------------------------------+

|  192 - 247   | reserved for future use                  |

+--------------+------------------------------------------+

|     248      | ULTRIX-32                                |

+--------------+------------------------------------------+

|  249 - 250   | see Appendix appendix(unannouncedprd), Table table(uncntrlrmodel)

|

+--------------+------------------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                       Page C-6

|  251 - 255   | unassigned                               |

+--------------+------------------------------------------+

| Notes: 1. The   preferred   method   for   assigning  a |

|           controller  model  value  is according to the |

|           port  protocol  used.  The  recommended model |

|           ranges are as follows:                        |

|                                                         |

|              o 1-31    SSP                              |

|              o 32-63   CI                               |

|              o 64-95   BVP                              |

|              o 96-127  DSSI                             |

|              o 128-159 Reserved for future use          |

|              o 160-191 FDDI                             |

|              o 192-247 Reserved for future use          |

|              o 248-255 Host Software Servers            |

|                                                         |

|        2.  The  model  values  that reference this note |

|            identify the  generic type  of  a   SSP/DSSI |

|            device   at  the  SSP  level.  Those  values |

|            are  never  returned  at  the MSCP level but |

|            appear here to ensure compatibility with the |

|            SSP specification assignments.               |

+---------------------------------------------------------+

+----------+---------+-------+---------------------------+--------------------------------+

|Model Byte|  Device | Media |   Media Type Identifier   |          Drive/Media           |

| (decimal)|Type Name| Name  +---------------+-----------+        Characteristics         |

|          |         |       |     octal     |    hex    |                                |

+----------+---------+-------+---------------+-----------+--------------------------------+

Table table(): Disk Drive/Media Identifier Values (cont.)

+----------+---------+-------+---------------------------+--------------------------------+

|Model Byte|  Device | Media |   Media Type Identifier   |          Drive/Media           |

| (decimal)|Type Name| Name  +---------------+-----------+        Characteristics         |

|          |         |       |     octal     |    hex    |                                |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     0    | reserved -- must not be assigned                                             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     1    |    DU   | RA80  | 022544,010120 | 2564,1050 | 121 MB, 14", fixed             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     2    |    DA   | RC25  | 020144,030031 | 2064,3019 | 26 MB, 8", removable           |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     3    |    DA   | RCF25 | 020144,031431 | 2064,3319 | 26 MB, 8", fixed               |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     4    |    DJ   | RA60  | 021244,010074 | 22A4,103C | 205 MB, 14", removable         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     5    |    DU   | RA81  | 022544,010121 | 2564,1051 | 456 MB, 14", fixed             |

+----------+---------+-------+---------------+-----------+--------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                       Page C-7

|     6    |    DU   | RD51  | 022544,040063 | 2564,4033 | 10 MB,  5.25",   fixed,   full |

|          |         |       |               |           | height                         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     7    |    DU   | RX50  | 022545,100062 | 2565,8032 | 400  KB,  5.25",  single-sided |

|          |         |       |               |           | 96  TPI  floppy,  full height, |

|          |         |       |               |           | dual drives (800 KB total)     |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     8    |    DU   | RD52  | 022544,040064 | 2564,4034 | 33  MB,   5.25",  fixed,  full |

|          |         |       |               |           | height                         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     9    |    DU   | RD53  | 022544,040065 | 2564,4035 | 71  MB,  5.25",   fixed,  full |

|          |         |       |               |           | height                         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    10    |    DU   | RX33  | 022545,100041 | 2565,8021 | 1200  KB,  5.25", double-sided |

|          |         |       |               |           | 96  TPI  floppy,  half height, |

|          |         |       |               |           | (also accepts RX50 media)      |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    11    |    DU   | RA82  | 022544,010122 | 2564,1052 | 622 MB, 14", fixed             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    12    |    DU   | RD31  | 022544,040037 | 2564,401F | 20  MB,  5.25",   fixed,  half |

|          |         |       |               |           | height                         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    13    |    DU   | RD54  | 022544,040066 | 2564,4036 | 160  MB,  5.25",  fixed,  full |

|          |         |       |               |           | height                         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    14    |    DU   | RRD50 | 022545,021062 | 2565,2232 | 500  MB,   4.75",   removable, |

|          |         |       |               |           | DAD (optical) format           |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    15    | see Appendix appendix(unannouncedprd), Table table(undskdrviden)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

|    16    | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    17    |    DU   | RX18  | 022545,100022 | 2565,8012 | 180  KB,  5.25",  single-sided |

|          |         |       |               |           | 96  TPI  floppy,  full height, |

|          |         |       |               |           | (aka RX180)                    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    18    |    DU   | RA70  | 022544,010106 | 2564,1046 | 280  MB,  5.25",  fixed,  full |

|          |         |       |               |           | height                         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    19    |    DU   | RA90  | 022544,010132 | 2564,105A | 1.216 GB, 9", fixed            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    20    | see Appendix appendix(unannouncedprd), Table table(undskdrviden)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

|    21    |    DI   | RF30  | 021144,060036 | 2264,601E | 150 MB, 5.25" , fixed, half    |

|          |         |       |               |           | height (DSSI Disk Server)      |

+----------+---------+-------+---------------+-----------+--------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                       Page C-8

|    22    |    DI   | RF71  | 021144,060107 | 2264,6047 | 400 MB, 5.25" , fixed, half    |

|          |         |       |               |           | height (DSSI Disk Server)      |

+----------+---------+-------+---------------+-----------+--------------------------------+

|  23 - 24 | see Appendix appendix(unannouncedprd), Table table(undskdrviden)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

|    25    |    DU   | ESE20 | 022513,031224 | 254B,3294 | 120 MB Solid State disk drive  |

|          |         |       |               |           | with data retention            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    26    |    DU   | RRD40 | 022545,021050 | 2565,2228 | 500  MB,   4.75",   removable, |

|          |         |       |               |           | DAD (optical) format           |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    27    |    DI   | RF31  | 021144,060037 | 2264,601F | 381 MB, 5.25" fixed, half      |

|          |         |       |               |           | height (DSSI Disk Server)      |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    28    |    DI   | RF72  | 021144,060110 | 2264,6048 | 1 GB, 5.25", fixed, full       |

|          |         |       |               |           | height (DSSI Disk Server)      |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    29    |    DU   | RA92  | 022544,010134 | 2564,105C | 1.5 GB, 9", fixed              |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    30    | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    31    |    DU   | ESE52 | 022513,031264 | 254B,32B4 | 120MB  Solid state disk        |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    32    | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    33    |    DI   | RFH31 | 021144,062037 | 2264,641F | 190.5 MB, 5.25" fixed, half    |

|          |         |       |               |           | height (DSSI Disk Server)      |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    34    |    DI   | RFH72 | 021144,062110 | 2264,6448 | 500 MB, 5.25", fixed, full     |

|          |         |       |               |           | height (DSSI Disk Server)      |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    35    |    DI   | RF73  | 021144,060111 | 2264,6049 | 2 GB, 5.25", fixed             |

|          |         |       |               |           | (DSSI Disk Server)             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    36    |    DI   | RFH73 | 021144,062111 | 2264,6449 | 1 GB, 5.25", fixed             |

|          |         |       |               |           | (DSSI Disk Server)             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    37    |    DU   | RA72  | 022544,010110 | 2564,1048 | 1.0 GB, 5.25", fixed           |

+----------+---------+-------+---------------+-----------+--------------------------------+

|  38 - 39 | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    40    |    DU   | RA71  | 022544,010107 | 2564,1047 | 700 MB, 5.25", fixed           |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    41    |    DI   | RFF31 | 021144,061437 | 2264,631F | 200 MB, 5.25", fixed, half     |

|          |         |       |               |           | height. (DSSI Disk Server)     |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    42    |    DI   | RF35  | 021144,060043 | 2264,6023 | 852 MB, 3.5", fixed            |

|          |         |       |               |           | (DSSI Disk Server)             |

+----------+---------+-------+---------------+-----------+--------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                       Page C-9

|    43    |    DI   | RFH35 | 021144,062043 | 2264,6423 | 426 MB,3.5", fixed             |

|          |         |       |               |           | (DSSI Disk Server)             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|  44 - 46 | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    47    |    DU   | RA73  | 022544,010111 | 2564,1049 | 2.0 GB 5.25" fixed             |

|          |         |       |               |           |                                |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    48    |    DU   | ESE56 | 022513,031270 | 254B,32B8 | 600 MB  Solid state disk       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    49    |    DU   | ESE58 | 022513,031272 | 254B,32BA | 1 GB  Solid state disk without |

|          |         |       |               |           | data retention.                |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    50    | see Appendix appendix(unannouncedprd), Table table(undskdrviden)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

|    51    |    DI   | EF51  | 021112,060063 | 224A,6033 | 100 MB Solid State Disk        |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    51    |    DI   | EF52  | 021112,060064 | 224A,6034 | 200 MB Solid State Disk        |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    51    |    DI   | EF53  | 021112,060065 | 224A,6035 | 250 MB Solid State Disk without|

|          |         |       |               |           | data retention.                |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    51    |    DI   | EF54  | 021112,060066 | 224A,6036 | 400 MB Solid State Disk        |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    51    |    DI   | EF58  | 021112,060072 | 224A,603A | 1 GB Solid State Disk without  |

|          |         |       |               |           | data retention.                |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    52    |    DI   | RF36  | 021144,060044 | 2264,6024 | 1.6 GB  3 1/2" fixed           |

|          |         |       |               |           | (DSSI Disk Server)             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    53    | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    54    |    DI   | RF74  | 021144,060112 | 2264,604A | 5 1/4" fixed, TBS GB.          |

|          |         |       |               |           | (DSSI Disk Server)             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|  55 - 57 | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    58    |    DI   | RF37  | 021144,060045 | 2264,6025 | 3 1/2", fixed, 2.1 GB.         |

|          |         |       |               |           | (DSSI Disk Server)             |

+----------+---------+-------+---------------+-----------+--------------------------------+

| 59 - 253 | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   254    |    DU   | HSX02 | 022521,036002 | 2551,3C02 | SCSI-embedded MSCP pseudo disk.|

|          |         |       |               |           | (Command disk for loaders, etc)|

+----------+---------+-------+---------------+-----------+--------------------------------+

|   254    | see Appendix appendix(unannouncedprd), Table table(undskdrviden)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                      Page C-10

|   255    |    DU   | HSX00 | 022521,036000 | 2551,3C00 | MSCP served SCSI disk.         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   255    |    DU   | HSX01 | 022521,036001 | 2551,3C01 | MSCP served SCSI disk array.   |

+----------+---------+-------+---------------+-----------+--------------------------------+

+----------+---------+-------+---------------------------+--------------------------------+

|Model Byte|  Device | Media |   Media Type Identifier   |          Drive/Media           |

| (decimal)|Type Name| Name  +---------------+-----------+        Characteristics         |

|          |         |       |     octal     |    hex    |                                |

+----------+---------+-------+---------------+-----------+--------------------------------+

Table table(): Tape Drive/Media Identifier Values (cont.)

+----------+---------+-------+---------------------------+--------------------------------+

|Model Byte|  Device | Media |   Media Type Identifier   |             Drive              |

| (decimal)|Type Name| Name  +---------------+-----------+        Characteristics         |

|          |         |       |     octal     |    hex    |                                |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     0    | reserved -- must not be assigned                                             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     1    |    MU   | TA78  | 066550,010116 | 6D68,104E | 9-track, PE/GCR modes, 125 ips |

|          |         |       |               |           | start/stop                     |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     2    |    MU   | TU81  | 066551,050121 | 6D69,5051 | 9-track,  PE/GCR modes,  25/75 |

|          |         |       |               |           | ips streamer                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     3    |    MU   | TK50  | 066550,130062 | 6D68,B032 | 94  MB,  Block Mode Cartridge; |

|          |         |       |               |           | emulates   9-track   operation |

|          |         |       |               |           | (except  read reverse), 75 ips |

|          |         |       |               |           | streamer                       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     4    |    MU   | TA81  | 066550,010121 | 6D68,1051 | STI version of the TU81        |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     5    |    MU   | TA79  | 066550,010117 | 6D68,104F | 9-track, PE/GCR modes, 125 ips |

|          |         |       |               |           | start/stop                     |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     6    | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     7    |    MU   | TA90  | 066550,010132 | 6D68,105A | 200  MB,  Cartridge,  Modified |

|          |         |       |               |           | GCR  (IBM  3480  .5" Cartridge |

|          |         |       |               |           | Tape  Format), 79 ips streamer |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     8    |    MU   | RV60  | 066545,060074 | 6D65,603C | 2  GB, 12.0", removable, Write |

|          |         |       |               |           | Once   (optical)  disk,  "tape |

|          |         |       |               |           | mode";     emulates    9-track |

|          |         |       |               |           | operation, machine loadable    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     9    | see Appendix appendix(unannouncedprd), Table table(untapdrviden)

|

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                      Page C-11

+----------+---------+-------+---------------+-----------+--------------------------------+

|    10    |    MI   | TF85  | 065150,060125 | 6A68,6055 | 2.6 GB .50 Cart. 100 ips,      |

|          |         |       |               |           | 800 KB/sec streamer. for DSSI. |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    11    |    MI   | TK70  | 065150,130106 | 6A68,B046 | 300 MB,  Block Mode Cartridge; |

|          |         |       |               |           | emulates   9-track   operation |

|          |         |(TF70) |               |           | (except read reverse), 100 ips |

|          |         |       |               |           | streamer                       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    12    |    MU   | TA91  | 066550,010133 | 6D68,105B | 200MB Cartridge, Modified GCR, |

|          |         |       |               |           | (IBM 3490 .5" Cartridge Tape   |

|          |         |       |               |           | Format), 79 ips Streamer. Store|

|          |         |       |               |           | up to 1 GB with Compaction.    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    13    | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    14    |    MU   | TK70  | 066550,130106 | 6D68,B046 | 300 MB,  Block Mode Cartridge; |

|          |         |       |               |           | emulates   9-track   operation |

|          |         |       |               |           | (except read reverse), 100 ips |

|          |         |       |               |           | streamer                       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    15    |    MU   | RV20  | 066545,060024 | 6D65,6014 | 2  GB, 12.0", removable, Write |

|          |         |       |               |           | Once   (optical)  disk,  "tape |

|          |         |       |               |           | mode";     emulates    9-track |

|          |         |       |               |           | operation                      |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    16    |    MU   | TM32  | 066550,150040 | 6D68,D020 | Gapless   tape   controller /  |

|          |         |       |               |           | interface                      |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    17    |    MU   | TA85  | 066550,010125 | 6D68,1055 | 2.6 GB .50 Cartridge, 100 ips  |

|          |         |       |               |           | streamer for KDM/HSC.          |

|          |         |       |               |           | STI version of the TF85        |

+----------+---------+-------+---------------+-----------+--------------------------------+

|  18 - 19 | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    20    |    MU   | TAD85 | 066550,011125 | 6D68,1255 | TZ857 with integrated STI to   |

|          |         |       |               |           | SCSI adapter. Supports direct  |

|          |         |       |               |           | cartridge access via DISPLAY   |

|          |         |       |               |           | command.  NOTE:  this entry    |

|          |         |       |               |           | has been incorrectly used to   |

|          |         |       |               |           | identify the TZ857 w/o adapter |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    21    |    MI   | TF86  | 065150,060126 | 6A68,6056 | 6.0 GB .50 Cart. 100 ips,      |

|          |         |       |               |           | 800 KB/sec streamer. for DSSI. |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    22    |    MU   | TA86  | 066550,010126 | 6D68,1056 | 6.0 GB .50 Cart. 100 ips,      |

|          |         |       |               |           | 800 KB/sec streamer. STI vers. |

|          |         |       |               |           | Uses separate STI to SCSI conv |

+----------+---------+-------+---------------+-----------+--------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                      Page C-12

|    23    |    MU   | TAD86 | 066550,011126 | 6D68,1256 | 6.0 GB .50 Cart. 100 ips,      |

|          |         |       |               |           | 800 KB/sec streamer. 7 cart mag|

|          |         |       |               |           | TZ867 with integrated STI to   |

|          |         |       |               |           | SCSI converter.  NOTE:  this   |

|          |         |       |               |           | entry has been incorrectly     |

|          |         |       |               |           | used to identify the TZ867.    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    24    |    MU   | TAD87 | 066550,011127 | 6D68,1257 | 15.0 GB .50 Cart. 100 ips,     |

|          |         |       |               |           | streamer. 7 cartridge magazine |

|          |         |       |               |           | TZ877 with integrated STI to   |

|          |         |       |               |           | SCSI converter.  NOTE:  this   |

|          |         |       |               |           | entry has been incorrectly     |

|          |         |       |               |           | used to identify the TZ877.    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    25    | see Appendix appendix(unannouncedprd), Table table(untapdrviden)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

|    26    |    MU   | TZ87  | 066551,120127 | 6D69,A057 | 10.0 GB (20.0 GB compressed)   |

|          |         |       |               |           | .50 Cart. 100 ips streamer.    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    27    |    MU   | TLZ04 | 066550,146404 | 6D68,CD04 | RDAT 1.2 GB                    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    28    |    MU   | TLZ06 | 066550,146406 | 6D68,CD06 | RDAT-2 4 GB                    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    29    |    MU   | TZK10 | 066551,122612 | 6D69,A58A | 320/525 MB QIC                 |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    30    |    MU   | TZ86  | 066551,120126 | 6D69,A056 | 6.0GB .50 Cart 100 ips         |

|          |         |       |               |           | streamer                       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    31    |    MU   | TA92  | 066550,010134 | 6D68,105C | 800MB Cartridge, Modified GCR  |

|          |         |       |               |           | (IBM 3490E Enhanced Capacity   |

|          |         |       |               |           | .5" Cartridge Tape Format),    |

|          |         |       |               |           | 36 track                       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    32    |    MU   | TZ88  | 066551,120130 | 6D69,A058 | 20.0 GB DLT tape drive         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    33    |    MU   | TKZ09 | 066550,136411 | 6D68,BD09 | 8MM tape drive, 8200/8500 mode |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    34    |    MU   | TKZ15 | 066550,136417 | 6D68,BD0F | 8MM tape drive, 8200/8500 mode |

|          |         |       |               |           | with compression               |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    35    |    MU   | TLZ07 | 066550,146407 | 6D68,CD07 | 4MM tape drive, DDS-2 mode     |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    36    | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    37    |    MU   | TKZ60 | 066550,136474 | 6D68,BD3C | 3480-compatible tape drive     |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    38    |    MU   | STK42 | 066547,042652 | 6D67,45AA | STK 3480-compatible tape drive |

+----------+---------+-------+---------------+-----------+--------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                      Page C-13

|    39    |    MU   | TZK11 | 066551,122613 | 6D69,A58B | QIC-2                          |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    40    |    MU   | TSZ07 | 066551,036407 | 6D69,3D07 | 9 track 1600/6250 bpi          |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    41    |    MU   | TZ85  | 066551,120125 | 6D69,A055 | 6.0GB .50 Cart 100 ips         |

|          |         |       |               |           | streamer                       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    42    | see Appendix appendix(unannouncedprd), Table table(untapdrviden)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

|    43    |    MU   | TSZ06 | 066551,036406 | 6D69,3D06 | 9 track 1600/6250 bpi          |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    44    |    MU   | TKZ08 | 066550,136410 | 6D68,BD08 | 8MM tape drive, 8200 mode      |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    45    |    MU   | TZ30  | 066551,120036 | 6D69,A01E | 94 MB block mode cartridge     |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    46    |    MU   | TSZ05 | 066551,036405 | 6D69,3D05 | 9 track                        |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    47    | see Appendix appendix(unannouncedprd), Table table(untapdrviden)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

|    48    |    MU   | TLZ09 | 066550,146411 | 6D68,CD09 | 4MM tape drive, DDS-3 mode     |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    49    | see Appendix appendix(unannouncedprd), Table table(untapdrviden)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

| 50 - 254 | unassigned                                                                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   255    |    MU   | HST00 | 066521,035000 | 6D51,3A00 | MSCP served SCSI tape device.  |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   255    |    MU   | HST01 | 066521,035001 | 6D51,3A01 | MSCP served SCSI tape device   |

|          |         |       |               |           | with media changer.            |

+----------+---------+-------+---------------+-----------+--------------------------------+

+----------+---------+-------+---------------------------+--------------------------------+

|Model Byte|  Device | Media |   Media Type Identifier   |                                |

| (decimal)|Type Name| Name  +---------------+-----------+ Device (see Note below)        |

|          |         |       |     octal     |    hex    |                                |

+----------+---------+-------+---------------+-----------+--------------------------------+

Table table(): Media Loader Identifier Values (cont.)

+----------+---------+-------+---------------------------+--------------------------------+

|Model Byte|  Device | Media |   Media Type Identifier   |                                |

| (decimal)|Type Name| Name  +---------------+-----------+ Device (see Note below)        |

|          |         |       |     octal     |    hex    |                                |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     0    | reserved -- must not be assigned                                             |

+----------+---------+-------+---------------+-----------+--------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                      Page C-14

|     1    |  none   | none  |      none     |    none   | 64 cartridge jukebox           |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     2    |  none   | none  |      none     |    none   | Quabbin Loader                 |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     3    |  none   | none  |      none     |    none   | TLZ06 Loader                   |

+----------+---------+-------+---------------+-----------+--------------------------------+

+----------+---------+-------+---------------------------+--------------------------------+

|Model Byte|  Device | Media |   Media Type Identifier   |          Drive/Media           |

| (decimal)|Type Name| Name  +---------------+-----------+        Characteristics         |

|          |         |       |     octal     |    hex    |                                |

+----------+---------+-------+---------------+-----------+--------------------------------+

Table table(): SCSI Storage Device/Media Identifier Values (cont.)

+----------+---------+-------+---------------------------+--------------------------------+

|Model Byte|  Device | Media |   Media Type Identifier   |          Drive/Media           |

| (decimal)|Type Name| Name  +---------------+-----------+        Characteristics         |

|          |         |       |     octal     |    hex    |                                |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     0    | reserved -- must not be assigned                                             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     1    |    DK   | RX23  | 021345,100027 | 22E5,8017 | 1.4 MB, 3.5", floppy           |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     2    |    DK   | RX26  | 021345,100032 | 22E5,801A | 2.8 MB, 3.5", floppy           |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     3    |    DK   | RX33  | 021345,100041 | 22E5,8021 | 1.2 MB, 5.25" HH, floppy       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     4    |    DK   | RZ22  | 021345,120026 | 22E5,A016 | 5.2 MB, 3.5", fixed            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     5    |    DK   | RZ23  | 021345,120027 | 22E5,A017 | 104 MB, 3.5", fixed            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     6    |    DK   | RZL23 | 021345,123027 | 22E5,A617 | 121 MB, 3.5", fixed (RZ23L)    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     7    |    DK   | RZ24  | 021345,120030 | 22E5,A018 | 209 MB, 3.5", fixed            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     8    |    DK   | RZ25  | 021345,120031 | 22E5,A019 | 426 MB, 3.5", fixed            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     9    |    DK   | RZL24 | 021345,123030 | 22E5,A618 | 245 MB, 3.5", fixed (RZ24L)    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    10    |    DK   | RZ26  | 021345,120032 | 22E5,A01A | 1.05 GB 3 1/2" fixed           |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    11    |    DK   | RZ35  | 021345,120043 | 22E5,A023 | 852 MB, 3.5", fixed            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    12    |    DK   | RZ27  | 021345,120033 | 22E5,A01B | 1.6 GB, 3.5", fixed            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    13    |    DK   | RZ28  | 021345,120034 | 22E5,A01C | 2.1 GB, 3.5", fixed            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    14    |    DK   | RZ55  | 021345,120067 | 22E5,A037 | 332 MB, 5.1/4", fixed          |

+----------+---------+-------+---------------+-----------+--------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                      Page C-15

|    15    |    DK   | RZ56  | 021345,120070 | 22E5,A038 | 665 MB, 5.1/4", fixed          |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    16    |    DK   | RZ57  | 021345,120071 | 22E5,A039 | 1.0 GB, 5.1/4", fixed          |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    17    |    DK   | RZ58  | 021345,120072 | 22E5,A03A | 1.38 GB, 5.1/4", fixed         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    18   |   unassigned                                                                  |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    19    |    DK   | RZ29  | 021345,120035 | 22E5,A01D | 4.3 GB, 3.5", fixed            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    20   |   unassigned                                                                  |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    21    |    DK   | RZ72  | 021345,120110 | 22E5,A048 | 1 GB, 5.25", fixed             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    22    |    DK   | RZ73  | 021345,120111 | 22E5,A049 | 2 GB, 5.25", fixed             |

+----------+---------+-------+---------------+-----------+--------------------------------+

|  23 - 24 | see Appendix appendix(unannouncedprd), Table table(unscsistodev)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

|  25 - 39 |   unassigned                                                                 |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    40    |    DK   | EZ51  | 021313,120063 | 22CB,A033 | 100 MB Solid State Disk        |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    41    | see Appendix appendix(unannouncedprd), Table table(unscsistodev)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

|    42    |   unassigned                                                                 |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    43    |    DK   | EZ54  | 021313,120066 | 22CB,A036 | 400 MB Solid State Disk        |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    44    |    DK   | EZ58  | 021313,120072 | 22CB,A03A | 1 GB 5.25" Solid State Disk    |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    45    |    DK   | RZ74  | 021345,120112 | 22E5,A04A | 3.57 GB, 5.25", fixed          |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    46    |    DK   | EZ31  | 021313,120037 | 22CB,A01F | 134 MB 3.5" Solid State Disk   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    47    |    DK   | EZ32  | 021313,120040 | 22CB,A020 | 268 MB 3.5" Solid State Disk   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|  48 - 54 |   unassigned                                                                 |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    55    |    DK   | EZ69  | 021313,120105 | 22CB,A045 | 950 MB 5.25" Solid State Disk  |

+----------+---------+-------+---------------+-----------+--------------------------------+

|    56    |    DK   | EZ64  | 021313,120100 | 22CB,A040 | 475 MB 5.25" Solid State Disk  |

+----------+---------+-------+---------------+-----------+--------------------------------+

| 57 - 190 |   unassigned                                                                 |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   191    | see Appendix appendix(unannouncedprd), Table table(unscsistodev)

|

+----------+---------+-------+---------------+-----------+--------------------------------+

CONTROLLER, UNIT, AND MEDIA TYPE IDENTIFIERS                                      Page C-16

|   193    |    DK   | RRD42 | 021345,021052 | 22E5,222A | 600 MB, 5.25" CD-ROM w/audio   |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   194    |    DK   | RRD43 | 021345,021053 | 22E5,222B | 600 MB 2X CD-ROM drive         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   195    |    DK   | RRD44 | 021345,021054 | 22E5,222C | 600 MB 2.2X CD-ROM drive       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   196    |    DK   | RRD45 | 021345,021055 | 22E5,222D | 600 MB 4.4X CD-ROM drive       |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   197    |    DK   | RWZ52 | 021345,076464 | 22E5,7D34 | 600/1200 MB Multifunction MO   |

|          |         |       |               |           | drive.                         |

+----------+---------+-------+---------------+-----------+--------------------------------+

|   198    |    DK   | RWZ53 | 021345,076465 | 22E5,7D35 | 1.2 GB Multifunction MO drive  |

+----------+---------+-------+---------------+-----------+--------------------------------+

|  |   199    | see Appendix appendix(unannouncedprd), Table table(unscsistodev)

|                                       |

|  +----------+---------+-------+---------------+-----------+--------------------------------+

|  | 200-255  |   unassigned                                                                 |

|  +----------+---------+-------+---------------+-----------+--------------------------------+

| Notes: 1. The   preferred   method   for   assigning  a model  value  is according to   |

|           the device type (i.e. disk, tape, or optical). The  recommended model byte    |

|           ranges are as follows:                                                        |

|                                                                                         |

|              o 1-191   Disk devices                                                     |

|              o 192-255 Optical disk devices                                             |

|                                                                                         |

+-----------------------------------------------------------------------------------------+