APPENDIX D

BUFFER DESCRIPTOR FORMATS

The  information  in  this Appendix is NOT a formal part of MSCP.

The formal specification for buffer descriptors is in the indivi-

dual  communications  mechanism specifications.  This information

is summarized here in order to provide a convenient reference for

all buffer descriptors in a single document.

It  is  recommended  that  communications  mechanisms  adhere  as

closely as possible to the following preferred or generic  buffer

descriptor  format.  Although not required by MSCP, similarity of

buffer descriptor format can only simplify the design of host and

controller   port  drivers.   The  preferred  or  generic  buffer

descriptor format is as follows:

31 0

+-------------------------------+

| offset to start of buffer |

+-------------------------------+

| buffer name |

+-------------------------------+

| connection identifier |

+-------------------------------+

The  "connection  identifier"  identifies  a  specific connection

across which the block data transfer  should  take  place.   Thus

this  field  identifies,  in a communications mechanism dependent

manner, the host or node with  which  the  transfer  should  take

place.    The  "buffer  name"  identifies,  in  a  communications

mechanism dependent manner, the buffer or  region  of  memory  on

that system to which the transfer should take place.  The "offset

to start of buffer" is the byte offset from the beginning of  the

named  buffer  to the point at which the transfer should actually

start.

BUFFER DESCRIPTOR FORMATS                                Page D-2

The  "connection  identifier" allows for third party transfers --

that is, it allows a host to request transfers to  or  from  some

other host's memory.  The "buffer name" provides a logical rather

than physical identification of the buffer, thus simplifying vir-

tual memory management and possibly providing a consistency check

that the buffer is valid.  The "offset to start of  buffer"  sim-

plifies  certain  aspects  of third party transfers.  It allows a

controlling host to accept a transfer from some other host, break

the  transfer up into segments, and forward each segment indepen-

dently to  a  controller.   Such  segmentation  of  transfers  is

especially  useful  with  file  systems,  in  which  a contiguous

transfer  within  a  file  may  require  transferring  physically

noncontiguous portions of a disk.

The  buffer  descriptor used with the CI communications mechanism

uses the exact format illustrated for the generic buffer descrip-

tor.  The "offset to start of buffer" is indeed a byte offset and

the "buffer name" is the CI  block  transfer  buffer  name.   The

"connection  identifier" is the controller's SCA connection iden-

tifier for the connection between the disk class driver  and  the

disk MSCP server.

There  are two buffer descriptor formats used with the UNIBUS and

QBUS in order to accommodate two different transfer methods:  un-

mapped and mapped.

The format for unmapped transfers is as follows:

31 0

+-+-----+-----------------------+

|0| chan|buffer physical address|

+-+-----+-----------------------+

| reserved |

+-------------------------------+

| reserved |

+-------------------------------+

The format for mapped transfers is as follows:

31 9 8 0

+-+-----+-----------------------+

|1| map register index | offset |

+-+-----+-----------------------+

|mbz| map register base |

+-------------------------------+

| reserved |

+-------------------------------+

The  following  interpretation  shows  that the unmapped transfer

format is consistent with the generic buffer  descriptor  format.

BUFFER DESCRIPTOR FORMATS                                Page D-3

The  UNIBUS and QBUS are inherently single host busses.  Further-

more, all anticipated controllers for these busses only support a

single  device  class.  Therefore there can only be a single con-

nection between the host and any controller.  Thus it is  reason-

able  to assign a fixed connection identifier of zero.  Secondly,

since these busses are not used with any (bus  visible)  form  of

virtual  memory,  there  is  no  use for the concept of a logical

buffer name.  Thus it is reasonable to assign a fixed buffer name

of  zero, denoting the entire address space available on the bus.

This leaves the offset field to denote the byte offset  from  ad-

dress  zero,  or  merely  the starting address of the buffer.  In

order to accommodate VAX-11/780 UBAs, it is necessary to  include

the  UBA  channel  number  so  that  controllers  can request UBA

purges.  This has been placed with the bus address for controller

convenience.

On  the other hand the mapped transfer format, with the exception

of the fixed connection identifier of zero, deviates totally from

the  generic  buffer  descriptor format.  See the Storage Systems

Port  Specification  (SSP)  for  complete   details   on   mapped

transfers.