APPENDIX E

WAIVERS AND EXCEPTIONS

E.1  UDA50

E.1.1  Unit and Controller Revision Numbers

Waiver:

Early  serial  number  UDA50  controllers  do not return unit

revision numbers in the GET UNIT STATUS end message  or  con-

troller  revision  numbers in the SET CONTROLLER CHARACTERIS-

TICS end message.

E.2  RC25

WAIVERS AND EXCEPTIONS                                   PAGE E-2

RC25

E.2.1  Error Log Message Ordering

Waiver:

Under  certain  circumstances  the RC25 will return error log

messages after the corresponding command's end message.  Sec-

tion  section(errorlog_mess) "Error Messages," requirement #3

of the requirements controllers must meet  in  order  to  not

provide  error  log sequence numbers, requires that all error

log messages be returned before the  corresponding  command's

end message.

The  particular circumstances where the error log is returned

after the corresponding command's end message are as follows:

1.  If  the  host does not abide by the credit limit

supplied by the controller,  and  posts  <credit

limit>+1 commands, and an event occurs which re-

quires the generation of an error log.

2.  If  the host abides by the credit limit supplied

by the controller, and posts <credit limit> com-

mands,  and two (or more) events occur which re-

quire the generation  of  two  (or  more)  error

logs.

3.  Sometimes,  if  while  processing Immediate type

commands the host posts <credit limit>  commands

and  an  event occurs which requires the genera-

tion of an error log.

E.2.2  Data Error (subcode "Forced Error") Reporting

Waiver:

The RC25 reports Success (subcode "Normal") status instead of

Data Error (subcode "Forced Error") status  upon  termination

of the following:

WAIVERS AND EXCEPTIONS                                   PAGE E-3

RC25

1.  a  COMPARE  HOST DATA command which encounters a

logical block containing a "Forced Error."

2.  the  compare portion of a "Compare, Force Error"

modified WRITE command.

3.  the  compare portion of a "Force Error" modified

WRITE command  when  the  "Compare  Write"  unit

characteristic  was enabled via an ONLINE or SET

UNIT CHARACTERISTICS command.

WARNING

A discovery was made after this waiver was approved

(and the RC25 already shipped) that  the  erroneous

behavior described above, especially with regard to

the last two cases listed, adversely  affected  the

operation  of the Host Initiated Bad Block Replace-

ment (HIBBR) algorithm described in early  versions

of  the  Digital  Storage  Architecture Disk Format

(DSDF) specification.  One of the most harmful con-

sequences  is  that  under certain conditions HIBBR

could mark all available  replacement  blocks  con-

tained  on  an RC25 disk as unusable (BAD), thereby

rendering the disk essentially unusable.   A  brief

description,  paraphrasing DSDF V1.4.0, Section 7.5

"Host Initiated Block  Replacement  Algorithm,"  of

how this problem might occur is as follows:

1.  HIBBR  determines  that  the  data ob-

tained from the block  being  replaced

were  invalid,  and that the block was

in fact bad.

2.  HIBBR   scans  the  RCT  to  obtain  a

replacement  block.   If  RBN   found,

proceed  to Step 3, below.  If RBN not

found proceed to Step 7, below.

3.  HIBBR  issues  a  REPLACE  command  to

revector the LBN to the RBN acquired.

4.  HIBBR  issues a "Force Error, Compare"

modified WRITE command to  the  revec-

tored LBN.

5.  Assuming  that  no  other errors occur

during the  WRITE,  the  RC25  reports

Success (subcode "Normal") status.

6.  HIBBR assumes that the RBN was bad be-

cause a Data  Error  (subcode  "Forced

Error") status was not returned.

WAIVERS AND EXCEPTIONS                                   PAGE E-4

RC25

7.  HIBBR  marks  the RCT entry of the RBN

in question as unusable, then  contin-

ues processing at Step 2, above.

8.  HIBBR  issues a "Force Error" modified

WRITE, of the saved data  to  the  LBN

that was being replaced, puts an entry

in the  host's  error  log  indicating

that the replacement failed, cleans-up

the RCT to indicate replacement is  no

longer in progress, and takes whatever

host dependent action  is  appropriate

for a failed bad block replacement.

Note  that  the  DSDF HIBBR has since been ECOed to

eliminate the  possibility  of  this  problem  (and

numerous others) occurring.

E.2.3  Improper Duplicate Unit Number Processing

Waiver:

RC25  operation does not fully comply with all Duplicate Unit

Number  processing  requirements.   The  specific   erroneous

behavior  is  described below in the form of four operational

cases.

The  case  descriptions assume two RC25 drives connected in a

master/slave configuration.  Throughout  the  discussion  the

master drive is referred to as "M" and the slave as "S".

o  Case #1

Set-up Conditions:

1.  "M"  drive  spun  up  with 0/1 unit plug in-

serted

2.  "S"  drive  spun down with no unit plug (un-

known to the controller)

WAIVERS AND EXCEPTIONS                                   PAGE E-5

RC25

3.  controller  initialized  and characteristics

set

4.  "M"  unit 0 and "M" unit 1 brought ONLINE to

host

5.  duplicate  unit  number condition created by

inserting 0/1 unit number plug into "S"  and

attempting to spin it up

Improper operation:

Status  code  Success  (subcode  "Duplicate Unit

Number") is not reported in the end messages  of

all  commands  which require it when a duplicate

unit number condition exists  and  the  affected

unit  is  ONLINE to a host.  The improper opera-

tion occurs on the following commands:

>  WRITE

>  READ

>  ACCESS

>  COMPARE HOST DATA

>  ERASE

>  REPLACE

>  SET UNIT CHARACTERISTICS

The  only  command which reports the status cor-

rectly is the  GET  UNIT  STATUS  command.   The

failure  occurs  on  commands issued to both "M"

unit 0 and "M" unit 1.

o  Case #2

Set-up Conditions:

1.  "M"  unit  0 and "M" unit 1 ONLINE from Case

#1

2.  AVAILABLE command issued to "M" unit 0

Improper operation:

"M" unit 0 enters the "Unit-Available" state in-

stead of the "Unit-Offline" state as required.

WAIVERS AND EXCEPTIONS                                   PAGE E-6

RC25

An ONLINE command directed to "M" unit 0, issued

immediately after the AVAILABLE command, reports

Success  (subcode  "Duplicate  Unit Number") in-

stead of Unit-Offline (subcode  "Duplicate  Unit

Number").    Success  (subcode  "Duplicate  Unit

Number") status is not valid for the ONLINE com-

mand.

In  addition,  "M"  unit  0  is actually brought

"Unit-Online" and  subsequent  commands  execute

successfully,    reporting    Success   (subcode

"Normal") status instead of being rejected  with

Unit-Offline  (subcode  "Duplicate Unit Number")

status.  That improper operation occurs  on  the

following commands:

>  GET UNIT STATUS

>  WRITE

>  READ

>  ACCESS

>  COMPARE HOST DATA

>  ERASE

>  REPLACE

>  SET UNIT CHARACTERISTICS

o  Case #3

Set-up Conditions:

1.  "M" unit 1 ONLINE from Case #3

2.  AVAILABLE command issued to "M" unit 1

Improper Operation:

Same as that described in Case #2 with one addi-

tional anomaly:  the drive is not spun  down  as

required.

o  Case #4

Set-up Conditions:

WAIVERS AND EXCEPTIONS                                   PAGE E-7

RC25

1.  controller  initialized and default host ac-

cess timeout allowed to expire resulting  in

the       controller       entering      the

"Controller-Available" state

2.  "M"  drive  spun down with 0/1 unit plug in-

serted

3.  "S"  drive  spun down with 0/1 unit plug in-

serted

4.  controller  initialized  and characteristics

set

Improper operation:

Status  code  Unit-Offline  (subcode  "Duplicate

Unit Number") is not reported in  the  end  mes-

sages  of  all  commands which require it when a

duplicate unit number condition exists  and  the

affected unit is not ONLINE to a host.  The RC25

reports status code Unit-Offline (subcode  "Unit

Disabled  by  Field Service or Internal Diagnos-

tic") instead.  The improper operation occurs on

the following commands:

>  ONLINE

>  GET UNIT STATUS

>  WRITE

>  READ

>  ACCESS

>  COMPARE HOST DATA

>  ERASE

>  REPLACE

>  SET UNIT CHARACTERISTICS

The  failure  occurs  on commands issued to both

"M" unit 0 and "M" unit 1.

WAIVERS AND EXCEPTIONS                                   PAGE E-8

RC25

E.3  HSC AND VAX/VMS

E.3.1  "Bundled" Shadowing

Temporary Implementation:

"Bundled  Shadowing"  is  a  VAX/VMS-HSC  (models  50 and 70)

specific temporary implementation.  A more  general  approach

to  shadowing, referred to as "Unbundled Shadowing," has been

proposed.  The "Unbundled Shadowing"  specification  is  cur-

rently in a stable state, lacking only a first implementation

before becoming an approved ECO  to  MSCP.   When  "Unbundled

Shadowing" is finally approved the temporary "Bundled Shadow-

ing" implementation will be eliminated in a timely fashion.

E.3.1.1  Shadowing Concepts  -

E.3.1.1.1  Shadowing Functionality  -

Shadowing  is  an  optional  feature  of  MSCP  for enhancing the

availability of critical data by the duplication of the  data  on

two  or  more compatible disk volumes so that the data will still

be  available  if  one  of  the  volumes  becomes   inaccessible.

Furthermore,  data  which  becomes  unreadable due to degradation

over a period of time may be repaired by reading  the  data  from

another volume and restoring the data to the failing volume.  The

individual volumes containing the  duplicate  data  may  be  con-

sidered  to  be  members of a 'virtual' volume, which is known as

the shadow set.

Note that shadowing may decrease the write performance, since new

data must be written to all members of the shadow set.  On an op-

timizing  controller  however, shadowing may enhance read perfor-

mance, as the data may be read from any member of the shadow set,

and  the  controller  may therefore optimize read requests across

the various members.

Conceptually,  shadowing  is implemented by inserting a shadowing

layer between the gatekeeper and  bad  block  replacement  layers

WAIVERS AND EXCEPTIONS                                   PAGE E-9

HSC AND VAX/VMS

(note  that  controller initiated bad block replacement is manda-

tory for shadowing).  The shadowing layer recognizes and  manages

shadow sets by issuing MSCP requests to the bad block replacement

layer.  This section describes the concepts that  shadowing  adds

to MSCP, the changes to MSCP control message formats, and the al-

gorithms used by the shadowing layer.  Note that the  actual  im-

plementation  of  shadowing  need not be a distinct layer nor use

the exact algorithms given here, as  long  as  all  class  driver

visible results are identical.

Class Driver

A

|

V

/ +----------------------------------+

/  |            Gatekeeper            |

/   +----------------------------------+

/    |            Shadowing             |

MSCP server +----------------------------------+

\    | Controller Bad Block Replacement |

\   +----------------------------------+

\  |            Basic MSCP            |

\ +----------------------------------+

E.3.1.1.2  Invalid Commands  -

Basic  MSCP  allows considerable flexibility in how extensively a

server checks for invalid commands.  For  example,  if  the  byte

count  in  a  transfer  command  is too large, the server may op-

tionally perform the transfer up to the maximum legal byte count.

As  another  example, checking that reserved fields actually con-

tain zero is usually optional.

This  flexibility  can cause problems with shadowing.  Consider a

write operation on a shadow set where the byte count is too large

or  some other field is invalid.  The shadowing layer will decom-

pose the write operations into one subcommand for each member  of

the  shadow  set  and,  if  the members are serviced by different

basic MSCP servers, one server may choose to perform the transfer

while  another  server  may  choose  to  not perform it.  The net

result of this series of events is that the shadow set has become

inconsistent, which is unacceptable.

For this reason, the shadowing layer may not just blindly reissue

commands that it  receives  as  subcommands,  but  instead,  must

guarantee  that  all  subcommands  that  it  issues  to the lower

layer(s) are valid.  In other words,  the  shadowing  layer  must

perform all command validity checking itself, without trusting to

WAIVERS AND EXCEPTIONS                                  PAGE E-10

HSC AND VAX/VMS

any checking which may be performed by the  lower  layers.   Note

that  if  the  shadowing  layer  has  knowledge of the basic MSCP

layer(s) below it (as will generally be the case in an integrated

controller),  and  it  can  guarantee  that all of the individual

servers will treat the the command identically, then the  shadow-

ing  layer may pass the responsibility of command verification to

the individual servers.

If  this is not the case, the shadowing layer must use one of two

approaches.  The first is that it may  explicitly  check  that  a

reserved  field  is zero before it begins processing the command,

and reject the command if the field is nonzero.   The  second  is

that it may totally ignore the contents of the reserved field for

its own internal processing, and explicitly zero the field in any

copies  of the command that it issues as subcommands.  These cor-

respond to the options allowed for reserved fields in basic MSCP,

namely  rejecting  the command or being totally unaffected by the

field's contents.

For  nonreserved fields, the shadowing layer generally must vali-

date the field before it begins processing the command,  and  re-

ject  the  command if the field's contents are invalid.  The only

exception to this general rule is the byte count  in  a  transfer

command.  In this case, the shadowing layer must still check that

the byte count is valid, however if the byte count is too  large,

the layer may either reject the command before beginning process-

ing or process the command up to the maximum  legal  byte  count.

The  latter  case effectively means that the shadowing layer sub-

stitutes the maximum legal byte count for the actual  byte  count

before processing the command.

Since  the shadowing layer guarantees that it will never issue an

invalid command to a lower layer, any  "Invalid  Command"  errors

returned by a lower layer are internal controller errors and must

be reported as such.  The  shadowing  layer  translates  "Invalid

Command"  status codes to a "Controller Error" status code before

returning such errors to higher layers.

E.3.1.1.3  Virtual and Member Units  -

The  shadow set itself exists as a virtual unit distinct from any

physical unit, that is, the shadow set itself has a  unit  number

and  context that is distinct from the unit number and context of

any member of the shadow set.  The shadow set -- i.e.,  its  unit

number   --   may   be   "Unit-Offline,"   "Unit-Available,"   or

"Unit-Online," in a like manner to a real  unit.   Class  drivers

access the shadow set with commands that specify the shadow set's

WAIVERS AND EXCEPTIONS                                  PAGE E-11

HSC AND VAX/VMS

unit number.  Although the full  16  bit  unit  number  range  is

available for virtual units, it is suggested that numbers outside

the SDI range (i.e., greater than 4095) be  utilized  to  prevent

possible operator confusion.

The  shadowing layer presents both the individual member units in

the shadow set as well as the virtual unit itself to upper layers

or  class drivers for access.  It provides full read/write access

to both the shadow set itself and to the member units.  It relies

upon the host class driver in combination with the host shadowing

layer to take appropriate actions to ensure shadow  set  internal

consistency.

E.3.1.1.4  Shadow Member Compatibility  -

Although  it is conceptually possible for a shadow set to consist

of a mixture of physical disk types, the overhead could be signi-

ficant,  and  for this reason all members of a shadow set must be

compatible.  Two disk volumes are compatible, and  therefore  al-

lowable  as  members  of  the same shadow set, if and only if the

following geometry parameters are identical:

1.  Unit size

2.  Track size

3.  Group size

4.  Cylinder size

5.  Hardware/software write protect status

6.  Sector size (512/576)

7.  Media type

Any attempt to create a shadow set with members which do not con-

form to the above requirements will be rejected with a status  of

"Media  Format  Error," subcode of "Characteristics Or Protection

Mismatch For Shadow Member." Note that  these  parameters,  which

are  returned in the end messages of the GET UNIT STATUS, ONLINE,

and SET UNIT CHARACTERISTICS commands, are  characteristic  of  a

specific  disk  volume, and that it is possible for a single disk

drive to  accept  different  disk  volumes  that  have  different

geometries.

WAIVERS AND EXCEPTIONS                                  PAGE E-12

HSC AND VAX/VMS

E.3.1.1.4.1  Hardware Write Protected Shadow Sets  -

When  there  is no intention to write to a shadow set, the shadow

set should  be  software  write  protected,  not  hardware  write

protected.   Hardware  write  protection  of  a  shadow set is an

anachronism.  Hardware write protection of a shadow set  prevents

both bad block replacement and shadow set data repair operations.

Because  hardware  write  protection prohibits bad block replace-

ments, blocks which degrade over time will produce  "Data  error"

subcode  values  other  than  "Forced Error." Such errors are not

repairable, and shadow set members on which a block spontaneously

becomes unreadable will be removed from the shadow set.

Since spontaneous decay of a block cannot produce a forced error,

all blocks containing forced errors must be present  due  to  ex-

plicit  host  action.   Therefore, there is no need to read addi-

tional members of a hardware write protected shadow set when  one

member has a forced error.  All members of the shadow set contain

a forced error because they were identical when  the  shadow  set

was  formed (an implicit assumption the shadowing layer must make

because no copy operations could be performed).

The  only  possible use for a hardware write protected shadow set

is during an attempt to recover data from a shadow set  in  which

all  the  members have suffered sufficient data corruption to re-

quire their mounting in a hardware write  protected  mode,  (e.g.

all member RCTs are invalid).  In such a case, the normal shadow-

ing layer algorithms are insufficient  to  provide  a  reasonable

probability  of  recovering  the data.  Therefore, an intelligent

MSCP controller  may  choose  to  implement  a  shadow  set  data

recovery  mode  which is activated by the presence of the "Ignore

Media Format Errors" modifier on an ONLINE command for a  virtual

unit.

If  supported,  "Ignore  Media Format Errors" modifier places the

shadow set in a special data recovery state.  In this state,  any

error detected while reading one shadow set member will result in

an attempt to read another shadow set member.  If all shadow  set

members  produce an error, the last error received along with any

data transferred will be returned to the host.  Member units will

be  removed  from  the shadow set only upon explicit request from

the host.

The  "Ignore  Media  Format  Errors" modifier is illegal when the

shadow set is not hardware write protected or when the shadow set

is  online  to  another  connection.   In these cases, the ONLINE

WAIVERS AND EXCEPTIONS                                  PAGE E-13

HSC AND VAX/VMS

command will produce a status of "Invalid Command" with a subcode

which  points to the modifiers field.  If the controller does not

support the special data recovery mode, an  ONLINE  command  with

the "Ignore Media Format Errors" modifier will be rejected with a

status of "Invalid Command" and a subcode pointing to  the  modi-

fiers field.

E.3.1.1.5  Shadow Set Management  -

This  section  provides an overview of shadow set management from

the perspective  of  both  the  host  and  the  controller,  with

numerous  details omitted to allow a general understanding of the

principles without adding unnecessary confusion.  Later  sections

of  this  specification  deal with the actual specifics in a more

rigorous fashion.

The  mechanism for managing the membership of a shadow set is via

the ONLINE, SET UNIT  CHARACTERISTICS,  and  AVAILABLE  commands.

Member  units are added to the shadow set one at a time, with the

first member also performing an implicit creation of the  virtual

unit  (effectively  causing  the  virtual unit to transition from

"Unit-Offline" to "Unit-Available").  If the member  unit  to  be

added  is  "Unit-Available" to the issuing host, it must be added

to the shadow  set  via  the  ONLINE  command,  while  if  it  is

"Unit-Online" to the issuing host, it must be added to the set by

the SET UNIT CHARACTERISTICS command.

When  adding  additional  members  to  the shadow set, a host may

specify that all data from the virtual unit is to  be  copied  to

the  newly  added  member (allowing an uninitialized volume to be

consistent with the shadow set).  During the process of the  con-

troller  copying  this  data, normal host I/O to all units except

the newly added member is unaffected, with the controller  having

the  responsibility for correctly segmenting the host I/O and the

internal disk to disk copy operation.

The  state  of the virtual unit relative to the issuing host will

remain unchanged by these  operations  (other  than  the  initial

transition from "Unit-Offline" to "Unit-Available"), so the issu-

ing host must also issue an ONLINE command addressed to the  vir-

tual  unit  itself  in order to access it with commands requiring

the unit to be in a "Unit-Online" state.  If a host issues an ON-

LINE  command  to  a virtual unit with no members, it will be re-

jected with a status of "Unit-Available," subcode of "No  Members

In Shadow Set."

Once  a  host  brings  a  virtual  unit  online, it may still add

WAIVERS AND EXCEPTIONS                                  PAGE E-14

HSC AND VAX/VMS

members with ONLINE  or  SET  UNIT  CHARACTERISTICS  commands  as

appropriate for the specific physical unit.  As previously noted,

such additions will not alter the unit state of the virtual unit;

i.e., it will remain "Unit-Online."

Removal  of  a  member from a shadow set (other than asynchronous

error states) is accomplished by the host  issuing  an  AVAILABLE

command  to  the specific member it is desired to remove.  On re-

ceipt of this command, the controller will  remove  the  affected

member   from   the   shadow   set,   place   that  member  in  a

"Unit-Available" state, and place the  shadow  set  itself  in  a

"Unit-Available" state.  All other members of the shadow set will

remain in a "Unit-Online" state.

If  an AVAILABLE command is addressed to the virtual unit itself,

the shadow set is placed in a  "Unit-Available"  state,  and  all

members  remain in a state of "Unit-Online." The host may issue a

modifier which removes all members from the shadow  set,  placing

the virtual unit in a "Unit-Offline" state.

Error  states  within  shadowing may be broken into three generic

cases:

1.  Repairable  (recoverable)  error - A repairable error is

defined as being a data error with a subcode  of  forced

EDC.   Note  that  although this is usually generated is

the last step of the bad block replacement layer, it may

also be present due to explicit action by the host.  The

controller is unable to distinguish these two cases, and

will proceed with the repair process as detailed below.

2.  Unrecoverable  (unrepairable)  error - This class of er-

rors encompasses all drive errors which  the  controller

is  unable  to  recover  from after its retry algorithms

have been exhausted.

3.  Controller  errors  -  This class of errors includes not

only those errors currently defined in the MSCP specifi-

cation,  but also any invalid command errors returned to

the shadowing layer by the MSCP layer.

If a forced EDC error (e.g., a repairable error) is detected on a

member of a shadow set while reading, the controller will attempt

to  repair the data by reading the equivalent data from an alter-

nate member of the set, writing the  data  to  the  member  which

failed,  and then returning the (corrected) data to the host.  If

a member is determined to be unrepairable,  the  affected  member

will be removed from the set, and the virtual unit will be placed

in a "Unit-Available" state.

WAIVERS AND EXCEPTIONS                                  PAGE E-15

HSC AND VAX/VMS

If  an  unrecoverable error occurs on a member which is not asso-

ciated with a read request, the failing member  will  be  removed

from  the  shadow  set,  and  the  shadow set will be placed in a

"Unit-Available" state.

E.3.1.1.6  MSCP Command Changes  -

One  of  the  goals  of shadowing is to attempt to make a virtual

unit present much the same appearance to a  host  as  a  physical

unit.   For this reason, no new MSCP commands have been added for

shadowing, and only those commands which are involved with  crea-

tion  and dissolution of a shadow set have new features (although

transfer commands now have a new status code to deal with  errors

detected during shadow set operations).

In  the descriptions below, the terms "host" and "connection" are

used interchangeably.  Unless otherwise  explicitly  stated,  the

descriptions    in    Sections   section(bunshadb1host)   through

section(bunshade1host) refer to single host operation.

E.3.1.1.6.1  ONLINE Command Changes  -

The  ONLINE  command  is  issued  by a host when it is desired to

bring a physical unit which is currently "Unit-Available" to  the

issuing host into a shadow set.  If the shadow set already exists

at the time of the ONLINE command, the physical unit is added  to

the  set.   If  the shadow unit is nonexistent at the time of the

command, the virtual unit representing the shadow set is  created

as  part of the online process.  As part of the addition process,

the host may optionally specify that  all  data  on  the  virtual

volume be copied to the newly added member, and if this option is

chosen, may also specify that a single definable portion  of  the

virtual volume be excluded from the copy operation.  As mentioned

previously, the state of the virtual unit relative to  all  hosts

remains  unaltered  by  the  addition of a new member to the set,

other  than  the  possible  transition  from  "Unit-Offline"   to

"Unit-Available" if the virtual unit did not previously exist.

If  the  unit  to  which  the  command  is  issued  is  currently

"Unit-Online" as a nonmember to any host, the command will be re-

jected  with  a  status  of  "Illegal  Command," with the subcode

WAIVERS AND EXCEPTIONS                                  PAGE E-16

HSC AND VAX/VMS

pointing to the modifiers field.  In this case,  the  command  is

rejected without changing the unit state of any unit.

E.3.1.1.6.1.1  Changes in the ONLINE Command Syntax  -

Volume  shadowing  introduces the following changes to the syntax

of the ONLINE command:

1.  Shadow  Unit  Specified - This modifier is applied to an

ONLINE command addressed to a physical unit to  indicate

to  the  controller that the remaining fields are valid.

If this modifier is clear, all the remaining fields  de-

tailed  below  are  undefined,  while  if it is set, all

fields must be examined to determine their  validity  as

detailed below.

2.  Shadow  Unit - This  field  refers  to  the virtual unit

number to which the new member is to be added.  It is 16

bits in length and may be fully utilized.  Four possible

states exist with reference to the  passed  shadow  unit

number:

a.  The specified shadow unit number is already known to

the controller as an existing  virtual  unit  number

with  known characteristics.  Note this includes the

possibility that the unit has no  members  but  that

the  characteristics  were previously defined by the

addition of one or  more  members  which  were  sub-

sequently  removed.  The virtual unit will be in ei-

ther the "Unit-Online"  or  "Unit-Available"  state.

In  this  instance,  the newly added member is added

into the already existing shadow set (upon  success-

ful completion of the ONLINE command), and the state

of the virtual unit remains unchanged  with  respect

to the host.

b.  The specified shadow unit number is already known to

the controller as an existing  virtual  unit  number

with unknown characteristics.  The virtual unit will

be in the  "Unit-Offline"  state,  substate  of  "No

Volume  Mounted." In this instance, the virtual unit

will be assigned the characteristics  of  the  newly

added  member.   The  virtual  unit is placed in the

WAIVERS AND EXCEPTIONS                                  PAGE E-17

HSC AND VAX/VMS

"Unit-Available" state and  an  AVAILABLE  ATTENTION

message  will be sent notifying all hosts of the ex-

istence of a new unit.

c.  The  specified  shadow unit number is unknown to the

controller.  In this instance, the controller  will,

during execution of the ONLINE command, 'create' the

virtual unit and assign it  the  characteristics  of

the  newly added member.  The virtual unit is placed

in a "Unit-Available" state and an AVAILABLE  ATTEN-

TION message will be sent notifying all hosts of the

existence of a new  unit.   If  the  ONLINE  command

fails after the virtual unit is 'created' but before

the characteristics are assigned, no  AVAILABLE  AT-

TENTION  message  will  be sent and the virtual unit

will be left in the "Unit-Offline"  state,  substate

of "No Volume Mounted."

d.  The  specified  shadow  unit  number is known to the

controller as an existing physical unit number.   In

this  instance,  the  controller must reject the at-

tempted ONLINE command with  a  status  of  "Invalid

Command,"  with  the subcode pointing to the "Shadow

Unit" field.

3.  Copy  Mode - This  field is used by the host to indicate

how the entire LBN space on the virtual volume should be

copied  to  the newly added shadow set member as part of

the online process.

The  virtual  unit and the newly added member must be in

the same state with reference to  write  protection,  or

the  command  will  be  rejected with a status of "Media

Format Error," subcode of "Characteristics or Protection

Mismatch For Shadow Member."

The  "Copy  Mode" field itself is settable at the option

of the host, and may currently take one  of  three  dis-

tinct  values.   Symbolic names for these values are de-

fined in Table "Bundled  Shadowing"  A-a,  "Shadow  Copy

Mode Operation Codes." The values are:

a.  ZERO,  indicating  that  no  copy operation is to be

performed.

b.  ONE,  indicating  that a FULL copy operation is per-

formed.  The controller will copy all  LBNs  in  the

host  area  from the virtual unit to the newly added

WAIVERS AND EXCEPTIONS                                  PAGE E-18

HSC AND VAX/VMS

shadow member (Note that the term 'virtual unit' im-

plies  the entire existing shadow set membership, so

the controller may elect to divide the READ requests

among the various membership).

c.  TWO, indicating that a MERGE copy operation is to be

performed.   This  operation  is  identical  to  the

NORMAL  copy operation with one exception.  If, dur-

ing the copy operation, all members  of  the  shadow

set  return  a forced error, the controller will at-

tempt to read  the  same  LBN  on  the  newly  added

member,  and  if  successful, will copy this data to

all members of the shadow set.  If the data  on  the

newly  added  member also has a forced error indica-

tor, the controller  will  merely  insure  that  all

members of the shadow set and the newly added member

contain consistent data, flagging  the  LBN  on  all

members  with  the  forced  error  indicator.   This

"backwards copy" operation encompasses ONLY the  LBN

in  error,  with the remainder of the copy operation

reverting to the normal mode  (unless  other  errors

are detected).

The  presence  of  a  shadow set member in the merge

copy  mode  also  effects  the  processing  of  READ

transfer  requests.   See  Section  section(bunshad-

percop) for details of this effect.

Note that although this mode of operation may result

in "repairing" an LBN on the shadow set  by  copying

data  from the newly added member, it should only be

invoked when the host has  a  high  degree  of  con-

fidence that the member to be added differs from the

rest of the shadow set only by write  operations  in

progress when the shadow set was "accidentally" dis-

solved.

All  other values are undefined and will result in a re-

turned status of  "Invalid  Command"  with  the  subcode

pointing  to  this field.  In all but one case, the com-

mand will be rejected without changing the unit state of

any  unit.   If the unit field specifies a unit which is

already online to the issuing host as  a  nonmember  and

the  command is SET UNIT CHARACTERISTICS, the unit state

will be changed to "Unit-Available."

The  presence of a nonzero copy mode field requires spe-

cial handling of shadow sets which are write  protected.

If  the  virtual  unit  is  in  a hardware write protect

state, then the ONLINE command will be rejected  with  a

WAIVERS AND EXCEPTIONS                                  PAGE E-19

HSC AND VAX/VMS

status  of  "Illegal Command," with the subcode pointing

to the "Copy  Mode"  field.   If  the  virtual  unit  is

software write protected, then the write protection will

be disabled on the newly added member for  the  duration

of  the  copy operation, and re-enabled when the copy is

completed.

4.  Excluded  LBN  Block Address - This field may be used by

the host in conjunction with the "Copy  Mode"  field  to

indicate  that  a specific portion of the virtual volume

is not to be copied to the  newly  added  volume.   This

field  is  valid  only if both the "Copy Mode" field and

the "Excluded LBN Count" fields are nonzero.   In  addi-

tion,  if  this  field is valid it must represent a host

accessible LBN, or the command will be rejected  with  a

status  of  "Invalid Command," with the subcode pointing

to the offset of this field.  If either the "Copy  Mode"

or "Excluded LBN Count" fields are zero, the contents of

this field are undefined.

5.  Excluded  LBN Count - This field may be used by the host

in conjunction with the "Copy Mode"  and  "Excluded  LBN

Block  Address"  fields to indicate that a specific por-

tion of the virtual volume is not to be  copied  to  the

newly  added  volume.   This  field is valid only if the

"Copy Mode" field is  nonzero.   In  addition,  if  this

field  is  nonzero,  the contents of this field added to

the "Excluded LBN Block Address" must represent a  range

of host accessible LBNs, or the command will be rejected

with a status of "Invalid  Command,"  with  the  subcode

pointing  to the offset of this field.  If this field is

zero, the contents of the "Excluded LBN  Block  Address"

field  is  undefined, and no LBN's will be excluded from

the copy operation.

WAIVERS AND EXCEPTIONS                                  PAGE E-20

HSC AND VAX/VMS

E.3.1.1.6.1.2  Changes Which Enforce Shadow Set Membership Rules

The ONLINE command modified by "Shadow Unit Specified" is respon-

sible for enforcing all shadow set  membership  rules.   Some  of

these  rules are listed in Section section(bunshadmemcom).  Other

of these rules are detailed in the paragraphs which follow.

A  shadow  set may consist of only physical volumes, e.g., a host

may not create a shadow set and then attempt  to  create  another

shadow set specifying the virtual unit number of the first set as

a member of the second set.  If a host attempts  such  an  opera-

tion,  the  controller  will  reject the command with a status of

"Invalid Command," with the subcode pointing to the "Unit" field.

In  this  case, the command is rejected without changing the unit

state of any unit.

A  physical  unit may belong to one and only one shadow set.  Any

attempt by a host to add a physical unit which is a member of one

set  to  another  set  will be rejected with a status of "Invalid

Command" with the subcode pointing to the  "Shadow  Unit"  field.

In  this  case, the command is rejected without changing the unit

state of any unit.

A physical unit cannot be "Unit-Online" both as a member and as a

nonmember.  Any attempt by a  host  to  bring  a  unit  which  is

already  a  member  of a shadow set online as a nonmember will be

rejected with a status of  "Invalid  Command"  with  the  subcode

pointing  to  the "Shadow Unit" field.  In this case, the command

is rejected without changing the unit state of any unit.

If any of the unit characteristics of a member proposed for addi-

tion to a shadow set fail to match those of the already  existing

shadow  set, the ONLINE command will be rejected with a status of

"Media Format Error," subcode of "Characteristics  Or  Protection

Mismatch  For  Shadow Member." In this case, the physical unit is

placed in the "Unit-Available" state.  The unit state of the vir-

tual unit remains unchanged.

E.3.1.1.6.1.3  Changes in Sequential Gatekeeping  -

Since  a  copy operation takes an extended period of time to com-

plete, the Sequential command operation of the ONLINE command  is

modified somewhat, and is detailed as follows:

WAIVERS AND EXCEPTIONS                                  PAGE E-21

HSC AND VAX/VMS

1.  The   ONLINE  command  is  deferred  by  the  sequential

gatekeeper until such time as all  outstanding  commands

addressed  to both the shadow unit and the physical unit

in question known to the controller have completed.

2.  The  ONLINE command is then performed on the controller,

with all commands to both the shadow unit and the physi-

cal  unit (except Immediate commands) being deferred un-

til the noncopy portion has completed.

3.  When  the  copy  portion of the command starts, the gate

imposed by  the  sequential  nature  of  the  ONLINE  is

lifted, and all commands directed to the shadow unit may

proceed in parallel with the copy operation.   Any  com-

mands (except Immediate commands) directed to the physi-

cal unit prior to completion of the copy operation  will

be  rejected  with a status of "Unit Available," subcode

of "Copy In Progress."

The  copy operation will not be affected by any commands

directed to the virtual unit with two exceptions:

a.  If  an AVAILABLE command is addressed to the virtual

unit which would cause the shadow  set  to  dissolve

(see   Section   section(bunshadavlcmd)),  the  copy

operation will be aborted with a status  of  "Shadow

Set Status Has Changed."

b.  If the host issues a SET UNIT CHARACTERISTICS to the

virtual unit which would cause the virtual unit (and

all member units) to become software write protected

(see Section section(bunshadsuccmd)), the member  to

which   the   copy   is  being  done  will  only  be

'logically' write protected until completion of  the

copy  (at which time it will become physically write

protected).  In other words, all  host  attempts  to

write  the virtual unit must be rejected by the con-

troller with a  status  of  "Write  Protected"  even

though  the  member  unit  is still physically write

enabled for the  duration  of  the  copy  operation.

WAIVERS AND EXCEPTIONS                                  PAGE E-22

HSC AND VAX/VMS

Note  that  any attempt to directly write the member

unit   will   be   rejected   with   a   status   of

"Unit-Available," subcode of "Copy In Progress."

E.3.1.1.6.1.4  Performing Shadow Copy Operations  -

Controllers may optionally allow concurrent addition of more than

one shadow set member requiring  a  shadow  copy.   Whenever  the

controller's  limit  on  concurrent  shadow  copy  operations  is

reached, further attempts to add members with shadow copy  opera-

tions must be rejected with a status of "Unit-Available," subcode

of "Copy In Progress."

At  any  given  point  in  time,  a shadow set may be composed of

members in one of three states:

1.  Full-fledged member, no copy in progress.

2.  New member, target of NORMAL copy.

3.  New member, target of MERGE copy.

Because  multiple  shadow copy operations may be in progress con-

currently, as few as zero units and as many units as permitted by

the  controller  may be in each of these states.  Note:  a shadow

set containing zero full-fledged members but  one  or  more  copy

members  of  either type is a short-lived degenerate case.  Based

upon the copy algorithms detailed below, lack of any full-fledged

members should quickly produce the expulsion of all copy members.

The remainder of this section will refer to shadow set member  in

the   various   states   as   full-members,   copy-members,   and

merge-members, respectively.

Due to the sequential gatekeeping rules described in Section sec-

tion(bunshadgatekp), write operations directed to the shadow  set

may  be  executed  concurrently with shadow copy operations.  The

controller is responsible for insuring the coordination of writes

to  the  newly  added member in such a manner as to insure shadow

set consistency.  Note:  this means  that  ALL  write  operations

must  be  propagated  to  ALL  merge-members.  Depending upon the

exact shadow copy implementation, write operations may or may not

need to be propagated to one or to all copy-members.

WAIVERS AND EXCEPTIONS                                  PAGE E-23

HSC AND VAX/VMS

Also,  due  to the sequential gatekeeping rules described in Sec-

tion section(bunshadgatekp),  read  operations  directed  to  the

shadow  set  may be executed concurrently with shadow copy opera-

tions.  The controller is responsible for insuring that the  data

read  is  NOT CHANGED by a copy operation, i.e., that an LBN read

before it is copied does not differ from when it is read after it

is  copied.  This possibility can occur if a merge copy operation

is taking place since the data which  ultimately  ends  up  in  a

particular LBN can be taken from one of two or more members which

MAY contain different data for that LBN.  This implies that  LBNs

which  are  read  from  a shadow set which is undergoing a shadow

merge copy operation MUST have the copy operation(s) performed on

them  before they are read.  Depending upon the exact shadow copy

implementation, this shadow copy operation may or may not be per-

formed redundantly.

Write  operations which fall completely within an excluded region

must NEVER be propagated to either copy-members or  merge-members

for  which  a excluded region has been specified.  Obviously, any

shadow copy operation must not copy the excluded region,  if  one

has  been  legally  specified.  The results of writing a multiple

block region some of which lies within the  excluded  region  and

some of which lies outside the excluded region are unpredictable.

Hosts must confine their shadow set write operations to being ei-

ther entirely inside or entirely outside the excluded region.

If the connection between the host class driver and the MSCP sha-

dowing server is broken during the copy  operation  or  an  ABORT

command  is  received,  the  controller  must  terminate the copy

operation within the controller timeout interval.

All  types of shadow copy operation treat the set of full-members

as the primary source of copy data.  Whenever copy data cannot be

successfully  read  from  the  set  of  full-members,  the set of

merge-members is used as a secondary source of data.   A  failure

to  locate  source  copy  data  occurs  only when both the set of

full-members and the set of merge-members  have  been  exhausted.

All  errors  detected during attempts to acquire source copy data

are subject to read repair (exactly as if they occurred during  a

normal  read  operation).  In addition, any read repair operation

which repairs using data obtained from a merge-member must  write

both all full-members and all merge-members.

As with normal read and write operations, any unrecoverable error

is cause for removing the failing unit from the shadow  set.   If

the failing unit is a full-member, the resulting shadow set state

change places the virtual unit in the "Unit-Available" state, but

the shadow copy operation continues (if possible).  Note:  when a

full-member is failed out  of  a  shadow  set  due  to  an  error

detected  during  a  shadow  copy operation, the host receives no

WAIVERS AND EXCEPTIONS                                  PAGE E-24

HSC AND VAX/VMS

notification other than  rejection  of  future  commands  with  a

"Unit-Available"  status.   If  the  failing  unit  is  either  a

copy-member or a merge-member, the ONLINE  command  is  completed

with a status of "Shadow Set Status Has Changed," and the virtual

unit will be placed in a "Unit-Available" state.   In  the  event

that  source copy data cannot be located (i.e.  no full-member or

merge-member is "readable"), the ONLINE command is completed with

a  status  of "Shadow Set Status Has Changed." This is equivalent

to having the shadow set membership reduced to zero volumes.

Whenever  all full-members and merge-members contain a forced er-

ror block, a forced error block is written  to  the  shadow  copy

target.    The   controller  will  select  the  contents  of  one

full-member copy of the block  and  insure  that  the  equivalent

block  in  all  shadow  set  members  including the target of the

shadow copy contains that data with the forced error flag set.

E.3.1.1.6.1.5  Restrictions and Miscellaneous Notes  -

Since the controller is responsible for maintaining the integrity

of the shadow set, it cannot allow the addition  of  members  for

which  data  reliability  cannot be maintained.  For this reason,

the "Allow Self Destruction" and "Ignore Media Format Error" com-

mand  modifiers  are not allowed.  If they are specified, the ON-

LINE command will be rejected with a status of "Invalid Command,"

with the subcode pointing to the offset of the modifier field.

If  in  the  process  of bringing a member unit online it is dis-

covered that the RCT is invalid or that an incomplete replacement

cannot  be  completed,  which results in the "Write Protect (data

safety)" unit flag being set, the ONLINE command will be rejected

with  a  status of "Media Format Error," subcode of "Characteris-

tics Or Protection Mismatch For Shadow Member."

E.3.1.1.6.2  ONLINE End Message Changes  -

This following changes are made to the ONLINE command end message

for shadowing:

1.  Two  additional  unit  flags  are defined for the ONLINE

command end message:

WAIVERS AND EXCEPTIONS                                  PAGE E-25

HSC AND VAX/VMS

a.  Shadow  Set  Master  -  This flag will be set in the

unit flags field by the  controller  if  the  "Unit"

field of the command references a virtual unit.

b.  Shadow  Set  Member  -  This flag will be set in the

unit flags field by the  controller  if  the  "Unit"

field  of  the  command references a unit which is a

member of a shadow set including members  which  are

the target of a shadow copy operation.

2.  Two  additional  fields  are  defined for the ONLINE end

message:

a.  Shadow Status - If the "Shadow Set Master" unit flag

is set (which will only be the case when referencing

the  virtual  volume  itself), this field represents

the total number of members (excluding  the  virtual

volume)  which  are  currently  "Unit-Online" in the

shadow set.  Note that this number will include  all

members  on  which  copy operations are currently in

progress.

If  the  "Shadow  Set Member" unit flag is set, this

field will contain the  copy  mode  for  this  unit.

Note  that the values are identical to the copy mode

values permitted in the "Copy Mode" field of the ON-

LINE and SET UNIT CHARACTERISTICS commands.  A value

of ZERO indicates that no copy operation is in  pro-

gress.

If  both  the  "Shadow  Set  Master" and "Shadow Set

Member" unit flags are clear, this  field  is  unde-

fined.

b.  Shadow Unit - This field may take on one of two dis-

tinct values:

1.  If  "Shadow  Set Master" is set (which will only

be the case when the  original  command  is  ad-

dressed  to  the  virtual unit), then this field

represents the lowest unit number of the members

in the shadow set.

2.  If "Shadow Set Member" is set (which will be the

case when the original command is addressed to a

member  of  the  shadow  set),  then  this field

represents the virtual unit number of the shadow

set of which this unit is a member.

WAIVERS AND EXCEPTIONS                                  PAGE E-26

HSC AND VAX/VMS

If  both  the  "Shadow  Set  Master" and "Shadow Set

Member" unit flags are clear, this  field  is  unde-

fined.

3.  Although no architectural limit is placed on the minimum

or maximum number of shadow sets a controller must  sup-

port  or  the  number of members in a shadow set, a con-

troller may elect to restrict the maximum number due  to

internal  resource  limitations.  If a controller elects

to reject an operation which would add  members  to  the

shadow set strictly because of internal resource limita-

tions, the controller must indicate this with a returned

status  of  "Controller Error," subcode of "Insufficient

Resources."

4.  If  the  "Shadow  Set Master" unit flag is set (implying

that the unit is the virtual volume), then the following

additional changes exist:

a.  The "Multiunit Code" field is zero.

b.  The "Unit Identifier" field is undefined.

c.  The "Media Type Identifier" field contains the media

type of the lowest member unit.

d.  The "Volume Serial Number" field contains zeroes.

E.3.1.1.6.3  SET UNIT CHARACTERISTICS Command Changes  -

The  SET  UNIT CHARACTERISTICS command exists as a way for a host

to bring a unit which is already  "Unit-Online"  to  the  issuing

host  into  a shadow set.  If the unit has different characteris-

tics than those of  the  shadow  set  (which  would  normally  be

grounds  for  rejecting its entry into the shadow set), this com-

mand allows the characteristics to be changed to  be  in  confor-

mance  with  the shadow set characteristics as a part of bringing

it into the shadow set.  The changes to the SET UNIT CHARACTERIS-

TICS  command  are identical to those of the ONLINE command, with

only three additions.

WAIVERS AND EXCEPTIONS                                  PAGE E-27

HSC AND VAX/VMS

If  a SET UNIT CHARACTERISTICS command is addressed to a physical

unit and the "Shadow" modifier is set (to bring it into a  shadow

set),  the  connection  issuing  the command MUST be the only one

which has it "Unit-Online." If it is not, the command will be re-

jected  with  a  status  of  "Invalid  Command," with the subcode

pointing to the "Unit" field.  The unit will  be  placed  in  the

"Unit-Available"  state  relative to the issuing connection.  The

state of the referenced virtual unit remains unchanged.

If a SET UNIT CHARACTERISTICS command is addressed to the virtual

unit, the successful completion of this command will  modify  not

only the virtual unit itself, but also all member units.

If a SET UNIT CHARACTERISTICS command is addressed to a member of

a shadow set, it may not alter the characteristics of the  member

in  such  a  manner as to violate the requirements set forth pre-

viously for shadow set membership compatibility  (hence,  a  suc-

cessful SET UNIT CHARACTERISTICS command addressed to a member of

a shadow set is nothing more than a Sequential NOP).  If  a  host

attempts to modify a member's characteristics in such a manner as

to make it incompatible with the shadow set, the command will  be

rejected  with  a  status  of "Invalid Command," with the subcode

pointing to the field in error.  In this special case,  the  unit

will  not be placed in the "Unit-Available" state but will remain

in the "Unit-Online" state and the state of the  referenced  vir-

tual unit will also remain unchanged.

E.3.1.1.6.4  SET UNIT CHARACTERISTICS End Message Changes  -

The SET UNIT CHARACTERISTICS end message modifications are ident-

ical to those for the ONLINE end message.

E.3.1.1.6.5  AVAILABLE Command Changes  -

The  AVAILABLE  command  is  the mechanism whereby members may be

removed from a shadow set, or the shadow set itself may  be  dis-

solved.   The  actual function performed by the AVAILABLE command

depends on whether the command is addressed  to  a  member  of  a

shadow set, or the virtual unit itself.

WAIVERS AND EXCEPTIONS                                  PAGE E-28

HSC AND VAX/VMS

1.  If the command is addressed to a member of a shadow set,

that member will  transition  from  "Unit-Online"  to  a

state  of "Unit-Available," and will be removed from the

shadow set, with the shadow set itself being placed in a

"Unit-Available"  state.   This  is  the means whereby a

host may remove members from a shadow set while preserv-

ing the shadow set.  Note that it is possible for a host

to remove all members from a shadow set while preserving

the virtual volume via this mechanism.

2.  If  the  command  is  addressed to the virtual unit, all

members will remain "Unit-Online," and  the  shadow  set

will  transition  to  a  "Unit-Available" state.  If the

"Spin-down" modifier is present, the  end  message  will

contain  a  status  of  "Success," subcode of "Spin-down

Ignored." This is the means whereby a host may  place  a

virtual  unit  in  the "Unit-Available" state while pre-

serving the shadow set.

3.  If  the  command  is addressed to a virtual unit and the

"Dissolve" modifier is specified, all member units  will

be  placed  in a state of "Unit-Available," and the vir-

tual unit will be placed in a  state  of  "Unit-Offline"

(i.e.,  it will disappear).  If the "Spin-down" modifier

is also specified, all member units will be spun down as

part  of  the  transition to the "Unit-Available" state.

This is the means whereby a host may totally dissolve  a

shadow set.

E.3.1.1.6.6  GET UNIT STATUS Command End Message Changes  -

The  end  message of a GET UNIT STATUS command has two additional

fields defined, "Shadow Unit" and "Shadow  Status." These  fields

function  in  an  identical  fashion to the fields described pre-

viously for the ONLINE command end message.

In  addition,  the  GET UNIT STATUS end message also contains the

additional  unit  flags  "Shadow  Set  Master"  and  "Shadow  Set

Member,"  which  also  operate  in  the same manner as previously

described.

If  the "Shadow Set Master" flag is set in the "Unit Flags" field

(implying that the unit is a virtual unit) and there are members,

then the following changes exist:

WAIVERS AND EXCEPTIONS                                  PAGE E-29

HSC AND VAX/VMS

1.  The "Multiunit Code" is zero.

2.  The "Unit Identifier" field is undefined.

3.  The  "Media  Type Identifier" contains the media type of

the lowest member unit.

4.  The  hardware  and  software  microcode  version  number

fields ("Usvrsn" and "Uhvrsn") are undefined.

5.  The "RCT Size" field is zero.

6.  The  "RBNs"  field  contains  the replacement blocks per

track for the lowest member unit.

7.  The "RCT Copies" field is zero.

If  the "Shadow Set Master" flag is set in the "Unit Flags" field

and there are no members ("Shadow Status" IS  zero),  all  fields

after the "Unit Flags" field will be zero.

E.3.1.1.6.7  DETERMINE ACCESS PATHS Command Changes  -

If  a  DETERMINE  ACCESS  PATHS command is addressed to a virtual

unit, the controller will treat the command as a  NOP,  but  must

follow  the  rules  for  sequential command gatekeeping.  If this

command is addressed to a member  of  the  shadow  set,  it  will

operate  as if the addressed unit were not a member of the shadow

set.

E.3.1.1.6.8  COMPARE CONTROLLER DATA command changes  -

If  a  COMPARE CONTROLLER DATA COMMAND is addressed to a physical

unit, it functions in a manner unchanged from that of basic MSCP.

If  this command is addressed to a virtual unit however, the con-

troller will interpret it to mean "Compare data on all members of

the   shadow   set  as  specified  by  the  LBN  and  byte  count

fields." The controller will perform a read-compare operation  on

the  specified  range on all members of the shadow set, comparing

the data from all members.  Six results are  possible  from  this

operation:

WAIVERS AND EXCEPTIONS                                  PAGE E-30

HSC AND VAX/VMS

1.  If  all  data  compares,  the  controller  will return a

status of "Success."

2.  If the data differs between members, the controller will

return a status of "Compare Error."

3.  If a forced error is detected on one unit with good (and

equivalent) data on all other units, the controller must

repair  the  forced  error  as  detailed in Section sec-

tion(bunshadreadop).

4.  If forced errors are detected on all units, and the data

in the sectors containing the forced error is  identical

on  all  units,  the  controller  will treat the data as

equal, and proceed with the remainder of the command.

5.  If forced errors are detected on all units, and the data

in the sectors containing the forced error is NOT ident-

ical  on  all units, the controller will return a status

of "Compare Error."

6.  If  an unrecoverable error occurs on a member during the

compare operation, the command will be terminated with a

status of "Shadow Set Status Has Changed."

It  is  important  for  a  host to realize that a write operation

directed to an area on which the controller is currently perform-

ing  a compare operation may result in a returned status of "Com-

pare Error" (see Section section(bunshadredwrt)).

E.3.1.1.6.9  Transfer Commands  -

The  shadowing  layer passes transfer operations on nonshadow set

units through to the basic MSCP layer  unaltered,  and  similarly

returns their end messages unaltered.

Although not recommended, it is perfectly legal for a host to is-

sue transfer commands to an individual member of  a  shadow  set.

It  must  be  remembered  that if a write is issued to a specific

member and not to the other members of the set, the volumes  will

no longer be consistent.

Since  the issuance of a transfer command to a specific member of

the set implies that the host is only interested in that specific

unit,  the  controller will interpret this command to have an im-

plied "suppress shadowing" action.  Failure of the transfer  com-

mand  will  NOT  result in an attempt by the controller to repair

WAIVERS AND EXCEPTIONS                                  PAGE E-31

HSC AND VAX/VMS

the data in the affected block(s) by replacing it with  the  data

from  other members of the shadow set, but will instead result in

the end packet being returned to the host  with  the  appropriate

error status.  The member on which the error occurred will NOT be

removed from the shadow set  (unless  the  controller  determines

that the unit is inoperative), and the shadow set membership will

continue as it was prior to the command.  It should be noted that

issuance  of a command to an individual member of a shadow set is

only legal if the issuing host has previously brought  that  unit

online with the ONLINE command.

As  implied  by  the  statements  about the changes in the ending

status for the GET UNIT STATUS command, a shadow  unit  does  not

have  an  RCT.   Thus,  transfer commands which attempt to access

LBNs beyond the "host LBN area" will be rejected with a status of

"Invalid Command," with a subcode pointing to either the "Logical

Block Number" or the "Byte Count" depending upon their values.

E.3.1.1.6.9.1  READ operations  -

Read  operations  encompass those MSCP commands which obtain data

from the virtual unit, and (may) return that data  to  the  host.

The read commands are ACCESS, COMPARE HOST DATA, and READ.  (COM-

PARE CONTROLLER DATA is a special READ operation,  and  has  been

detailed in Section section(bunshadccdcmd)).

The  shadowing layer performs a shadow read operation as follows.

For each block of the transfer, the layer chooses a member of the

shadow  set  and  attempts the transfer on that unit (note that a

layer may elect to either perform the entire read on  one  member

or  divide  that  read  among  several members).  If that attempt

succeeds, then that block of the transfer is completed.  If  that

attempt  fails,  then  either that block of the transfer fails or

that block must be  repaired,  depending  on  the  cause  of  the

failure  as  indicated  by its status code and the hardware write

protect status of the shadow set.

For  hardware  write protected shadow sets, if the status code is

"Data Error," subcode of "Forced Error," the  transfer  completes

with  that  status.   The data obtained at that point is the data

returned.  If any other  error  is  encountered,  the  member  is

removed  from  the  shadow set and the status returned is "Shadow

Set Status Has Changed."

For shadow sets that are not hardware write protected, the status

WAIVERS AND EXCEPTIONS                                  PAGE E-32

HSC AND VAX/VMS

code "Data Error," subcode of "Forced Error," is considered to be

repairable.   Any  other error is considered to be nonrepairable,

and the layer will not attempt to repair  it,  but  will  instead

remove  the  unit  in  accordance  with  Section section(bunshad-

memchg).

Note  that  the  layer must attempt to repair forced errors, even

though they may have been caused by deliberate host  action.   If

the  forced  error was caused by deliberate host action, then the

repair  attempt  is  pointless  as  it  is  guaranteed  to  fail.

However,  there  is  no  way for the layer to distinguish between

forced errors due to deliberate host action and forced errors due

to  a  bad  block  replacement  operation.   Forced errors due to

replacements can be repaired, therefore the layer must attempt to

repair  all forced errors.  If all members of the shadow set con-

tain a forced error, the controller will report that fact via the

end  packet  with  a  status  of "Data Error," subcode of "Forced

Error." Note that the virtual unit will remain "Unit-Online,"  as

will  all  shadow  set members.  This may be considered a special

case of error handling in that the normal  mechanism  of  placing

the virtual unit in a "Unit-Available" state is bypassed.

The  repair  process  consists  of  two  phases.  The first phase

searches the other members of the shadow set  to  find  a  member

from  which  the data can be successfully obtained.  That is, the

layer tries to read the block on successive members of the shadow

set  until it is read without error.  The second phase writes the

data obtained by the first phase out to all the members on  which

the  read  failed,  including the original member.  Note that the

repair process is always  performed  on  a  specific,  individual

block.   Repairable  errors  on  several blocks of a transfer are

handled as several distinct repair operations, each of which  can

succeed or fail independently of the others.

To  accomplish  the  first phase of the repair process, the layer

first initializes a list of the members needing repair to contain

just  the  one  member on which the transfer was originally tried

(and failed).  The layer chooses a member of the shadow  set  not

in  the  list  of members needing repair and attempts to read the

block's data from it with a read operation.  If  another  repair-

able error occurs on that read attempt, then that member is added

to the list of members needing repair, the layer chooses  another

member  of  the shadow set, and attempts to read the block's data

from it.  This continues until the data is successfully read from

some  member or every member of the shadow set is listed as need-

ing repair.

If  an  unrepairable  error is detected on a member while reading

data for the repair operation, that member is  removed  from  the

shadow set, the shadow set is placed in a "Unit-Available" state,

WAIVERS AND EXCEPTIONS                                  PAGE E-33

HSC AND VAX/VMS

the original command (i.e.,  the  one  which  caused  the  repair

operation  to  be initiated) is returned with a status of "Shadow

Set Status has Changed," and the attempt  to  find  an  alternate

unit to read from continues.  Note that detection of an unrepair-

able error does NOT terminate the repair  operation  (which  must

continue  to  insure  shadow  set compatibility), but merely ter-

minates the original read operation.

If  a  repairable error occurs on every member of the shadow set,

then correct data cannot be obtained for that block.   The  block

is  not  repaired  on  any  member and that block of the original

transfer command fails with a status of "Data Error," subcode  of

"Forced  Error." The  controller must then insure shadow set con-

sistency by writing  identical  data  (copied  from  one  of  the

members)  to  all members in the shadow set with the forced error

indicator set.

If  the data is successfully read from some member, the shadowing

layer enters phase two of the repair process.   The  layer  takes

the  data  and  attempts  to  write it to all the members needing

repair.  This is done as a write-compare operation (with the con-

troller  disabling software write protect for the duration of the

repair as necessary).  The repair of a  member  succeeds  if  the

write-compare   to   that  member  succeeds,  and  fails  if  the

write-compare fails.  If the write-compare operation  fails,  the

error  is considered to be unrepairable and the following actions

take place:

1.  The  failed member(s) are placed in the "Unit-Available"

state.

2.  Other members remain in the "Unit-Online" state.

3.  The    shadow   volume   itself   is   placed   in   the

"Unit-Available" state.

4.  An  end message is returned for the READ operation, with

a failure  status  indicating  "Shadow  Set  Status  Has

Changed."

In  addition  to  writing the data to the members needing repair,

the layer also uses the data to  satisfy  the  original  transfer

command.   In  effect,  the  layer  retries  the one block of the

transfer command on the member from  which  it  successfully  ob-

tained  the  data.   It can either actually retry the transfer on

that member, retry the transfer on some other member  after  that

member has been successfully repaired (the write-compare has suc-

ceeded), retry the entire transfer, or  accomplish  the  original

transfer directly using the data it has read.

WAIVERS AND EXCEPTIONS                                  PAGE E-34

HSC AND VAX/VMS

The  layer must initiate another new repair sequence if the retry

of the original transfer fails with a repairable error.  This  is

necessary  to recover from such errors as a member becoming inac-

cessible in the middle of a repair sequence.

The  shadowing  layer  must  not  allow the repair sequence to be

aborted.  That is, once a repairable error has been detected, the

layer  must complete the repair sequence regardless of whether or

not it receives an ABORT command for the original  transfer.   It

may  omit  the original transfer's retry if the original transfer

is aborted, but the repair sequence itself -- the read-compare(s)

followed by the write-compare(s) -- must not be affected.

E.3.1.1.6.9.2  WRITE operations  -

Write  operations encompass those MSCP commands which modify data

on the virtual unit, possibly requesting that data from the host.

The write commands are ERASE, FLUSH, and WRITE.

The  shadowing  layer  performs a shadow write operation in three

phases:

1.  Issue  one  copy  of  the  original command to the lower

layer for each member of the shadow set  (these  command

copies are called subcommands).

2.  Handle  any  errors  that  occur  on  the subcommands by

removing the affected members from the shadow set.

3.  Determine  the  appropriate completion status and report

it to the higher layer or class driver.

By  definition,  all members in the shadow set have the identical

write protect status (both hardware and software), so the shadow-

ing layer need not be concerned about the write protect status of

the individual members.  If all the members  are  write  enabled,

the shadowing layer proceeds to the first phase and issues a copy

of the original command to the lower layer for each member of the

shadow set.

The  subcommands to the individual members can be issued and pro-

cessed (executed) in any order or concurrently.  As  the  subcom-

mands  complete,  the shadowing layer examines their status codes

WAIVERS AND EXCEPTIONS                                  PAGE E-35

HSC AND VAX/VMS

and divides the  subcommands  and  their  associated  shadow  set

members into three classes:

1.  Subcommands  that succeed.  This class includes commands

that complete with the following status codes:

a.  Success

b.  Data Error with subcode zero (Forced Error)

c.  Command Aborted

Note   that   a   forced  error  can  only  occur  on  a

write-compare operation with the  "Forced  Error"  modi-

fier.

2.  Subcommands  that  indicate that the transfer failed due

to loss of access to the transfer data in  host  memory.

All  failures  of  this type have a major status code of

"Host Buffer Access Error."

3.  Subcommands  that  indicate  that  the  transfer failed.

This class includes  commands  that  complete  with  all

other status codes, including:

a.  Unit-Offline

b.  Unit-Available

c.  Controller Error

d.  Drive Error

e.  Media Format Error

f.  Compare Error

g.  Data Error with nonzero subcode

Note  that an "Invalid Command" status code is converted

to and treated as a "Controller Error," since  the  sha-

dowing  layer  guarantees  that it will only issue valid

subcommands.

After  all  the  subcommands  have  been completed, the shadowing

layer examines classes 1, 2, and 3 and  returns  the  appropriate

status code as follows:

WAIVERS AND EXCEPTIONS                                  PAGE E-36

HSC AND VAX/VMS

1.  If  there  are  no  members  in classes 2 or 3, then the

transfer has succeeded and a status of "Success" is  re-

turned.

2.  If there are any members in class 2, then the shadow set

enters host write failure recovery as described in  Sec-

tion  section(bunshadhsfarc).  If there are also members

in class 3, those members are ignored by the host  write

failure   recovery  operation.   After  the  host  write

failure recovery is completed,  shadow  set  members  in

class  3  are  removed  from the shadow set as described

below.  The status returned in the end message should be

the "Host Buffer Access Error" status and substatus that

placed at least one of the shadow set members into class

2.

The  presence of a shadow set member in class 2 does not

affect the shadow  set  membership.   In  this  respect,

classes 1 and 2 are identical.

3.  If  there  are no members in class 2 and some members in

class 3, then:

a.  All  class  1  member(s) remain in the "Unit-Online"

state.

b.  All   class   3   member(s)   are   placed   in  the

"Unit-Available" state.

c.  The   shadow   volume   itself   is  placed  in  the

"Unit-Available" state.

d.  An  end message is returned for the write operation,

with a failure status indicating "Shadow Set  Status

Has Changed."

E.3.1.1.6.9.3  Host write failure recovery  -

Any  write  command  that  fails  due to inability to access host

memory must initiate a recovery sequence performed by the shadow-

ing layer.  This recovery sequence has several objectives:

WAIVERS AND EXCEPTIONS                                  PAGE E-37

HSC AND VAX/VMS

1.  over  the region affected by the write operation, ensur-

ing that all shadow set members contain the same data.

2.  within  the  constraints  imposed by inability to access

the data buffer described by the write  operation,  pre-

serving previously written user data.  In many respects,

this objective is identical to the objectives of a merge

copy operation.

3.  if at least one shadow set member has been fully written

by the write operation, cause the data  stored  on  that

member  to  be  propagated  to  the remaining shadow set

members.

The remainder of this section describes the actions that the sha-

dowing layer must perform during a host  write  failure  recovery

process.   This  description  assumes  a shadowing implementation

that contains a separate shadowing layer.  Note that  the  actual

implementation of host write failure recovery processing need not

be done in a distinct layer nor use  the  exact  algorithm  given

here, as long as all class driver visible results are identical.

The  host  write  failure  recovery  process is an error recovery

operation and is performed as a part of the  write  command  that

failed.   Within  this  context,  the host write failure recovery

process is a non-Sequential MSCP  operation.   Also  within  this

context,  the  host  write  failure  recovery  process blocks any

Sequential commands received after  the  original  write  command

began  execution  but before the host write failure recovery pro-

cess completes.  Any Sequential  commands  received  during  this

time  cannot  begin  execution until after the host write failure

recovery process completes.

Although  the  description above exactly matches the MSCP defini-

tion of a non-Sequential command's effects on a  Sequential  com-

mand,  it  is  repeated  here for emphasis.  This feature of host

write failure recovery processing is critical to host  synchroni-

zation  of  I/O  requests to the disk region affected by the host

write failure recovery process.

The  host  write failure recovery process is composed of the fol-

lowing steps:

1.  Starting  at  the  specified  LBN and proceeding for the

specified number of bytes, the contents  of  the  shadow

set are read.  This read proceeds according to the rules

for reading a  shadow  set  described  in  Section  sec-

tion(bunshadreadop).   If at least one shadow set member

has been fully  written  by  the  write  operation,  the

WAIVERS AND EXCEPTIONS                                  PAGE E-38

HSC AND VAX/VMS

controller  may at its option use a fully written member

as the preferred source for the read data.

2.  If  the  read  operation  results in the removal of some

shadow set members,  that  removal  is  postponed  until

write  failure processing finishes.  Whenever the shadow

set contains zero  or  one  operational  members,  write

failure recovery processing finishes immediately.

3.  After  the  read  has finished, the data that it read is

written back the shadow set  virtual  unit.   Note  this

operation cannot generate any class 2 shadow set members

(as defined in Section  section(bunshadwritop))  because

no access to host memory is required.

E.3.1.1.6.9.4  Read/Write Concurrency  -

It  is  important  for  the  host  to  be  aware that if multiple

transfer operations are outstanding, the controller  provides  no

guarantee  of  the order of the transfer execution and/or comple-

tion.  This restriction may result in any of the following possi-

ble inconsistencies:

1.  If  both a read and write command are outstanding to the

same LBN, although the final data on the  disk  will  be

the  specified  write  data,  the data returned from the

read command may consist of the original data before the

write,  the new data after the write, or even a combina-

tion of  both  (if  the  transfer  encompasses  multiple

LBN's).

2.  If  two  or  more  write commands are outstanding to the

same LBN, the final data on the disk may consist of  the

first  write, the second write, or a combination of both

(if the transfer encompasses multiple LBN's).

3.  When  a shadow set consists of more than one member, the

issuance of multiple write commands to the same LBN  may

result  in the above mentioned inconsistencies, with the

added complication of varying  combinations  across  the

individual members of the shadow set.

WAIVERS AND EXCEPTIONS                                  PAGE E-39

HSC AND VAX/VMS

4.  When  a shadow set consists of more than one member, the

issuance of interleaved read and write commands  to  the

same  LBN  which  result in the controller attempting to

repair the data from the read may result in inconsistent

data across the individual members of the shadow set.

The above problems may be avoided if the host(s) provide internal

gatekeeping  on  write  commands  to  an  individual  LBN.   This

gatekeeping  may be summarized as "No write command may be issued

which encompasses an LBN if any other I/O operation is  currently

outstanding which also encompasses that same LBN."

E.3.1.1.6.10  Shadow Set Membership Changes  -

If  a  member  is  removed from the shadow set by either the con-

troller (e.g., as a result of an unrecoverable error on a drive),

an  operator  action  (e.g.,  changing the hardware write protect

status), or the host (e.g., as a result of an AVAILABLE command),

the  affected  member  will  be  placed  in a "Unit-Available" or

"Unit-Offline" state, depending on the nature of the change,  and

the virtual unit will be placed in a "Unit-Available" state.

As  a result of this change, all commands known to the controller

which are directed to the affected virtual unit and  require  the

virtual  unit  to  be in a "Unit-Online" state will be terminated

with a status code of "Shadow Set Status Has  Changed." Any  com-

mands  received  by  the  controller  subsequent  to  this change

directed to the virtual unit which require the virtual unit to be

in  a  "Unit-Online"  state  will  be  rejected  with a status of

"Unit-Available."

E.3.1.1.6.11  Multihost Access  -

A concept which is unique in the multihost environment is that of

"ownership" and "online  counts." The  controller  must  maintain

knowledge of all hosts which have brought a specific unit online,

and also know which hosts have which  units  online.   A  command

which  is  addressed to a specific unit will only succeed if that

unit has previously been brought online by the issuing host.   At

WAIVERS AND EXCEPTIONS                                  PAGE E-40

HSC AND VAX/VMS

the   same   time,  the  knowledge  of  "online  hosts"  must  be

maintained.

Although  the  higher  level  management of the shadow set is the

responsibility of the host, many implicit actions  are  performed

by  the controller on behalf of the hosts in a multihost environ-

ment.  The sections which follow describe  the  implicit  actions

performed by a controller in a multihost environment.

E.3.1.1.6.11.1  Member Unit Actions  -

When an AVAILABLE command is issued by a host which currently has

the designated unit "Unit-Online," the unit will transition to  a

"Unit-Available"  state  relative  to  the issuing host, but will

remain "Unit-Online" to all other hosts.  The  unit  itself  will

not  transition  to a "Unit-Available" state relative to the con-

troller until such time as ALL hosts which have the  unit  online

issue an AVAILABLE command or the "All Class Drivers" modifier is

used.  When no host has the unit "Unit-Online," the unit will  be

removed  from the shadow set, and the virtual unit will be placed

in a "Unit-Available"  state  (since  the  membership  count  has

changed).   In this case, future commands directed to the virtual

unit would be rejected with a status of  "Unit-Available"  rather

than  "Shadow  Set  Status  Has  Changed,"  since  the sequential

gatekeeping mechanism would preclude the possibility of any  out-

standing commands when the (last) AVAILABLE message is processed.

Note  that if only one host had the specific unit ONLINE, then if

that host issued an AVAILABLE command, the unit would be  removed

from  the  shadow  set  and the virtual unit would be placed in a

"Unit-Available" state.   This  will  happen  regardless  of  the

number of hosts which have the shadow set virtual unit ONLINE.

When  processing an ONLINE command with the shadow unit specified

modifier, the controller must first check to see if the specified

virtual  unit  has  already been created by another host.  If the

virtual unit is not in existence,  it  will  be  created  and  an

AVAILABLE  ATTENTION  message  will be sent to all other hosts in

accordance with Section section(availabatnmes).  If  the  virtual

unit  already  exists  and  the  ONLINE unit flags are identical,

several actions will be taken by the controller as follows:

1.  The  shadow  set membership will be updated as necessary

to reflect the addition of the new member.  Note that if

the member which the requesting host added was already a

member of the shadow set, this step is  nugatory  (other

WAIVERS AND EXCEPTIONS                                  PAGE E-41

HSC AND VAX/VMS

than updating the knowledge that the requesting host now

has this unit in a "Unit-Online" state).

2.  The  requesting host is informed of the total membership

by the mechanism of the ONLINE (or SET UNIT CHARACTERIS-

TICS) end message.

3.  Any future requests to the virtual unit by any host will

be addressed to the shadow set  as  a  whole,  including

both the new and any pre-existing members.

The implication of the above items is that if a shadow set is ex-

tended by one host, other hosts will have implicit access to  the

new  member  via the normal shadowing mechanisms, but will not be

explicitly informed of the new members by the controller.  At the

same  time,  only those hosts which have explicitly issued an ON-

LINE command to the unit will have access to the individual unit.

Finally, when those hosts which have "privately" added units to a

shadow set crash or issue AVAILABLE  commands  for  those  units,

they will be removed from the shadow set.

In a multihost environment, it is expected that several hosts may

desire to bring members of a shadow set online relative to  their

connections.   Therefore,  commands  which  would produce results

identical with the current  state  for  a  specified  "Unit"  and

"Shadow Unit" are allowed and are treated as redundant except for

changing the unit state relative to the connection.

As  detailed  previously, a unit may not be "Unit-Online" in both

shadowed and nonshadowed mode simultaneously.  The implication of

this in a multihost environment is that the individual hosts must

coordinate their activity when bringing a unit into a shadow set.

E.3.1.1.6.11.2  Shadow Set Virtual Unit Actions  -

As  noted  above,  the membership of a shadow set is the union of

all units added to the shadow set by all hosts.  Any host access-

ing  the  shadow  set effects all units in the shadow set whether

that host added said units or not.

When  a  host  issues an AVAILABLE command to the shadow set as a

whole, that shadow set  will  transition  to  a  "Unit-Available"

state   relative   to   the   requesting  host  but  will  remain

"Unit-Online" to all hosts which have not  issued  the  AVAILABLE

command  (unless  the  "All Class Drivers" modifier is supplied).

When multiple hosts have the shadow set  online,  the  "Dissolve"

modifier  will make all shadow set members which the issuing host

WAIVERS AND EXCEPTIONS                                  PAGE E-42

HSC AND VAX/VMS

has online available with respect  to  that  host.   If  a  given

shadow  set  member was online only to that host, it will transi-

tion into the "Unit-Available" state and, if the "Spin-down" mod-

ifier  was  specified, it will spin down.  When ONLY one host has

the shadow set and its members online,  the  "Dissolve"  modifier

will   remove   all  members,  place  the  virtual  unit  in  the

"Unit-Offline" state and, if the "Spin-down" modifier  is  speci-

fied, all members will be spun down.

When a shadow set membership change is detected, the virtual unit

is placed in the  "Unit-Available"  state  with  respect  to  all

hosts.   To  regain access to the virtual unit each host must in-

dividually issue an ONLINE command addressed to the virtual unit.

The  first host to issue an ONLINE command makes the virtual unit

"Unit-Online" only with respect to itself.  The unit  remains  in

the "Unit-Available" state with respect to other hosts.

When  a  host  issues  an  AVAILABLE  command with the "All Class

Drivers" modifier to the shadow set as a whole, that  shadow  set

will  transition  to  the  "Unit-Available" state relative to all

hosts.  This action will be carried out even if  the  shadow  set

was "Unit-Available" to the issuing host.  If the "Dissolve" mod-

ifier is specified in addition, all members will be removed  from

the  shadow  set  and  the  virtual  unit  will  be placed in the

"Unit-Offline" state and, if the "Spin-down" modifier  is  speci-

fied, all members will be spun down.

E.3.1.1.6.11.3  Shadow Copy Actions  -

For any given physical unit, only one host may have a shadow copy

operation in progress.  Subsequent ONLINE commands  requesting  a

shadow   copy  operation  will  be  rejected  with  a  status  of

"Unit-Available," subcode of "Copy in Progress."

If  an  ONLINE  command  which  requests some form of shadow copy

operation is received for a unit which was added without  a  copy

operation  or  which  has  completed  a previously requested copy

operation, the copy mode field of the command  will  be  ignored.

The  unit will be added to the shadow set without any copy opera-

tion being performed.

If  an  ONLINE  command  which  requests some form of shadow copy

operation is received for a unit which is the first member  of  a

shadow  set,  the copy mode field of the command will be ignored.

The unit will be added to the shadow set without any copy  opera-

tion being performed.

WAIVERS AND EXCEPTIONS                                  PAGE E-43

HSC AND VAX/VMS

E.3.1.1.6.11.4  Get Unit Status Actions  -

The  GET  UNIT STATUS command will return the "shadow set master"

and "shadow set member" unit flags, and  the  Shadow  Status  and

Shadow  Unit  fields  correctly  regardless of whether on not the

specified physical or virtual unit is "Unit-Online" with  respect

to  the host issuing the command.  If one host forms a shadow set

composed of two units, another host can use GET  UNIT  STATUS  to

determine  the  status  of  either the virtual unit or the member

units without bringing any of them "Unit-Online" with respect  to

itself.

E.3.1.1.6.12  Error Logs  -

Any  drive  specific  error log messages generated as a result of

shadowing  operations  will  be  in   accordance   with   Section

section(errorlog_mess).  The command reference number of the mes-

sage will be  that  of  the  original  command  reference  number

directed to the virtual unit (as opposed to the one issued by the

shadowing layer directed  to  the  MSCP  layer),  and  the  drive

specific  information will be that of the drive which had the er-

ror.  The restriction limiting to three the number of error  logs

generated for a single error is extended in the shadowing case to

a physical drive basis -- if errors occur on all  members  of  an

'n'  member  shadow set, then the controller must limit its error

log messages to three messages per drive, giving a maximum of 'n'

times three messages.

WAIVERS AND EXCEPTIONS                                  PAGE E-44

HSC AND VAX/VMS

E.3.1.1.7  "Bundled Shadowing" Definitions  -

The  following  section  contains  specific  modifier,  flag, and

offset information.  The tables which follow contain the  command

modifier,  controller  and  unit flag, and message offset defini-

tions specific to "Bundled Shadowing." The tables  are  formatted

in  the  same  manner  as their Appendix appendix(opcdsflgsoffs),

"Opcode, Flag,  and  Offset  Definitions"  counterparts  to  ease

cross-referencing.

In  addition,  Table  "Bundled  Shadowing" A-a, "Shadow Copy Mode

Operation Codes" contains other definitions specific  to  Shadow-

ing.   However,  it has no Appendix appendix(opcdsflgsoffs), "Op-

code, Flag, and Offset Definitions" counterpart.

WAIVERS AND EXCEPTIONS                                  PAGE E-45

HSC AND VAX/VMS

Table "Bundled Shadowing" table(commandmodifer):  Command Modifiers

+------+--------------+--------------------------+----------------------------------------+

| Bit  |   Bit Mask   |   Preferred Mnemonics    |                                        |

|Number+-------+------+--------+-----------------+     Command Modifier                   |

|      | Octal | Hex. | 16 bit |32 bit (see note)|                                        |

+------+-------+------+--------+-----------------+----------------------------------------+

| AVAILABLE Command Modifiers:                   |                                        |

|   4  |    20 |   10 | MD.DIS | MSCP$x_MD_DSOLV | Dissolve Shadow Set                    |

+------+-------+------+--------+-----------------+----------------------------------------+

| ONLINE and SET UNIT CHARACTERISTICS Command Modifiers:                                  |

|   4  |    20 |   10 | MD.SHD | MSCP$x_MD_SHDSP | Shadow Unit Specified                  |

+------+-------+------+--------+-----------------+----------------------------------------+

| Note: The  "x"  in  a  32  bit  mnemonic  for  a  bit flag will be "V" if the symbol is |

|       defined as a bit number (offset) or an "M" if defined as a mask.                  |

+-----------------------------------------------------------------------------------------+

Table "Bundled Shadowing" table(controllerflag): Controller Flags

+------+--------------+--------------------------+----------------------------------------+

| Bit  |   Bit Mask   |   Preferred Mnemonics    |                                        |

|Number+-------+------+--------------------------+     Controller Flag                    |

|      | Octal | Hex. | 16 bit |32 bit (see note)|                                        |

+------+-------+------+--------+-----------------+----------------------------------------+

|   1  |     2 |    2 | CF.SHD | MSCP$x_CF_SHADW | Shadowing                              |

+------+-------+------+--------+-----------------+----------------------------------------+

| Note: The  "x"  in  a  32  bit  mnemonic  for  a  bit flag will be "V" if the symbol is |

|       defined as a bit number (offset) or an "M" if defined as a mask.                  |

+-----------------------------------------------------------------------------------------+

Table "Bundled Shadowing" table(unitflagsdefs): Unit Flags

+------+--------------+--------------------------+----------------------------------------+

| Bit  |   Bit Mask   |   Preferred Mnemonics    |                                        |

|Number+-------+------+--------+-----------------+     Unit Flag                          |

|      | Octal | Hex. | 16 bit |32 bit (see note)|                                        |

+------+-------+------+--------+-----------------+----------------------------------------+

|  14  | 40000 | 4000 | UF.MEM | MSCP$x_UF_SSMEM | Shadow Set Member                      |

+------+-------+------+--------+-----------------+----------------------------------------+

|   9  |  1000 |  200 | UF.MST | MSCP$x_UF_SSMST | Shadow Set Master                      |

+------+-------+------+--------+-----------------+----------------------------------------+

| Note: The  "x"  in  a  32  bit  mnemonic  for  a  bit flag will be "V" if the symbol is |

|       defined as a bit number (offset) or an "M" if defined as a mask.                  |

+-----------------------------------------------------------------------------------------+

WAIVERS AND EXCEPTIONS                                                                    PAGE E-46

HSC AND VAX/VMS

Table "Bundled Shadowing" table(commandmesoff): Command Message Offsets

+--------------------+--------------------------+-------+---------------------------------+

|    Offset Value    |   Preferred Mnemonics    | Field |                                 |

+------+------+------+--------+-----------------+ Size  | Field Description               |

| Dec. | Oct. | Hex. | 16 bit |      32 bit     |(bytes)|                                 |

+------+------+------+--------+-----------------+-------+---------------------------------+

| ONLINE and SET UNIT CHARACTERISTICS Command Message Offsets:                            |

|  20  |  24  |  14  | P.EXBA | MSCP$L_EXCL_LBA |   4   | Excluded LBN Address            |

|  24  |  30  |  18  | P.EXBC | MSCP$W_EXCL_LBC |   2   | Excluded LBN Count              |

|  32  |  40  |  20  | P.SHUN | MSCP$W_SHDW_UNT |   2   | Shadow unit                     |

|  34  |  42  |  22  | P.CPMD | MSCP$W_COPY_MOD |   2   | Copy mode  (see  Table "Bundled |

|      |      |      |        |                 |       | Shadowing" A-a)                 |

+------+------+------+--------+-----------------+-------+---------------------------------+

| Note: An asterisk (*) in the field size indicates that the size varies.                 |

+-----------------------------------------------------------------------------------------+

Table "Bundled Shadowing" table(endattmesoff): End and Attention Message Offsets

+--------------------+--------------------------+-------+---------------------------------+

|    Offset Value    |   Preferred Mnemonics    | Field |                                 |

+------+------+------+--------+-----------------+ Size  | Field Description               |

| Dec. | Oct. | Hex. | 16 bit |      32 bit     |(bytes)|                                 |

+------+------+------+--------+-----------------+-------+---------------------------------+

| GET UNIT STATUS End Message Offsets:          |       |                                 |

|  32  |  40  |  20  | P.SHUN | MSCP$W_SHDW_UNT |   2   | Shadow unit                     |

|  34  |  42  |  22  | P.SHST | MSCP$W_SHDW_STS |   2   | Shadow   status    (see   Table |

|      |      |      |        |                 |       | "Bundled Shadowing" A-a)        |

+------+------+------+--------+-----------------+-------+---------------------------------+

| ONLINE and SET UNIT CHARACTERISTICS End Message offsets:                                |

|  32  |  40  |  20  | P.SHUN | MSCP$W_SHDW_UNT |   2   | Shadow unit                     |

|  34  |  42  |  22  | P.SHST | MSCP$W_SHDW_STS |   2   | Shadow   status    (see   Table |

|      |      |      |        |                 |       | "Bundled Shadowing" A-a)        |

+------+------+------+--------+-----------------+-------+---------------------------------+

WAIVERS AND EXCEPTIONS                                                                    PAGE E-47

HSC AND VAX/VMS

Table "Bundled Shadowing" A-a: Shadow Copy Mode Operation Codes

+--------------------+--------------------------+-----------------------------------------+

|    Opcode Value    |   Preferred Mnemonics    |                                         |

+------+------+------+--------+-----------------+   Operation                             |

| Dec. | Oct. | Hex. | 16 bit |     32 bit      |                                         |

+------+------+------+--------+-----------------+-----------------------------------------+

|   0  |   0  |   0  | CM.NOC | MSCP$K_CM_NOCPY | No Copy                                 |

+------+------+------+--------+-----------------+-----------------------------------------+

|   1  |   1  |   1  | CM.NCP | MSCP$K_CM_COPY  | Normal Copy                             |

+------+------+------+--------+-----------------+-----------------------------------------+

|   2  |   2  |   2  | CM.MCP | MSCP$K_CM_MGCPY | Merge Copy                              |

+------+------+------+--------+-----------------+-----------------------------------------+

E.3.1.1.8  "Bundled Shadowing" Specific Status Code Definitions  -

The  following  tables  contain  the  status  code definitions specific to "Bundled Shadowing." The

tables are formatted in the same manner as  their  Appendix  appendix(statusevntdef),  "Status  and

Event Code Definitions" counterparts to ease cross-referencing.

Table "Bundled Shadowing" table(statuseventcd):  Status and Event Codes

+--------------------+--------------------------+-----------------------------------------+

|       Value        |   Preferred Mnemonics    |                                         |

+------+------+------+--------+-----------------+     Status or Event Code                |

| Dec. | Oct. | Hex. | 16 bit |     32 bit      |                                         |

+------+------+------+--------+-----------------+-----------------------------------------+

|   12 |   14 |    C | ST.SSS | MSCP$K_ST_SHST  | Shadow Set Status Has Changed           |

+------+------+------+--------+-----------------+-----------------------------------------+

WAIVERS AND EXCEPTIONS                                                                    PAGE E-48

HSC AND VAX/VMS

Table "Bundled Shadowing" table(statusonleesc): Status Only Subcode Values

+------+---------------------+------------------------------------------------------------+

| Sub- |   Code + Subcode    |                                                            |

| code +------+-------+------+                 Status Subcode                             |

|      | Dec. |  Oct. | Hex. |                                                            |

+------+------+-------+------+------------------------------------------------------------+

| "Unit-Available" Subcode values:                                                        |

|    2 |   68 |   104 |   44 | Shadow Set Copy In Progress                                |

|      |      |       |      |   This  subcode  is  returned while a copy operation is in |

|      |      |       |      |   progress  if  any  of the following circumstances occur: |

|      |      |       |      |   1) any command is issued directly to the copy recipient; |

|      |      |       |      |   2) the controller dependent number of copy operations in |

|      |      |       |      |   progress  limit  has already been reached; or 3) another |

|      |      |       |      |   host has already initiated a copy operation to the unit. |

|      |      |       |      |   Note  that  this bit may also be returned in the "Shadow |

|      |      |       |      |   Status"   field   of  a  GET  UNIT  CHARACTERISTICS  end |

|      |      |       |      |   message  if  the  command  is  directed  to a shadow set |

|      |      |       |      |   member on which a copy is in progress.                   |

|    4 |  132 |   204 |   84 | No Members In Shadow Set                                   |

|      |      |       |      |   An  ONLINE  command was addressed to the virtual unit of |

|      |      |       |      |   an  existing  shadow  set  from  which  all members have |

|      |      |       |      |   been removed.                                            |

+------+------+-------+------+------------------------------------------------------------+

| "Shadow Set Status Has Changed" Subcode values:                                         |

|    0 |   12 |    14 |    C | Subcodes are not used.                                     |

|      |      |       |      |   In general this status code is returned as the result of |

|      |      |       |      |   a shadow set member being removed from a shadow set. See |

|      |      |       |      |   specific    references   in   the   "Bundled  Shadowing" |

|      |      |       |      |   specification  for  details  of particular circumstances |

|      |      |       |      |   under which this status code is returned.                |

+------+------+-------+------+------------------------------------------------------------+

Table "Bundled Shadowing" table(mediafmterrsc): "Media Format Error" Status or Event Subcode Values

+------+---------------------+-+-+--------------------------------------------------------+

| Sub- |   Code + Subcode    |E|S|                                                        |

| code +------+-------+------+V|T|     Status or Event Subcode                            |

|      | Dec. |  Oct. | Hex. | | |                                                        |

+------+------+-------+------+-+-+--------------------------------------------------------+

|    4 |  133 |   205 |   85 | |*| Characteristics  Or  Protection  Mismatch  For  Shadow |

|      |      |       |      | | | Member                                                 |

|      |      |       |      | | |   The  geometry  or  write protect state of a physical |

|      |      |       |      | | |   unit  being  added  to  a  shadow set does not match |

|      |      |       |      | | |   that  of the shadow set or the physical unit's Write |

|      |      |       |      | | |   Protect (data safety) unit flag is set.              |

+------+------+-------+------+-+-+--------------------------------------------------------+

WAIVERS AND EXCEPTIONS                                                                    PAGE E-49

HSC AND VAX/VMS

Table "Bundled Shadowing" table(controlrerrsc): "Controller Error" Status or Event Subcode Values

+------+---------------------+-+-+--------------------------------------------------------+

| Sub- |   Code + Subcode    |E|S|                                                        |

| code +------+-------+------+V|T|     Status or Event Subcode                            |

|      | Dec. |  Oct. | Hex. | | |                                                        |

+------+------+-------+------+-+-+--------------------------------------------------------+

|   10 |  330 |   512 |  14A | |*| Insufficient Resources                                 |

|      |      |       |      | | |   The  controller  is  unable  to  honor  a request to |

|      |      |       |      | | |   create  a  shadow set or to add an additional member |

|      |      |       |      | | |   to an existing shadow set, due to a lack of internal |

|      |      |       |      | | |   resources  to support the new entity. Note that this |

|      |      |       |      | | |   subcode must not be used for any other purpose.      |

+------+------+-------+------+-+-+--------------------------------------------------------+

WAIVERS AND EXCEPTIONS                                  PAGE E-50

HSC AND VAX/VMS

E.4  TU81

E.4.1  "Enable  Miscellaneous  Error  Log  Messages" Modifier Not

Fully Supported

Waiver:

The TU81 does not fully support the "Enable Miscellaneous Er-

ror Log Messages" modifier to the SET CONTROLLER CHARACTERIS-

TICS  command as required by Section section(controlerflags),

"Controller Flags." If the modifier is set by  the  host  the

TU81  merely  ignores it and returns it clear in the end mes-

sage of the SET CONTROLLER CHARACTERISTICS command.

E.5  VAX/VMS

E.5.1  Bad Block Replacement Attempt Error Log Usage

Usage Note:

VAX/VMS implements usage of the BAD BLOCK REPLACEMENT ATTEMPT

error  log  message  format  for  host-initiated  bad   block

replacement  operations performed by VMS.  Error log messages

so generated supply all message fields in conformance to  the

MSCP  specification  with  the  exception  of  the  "sequence

number" field.  In VMS generated error log messages, the "se-

quence number" field has all bits set (numeric value -1).

WAIVERS AND EXCEPTIONS                                  PAGE E-51

RV80

E.6  RV80

E.6.1  Media Loader Error Logs

Waiver:

Add or modify the following sections of the MSCP specification to

include the means by  which  an  MSCP  or  TMSCP  controller  can

generate Media Loader Error Logs:

o  Modify section section(cntrlrunitids)    "Controller and

Unit Identifiers"

o  Add new section        "Media Loader Error Log"

o  Modify table table(errlogmesoff)       "Error Log Message

Offsets

o  Modify table table(errlogmesfmt)       "Error Log Message

Format Codes"

o  Modify table table(statuseventcd)       "Status and Event

Codes"

o  Add table table(medialdrerrsc)         "Media Loader Error

Status and

Event Subcode Values"

o  Modify table table(contrlunclass)       "Controller and

Unit Class Values"

o  Add table table(medloaderiden)          "Media Loader

Values"

Impact:

Host  error  log report generators must be modified to respond to

the new format appropriately.  If the error report generator mod-

ification  is  not  implemented  for VMS Version 5.0, the Ptolemy

Optical Library schedule will be delayed until the  next  version

release of VMS.

This  ECO does *not* request that any operating system respond to

a media loader as a new device, just to interpret its error logs.

WAIVERS AND EXCEPTIONS                                  PAGE E-52

RV80

E.6.1.1  Controller   and   Unit   Identifiers    - Section  sec-

tion(cntrlrunitids), "Controller and Unit Identifiers," is  modi-

fied to read as follows:

section(cntrlrunitids) Controller, Media Loader, and Unit Identi-

fiers

MSCP  requires  that  all  controllers, media loaders, and drives

have unique identifiers,  called  controller  identifiers,  media

loader identifiers, and unit identifiers.  The structure of these

identifiers is as follows:

31 0

+-------------------------------+

| unique device number |

+-------+-------+ +

| class | model | |

+-------+-------+---------------+

The  "class"  byte  identifies  the type of the subsystem -- con-

troller, media loader, disk drive, etc.  The "model" byte identi-

fies  the  exact  model  of  the subsystem within its class.  All

valid class and model codes are nonzero, implying that all  valid

identifiers  are  nonzero.  The "unique device number" field must

uniquely identify the device among all devices of that same class

and model.  The device serial number could be used as the "unique

device number," though that isn't  required.   Currently  defined

values  for  the "class" and "model" bytes are listed in Appendix

appendix(ctruntmedids),  "Controller,  Unit,   and   Media   Type

Identifiers." Table  table(contrlunclass),  "Controller  and Unit

'Class' Values" contains  the  "class"  values  for  controllers,

media  loaders,  and  drives.  Table table(controllrmodel), "Mass

Storage Controller 'Model' Values" contains  the  "model"  values

for both disk and tape controllers.  Tables table(diskdriveiden),

"Disk Drive/Media Identifier Values," table(tapedriveiden), "Tape

Drive/Media  Identifier Values," and table(medloaderiden), "Media

Loader Identifier Values," contain the "model"  values  for  disk

drives, tape drives, and media loaders, respectively.  Values for

new devices must be added to those tables  via  an  ECO  to  this

specification as new products are developed.  Note that different

units of multiunit drives are distinguished by  having  different

"model" bytes;  the "class" and "unique device number" fields are

typically identical.  Note also that all  MSCP  servers  for  the

same device class within the same controller must return the same

controller identifier.

As  previously  stated,  MSCP  requires  that controller and unit

identifiers be unique across all  devices  accessible  via  MSCP.

This  clearly cannot be checked by controllers.  Controllers can,

however, enforce unique unit identifiers across  the  units  that

WAIVERS AND EXCEPTIONS                                  PAGE E-53

RV80

are  attached  to  themselves.   This is done using the following

algorithms:

1.  Controllers  should detect and respond to duplicate unit

identifiers across all units whose unit identifiers  the

controller  can  obtain,  including all units that would

otherwise          be          "Unit-Online"          or

Detection  of a duplicate unit identifier on one unit of

a multiunit drive is treated as a duplicate unit identi-

fier condition on all other units that share one or more

of the following components with the unit having the du-

plicate unit identifier:

a.  A unit number select mechanism.

b.  A Run/Stop or Load/Unload switch.

c.  A spindle or other mechanical components.

Note  that  duplicate  unit identifiers are detected re-

gardless of the state of a unit's Run/Stop  or  Load/Un-

load switch.

2.  Whenever  a controller becomes aware of a duplicate unit

identifier, it immediately spins-down all units with the

duplicate   identifier   and   forces   them  to  remain

spun-down.  The controller spins-down the units  regard-

less  of  their current state.  The controller typically

forces them to remain spun-down by  spinning  them  down

again whenever an operator spins them up.

3.  The    controller    reports    "Unit-Offline   (subcode

'Inoperative')" status for all units with duplicate unit

identifiers.  In addition, if the unit might potentially

be  connected  to  another  controller,  the  controller

should  flag  the presence of the duplicate unit identi-

fier in the drive.   Other  controllers,  if  any,  must

check   this   flag   and   also   treat   the  unit  as

"Unit-Offline" if the flag is set.

Whether or not a controller does in fact check for duplicate unit

identifiers is controller dependent.  Note that a duplicate  unit

identifier is a drastic failure, indicative of some otherwise un-

diagnosed hardware malfunction.

WAIVERS AND EXCEPTIONS                                  PAGE E-54

RV80

E.6.1.2  Media  Loader  Error Log Format  - The following section

is added after section(bbratmlogfmt), "BAD BLOCK REPLACEMENT  AT-

TEMPT Error Log Format":

5.n.m MEDIA LOADER ERRORS Error Log Format

The  following  format is used to report errors that occur during

media loader operations.

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| event code | flags | format|

+---------------+-------+-------+

| |

+--- controller identifier ---+

| |

+---------------+-------+-------+

|multiunit code |chvrsn | csvrsn|

+---------------+-------+-------+

| |

+--- unit identifier ---+

| |

+-------+-------+-------+-------+

| reserved | uhvrsn| usvrsn|

+-------+-------+-------+-------+

| |

+--- media loader identifier ---+

| |

+-------+-------+-------+-------+

| ml unit number|mlhvrsn|mlsvrsn|

+-------+-------+-------+-------+

| |

/ controller or media loader /

/ dependent information /

| |

+-------------------------------+

The  fields  in  the  MEDIA LOADER ERROR error log message are as

follows (unit number, multiunit code,  unit  identifier,  usvrsn,

and  uhvrsn  refer to the disk or tape unit that is the target of

the media loader operation):

command reference number

unit number

sequence number

format

WAIVERS AND EXCEPTIONS                                  PAGE E-55

RV80

flags

event code

csvrsn

chvrsn

multiunit code

unit identifier

usvrsn

uhvrsn

See  Section  section(generrlogfmt),  "Generic  Error Log

Message Format."

controller identifier

Uniquely  identifies the controller among all devices ac-

cessible via MSCP.  See  Section  section(cntrlrunitids),

"Controller  and  Unit  Identifiers".  If zero, the error

log was generated by host software  and  not  by  a  con-

troller.

media loader identifier

Uniquely  identifies  the  media loader among all devices

accessible via MSCP.  See Section section(cntrlrunitids),

"Controller  and  Unit  Identifiers."  This field is only

present for  errors  that  relate  to  a  specific  media

loader.

mlsvrsn

The media loader's software, firmware, or microcode revi-

sion number.  Always zero if  the  product's  development

and  maintainability  groups  determine  that there is no

benefit  from  supporting   machine   readable   revision

numbers.

mlhvrsn

The media loader's hardware revision number.  Always zero

if the product's development and  maintainability  groups

determine  that  there  is  no  benefit  from  supporting

machine readable revision numbers.

ml unit number

The  unit  number  of the media loader to which the error

log message relates, or zero if either the  message  does

not relate to a specific unit or if the media loader does

not support unit numbers.

WAIVERS AND EXCEPTIONS                                  PAGE E-56

RV80

controller or media loader dependent information

A  variable (controller or media loader dependent) amount

of information.  The length of this  information  is  im-

plied  by  the  total  length  of  the error log message,

passed to the class driver by the port driver.  This  in-

formation  will typically not be interpreted by error log

formatting programs, instead being printed as a series of

values.

WAIVERS AND EXCEPTIONS                                                                    PAGE E-57

RV80

E.6.1.3  Error  Log  Message  Offsets   - Table table(errlogmesoff), "Error Log Message Offsets" is

modified to appear as follows:

Table table(errlogmesoff):  Error Log Message Offsets

+--------------------+--------------------------+-------+---------------------------------+

|    Offset Value    |   Preferred Mnemonics    | Field |                                 |

+------+------+------+--------+-----------------+ Size  | Field Description               |

| Dec. | Oct. | Hex. | 16 bit |      32 bit     |(bytes)|                                 |

+------+------+------+--------+-----------------+-------+---------------------------------+

|                                            .                                            |

|                                            .                                            |

|                                            .                                            |

+------+------+------+--------+-----------------+-------+---------------------------------+

| Bad Block Replacement Attempt Error Log Message Offsets:                                |

|  34  |  42  |  22  | L.RPFL | MSLG$W_RPL_FLGS |   2   | Replace Flags (see Table

table(bbratmprpflg))  |

|  36  |  44  |  24  | L.VSER | MSLG$L_VOL_SER  |   4   | Volume serial number            |

|  40  |  50  |  28  | L.LBN  | MSLG$L_BAD_LBN  |   4   | Bad LBN                         |

|  44  |  54  |  2C  | L.ORBN | MSLG$L_OLD_RBN  |   4   | Old RBN                         |

|  48  |  60  |  30  | L.NRBN | MSLG$L_NEW_RBN  |   4   | New RBN                         |

|  52  |  64  |  34  | L.RPEV | MSLG$W_CAUSE    |   2   | Cause (an event code)           |

+------+------+------+--------+-----------------+-------+---------------------------------+

| Media Loader Errors Error Log Message Offsets:                                          |

|  36  |  44  |  24  | L.MLID | MSLG$Q_ML_ID    |   8   | Media loader identifier         |

|  44  |  54  |  2C  | L.LSVR | MSLG$B_ML_SVR   |   1   | Media loader software version   |

|  45  |  55  |  2D  | L.LHVR | MSLG$B_ML_HVR   |   1   | Media loader hardware version   |

|  46  |  56  |  2E  | L.MLUN | MSLG$W_ML_UNIT  |   2   | Media loader unit number        |

|  48  |  60  |  30  |        |                 |   *   | Controller or media loader      |

|      |      |      |        |                 |       | dependent information (optional)|

+------+------+------+--------+-----------------+-------+---------------------------------+

| Note:  An asterisk (*) in the field size indicates that the size varies.                |

+-----------------------------------------------------------------------------------------+

WAIVERS AND EXCEPTIONS                                                                    PAGE E-58

RV80

E.6.1.4  Error  Log  Message  Format  Codes  - Table table(errlogmesfmt), "Error Log Message Format

Codes" is modified to appear as follows:

Table table(errlogmesfmt):  Error Log Message Format Codes

+--------------------+--------------------------+-----------------------------------------+

|    Format Code     |   Preferred Mnemonics    |                                         |

+------+------+------+--------+-----------------+   Format Description                    |

| Dec. | Oct. | Hex. | 16 bit |      32 bit     |                                         |

+------+------+------+--------+-----------------+-----------------------------------------+

|   0  |   0  |   0  | FM.CNT | MSLG$K_CNT_ERR  | Controller Errors                       |

+------+------+------+--------+-----------------+-----------------------------------------+

|   1  |   1  |   1  | FM.BAD | MSLG$K_BUS_ADDR | Memory Errors  with  (Host/Controller)  |

|      |      |      |        |                 | Memory Address                          |

+------+------+------+--------+-----------------+-----------------------------------------+

|   2  |   2  |   2  | FM.DSK | MSLG$K_DISK_TRN | Disk Transfer Errors                    |

+------+------+------+--------+-----------------+-----------------------------------------+

|   3  |   3  |   3  | FM.SDI | MSLG$K_SDI      | SDI Errors                              |

+------+------+------+--------+-----------------+-----------------------------------------+

|   4  |   4  |   4  | FM.SDE | MSLG$K_SML_DSK  | Small Disk Errors                       |

+------+------+------+--------+-----------------+-----------------------------------------+

|   9  |  11  |   9  | FM.RPL | MSLG$K_REPLACE  | Bad Block Replacement Attempt           |

+------+------+------+--------+-----------------+-----------------------------------------+

|  10  |  12  |   A  | FM.LDR | MSLG$K_LDR_ERR  | Media Loader Errors (see Note below)    |

+------+------+------+--------+-----------------+-----------------------------------------+

E.6.1.5  Status and Event Codes  - Table table(statuseventcd), "Status and Event Codes" is modified

to appear as follows:

Table table(statuseventcd):  Status and Event Codes

+--------------------+--------------------------+-----------------------------------------+

|       Value        |   Preferred Mnemonics    |                                         |

+------+------+------+--------+-----------------+     Status or Event Code                |

| Dec. | Oct. | Hex. | 16 bit |     32 bit      |                                         |

+------+------+------+--------+-----------------+-----------------------------------------+

|                                   .                                                     |

|                                   .                                                     |

|                                   .                                                     |

+------+------+------+--------+-----------------+-----------------------------------------+

|   20 |  24  |  14  | ST.BBR | MSCP$K_ST_BBR   | Bad Block Replacement Completion        |

+------+------+------+--------+-----------------+-----------------------------------------+

|   21 |  25  |  15  | ST.LDR | MSCP$K_ST_LOADR | Media Loader Error                      |

+------+------+------+--------+-----------------+-----------------------------------------+

|   31 |  37  |  1F  | ST.DIA | MSCP$K_ST_DIAG  | Message from an internal diagnostic     |

+------+------+------+--------+-----------------+-----------------------------------------+

WAIVERS AND EXCEPTIONS                                                                    PAGE E-59

RV80

E.6.1.6  Media  Loader  Error Status or Event Subcodes  - Table table(medialdrerrsc), "Media Loader

Error" Status or Event Subcodes, is added to Appendix appendix(statusevntdef),  "Status  and  Event

Code Definitions" and appears as follows:

Table table(medialdrerrsc):  "Media Loader Error" Status or Event Subcodes

+------+---------------------+-+-+-----------------------------------------------------+

| Sub- |   Code + Subcode    |E|S|                                                     |

| code +------+-------+------+V|T|     Status or Event Subcode (see Note below)        |

|      | Dec. |  Oct. | Hex. | | |                                                     |

+------+------+-------+------+-+-+-----------------------------------------------------+

|    1 |   55 |   67  |   37 |*|*| Loader command time out                             |

+------+------+-------+------+-+-+-----------------------------------------------------+

|    2 |   87 |  127  |   57 |*|*| Controller detected transmission error              |

+------+------+-------+------+-+-+-----------------------------------------------------+

|    3 |  119 |  167  |   77 |*|*| Controller detected protocol error                  |

+------+------+-------+------+-+-+-----------------------------------------------------+

|    4 |  151 |  227  |   97 |*|*| Loader detected error                               |

+------+------+-------+------+-+-+-----------------------------------------------------+

E.6.1.7  Controller and Unit Identifier Class Values  - Table table(contrlunclass), "Controller and

Unit Identifier Class Values" is modified to appear as follows:

Table table(contrlunclass):   Controller and Unit Identifier "Class" Values

+----------+----------------------------------------------+

|Class Byte|                                              |

| (decimal)|     Subsystem Type                           |

+----------+----------------------------------------------+

|     0    | reserved -- must not be assigned             |

+----------+----------------------------------------------+

|     1    | Mass storage controllers                     |

+----------+----------------------------------------------+

|     2    | Disk class devices -- DEC Standard 166 disks |

+----------+----------------------------------------------+

|     3    | Tape class devices                           |

+----------+----------------------------------------------+

|     4    | Disk class devices -- DEC Standard 144 disks |

+----------+----------------------------------------------+

|     5    | Loader class devices                         |

+----------+----------------------------------------------+

WAIVERS AND EXCEPTIONS                                                                    PAGE E-60

RV80

E.6.1.8  Media  Loader  Identifier  Values   - Table  table(medloaderiden) is added to Appendix ap-

pendix(ctruntmedids), "Controller, Unit, and Media Type Identifiers" and appears as follows:

Table table(medloaderiden):   Media Loader Identifier Values

+----------+---------+-------+---------------------------+--------------------------------+

|Model Byte|  Device | Media |   Media Type Identifier   |                                |

| (decimal)|Type Name| Name  |     octal     |    hex    |     Device                     |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     0    |  Reserved -- must not be assigned                                            |

+----------+---------+-------+---------------+-----------+--------------------------------+

|     1    |  none   | none  |     none      |   none    | 64 cartridge jukebox           |

+----------+---------+-------+---------------+-----------+--------------------------------+