APPENDIX F

PORT ADDRESS AND SYSTEM ADDRESS FORMATS

F.1  PORT ADDRESS FORMATS

The  format  of the "storage subsystem port address" field varies

depending on the communications mechanism being used.  The  field

format  for  the  currently  defined  (multihost)  communications

mechanism is described in the subsection that follows.

F.1.1  CI Port Address

The  format  of  the  "storage subsystem port address" field when

used with the Computer Interconnect (CI) communications mechanism

is as follows:

31 0

+---------------+---------------+

| | xport_addr |

+ +---------------+

| mbz |

+-------------------------------+

xport_addr

16-bit  CI transport address.  See the Computer Intercon-

nect Specification (DEC STD 161-0) for a  description  of

the internal structure of this field.

PORT ADDRESS AND SYSTEM ADDRESS FORMATS                  PAGE F-2

PORT ADDRESS FORMATS

mbz

This portion of the field Must Be Zero.

Note  that  the  Digital Storage Systems Interconnect (DSSI) com-

munications mechanism uses the same port addressing scheme as the

CI.

F.2  SYSTEM ADDRESS FORMATS

The format of the "storage subsystem system address" field varies

depending on the communications protocol being used.   The  field

format  for the currently defined (multihost) communications pro-

tocol is described in the subsection that follows.

F.2.1  SCA System Address

The  format  of the "storage subsystem system address" field when

used with the Systems Communications Architecture (SCA)  communi-

cations protocol is as follows:

31 0

+-------------------------------+

| system address |

+---------------+ +

| mbz | |

+---------------+---------------+

system address

48-bit  system  address.   See the Systems Communications

Architecture Specification for a description of  the  in-

ternal structure of this field.

mbz

This portion of the field Must Be Zero.