CHAPTER 1

INTRODUCTION

1.1  OVERVIEW OF MSCP SUBSYSTEM

Mass  Storage  Control  Protocol (MSCP) is the protocol used by a

family of mass storage controllers and devices designed and built

by  Digital Equipment Corporation.  In a system that uses an MSCP

storage subsystem, the controller contains intelligence  to  per-

form  the  detailed  I/O handling tasks.  This arrangement allows

the host to simply send command messages (requests for  reads  or

writes) to the controller and receive response messages back from

the controller.  The host need not concern  itself  with  details

such  as  device  type,  media  geometry,  media format, or error

recovery.

The  host  uses  two  levels of software to accomplish its tasks.

The higher level is called a "class driver." The  class  driver's

knowledge  of  devices  is  limited  to the device class (such as

disks) and their capacity.  The class driver  does  not  have  to

know  the  detailed  nature of the communications link (I/O bus),

controller, or devices that are being used.

The  second level of host software is called a "port driver." The

port driver passes messages to/from the  communications  link  or

bus.   It is not aware of the messages' meaning.  The port driver

does have to know the exact nature of the communications link  or

bus (communications mechanism).

In  the  controller  architecture,  there  are also two levels of

software.  The lower of these two is also a  "port  driver"  and,

like  the port driver in the host, is concerned only with passing

INTRODUCTION                                             PAGE 1-2

OVERVIEW OF MSCP SUBSYSTEM

messages on and off of the bus.  The higher level  of  controller

software is the "MSCP Server." It constitutes the intelligence of

the controller and therefore defines  the  functionality  of  the

controller.

The  MSCP  server  concerns itself with determining the number of

devices, their type, geometry, unit number, availability, status,

etc.   The  MSCP server receives requests from the host and sends

responses to the host.  It optimizes the requests,  performs  the

operations,  transfers  the  data to/from the host, transfers the

data to/from the device, and buffers the data as necessary.   The

MSCP  server  performs  error detection and recovery, and reports

any significant errors to the host.

Because  the MSCP server handles the error detection and recovery

by itself, the host sees "perfect media," an important character-

istic  of  an MSCP subsystem.  That is, the host need only report

errors to higher level (user) software, as the MSCP  server  per-

forms all error recovery and media defect (bad block) handling.

The  host's  class  driver and the controller's MSCP server route

their messages through the path of the two  port  drivers  and  a

hardware   interconnect.   This  is  their  physical  connection.

However, their logical communication is a direct  connection  be-

cause  the  port driver details are below their level of concern.

Therefore, there are two paths to consider:  a  physical  message

path  and a logical MSCP connection.  This is illustrated in Fig-

ure figure(example_system).

Host Mass Storage Controller

+ - - - - - - - - + + - - - - - - - - +

| | | |

+-----------+ +-----------+

| | Class | | MSCP | | MSCP | |

| Driver | <------------------> | Server |

| +-----------+ | | +-----------+ |

A A

| | | | | |

V V

| +-----------+ | Communications | +-----------+ |

| Port | <------------------> | Port |

| | Driver | | Protocols | | Driver | |

+-----------+ +-----------+

| A | | A |

+ - - - -|- - - - + + - - - -|- - - - +

| |

V V

INTRODUCTION                                             PAGE 1-3

OVERVIEW OF MSCP SUBSYSTEM

+------------------------------------------+

| Port | | Port |

| - - + + - - |

| Communications Mechanism |

| |

+------------------------------------------+

In  summary, an MSCP subsystem is characterized by an intelligent

controller that provides the host with the view of perfect media.

It  is further characterized by host independence from a specific

bus, controller, or device type.

1.2  PURPOSE

The purpose of this manual is to provide information on the rules

of MSCP to the detail necessary for both  writing  a  host  class

driver and implementing an MSCP server.

1.3  METHOD OF PRESENTATION

The method of presentation used in this manual is:

o  to define new terms and concepts.

o  list the responsibilities of the class driver.

INTRODUCTION                                             PAGE 1-4

METHOD OF PRESENTATION

o  list  the  responsibilities  of the MSCP server that are

applicable to the class driver.

o  list  the  responsibilities of the storage unit that are

applicable to the class driver.

o  explain each command and response message.

o  explain each error message.

o  provide appropriate tables of consolidated information.

1.4  SCOPE

The  scope  of  this manual is limited to the details of the MSCP

itself and does not provide information on any specific  type  of

host processor or any specific operating system.  It does not as-

sume any particular bus, controller, device type, host,  or  port

driver implementation.