CHAPTER 2

TERMINOLOGY

Aborted Command

From  the  viewpoint of a class driver, any command for which

it has issued an ABORT command.  From  the  viewpoint  of  an

MSCP  server,  a  command  for which it has received an ABORT

command, located the specified command,  and  taken  explicit

action  to  abort  it.  An aborted command will be either re-

jected or  terminated.   See  Section  section(cmdcatsexord),

"Command Categories and Execution Order."

Backing-store

In  a  cached  memory  system, the memory being served by the

cache.  This memory is usually much larger  and  much  slower

than  the cache memory and its performance is enhanced by the

cache between it and the user.  In the case  of  cached  mass

storage, the backing store is the disk or tape media.

Cache

A  fast  memory  placed between a larger slow memory known as

the backing-store and the user.  By retaining frequently  ac-

cessed  data  in  the  cache, the apparent performance of the

backing-store is improved.  A number of different  algorithms

may be used for the selection of data retained in the cache.

TERMINOLOGY                                              Page 2-2

Cache Consistency

The property that ensures that cached data behaves exactly as

if no cache existed.  That is, only  the  last  written  data

will  be  returned  to  users  and  in the case of write-back

cache, the last written data will be properly written to  the

backing-store even in the event of power failure.

Command Categories

All  MSCP  commands fall into one of four command categories:

Immediate commands, Non-Sequential commands, Sequential  com-

mands,  and Special commands.  Each command category has cer-

tain constraints on when those commands may be executed, thus

limiting  the scope of controller optimizations.  See Section

section(cmdcatsexord),  "Command  Categories  and   Execution

Order."

Completed Command

A  command  that  the MSCP server has executed through to its

normal completion, which may be either successful  completion

or    an    unrecoverable    error.     See    Section   sec-

tion(cmdcatsexord),   "Command   Categories   and   Execution

Order."

Controller Timeout Interval

A  time  interval,  measured in seconds, supplied by the con-

troller or MSCP server in the SET CONTROLLER  CHARACTERISTICS

command's end message.  Controllers or MSCP servers guarantee

that they will complete  all  Immediate  commands  plus  some

measurable  amount of useful work on their oldest outstanding

non-Immediate command within the controller timeout interval.

See Section section(command_timos), "Command Timeouts."

TERMINOLOGY                                              Page 2-3

Forced Error

A  data  error  (in  a disk block) that has been deliberately

caused by use of the "Force Error" command modifier.  Used to

indicate  that  the  data  in  the  block  is of questionable

validity.  For example, an unrecoverable error occurred  when

the data was copied from some other block.

Forced Error Indicator

The  logical flag, present in each disk block, used to record

the presence of a Forced Error.  Depending upon the detailed,

low  level format of the disk device, this may be implemented

either as an actual bit flag or as a special pattern (such as

the  complement  of  the  normal  value)  of error correcting

and/or error detecting codes.

Immediate Commands

Commands that MSCP servers shall execute immediately, without

waiting for any other commands to complete.   Immediate  com-

mands  are typically status inquiries, and shall be completed

within the controller timeout interval.

May

Denotes  a  lack of prohibition when used as a directive.  It

is used to indicate an option or options architecturally per-

missible  for  an implementation.  No bias for or against the

option is implied.  "May not" denotes strict prohibition when

used as a directive.

Non-Sequential Commands

Commands  whose execution order MSCP servers may rearrange in

order to optimize performance.  The  optimization  shall  not

move  a  Non-Sequential command past the barrier imposed by a

Sequential command.

TERMINOLOGY                                              Page 2-4

Nugatory

Of little or no consequence:  Trifling, Inconsequential.  See

Section section(available_cmd), "AVAILABLE Command."

Prefetch

The  act  of  reading data logically following requested data

during a data transfer command.  If the user access  patterns

frequently  access  the  data  following previously requested

data, performance improvement will result.

Rejected Command

A  command that the MSCP server rejects, discards, aborts, or

otherwise finishes before it begins to execute it.  See  Sec-

tion section(cmdcatsexord), "Command Categories and Execution

Order."

Sequential Commands

Commands  that  MSCP servers shall execute in the exact order

that they were received from class drivers.  Sequential  com-

mands typically change a unit's state or context.

Shall

Denotes an architectural requirement.  It is used as a direc-

tive to express what  is  mandatory  for  an  implementation.

"Shall not" denotes strict prohibition.

Should

Denotes  an emphatic recommendation when used as a directive.

It is used where "shall" might be too strong because  of  the

possibility  of  technical, implementation specific obstacles

or other overriding considerations.

TERMINOLOGY                                              Page 2-5

Special Commands

Commands  that  have  both the execution order constraints of

Non-Sequential commands plus certain special, command  depen-

dent execution order constraints.

Terminated Command

A  command  that the MSCP server terminates after it has been

partially executed.  See Section section(cmdcatsexord), "Com-

mand Categories and Execution Order."

Write-back Cache

A  cache  in  which  a write operation places the data in the

cache and the completion response is returned before the data

is written to the backing-store.

Write-through Cache

A  cache  in  which  a write operation places the data in the

cache and writes it to the backing-store before  the  comple-

tion response is returned.