CHAPTER 3

CLASS DRIVER / MSCP SERVER COMMUNICATIONS

3.1  CONNECTION

Host  class  drivers use the host port driver to communicate with

MSCP servers in controllers.  MSCP servers similarly use the con-

troller  port  driver to communicate with class drivers in hosts.

This communication takes place across a link called a connection.

The  state  of the connection is directly equivalent to the state

of the controller or  MSCP  server  with  respect  to  the  class

driver.  The controller is "Controller-Online" if and only if the

connection is established and  functioning.   The  controller  is

"Controller-Available"  if the connection is not established, but

it is believed that it could be established.  The  controller  is

"Controller-Offline"  if the connection is not established and it

is believed that it cannot be established.

Three  types  of communications services are used across the con-

nection between a class driver and an MSCP server:

o  A  sequential  message  communications service, used for

MSCP control messages -- i.e., command, end, and  atten-

tion  messages.   This  service  guarantees  sequential,

duplicate-free delivery for all messages sent across the

same  connection.   This service supports messages of at

least 48 bytes in length.

CLASS DRIVER / MSCP SERVER COMMUNICATIONS                PAGE 3-2

CONNECTION

o  A  datagram  communication  service, used for MSCP error

log messages.  This service delivers messages sent on it

with  very  high probability.  Messages may be delivered

out of sequence, lost, or duplicated, but the  probabil-

ity  of  any of these occurring shall be very low.  This

service supports messages of no more than 128  bytes  in

length.

o  A  block  data  communication service, used to move data

between hosts and mass storage controllers.   This  ser-

vice  provides a reliable, efficient method of transfer-

ring the contents of a named buffer in one subsystem  to

a  named buffer in another subsystem.  Buffers are iden-

tified by buffer descriptors, which  identify  both  the

buffer  and the subsystem (host) in which the buffer re-

sides.

The communications mechanism or port drivers discard all messages

that, at the time a connection is terminated, have been  sent  or

queued  to  be  sent via the sequential message and datagram ser-

vices but have not yet been delivered.  Block data transfers  may

or  may  not  be  terminated when a connection is terminated.  If

terminated, they  may  have  already  been  partially  completed.

Block  data transfers from a previous incarnation of a connection

are guaranteed to have been terminated  when  the  connection  is

re-established.

Besides  using these three communications services directly, MSCP

uses the establishment of the connection  itself  to  synchronize

class  drivers  and MSCP servers.  Either the class driver or the

MSCP server will terminate the connection  between  them  (become

"Controller-Available")  if  it  determines that they must resyn-

chronize with each other.  Events that  require  class  driver  /

MSCP  server  resynchronization include certain errors or loss of

context by either process.  The connection is also terminated, by

a  port  driver, if an unrecoverable communications error occurs.

Termination of the connection signals the processes  that  resyn-

chronization is necessary.  The resynchronization is accomplished

by each process discarding all context regarding outstanding com-

mands   or   transactions,   after  which  a  new  connection  is

established.

Following  resynchronization, commands which were outstanding be-

fore the resynchronization was performed may have completed to an

indeterminate  extent.   Such  commands  may  have  been rejected

(never started), may have been terminated (partially  completed),

or  may  have  been  fully completed.  The only guarantee is that

they are no longer outstanding, implying that the  controller  is

CLASS DRIVER / MSCP SERVER COMMUNICATIONS                PAGE 3-3

CONNECTION

no longer performing work for them and that the class driver will

not receive an end message for them.   The  fact  that  the  con-

troller  is  no  longer  performing work for them implies that no

state changes or modification of data will take place as a result

of such commands.

3.2  FLOW CONTROL

Especially  critical  to  MSCP is the concept of flow control and

the flow control based requirements that MSCP  imposes  on  class

drivers and MSCP servers.  These items are discussed below.

Flow  control arises from the need to avoid the congestion and/or

deadlock which can  occur  if  one  process  sends  messages  too

quickly  to  another  process.   The  receiving process must have

buffers in which to place the incoming messages.  When  all  such

buffers are full, additional messages cannot be handled.

The  datagram  communications  service does not use flow control.

If no buffers are available,  incoming  datagrams  will  be  dis-

carded.   Thus  the characteristic that the datagram service does

not guarantee delivery, instead only assuring a high  probability

of  delivery.   This  high  probability is dependent upon the re-

ceiver (i.e., the class driver) always having buffers queued  for

incoming datagrams.

The  sequential message communications service does use flow con-

trol.  When a potential receiving process queues a buffer for re-

ceiving  messages on a connection, the presence of this buffer is

communicated (via the underlying communications service)  to  the

potential  sending  process  at  the other end of the connection.

This message notifying  the  potential  sending  process  of  the

queued  buffer  grants the sending process a credit, which is the

privilege to send a message.  Therefore  messages  will  only  be

sent  when  the  sending process knows that the receiving process

has queued a buffer into which the message can be  received,  en-

suring that the receiving process will be able to handle the mes-

sage.

A typical implementation of flow control will be somewhat as fol-

lows.  The port driver maintains a counter on behalf of each pro-

cess  participating  in  a  connection.   That  counter holds the

process's current credit balance -- i.e., the number  of  receive

buffers  that  its partner has queued less the number of messages

CLASS DRIVER / MSCP SERVER COMMUNICATIONS                PAGE 3-4

FLOW CONTROL

that it has sent.  Every time the process's partner queues a  re-

ceive  buffer,  a  message  is sent causing the counter to be in-

cremented.  Every time the process sends a message,  the  counter

is  decremented.   Messages  may  only  be  sent when the counter

(credit balance) is greater than zero, thus guaranteeing that the

counter  will  never be negative.  Indeed, we will see later that

some messages require that the counter be greater than  a  thres-

hold larger than zero.

Due to the inherent asynchrony of communications between multiple

processes or  subsystems,  revoking  or  canceling  a  previously

queued  receive  buffer  is  not straightforward.  The problem is

that the buffer cannot be revoked and returned to  the  receiving

process  until after the sending process has acknowledged its re-

vocation, as otherwise the sending process may attempt to send  a

message  that  requires  the  revoked  buffer.  Therefore the al-

gorithm for revoking receive buffers is as follows:

1.  The  revoking  process  (the  process  which  originally

queued the receive buffers) requests that some number of

buffers be revoked.

2.  The  revocation  request is communicated to the revoking

process's partner (actually to its port driver).

3.  The   revoking  process's  partner  (actually  its  port

driver) compares the number of  buffers  to  be  revoked

against  its current credit balance and a threshold.  If

the requested number of buffers / credits can be revoked

(i.e.,  subtracted  from  the  credit  balance)  without

lowering the credit balance below  the  threshold,  then

all  of  them will be revoked.  Otherwise, if the credit

balance is above the threshold, it is set to the  thres-

hold and the difference between its former value and the

threshold is the number of buffers  /  credits  actually

revoked.   If  the credit balance is already at or below

the threshold, then it stays the same and no  buffers  /

credits are revoked.

4.  The  actual  number of buffers / credits revoked is com-

municated back to the revoking process's port driver.

5.  The  revoking  process's port driver returns the buffers

actually revoked to the revoking process.

If a threshold of zero is used, the revoking or receiving process

can always get back all of its buffers.  The  fact  that  an  at-

tempted  revocation  failed implies that the buffers have already

CLASS DRIVER / MSCP SERVER COMMUNICATIONS                PAGE 3-5

FLOW CONTROL

been returned to the process, since messages have  been  received

into them.

The  above algorithm uses a threshold to prevent revocation below

some lower limit.  The mechanism by which this threshold  is  ob-

tained  is not critical to either MSCP or to the above algorithm.

The rules below are phrased as if the threshold were supplied  by

the  revoking process as part of the revocation request.  This is

purely to simplify the wording of those rules;  often the  thres-

holds   will   be  constants  determined  when  a  connection  is

established.  In such  an  implementation  a  threshold  of  zero

should  be  used when the class driver is revoking credits it has

granted to the MSCP server and a threshold of one should be  used

when  the  MSCP  server is revoking credits it has granted to the

class driver.

MSCP  is  only  concerned  with credits required by the sequenced

message  service.   Some  communications  services  may   require

credits  for  the  block data communication service as well.  Any

such credits are invisible to MSCP, being communications  service

dependent,  and  shall be provided in addition to the credits re-

quired by the rules below.

Note  that  the  above  discussion  merely describes a conceptual

model for flow control within the sequential  message  communica-

tions  service.   There  is  no requirement that flow control ac-

tually be implemented this way, provided that the results are the

same.   For example, almost all implementations will carry credit

information in a header added to messages and  processed  by  the

receiving   process's  port  driver,  rather  than  communicating

credits with separate messages.  Some extremely well behaved com-

munications  mechanisms  may  not need to implement explicit flow

control at all, since the underlying communications mechanism may

provide it implicitly.

3.2.1  Class Driver Responsibilities

Given the above model for flow control, we can state the require-

ments MSCP places on  class  drivers  and  MSCP  servers.   Class

drivers shall obey the following rules:

CLASS DRIVER / MSCP SERVER COMMUNICATIONS                PAGE 3-6

FLOW CONTROL

1.  All  MSCP  commands fall into one of several categories;

for this discussion  we  distinguish  between  Immediate

commands    (one    specific   command   category)   and

non-Immediate commands (the union of all  other  command

categories).   When the class driver's credit balance is

zero, the class driver may not issue any commands.  When

it  is  one,  the  class driver may only issue Immediate

commands.  When it is two or larger,  the  class  driver

may issue both Immediate and non-Immediate commands.  If

the class driver's credit balance is one and there is  a

GET  COMMAND STATUS command waiting to be issued for the

command timeout algorithm, then the class  driver  shall

issue  that  GET COMMAND STATUS command as the next com-

mand.

In  essence, this rule means that the class driver shall

reserve one credit for the exclusive use of the  command

timeout  algorithm.   This  credit may be "borrowed" for

issuing Immediate commands, since such  commands  always

complete  quickly.   The  goal  is to guarantee that the

command  timeout  algorithm  will  always  be  able   to

promptly issue a GET COMMAND STATUS command.

2.  The  class  driver shall queue a receive buffer for each

command that it sends to an MSCP  server.   The  receive

buffer  will  be used to hold the command's end message.

The receive buffer shall be  queued  either  before  the

command  is  sent  or as part of an atomic (indivisible)

action that includes sending the command.  The important

point  is  that  the MSCP server must receive the credit

for the receive buffer  either  before  or  concurrently

with receiving the command.

3.  In addition to queuing receive buffers for end messages,

class drivers that enable attention messages shall queue

at  least  one receive buffer in which to receive atten-

tion messages.  Such a receive buffer  shall  be  queued

before  the  class  driver enables attention messages --

i.e., before the class driver  sends  a  SET  CONTROLLER

CHARACTERISTICS command that enables attention messages.

Additional receive buffers may be queued  at  any  time.

Note that, with most controllers, there is no benefit to

queuing more than one attention message receive buffer.

CLASS DRIVER / MSCP SERVER COMMUNICATIONS                PAGE 3-7

FLOW CONTROL

4.  Upon  receiving  an  attention message, the class driver

shall immediately queue another (or  the  same)  receive

buffer  back  for  more  attention  messages.   The only

resource that the class driver may require  between  re-

ceiving an attention message and queuing another receive

buffer is host CPU cycles.  The class driver  shall  not

require  that  it  be  able to send or receive any other

messages or wait for any I/O to complete before  queuing

another  receive buffer.  This effectively requires that

all code and data structures needed to process attention

messages  be permanently resident in physical memory, so

that an incoming attention message  can  be  immediately

processed  and  the  buffer in which it was received im-

mediately requeued.

5.  With  one exception, the class driver shall never revoke

receive buffers.  The one exception is  after  disabling

attention messages -- i.e., after receiving the end mes-

sage for the SET CONTROLLER CHARACTERISTICS command that

disabled  attention  messages.   At  that time the class

driver may revoke as many buffers as it has  queued  for

attention messages (i.e., total number of buffers queued

less number of outstanding commands).   This  revocation

should  specify a threshold of zero.  Note that this re-

vocation is guaranteed to succeed if the MSCP server  is

operating correctly.

Note  that the class driver can accomplish the effect of

revoking end message receive buffers  by  aborting  MSCP

commands.   The  receive buffers will be returned to the

class driver as soon as the (aborted) commands complete.

Failure  of  a class driver to follow the above rules may lead to

controller deadlock, command  timeouts,  or  the  controller  not

obeying  its rules given below.  Any connection or process in the

controller's subsystem may be affected, rather than just the MSCP

server  with  which the class driver is communicating.  Note that

class drivers that enable error logging must also  keep  datagram

buffers  queued so that they can receive error log messages.  Not

keeping datagram buffers queued may result in loss of  error  log

CLASS DRIVER / MSCP SERVER COMMUNICATIONS                PAGE 3-8

FLOW CONTROL

messages.

3.2.2  MSCP Server Responsibilities

The rules for MSCP servers vary depending upon certain character-

istics of the server.  There is one general set of rules, plus  a

simplification  that  certain  classes of servers may follow.  In

all cases, an MSCP server's implementation of these rules may re-

quire,  for  its  correct  operation (and thus adherence to these

rules), that class drivers correctly follow the rules  given  for

them above.  The general set of rules, which shall be followed by

all MSCP servers that are not in any of the special cases identi-

fied later, are as follows:

1.  So  long as it is "Controller-Online" to a class driver,

an MSCP server shall ensure that the sum of  the  number

of  commands that are outstanding from that class driver

plus the number of unused receive buffers / credits that

it  has granted to that class driver is never lower than

the values given below.  That is, the sum of the  actual

and potential outstanding commands shall be at least the

values below.  Between the time that the controller  be-

comes  "Controller-Online"  and  the  completion  of the

first SET CONTROLLER CHARACTERISTICS command,  this  sum

shall  be at least one.  Following the completion of the

first SET CONTROLLER  CHARACTERISTICS  command,  and  so

long as the controller remains "Controller-Online," this

sum shall be at least two.  The first unit of  this  sum

allows  the  class  driver  to issue Immediate commands.

Any excess, beyond the value one, may be used  to  issue

"real" commands such as data transfers.  Note that these

requirements imply  that,  following  a  SET  CONTROLLER

CHARACTERISTICS  command,  the  MSCP server shall either

grant a minimum of two receive buffers / credits to  the

class  driver  or  else terminate the connection (become

"Controller-Available") with the class driver.

2.  So  long as it is "Controller-Online" to a class driver,

an MSCP server shall ensure that the sum of  the  number

of  Immediate  commands  that  are outstanding from that

class driver plus the number of unused receive buffers /

credits  that  it  has  granted  to that class driver is

CLASS DRIVER / MSCP SERVER COMMUNICATIONS                PAGE 3-9

FLOW CONTROL

always at least one.  That is, the sum of the actual and

potential  outstanding  Immediate  commands  shall be at

least one.  This is in addition to requirement 1 above.

3.  An  MSCP server may revoke receive buffers or credits at

any time so long as it continues to meet requirements  1

and 2 above.  Note that, in order to meet requirement 2,

the sum of the threshold used  for  the  revoke  request

plus  the number of outstanding Immediate commands (from

that class driver) must be at least one.  This  restric-

tion  will  typically be met by always using a threshold

of one.

4.  Notwithstanding  all  that  has  been  said  above, MSCP

servers shall allow hosts to perform  transfers  without

the  host  having to issue a SET CONTROLLER CHARACTERIS-

TICS command.  This is necessary  to  simplify  the  im-

plementation   of  host  bootstraps.   The  commands  to

perform such transfers shall be issued  one  at  a  time

(rule 1 above ensures that the host has at least the one

credit it needs to  issue  these  commands).   The  MSCP

server  shall  either  grant  the  host a minimum of two

credits when it becomes "Controller-Online" or else per-

form  commands  regardless  of the fact that the host is

nominally violating the requirements described  in  Sec-

tion      section(classdrvrresp),      "Class     Driver

Responsibilities." The specific requirement violated  is

that  the host will be issuing non-Immediate commands in

spite of its credit balance being only one.

5.  An  MSCP  server shall keep track of the excess, if any,

of its credit balance over  the  number  of  outstanding

commands.   The server may only issue attention messages

when this excess is greater than zero.   Attention  mes-

sages  that cannot be issued immediately should be saved

until credits are available with which  to  issue  them.

The  controller  shall  continue to accept new commands,

process outstanding commands, and issue end messages for

completed  commands  while  attention messages are being

saved.  If the conditions that triggered the  generation

CLASS DRIVER / MSCP SERVER COMMUNICATIONS               PAGE 3-10

FLOW CONTROL

of  an attention message disappear before that attention

message can be issued (sent), then the attention message

may  or  may not still have to be sent.  See the indivi-

dual attention message descriptions contained in Section

section(atten___mess), "Attention Messages."

With  many  communications mechanisms there is an inherent or un-

avoidable race that MSCP servers must be prepared to resolve.  It

can occur whenever the credit for an end message might be sent as

a distinct message, rather than always being embedded in the com-

munications  mechanism  header for the corresponding command mes-

sage.

Suppose  the  host  class  driver grants one credit for attention

messages, and that some combination of  events  causes  the  MSCP

server  to  want  to  send two attention messages.  The attention

message credit is used to send the first  attention  message  and

the  second  is  queued  for  later transmission.  Meanwhile, the

class driver sends a  command  and  its  associated  end  message

credit  to  the  MSCP  server  as two distinct messages.  Per the

class   driver   requirements   described   in    Section    sec-

tion(classdrvrresp), "Class Driver Responsibilities," the notifi-

cation of the end message credit must be sent first.   When  this

notification arrives at the controller, the MSCP server thinks it

is the attention message credit being returned (since credits are

anonymous)  and  sends the second attention message.  Immediately

thereafter the command arrives and the MSCP server  discovers  an

apparent  rule  violation by the class driver, namely that it ap-

pears to have sent a command without an accompanying  credit  for

the end message.

This  credit  imbalance  problem will ultimately be resolved when

the class driver returns the attention message  credit.   As  the

class  driver  is not allowed to perform any I/O before returning

attention message credits, we are assured that the credit will be

returned  both  promptly and without any possibility of deadlock.

Therefore, whenever this race occurs, the MSCP  server  may  hang

until  the  attention message credit is returned.  Although it is

permissible for all MSCP servers for all device classes and  con-

nections  to hang, it is highly desirable if only the one connec-

tion is affected.

Typically  the  command(s)  that  is(are)  lacking an end message

credit would be held in a queue, with no work done on them  until

their  end message credits arrive.  Note that if the host did not

return attention message credits promptly, this would  result  in

command  timeouts.   The  MSCP  server  shall service the waiting

commands in their order of  arrival.   Thus  if  another  command

CLASS DRIVER / MSCP SERVER COMMUNICATIONS               PAGE 3-11

FLOW CONTROL

arrives  with  a  credit embedded in its communications mechanism

header, the MSCP server shall transfer that credit to the  oldest

command  waiting for an end message credit and place the new com-

mand on the waiting queue.

MSCP servers that limit the rate at which they generate attention

messages can replace rule 5 above with a simpler rule.  This  al-

ternate  rule  may  be  used, at the server's option, by any MSCP

server that will generate no more than an average of  two  atten-

tion  messages  per  second, averaged over the controller timeout

interval   (see    Section    section(command_timos),    "Command

Timeouts").   That  is,  if  the controller timeout interval is N

seconds, then the server will generate no more than N*2 attention

messages within any N second period.  The MSCP server may assume,

in determining its maximum attention  message  rate,  that  human

operators  do  not engage in pathological activity.  I.e., it may

assume that cases such as an operator continuously actuating  the

Run/Stop  switches  on  one or more drives will never occur.  Any

MSCP server that can meet this attention message rate restriction

may substitute the following alternate rule for rule 5 above:

5'. An MSCP server may issue attention messages and end mes-

sages whenever its credit balance is greater than  zero.

Attention  messages  and end messages that cannot be is-

sued immediately  should  be  saved  until  credits  are

available  with  which to issue them.  If the conditions

that triggered the generation of  an  attention  message

disappear  before  the  attention  message can be issued

(sent), then that attention message may or may not still

have  to  be sent.  See the individual attention message

descriptions contained in Section section(atten___mess),

"Attention Messages." Saved end messages shall always be

sent,   unless   the   MSCP   server    first    becomes

"Controller-Available"  (i.e., terminates its connection

with the class driver), in which case the saved end mes-

sages  shall  not be sent.  Note that end messages shall

always be sent in the  order  that  their  corresponding

commands completed.

An MSCP server may deadlock or cease operating on a con-

nection whenever it has saved attention messages or  end

messages  waiting  to be issued on that connection.  The

only operations that it is required to perform  when  it

has  such messages waiting are to accept incoming credit

notifications, send the waiting  messages  when  credits

become  available,  and resume normal operation when all

waiting messages have been sent.   Note  that  accepting

incoming  credit  notifications  will often require that

CLASS DRIVER / MSCP SERVER COMMUNICATIONS               PAGE 3-12

FLOW CONTROL

the MSCP server also accept new  commands,  although  it

need  not  begin  processing of those new commands until

all waiting messages have been  sent.   Note  also  that

only   the   one  MSCP  server  may  deadlock  or  cease

operating;  other processes (i.e., other  MSCP  servers)

in  the  same  subsystem or controller and other connec-

tions to the same MSCP server shall not be affected.

The  effect  of  this  modified  rule is to allow MSCP servers to

"borrow" end message credits for the purpose of sending attention

message credits.

3.3  MULTIHOST COMMUNICATION

Chapter    chapter(docintroduction),    "Introduction,"    Figure

figure(example_system), "Example System," shows a  "Single  Host"

system,  where a single host is connected to a single controller.

Certain communications mechanisms permit  communications  between

multiple  entities (nodes) interconnected along the same physical

path.  With such a communications mechanism  it  is  possible  to

create  a  "Multihost" system, where multiple hosts are connected

to a single controller.

Controllers  which  support  multinode  communications mechanisms

will usually (not an absolute requirement of MSCP) implement  the

Multihost  Support  option  (see Section section(controlerflags),

"Controller Flags").  Multihost Support allows simultaneous  com-

munication  between  multiple  host  class  drivers  and  an MSCP

server.  The communication takes place  across  separate  logical

connections  established  between  the class drivers and the MSCP

server.  The MSCP server shall treat each connection with a class

driver  as  a  separate  and  unique entity, with each connection

obeying all the communication rules  detailed  in  the  preceding

sections of this chapter.  New connections may be accepted or re-

jected without regard  to  the  number  of  existing  connections

(other  than  resource  limitations), and no notification will be

given to the new connection as to  the  presence  or  absence  of

other  connections.   Class driver/server synchronization is on a

per connection basis, and the loss of synchronization  between  a

class  driver  and the controller on a specific connection should

not affect any other connections which may be  open,  unless  all

connections  are  affected  and a hard initialization of the con-

troller is required.

CLASS DRIVER / MSCP SERVER COMMUNICATIONS               PAGE 3-13

MULTIHOST COMMUNICATION