CHAPTER 4

ALGORITHMS AND USAGE RULES

4.1  CONTROLLER STATES

The  controller  may be in any of three states relative to a host

class driver.  The controller may be in a different  state  rela-

tive  to  each  host or each class driver.  The controller states

are:

Controller-Offline

A  controller  is  "Controller-Offline"  to  a  class  driver

whenever it is not available to that class driver and  cannot

perform  any  operations  on its behalf.  Possible causes in-

clude inoperative hardware or an operator disabling the  con-

troller.   A  controller is "Controller-Offline" exactly when

it is not possible to  establish  a  connection  between  the

class driver and the MSCP server within the controller.  Note

that a controller may be "Controller-Offline" to  some  of  a

host's   class   drivers  yet  be  "Controller-Available"  or

"Controller-Online" to others.

Controller-Available

A  controller  is  "Controller-Available"  to  a class driver

whenever it could perform operations for that  class  driver,

but  the driver has not yet synchronized with the controller.

ALGORITHMS AND USAGE RULES                               PAGE 4-2

CONTROLLER STATES

A controller is "Controller-Available" exactly when it  would

be  possible  to  establish  a  connection  between the class

driver and the MSCP server within the controller, but no con-

nection has yet been established.

Controller-Online

A   controller  is  "Controller-Online"  to  a  class  driver

whenever it can both perform operations for that class driver

and  the driver has synchronized with the controller.  A con-

troller is "Controller-Online" exactly when a connection  ex-

ists  between the class driver and the MSCP server within the

controller.  This is the state used for normal operation.

Strictly  speaking,  the  term  "controller state" is a misnomer.

The states described above actually exist between  an  individual

class  driver  and  an  individual  MSCP server.  A host may have

several class drivers and a controller subsystem may have several

MSCP  servers  (usually  one  for  each  device class supported).

Furthermore, controllers that  provide  Multihost  Support  allow

multiple  connections  between  its  server(s) and multiple class

drivers resident in one or more hosts.

Note  also that the controller state (MSCP server state?) is dis-

tinct from the state of any unit connected to the controller.

An MSCP server (controller) enters the "Controller-Offline" state

relative to a host whenever the MSCP server ceases to function or

otherwise  becomes  unable  to  perform  operations for the host.

Possible causes include:

1.  Controller hardware, software, or power failure.

2.  Controller initialization, either requested or spontane-

ous.

3.  An  operator  (typically  Field Service) disables all or

part of the controller.

4.  Communications mechanism failures.

5.  The controller has not been built yet, the controller is

still in its shipping crate, or it has otherwise not yet

been installed.

ALGORITHMS AND USAGE RULES                               PAGE 4-3

CONTROLLER STATES

An  MSCP  server enters the "Controller-Available" state relative

to a host class driver when:

1.  The  controller  or MSCP server is "Controller-Offline,"

and all causes  of  it  being  "Controller-Offline"  are

removed.

2.  The  MSCP  server  is  "Controller-Online," and the MSCP

server cannot successfully send a control message (i.e.,

an  MSCP  end  or  attention  message) to the host class

driver.

3.  The MSCP server is "Controller-Online," and the host ac-

cess  timeout  expires.   See  Section   section(hostac-

cestimo), "Host Access Timeouts."

4.  The  MSCP  server  is  "Controller-Online," and the MSCP

server receives an "Invalid Command" (see "Invalid  Com-

mand"  status in Section section(status__codes), "Status

Codes") from the host.  Note  that  this  transition  to

"Controller-Available"  is  optional, and therefore con-

troller dependent.

5.  The  host class driver terminates the connection between

the class driver and the MSCP server.

6.  A port driver or the communications mechanism terminates

the connection between the class  driver  and  the  MSCP

server, generally due to a communications error.

7.  An  MSCP server terminates the connection between itself

and a host class driver as a result of the execution  of

a  TERMINATE  CLASS  DRIVER/SERVER CONNECTION command as

described in Section section(termcdsconmcmd), "Multihost

TERMINATE CLASS DRIVER/SERVER CONNECTION Command."

The  port driver should inform the class driver whenever the MSCP

server enters the "Controller-Available"  state.   How  the  port

driver  obtains  this  information  is  communications  mechanism

dependent.  Note that the notification that  the  controller  has

become  "Controller-Available"  is  not  necessarily  prompt.  In

particular, with some communications mechanisms the  notification

may  not occur until the next time the class driver issues a com-

mand to the controller.  Furthermore, the port  driver  need  not

notify  the class driver at all if a compound (multiple) error is

associated      with      the      MSCP      server      becoming

"Controller-Available." In  such a case the class driver will ul-

timately become aware  of  the  state  change  when  its  Command

Timeout  expires  (see  Section  section(command_timos), "Command

ALGORITHMS AND USAGE RULES                               PAGE 4-4

CONTROLLER STATES

Timeouts").

Since   no   connection   exists   to  an  MSCP  server  that  is

"Controller-Offline" or  "Controller-Available,"  the  communica-

tions  mechanism will either reject or discard any messages (com-

mands) that a class driver attempts  to  send  to  it.   An  MSCP

server       that       becomes      "Controller-Offline"      or

"Controller-Available" may either terminate commands in  progress

or  else continue processing the commands that it has already re-

ceived.  However, if a Sequential command from a given connection

is   terminated  or  rejected,  then  all  subsequently  received

non-Immediate commands from the same connection shall also be re-

jected (i.e., shall never begin processing).

Typically,  the  MSCP server will continue processing outstanding

commands until it "notices" that  the  connection  to  the  class

driver  has been terminated, at which point it will terminate any

commands still outstanding and reject any commands  that  haven't

begun  processing yet.  Note that the MSCP server shall guarantee

that all outstanding commands have been completed,  aborted,  re-

jected,  or  terminated (i.e., that there are no outstanding com-

mands)    before    it    completes     a     transition     from

"Controller-Available" to "Controller-Online."

The  MSCP server enters the "Controller-Online" state relative to

a host class driver  upon  successful  synchronization  with  the

class driver.  The class driver synchronizes with the MSCP server

by establishing a connection with the MSCP server.  Note that the

MSCP  server  shall  guarantee that there are no outstanding com-

mands "leftover" from a previous incarnation  of  the  connection

before  it  allows  the  new  incarnation of the connection to be

established and enters the "Controller-Online" state.

4.2  CONTROLS AND INDICATORS

All storage units used with MSCP must have the following controls

and indicators:

o  Unit number select mechanism.

ALGORITHMS AND USAGE RULES                               PAGE 4-5

CONTROLS AND INDICATORS

o  Unit number display mechanism.

o  Run/Stop  switch (on disk units) or a Load/Unload switch

(on tape units).

o  Write protect switch or mechanism.

o  Write protect status indicator.

The unit number select mechanism on an MSCP storage unit shall be

capable of specifying any unit number in the range 0 through  251

inclusive.   Larger unit number ranges are acceptable, so long as

they include the range 0 through 251.   The  unit  number  select

mechanism  shall operate without host intervention and shall pre-

serve unit numbers across power failures and other losses of con-

text.

Alteration  of  the  unit  number  must be possible in the field.

That is, the unit number shall be  alterable  by  Field  Service,

both  when  a  device  is first installed and subsequently when a

system is reconfigured.  The preferred unit select mechanism is a

removable  unit  number  plug, allowing the unit number to be al-

tered by users as well as by Field Service.  Alternatives to unit

number  plugs, however, are acceptable so long as they can be al-

tered by Field Service and they provide the full  0  through  251

unit number range.

A  single unit number select mechanism may be shared by the units

of a multiunit  drive  or  formatter  (see  Section  section(mul-

tidrvsfmts),  "Multiunit  Drives  and  Formatters").   Units that

share a unit number select mechanism shall always  have  consecu-

tive  unit  numbers.   If  exactly  two units share a unit number

select mechanism, it is acceptable for the first unit  to  always

have  an even unit number and the last unit to always have an odd

unit number.  If exactly N  units  share  a  unit  number  select

mechanism,  it  is acceptable for the first unit's unit number to

ALGORITHMS AND USAGE RULES                               PAGE 4-6

CONTROLS AND INDICATORS

always be a multiple of N, the next unit's unit number to  always

be a multiple of N plus 1, etc.

The  unit  number display mechanism on an MSCP storage unit shall

display the unit number(s) specified by the  unit  number  select

mechanism.   The  display  shall  be visible to normal (non-Field

Service) human operators.  If the unit number select mechanism is

a removable unit plug, then the unit number display mechanism may

merely be a number printed on the plug.  If several units share a

unit  number  select  mechanism,  then they may also share a unit

number display mechanism.

The  Run/Stop  switch or Load/Unload switch shall be alterable by

normal (non-Field Service) human operators to allow  or  disallow

host  access  to  the unit(s).  When in the Run or Load position,

this switch indicates that hosts should be allowed to access  the

unit(s).   When in the Stop or Unload position, this switch indi-

cates that hosts should not be allowed to access the unit(s).  If

the  unit(s) have removable media, the switch also indicates that

human operators should be allowed  to  remove  the  units'  media

(i.e.,  that  the  unit  should be spun-down).  A single Run/Stop

switch may be shared by any units of a multiunit drive that share

a spindle (i.e., that must be spun-up and spun-down together).

The write protect switch or mechanism shall either be an operator

accessible switch or else some kind of mechanical deformation  of

the  media, such as a tape write-ring or the write-lockout tab on

a cassette.  In either case, it  shall  be  alterable  by  normal

(non-Field  Service)  human  operators.  A separate write protect

switch or mechanism shall be provided for each unit of  a  multi-

unit drive.  When actuated, the write protect switch or mechanism

prevents write access to the unit by hosts and  controllers.   In

the  case  of a write protect switch, the transition to the write

protected state shall be "smooth"  rather  than  immediate.   See

Section section(write_protect), "Write Protection."

The  write  protect  status  display  mechanism shall display the

write protect status of  the  unit.   A  separate  write  protect

status  display  mechanism  shall  be provided for each unit of a

multiunit drive.  The write protect status display shall be  user

visible while a volume is mounted in the unit.  That is, the user

shall not be required to remove  the  volume  from  the  unit  to

determine whether or not it is write protected.

The  preferred write protect status display mechanism is a light,

located within the write protect switch, that is on whenever  the

unit  is  write  protected.  The light shall be lit regardless of

the reason for the unit being write protected -- i.e.,  it  shall

be  lit  when the unit is Software Write Protected or Data Safety

Write Protected as well  as  when  the  unit  is  Hardware  Write

ALGORITHMS AND USAGE RULES                               PAGE 4-7

CONTROLS AND INDICATORS

Protected  due  to  its  write protect mechanism being activated.

See Section section(write_protect), "Write Protection."

4.3  UNIT STATES

Each  unit  may  be in one of three states relative to each class

driver that is "Controller-Online" to an MSCP server.  (Actually,

it  is really each unit number that these states apply to, rather

than to a unit proper.) Each unit may be in a different state re-

lative  to  each  "Controller-Online"  class driver.  Controllers

shall therefore maintain the unit state separately for each  con-

nection.  Note that a unit's state is meaningless with respect to

a class driver that is not "Controller-Online" to the MSCP server

or  when  no  class  driver  is  "Controller-Online"  to the MSCP

server.

The three unit states are:

Unit-Offline

A  unit  is  "Unit-Offline"  whenever it is unable to satisfy

normal host requests.  Except for status queries,  MSCP  com-

mands addressed to a unit that is "Unit-Offline" shall be re-

jected.  Furthermore, many device characteristics may not  be

available to status queries.

Unit-Available

A  unit  is "Unit-Available" whenever it would be able to sa-

tisfy normal host requests, except that the host has not  yet

issued an ONLINE command to bring the unit "Unit-Online."

Unit-Online

A unit is "Unit-Online" whenever it is able to satisfy normal

host requests and the host has  issued  a  successful  ONLINE

command.

A  fourth  unit  state, actually a pseudo-state, exists with con-

trollers that provide Multihost Support.  Each MSCP  server  con-

tained  in  such  a  controller  shall  implement  the "Exclusive

Access" feature, which allows  a  single  host  class  driver  to

ALGORITHMS AND USAGE RULES                               PAGE 4-8

UNIT STATES

reserve a unit for its exclusive use.  The host that has obtained

"Exclusive Access" of a unit has complete control  of  that  unit

and  may perform all normal unit operations.  All other hosts are

prevented from performing any  operation  that  would  alter  the

state or context of the unit in any manner.  The other hosts are,

however, permitted to obtain the unit's characteristics as  main-

tained  by  the  controller  via  all  commands that return those

characteristics.  While a unit is under the "Exclusive Access" of

a host the controller shall prevent any other controller to which

the unit may be connected from bringing that unit or  some  other

unit  with  which it shares an access path online with respect to

that controller (see Sections section(multiaccesdrvs),  "Multiac-

cess  Drives"  and  section(multidrvsfmts), "Multiunit Drives and

Formatters").  "Exclusive Access" is termed  a  pseudo-state  be-

cause that context is maintained solely by the controller without

the knowledge nor direct participation of the unit.  The state of

the  unit with respect to the host that has "Exclusive Access" is

totally dependent on the operations that host has requested:   it

may   be   "Unit-Online,"   "Unit-Available,"  or  "Unit-Offline"

(provided the unit  remains  operative  and  known  to  the  con-

troller).   The  unit is "Unit-Offline" with respect to all other

hosts.  Host control of "Exclusive Access"  is  provided  through

the  operation  of the AVAILABLE, ONLINE or SET UNIT CHARACTERIS-

TICS commands.  See the description of those  commands  contained

in  Chapter chapter(multihostfunct), "Multihost Support" for more

details.

The  "Unit-Offline"  state  has  seven  substates, related to the

exact reason for the unit being  "Unit-Offline." These  substates

are:

1.  Unit  inoperative.   Some  fatal  error condition in the

drive prevents the unit from  becoming  "Unit-Available"

or "Unit-Online."

2.  Unit  disabled.   Field  Service  or  a  diagnostic  has

decided that continued operation  of  the  unit's  drive

will  lead  to  progressive  deterioration  and eventual

destruction of some portion of the drive or media.   The

unit  has  been  disabled  to  prevent its use, and con-

sequent destruction, until Field Service can repair  the

problem.   This  cause  of the unit being "Unit-Offline"

can be overridden, and the unit  brought  "Unit-Online,"

by  use  of the "Allow Self Destruction" modifier to the

ONLINE command.  Note that some devices may have no  way

of detecting progressive deterioration, and consequently

will never enter this substate.

ALGORITHMS AND USAGE RULES                               PAGE 4-9

UNIT STATES

3.  Unit  known.   The  controller  knows that the specified

unit (i.e., a unit with the specified unit  number)  ex-

ists,  but  the  unit  is "Unit-Offline" for some normal

(nonerror) condition.  Typical causes of  this  substate

include  the unit's Run/Stop or Load/Unload switch being

in the Stop  or  Unload  position  or  no  volume  being

mounted in the unit.

4.  Under exclusive use of another host.  The specified unit

exists  and  would  be  "Unit-Available,"  except   that

another  host connected via the same controller has been

granted "Exclusive Access" of the unit.

5.  Online to another controller.  The specified unit exists

and would be "Unit-Available," except that  it  or  some

other  unit  with  which it shares an access path (i.e.,

some other unit on the  same  multiunit  drive  or  for-

matter)  is  "Unit-Online"  to  one  or more hosts or is

under the "Exclusive Access" of a host via another  con-

troller.  The unit will become "Unit-Available" via this

controller if it and all units with which it  shares  an

access  path  cease  being "Unit-Online" to any hosts or

under the "Exclusive Access" of a  particular  host  via

that  other  controller.  In terms of the "Unit-Offline"

substate that is visible  to  and  reported  by  a  con-

troller,  a unit that is actually online to another con-

troller will typically oscillate between the "Online  to

another controller" substate and the "Unit unknown" sub-

state.  The frequency of the oscillation  is  determined

by  the  frequency at which hosts issue DETERMINE ACCESS

PATHS commands to the other controller for this unit  or

any  unit with which it shares an access path.  See Sec-

tions section(multiaccesdrvs), "Multiaccess Drives"  and

section(multidrvsfmts),   "Multiunit   Drives  and  For-

matters" for more information.

6.  Duplicate  unit  numbers.  That is, two or more distinct

units have the same unit number assigned to them.   Con-

trollers  shall  check for duplicate unit numbers across

all  units  of  the   same   device   class   that   are

"Unit-Online," that are "Unit-Available," or that are in

one of the  following  "Unit-Offline"  substates:   unit

disabled,  unit  known,  under  exclusive use of another

host, duplicate unit numbers, or online to another  con-

troller.   A  controller  may or may not, at its option,

include inoperative units when  checking  for  duplicate

unit  numbers.  Controllers may not check units that are

unknown (as described below) nor may they check for  du-

plicate  unit  numbers  across different device classes.

Note that a duplicate unit number does not affect a unit

ALGORITHMS AND USAGE RULES                              PAGE 4-10

UNIT STATES

that is already "Unit-Online" to or under the "Exclusive

Access" of a host.

7.  Unit  unknown.   As far as the controller can determine,

no unit exists with the specified unit number.

It  is  possible for a unit to be "Unit-Offline" for several rea-

sons at the same time (unless the unit is unknown).   If  a  unit

has a duplicate unit number and is not inoperative, then the con-

troller shall report the duplicate unit number;  other causes  of

the  unit  being  "Unit-Offline"  may  also  be  reported  at the

controller's option.  In all other  cases  the  controller  shall

report  at  least one cause of the unit being "Unit-Offline," but

which one it reports and whether or not it reports more than  one

is optional with the controller.

The fact that a unit is inoperative may not be detectable until a

host attempts to bring the unit "Unit-Online." Such units will be

treated  as and appear to be "Unit-Available" until a host issues

an ONLINE command.  The ONLINE command will fail, typically  with

a  "Drive  Error" status code.  At this time either the fact that

the unit is inoperative shall be recorded in the unit  itself  or

else  AVAILABLE  attention  messages  shall be suppressed for the

unit, exactly as if an AVAILABLE  command  with  the  "Spin-down"

modifier  set  had been issued for the unit as described later in

this section.  In either case, AVAILABLE attention messages shall

not  be  generated  for that unit by any controller until a human

interacts with the unit or some other event occurs that may clear

the error, such as a power failure.

Controllers   and/or  drives  should  keep  all  units  that  are

"Unit-Offline" due to being inoperative, disabled, or having  du-

plicate  unit numbers spun-down, except when such units are under

the control of a diagnostic.  The handling of units that  are  in

fact inoperative, but that the controller and drive believe to be

operative, is described in the preceding paragraph.

Controllers may spontaneously release a host's "Exclusive Access"

of a unit only if the unit becomes inoperative, disabled, or  un-

known.

For  the  purpose  of  automatic configuration (i.e., for the GET

UNIT STATUS command with the "Next  Unit"  modifier)  controllers

shall   acknowledge   the   existence   of  all  units  that  are

"Unit-Online" or "Unit-Available," or that are in one of the fol-

lowing  "Unit-Offline"  substates:   unit  disabled,  unit known,

under exclusive use of another host, or duplicate  unit  numbers.

Unknown   units  shall  not  be  acknowledged.   Units  that  are

ALGORITHMS AND USAGE RULES                              PAGE 4-11

UNIT STATES

inoperative or online to another controller may  or  may  not  be

acknowledged at the controller's option.

Controllers  shall report duplicate unit numbers with a DUPLICATE

UNIT NUMBER attention message, then monitor  the  affected  units

for  the  cessation of the duplicate unit number condition.  When

all units except one have had their unit number changed  or  have

become unknown, the remaining unit becomes "Unit-Available."

Controllers  shall  report  units  that they know to be online to

other controllers with an ACCESS  PATH  attention  message.   See

Section  section(multiaccesdrvs),  "Multiaccess Drives," for more

information.

The  "Unit-Offline"  substates  are  all  reported  with a single

status code;  they are partially distinguished via different sub-

codes.  In addition, the substates are distinguished by the func-

tioning of the "Allow Self Destruction" modifier  to  the  ONLINE

command, the "Next Unit" modifier to the GET UNIT STATUS command,

what unit characteristics are returned by the  GET  UNIT  STATUS,

ONLINE,  and SET UNIT CHARACTERISTICS commands, and by the DUPLI-

CATE UNIT NUMBER and ACCESS PATH attention messages.

Possible causes of a unit being "Unit-Offline," and the resulting

"Unit-Offline" substate, include:

1.  The unit is "Unit-Online" or is under "Exclusive Access"

via another controller.  The unit is either  unknown  or

else known to be online to another controller, depending

upon how recently the other controller has  processed  a

DETERMINE  ACCESS  PATHS command for this unit or a unit

with which it shares an access path.  See Sections  sec-

tion(multiaccesdrvs),   "Multiaccess  Drives"  and  sec-

tion(multidrvsfmts), "Multiunit Drives  and  Formatters"

for more information.

2.  A  power  failure that affects the unit but not the con-

troller.  The unit is unknown.

3.  Hardware  failure  in  the  unit  or  in  the connection

between the unit and the controller.  The unit is either

unknown  or  inoperative.  Note that the unit may appear

to be "Unit-Available" until an ONLINE command is issued

for  it, at which time it will be recognized as inopera-

tive.

4.  Disconnecting the unit from the controller.  The unit is

unknown.

ALGORITHMS AND USAGE RULES                              PAGE 4-12

UNIT STATES

5.  Disabling the unit number select mechanism (e.g., remov-

ing the unit number select plug).  The unit is unknown.

6.  Duplicate unit numbers.  This condition has its own sub-

state.  Note that this condition will not affect a  unit

that  is  already  "Unit-Online"  or is under "Exclusive

Access."

7.  Duplicate  unit  identifiers.   The unit is inoperative.

Note that the controller need not  check  for  duplicate

unit identifiers.

8.  Disabling  the  unit  with  the  Run/Stop or Load/Unload

switch.  The unit is known.

9.  Disabling  the  unit  with port selection switches.  The

unit is unknown.

10.  Removal  of  the  unit from service by operator command,

typically for diagnostics,  formatting,  maintenance  or

repair.  The unit is inoperative.

11.  An  internal  controller  diagnostic decides the unit is

sick and removes it from service.  The unit is disabled.

12.  No volume is present in the drive.  The unit is known.

13.  The  unit  has  not been built yet, the unit is still in

its shipping crate, or it has  otherwise  not  been  in-

stalled yet.  The unit is unknown.

In  general,  a  unit  that  is  "Unit-Offline" to one host class

driver via a specific MSCP server is "Unit-Offline" for the  same

reason  (same  substate)  to all host class drivers via that same

MSCP server.  There are two exceptions:

1.  A  duplicate  unit  number condition that arises while a

unit is "Unit-Online" to one or more class drivers.  The

unit  remains  "Unit-Online"  to  those class drivers to

which it is already "Unit-Online," yet is  "Unit-Offline

(substate 'Duplicate Unit')" to all other class drivers.

2.  A  host has been granted "Exclusive Access" of the unit.

The state of the unit relative  to  the  host  that  was

granted  "Exclusive Access" remains unchanged.  The unit

is "Unit-Offline  (substate  'Exclusive  Use')"  to  all

other class drivers.

ALGORITHMS AND USAGE RULES                              PAGE 4-13

UNIT STATES

All  normal  operator initiated transitions to the "Unit-Offline"

state shall be smooth, rather than abrupt.  Attempts to disable a

unit  with  its Run/Stop or Load/Unload switch and/or attempts to

dismount (remove) a  volume  from  a  unit  are,  by  definition,

"normal  operator  initiated transitions," and shall therefore be

smooth.  Any other action that a human operator  would  typically

use  to disable a drive in a nonemergency situation shall also be

smooth.  Note that this does not preclude the existence  of  some

special mechanism for immediately disabling a unit or drive in an

emergency, provided that it is not the normal way of disabling  a

unit or drive.

A  "smooth"  transition  to the "Unit-Offline" state implies that

all write  operations,  including  multiblock  write  operations,

shall  either  be  completed  in  their entirety or else rejected

(never begun).  To accomplish a smooth transition the  controller

shall  complete all write operations (commands) that have already

been initiated.  Outstanding write operations that  haven't  been

initiated  yet  may  either be completed or rejected.  Other out-

standing commands, including read operations, may  be  completed,

rejected,  or  terminated, regardless of whether or not they have

been initiated.  However, if a Sequential command  from  a  given

connection  is  rejected or terminated, then all subsequently re-

ceived non-Immediate commands from the same connection  shall  be

rejected.  Any new commands issued by a host should be rejected.

Note  that  establishing  write protection shall also be a smooth

transition.

A unit enters the "Unit-Available" state when:

1.  The  unit  is  "Unit-Offline" and all causes of the unit

being "Unit-Offline"  are  removed.   The  unit  becomes

"Unit-Available" with respect to all "Controller-Online"

class drivers unless the unit is  under  the  "Exclusive

Access"  of  a  host.   In  that  case  the unit becomes

"Unit-Available" only to the class driver that has  "Ex-

clusive Access" of the unit.

2.  The unit is "Unit-Online" and a host class driver issues

an AVAILABLE command for the  unit.   The  unit  becomes

"Unit-Available"  with respect to that class driver.  If

the "All Class Drivers" modifier was set, the unit  also

becomes "Unit-Available" with respect to all other class

drivers to which  it  is  "Unit-Online"  via  that  MSCP

server,  unless the "Exclusive Access" modifier was also

set.  If the "Exclusive Access"  modifier  was  set  the

unit  becomes  "Unit-Available"  to as well as under the

ALGORITHMS AND USAGE RULES                              PAGE 4-14

UNIT STATES

"Exclusive Access"  of  the  issuing  class  driver  and

"Unit-Offline  (substate  'Exclusive Use')" to all other

class drivers.

3.  The  unit  is  "Unit-Online"  or "Unit-Available" to and

under the "Exclusive Access" of a host and  that  host's

class  driver  issues  an AVAILABLE, ONLINE, or SET UNIT

CHARACTERISTICS command with the "Exclusive Access" mod-

ifier clear, thereby releasing "Exclusive Access" of the

unit.  The state of the unit with respect to the issuing

class  driver depends on which of those commands was is-

sued as well as the state of the unit prior to the  com-

mand  being  issued.   (See  the  descriptions  of those

commands in Chapter chapter(multihostfunct),  "Multihost

Support"   for   complete  details.)  The  unit  becomes

"Unit-Available" to all other class drivers,  regardless

of  the  unit  state  with  respect to the issuing class

driver.

If a class driver has enabled attention messages, the MSCP server

uses AVAILABLE attention messages to notify the class driver that

a  unit  has  asynchronously  become "Unit-Available." If a class

driver itself sends the MSCP server an  AVAILABLE  command,  then

the  transition  to "Unit-Available" is synchronous to that class

driver and an AVAILABLE attention message need not be sent to it.

Other class drivers, however, are notified.

The   possible   causes   of   an   asynchronous   transition  to

"Unit-Available" are as follows:

1.  Any      transition      from      "Unit-Offline"     to

"Unit-Available." This specifically includes the follow-

ing cases:

a.  The unit was online to another controller and ceases

to be online to that other  controller.   Note  that

this  includes  the  possibility  that  the unit was

"Unit-Online" to or under the "Exclusive Access"  of

the same class driver via that other controller.

b.  The  unit was under the "Exclusive Access" of a host

and that host has released the "Exclusive Access" of

the unit.

2.  A  transition  from  "Unit-Online"  to "Unit-Available,"

with respect to this class driver,  that  is  caused  by

some  other  class  driver  issuing an AVAILABLE command

with the "All Class Drivers" modifier set.

ALGORITHMS AND USAGE RULES                              PAGE 4-15

UNIT STATES

3.  Any   spontaneous   transition   from  "Unit-Online"  to

"Unit-Available." I.e.,     any     transition      from

"Unit-Online"  to "Unit-Available" that is not caused by

an AVAILABLE command.

The  one  exception  to  this  applies  to  a  unit  that becomes

"Unit-Available"  due  to  an  AVAILABLE  command  that  has  the

"Spin-down"  modifier  set  or as a side effect of certain errors

that also spin-down the unit.  Such commands or  errors  indicate

that all class drivers are disinterested in the volume mounted on

the unit, so that no class driver should be notified of the tran-

sition  until an operator mounts a new volume or otherwise inter-

acts with the unit.  Therefore the request to spin-down the  unit

suppresses  AVAILABLE  attention  messages for that unit until an

operator interacts with the unit.  Note that the  messages  shall

be  suppressed for all class drivers, MSCP servers, and controll-

ers that may connect to the unit, regardless of which  individual

MSCP  server  and  controller  actually  requested  that the unit

spin-down.  This effectively means that, for multiaccess  drives,

the fact that AVAILABLE attention messages are suppressed must be

recorded in the drive itself, rather than in the controller.

AVAILABLE  attention  messages shall be suppressed at least until

an operator interacts with the unit's  drive,  the  unit  becomes

"Unit-Online"  to  any  class  driver via any MSCP server, or the

unit's drive loses context.  It is not acceptable for the  unit's

drive to "lose context" solely to forget that AVAILABLE attention

messages have been suppressed.  Loss of context must  be  due  to

some  external  reason, such as a power failure.  The suppression

of AVAILABLE attention messages shall be canceled or forgotten if

the  unit  becomes "Unit-Online" to any class driver via any MSCP

server or if an operator actuates the unit's Run/Stop or Load/Un-

load  switch, changes the unit number selected by the Unit Number

Select Mechanism, or mounts a different volume in the unit.  Note

that AVAILABLE attention messages are not suppressed (i.e., their

suppression is instantly canceled or forgotten) if, after a class

driver  issues an AVAILABLE command with the "Spin-down" modifier

set, the unit is still "Unit-Online" with respect to one or  more

other class drivers.

MSCP  servers  may send redundant or unnecessary AVAILABLE atten-

tion messages at any time, provided that attention messages  have

been  enabled, the messages have not been suppressed as described

above, and the extra messages  are  infrequent  enough  to  avoid

causing  any  significant  overhead  in either the communications

mechanism or a host.  It is specifically allowable  for  an  MSCP

server  to  precede every DUPLICATE UNIT NUMBER attention message

with an AVAILABLE attention message for the same unit number.

ALGORITHMS AND USAGE RULES                              PAGE 4-16

UNIT STATES

A  unit  enters  the  "Unit-Online" state with respect to a class

driver upon the successful completion of an ONLINE command issued

by that class driver.

4.4  UNIT NUMBERS

As  described  in  Section section(controlsindics), "Controls and

Indicators," all disk and tape drives accessible  via  MSCP  must

have a user visible and Field Service alterable unit number.  The

characters displayed as the unit number, interpreted as a decimal

number,  shall match the binary value passed in the "unit number"

field of MSCP control messages.  Multiunit drives and  formatters

may use a single unit number select mechanism and display for the

range of consecutive unit numbers assigned to their units.

MSCP  supports  unit  numbers  in the range 0 through 65535.  All

controllers and units shall support unit numbers in the  range  0

through 251.  Unit numbers between 252 and 65535 may be supported

or not at the controller's and/or unit's  option.   This  implies

that  all  units  accessible  via  MSCP  shall have a unit number

select mechanism capable of specifying and a unit number  display

mechanism  capable  of  displaying any unit number in the range 0

through 251.  Controllers shall treat unsupported unit numbers as

if the specified unit is "Unit-Offline" due to being unknown (see

Section section(unit__states), "Unit States").

The  intent of this broad range of unit numbers is that, within a

device class, all units that are accessible to a single host have

unique  unit  numbers.  In pursuit of this goal units with dupli-

cate unit numbers are considered to  be  "Unit-Offline." However,

this must be balanced against another goal, namely that transient

actions performed on another unit should not be allowed to affect

continued  host access to a unit that is already "Unit-Online" or

under "Exclusive Access." Controllers  shall  balance  these  two

goals with the following algorithms:

1.  Controllers detect and respond to duplicate unit numbers

across  all  units  that  are  "Unit-Online,"  that  are

"Unit-Available,"  or  that  are in one of the following

"Unit-Offline" substates:  unit  disabled,  unit  known,

under  exclusive  use  of  another  host, duplicate unit

numbers, or online to another  controller.   Units  that

are  "Unit-Offline"  due  to  being unknown shall not be

considered for duplicate unit  number  detection.   Con-

trollers  may  or  may  not, at the controller's option,

consider units that are  inoperative.   Detection  of  a

ALGORITHMS AND USAGE RULES                              PAGE 4-17

UNIT NUMBERS

duplicate  unit number condition on one unit of a multi-

unit drive is treated as a duplicate unit number  condi-

tion  on  all  other units that share one or more of the

following components with the unit having the  duplicate

unit number:

a.  A unit number select mechanism.

b.  A Run/Stop or Load/Unload switch.

c.  A spindle or other mechanical components.

2.  Discovery  of a duplicate unit number condition does not

affect any unit that is already "Unit-Online"  or  under

"Exclusive Access." The state of such a unit remains un-

changed until some other event  (such  as  an  AVAILABLE

command or loss of controller context) occurs that would

otherwise cause it to  become  "Unit-Available"  to  all

hosts, at which time the unit becomes "Unit-Offline" and

is spun-down as  described  below.   Note  that  in  the

already    "Unit-Online"    case    the   unit   becomes

"Unit-Offline" to all other hosts (and therefore  cannot

be  brought "Unit-Online" by them) even while it remains

"Unit-Online" to its current hosts.

3.  When  a  controller  becomes  aware  of a duplicate unit

number condition on two or more units connected  to  it,

it  immediately  spins-down  all  such  units  that  are

"Unit-Available" or "Unit-Offline" with respect  to  all

hosts.  When a unit that was "Unit-Online" or under "Ex-

clusive Access" with a duplicate unit  number  condition

becomes  "Unit-Available"  to  all hosts for any reason,

the controller  immediately  spins  it  down.   In  both

cases,  units that belong to a multiunit drive and share

a spindle or other mechanical components are to be  spun

down  only  if  all of the units of the drive are either

"Unit-Offline" or "Unit-Available" to all hosts.

4.  The  controller  reports  "Unit-Offline" due to having a

duplicate unit number as the state for  all  units  with

duplicate  unit  number  conditions that are not already

"Unit-Online" or under "Exclusive Access." Other  causes

of  the  units  being  "Unit-Offline"  may or may not be

reported at the controller's option.

5.  The  controller  recognizes when a duplicate unit number

condition goes away.  That is, the controller recognizes

when  all  units except one with the same duplicate unit

number have their unit  numbers  changed  and/or  become

ALGORITHMS AND USAGE RULES                              PAGE 4-18

UNIT NUMBERS

unknown.   When this occurs, the controller sends AVAIL-

ABLE attention messages for the remaining unit if  there

is no other reason for it being "Unit-Offline," since it

has just become "Unit-Available."

In addition to the above algorithms, controllers report duplicate

unit number conditions to class drivers using the DUPLICATE  UNIT

NUMBER attention message.  This allows hosts to complain to a hu-

man operator, who will presumably  remedy  the  condition.   This

message  is  sent  to  all "Controller-Online" class drivers that

have attention messages enabled when a duplicate unit number con-

dition  is first detected.  It is permissible for a controller to

send redundant or extra DUPLICATE UNIT NUMBER attention messages,

provided  that  the  reported duplicate unit number condition ac-

tually exists.   See  Section  section(dupuntatnmes),  "Duplicate

Unit Number Attention Message," for more details.

The   above  algorithms  effectively  enforce  nonduplicate  unit

numbers across the  drives  connected  to  the  same  controller.

Although  not  required  by  MSCP,  it  is recommended that class

drivers use the following algorithm to enforce nonduplicate  unit

numbers across the drives accessible to that class driver:

1.  Upon  becoming  aware of a unit (i.e., upon receiving an

AVAILABLE attention message), the  class  driver  should

check  if  it  is  already aware of a unit with the same

unit number on a different MSCP server.  If  it  is  not

aware of such a unit, it should exit.  If it is aware of

such a unit, it should check if that unit has  the  same

unit identifier.

2.  If  the  unit  identifiers  are the same, it is the same

unit multiaccessed to several controllers, so exit.   If

the  unit  identifiers  are  different, the class driver

should issue a GET UNIT STATUS command to see if the old

unit still exists.

3.  If   the   old   unit   no  longer  exists  (i.e.,  it's

"Unit-Offline"), exit.  If the old  unit  still  exists,

the  class  driver should check that the unit identifier

returned by GET UNIT STATUS is also different  from  the

unit identifier that was in the AVAILABLE attention mes-

sage.  If the unit identifiers are the same,  exit.   If

the  unit  identifiers  are  different, the class driver

should issue an AVAILABLE command with  the  "Spin-down"

modifier  set for the new unit.  The class driver should

also issue an AVAILABLE  command  with  the  "Spin-down"

modifier set for the old unit if the class driver is not

already "Unit-Online" to that unit.

ALGORITHMS AND USAGE RULES                              PAGE 4-19

UNIT NUMBERS

4.  Whenever   a   class   driver   brings  an  MSCP  server

"Controller-Online,"  the  class  driver  should  issue,

through  that  MSCP  server, AVAILABLE commands with the

"Spin-down" modifier set  for  all  units  that  it  has

"Unit-Online" via another MSCP server.

5.  Whenever a class driver brings a unit "Unit-Online," the

class driver should issue ONLINE commands for that  same

unit  number  to all MSCP servers.  If more than one ON-

LINE command succeeds, the  class  driver  should  check

that the unit identifiers returned by all the successful

ONLINE commands are the same.  If any of the unit  iden-

tifiers  are  different,  then  the  class driver should

treat all the ONLINE commands as having failed and issue

AVAILABLE  commands with the "Spin-down" modifier set to

all MSCP servers on which the ONLINE command succeeded.

Note  that this algorithm uses the fact that an AVAILABLE command

with the "Spin-down" modifier set suppresses AVAILABLE  attention

messages until a human operator interacts with the unit.

4.5  COMMAND CATEGORIES AND EXECUTION ORDER

MSCP  commands  fall  into  one of four command categories.  Each

category has a set of  execution  order  restrictions  that  MSCP

servers must satisfy.  The four categories and their restrictions

are described below.

Immediate  commands are those commands, such as status inquiries,

that both require very little time to complete and do  not  cause

any  unit  context changes.  MSCP servers shall process Immediate

commands immediately, without waiting for any other  commands  to

complete.   MSCP servers shall guarantee that all outstanding Im-

mediate commands plus an additional GET  COMMAND  STATUS  command

issued  by  each  "Controller-Online"  class driver will complete

within the controller timeout interval.

A  class  driver may issue Immediate commands whenever its credit

balance is greater than zero, whereas all other commands may only

be  issued  when  the  class  driver's  credit  balance is two or

larger.     This    is    discussed    further     in     Chapter

chapter(drvrsrvrcommu),    "Class    Driver    /    MSCP   Server

Communications." Class drivers are thus guaranteed to be able  to

issue  at  least one Immediate command, and have it executed, re-

gardless of what other commands they may  have  outstanding.   In

particular,  the class driver is guaranteed to be able to issue a

ALGORITHMS AND USAGE RULES                              PAGE 4-20

COMMAND CATEGORIES AND EXECUTION ORDER

GET COMMAND STATUS command and have it complete within  the  con-

troller timeout interval.

Sequential  commands  are those commands that, for the same unit,

shall be executed in precise order.   Sequential  commands  typi-

cally  alter a unit's context, such as by changing a unit charac-

teristic or by altering  the  position  of  a  tape  drive.   All

Sequential  commands  for  a particular unit shall be executed in

the exact order that the MSCP server receives them.   The  execu-

tion of a Sequential command may not be interleaved with the exe-

cution of any other Sequential or Non-Sequential commands for the

same unit.  Furthermore, any Non-Sequential commands received be-

fore a particular Sequential command shall  be  completed  before

execution   of   that   Sequential   command   begins,   and  any

Non-Sequential commands received after  a  particular  Sequential

command  shall  not  begin  execution until after that Sequential

command is completed.  Sequential commands are, in effect, a bar-

rier  that  Non-Sequential  commands  may  not pass or penetrate.

Note that Sequential command execution is  performed  on  a  unit

basis  rather  than  a single connection basis.  Controllers that

provide Multihost Support shall therefore enforce the  Sequential

command  execution guarantees across all "Controller-Online" con-

nections.

Non-Sequential  commands  are those commands that controllers may

reorder so as to optimize performance.  Controllers may  further-

more  interleave the execution of several Non-Sequential commands

among themselves, performing each command a fragment at  a  time.

The only restrictions are:

1.  Execution  of  a  Non-Sequential command shall not cross

the barrier created by a Sequential command for the same

unit.

2.  The  controller shall complete useful work on its oldest

outstanding command on each "Controller-Online"  connec-

tion  within  the controller timeout interval, so as not

to   cause   a    command    timeout.     See    Section

section(command_timos), "Command Timeouts."

Note  that  controller performance optimization of Non-Sequential

commands, although not a strict requirement of  MSCP,  is  highly

recommended.  The optimization algorithms used are controller im-

plementation dependent and  will  undoubtedly  differ  from  con-

troller   to  controller.   Controllers  that  provide  Multihost

Support are expressly permitted  to  include  all  Non-Sequential

commands  received  from  all  "Controller-Online" class drivers,

directed to the same unit, in the optimization (algorithm)  being

performed on the unit.  One consequence of such optimization that

ALGORITHMS AND USAGE RULES                              PAGE 4-21

COMMAND CATEGORIES AND EXECUTION ORDER

must be noted is that if more than one data transfer request with

the  same  target  received from different class drivers (connec-

tions) are outstanding at the same time, the ordering of the data

written/read will be indeterminate.

Special commands are similar to Non-Sequential commands, but have

additional, unique execution order requirements.   The  execution

order  requirements  for  Special  commands  are described in the

commands' description.

Note  that  tape class devices only have Immediate and Sequential

commands.  There is no such thing as a Non-Sequential or  Special

tape class command.

Execution  order  is based on the order in which commands are re-

ceived by the controller's MSCP server.  For commands sent  by  a

single  class  driver,  the order of transmission is identical to

the order of reception.   For  commands  sent  by  several  class

drivers,  the order of reception is essentially random.  The only

way that a class driver can ensure that one of its commands  will

be  received  after  some  other  command issued by another class

driver is to wait until the other class driver receives the first

command's  end  message  (i.e., wait until the first command com-

pletes) before sending the command.

For the purpose of determining execution order, a command is com-

pleted when the controller's MSCP server queues the command's end

message  for  transmission  to the host.  The controller need not

wait until the host has confirmed its receipt,  although  sequen-

tial  message delivery guarantees must be preserved.  The gist of

this is that after termination of the connection between  a  host

class  driver  and  an  MSCP  server, the absence of a Sequential

command's end message implies  nothing  about  the  execution  or

nonexecution of the Sequential command.

When  an  MSCP server finishes processing or executing a command,

the command can finish in any of four ways.  Based on  which  way

the  server  finishes a command, the command is said to have been

completed, rejected, terminated, or aborted.

A  completed  command is a command that the server has fully exe-

cuted to its normal conclusion.  A command that is completed  has

either  succeeded  or  encountered  an  unrecoverable  error.  In

normal operation all commands are completed.

A  rejected  command is a command for which the server never even

starts processing.  Some commands are rejected due to termination

of  the  connection between the MSCP server and the class driver.

ALGORITHMS AND USAGE RULES                              PAGE 4-22

COMMAND CATEGORIES AND EXECUTION ORDER

Such commands are discarded by the MSCP server, port  driver,  or

communications  mechanism before any work is done on them.  Other

commands are rejected due to being illegal commands or due  to  a

unit  being  in an illegal state.  For example, transfer commands

are rejected if the specified unit is not "Unit-Online."

A  terminated  command  is  a command for which the server begins

processing, but then terminates processing part way through  com-

mand  execution.   Transfer commands are terminated if their unit

is disconnected (cable breaks or  drive  loses  power)  part  way

through  the  transfer.  Transfer commands may also be terminated

if the connection between the MSCP server and the class driver is

terminated   part  way  through  the  transfer.   Generally  only

Non-Sequential  commands  can  be  terminated.   Termination   of

Sequential commands is discussed below.

An  aborted  command is a command that the class driver has asked

to have aborted via an ABORT command, and  that  furthermore  has

been located and explicitly aborted by the MSCP server.  That is,

commands that the MSCP server just allows to  complete  are  com-

pleted,  not  aborted.   An aborted command is effectively either

rejected or terminated, depending upon whether or not the  server

has  begun  execution of the command when it processes the ABORT.

(This definition of an aborted command is from the  viewpoint  of

an MSCP server.  From the viewpoint of a class driver, an aborted

command is any command for which an ABORT command  has  been  is-

sued.)  The  end  message of an aborted command shall be returned

within one controller timeout  interval  following  rejection  or

termination  of  the  command regardless of the aborted command's

command category.

Immediate commands are always completed if the connection between

MSCP server and class driver remains intact;  they may not be re-

jected,  terminated, or aborted.  If the connection is terminated

before an Immediate command can be completed,  then  the  command

may  be completed, rejected, or terminated.  Since Immediate com-

mands do not change the state, context,  or  data  of  the  unit,

these  three  command completion modes are equivalent if the con-

nection has been terminated.

Non-Sequential  commands may, in general, be completed, rejected,

terminated, or aborted.  This is  true  both  if  the  connection

remains  intact  or  if  it is terminated.  However, there is one

restriction regarding write operations.  Write  operations  shall

not be terminated as a result of any normal operator interactions

with the unit.  That is, write operations shall not be terminated

(partially  completed)  as  a result of an operator attempting to

spin-down, dismount, or write protect the unit.  The  purpose  of

this  restriction  is  to  prevent data corruption due to partial

updates.  This is  further  discussed  under  the  term  "smooth"

ALGORITHMS AND USAGE RULES                              PAGE 4-23

COMMAND CATEGORIES AND EXECUTION ORDER

transitions  in Section section(unit__states), "Unit States," and

Section section(write_protect), "Write Protection."

Sequential  commands may be completed or rejected, but in general

may not be terminated.  If a Sequential command  is  aborted,  it

shall  be  aborted  before  it begins execution (rejected) rather

than in the middle of execution  (terminated).   The  reason  for

this  is  that Sequential commands appear to hosts as atomic (in-

divisible) operations.  Thus, there  is  both  this  requirement,

that Sequential commands cannot be terminated, therefore prevent-

ing subsequent commands from acting  on  partially  updated  unit

state  and  context,  and  the  requirement  that  execution of a

Sequential command cannot be interleaved with any  other  command

on  the same unit, therefore preventing a concurrent command from

acting on partially updated unit state and context.

There  are  two  exceptions  to the requirement that a Sequential

command may not be terminated.  The first is that  a  server  may

terminate  a  Sequential  command  in  the  middle  of execution,

provided that the termination process undoes all unit  state  and

context  changes  that  have  been  made  on  the unit.  From the

viewpoint of the class driver, this case is exactly equivalent to

the  command  having been rejected.  The second exception is that

the controller and/or unit loses  context,  generally  due  to  a

power  failure or similar event.  Since the unit's state and con-

text will be completely reset when power and/or normal  operation

are  restored,  the  fact  that it was left in some indeterminate

state does not matter.  Note that if the  controller  loses  con-

text,  it  must  lose context for all connections to the affected

unit's device class (implying that all such connections have been

terminated),  as  otherwise  some  other class driver may see the

unit in an indeterminate state.

Note  that  any command that fails due to an unrecoverable device

or data error is a completed  command.   Commands  are  rejected,

terminated,  or  aborted  only  in response to inappropriate unit

state or context, connection termination, and being  aborted  via

ABORT commands.

Commands  that  are  rejected, terminated, or aborted shall still

return their proper end message format with valid  parameters  as

defined  in  the  individual command descriptions.  The only case

where a different end message format may be returned  is  if  the

command is rejected as a protocol error type "Invalid Command." A

special end message format and end message  opcode  is  used  for

that   purpose.    See   "Invalid   Command"  status  in  Section

section(status__codes), "Status Codes" for more detail.

ALGORITHMS AND USAGE RULES                              PAGE 4-24

COMMAND CATEGORIES AND EXECUTION ORDER

4.6  CLASS DRIVER / MSCP SERVER SYNCHRONIZATION

Synchronization  of  a class driver with an MSCP server is accom-

plished by establishing or re-establishing the connection between

the  class  driver  and  the MSCP server.  When the connection is

established or re-established, the  MSCP  server  terminates  all

commands  that  are  outstanding  from  that  class driver.  This

forces the dialogue between the class driver and MSCP server to a

known,  synchronized  state:   that of having no outstanding com-

mands.  After establishing the connection the  class  driver  can

issue  commands without worrying about duplicating command refer-

ence numbers or other unfortunate side effects.  Note  that  syn-

chronizing  with  the MSCP server, if successful, causes the MSCP

server to become "Controller-Online."

As  stated  above,  the  main  purpose  of  synchronization is to

guarantee that there are no outstanding  commands,  thus  forcing

the  dialogue between the class driver and MSCP server to a known

state.  MSCP servers shall ensure that this guarantee is met  be-

fore  they  allow  synchronization to complete (i.e., before they

become "Controller-Online").  In particular, MSCP  servers  shall

guarantee  that  no  end  messages will be sent and that no units

will have their state, context, or (data)  contents  changed  for

any  commands  that  were issued on an earlier incarnation of the

connection between the class driver and MSCP server.   Note  that

an MSCP server may allow outstanding commands to complete, either

partially or entirely, but if it does it must delay  the  comple-

tion    of    synchronization    (delay    the    transition   to

"Controller-Online") until all such commands  have  completed  or

terminated.

Class drivers shall synchronize with the MSCP server whenever the

host boots, recovers from a power failure, loses context,  or  is

recovering from certain errors.  After synchronizing with an MSCP

server, the class driver should do the following:

1.  Issue a SET CONTROLLER CHARACTERISTICS command to estab-

lish host-settable controller characteristics and obtain

non-host-settable controller characteristics.

2.  Issue  ONLINE  commands  for  all  units  that the class

driver wishes to be "Unit-Online."

3.  Reissue  all commands, if any, that were outstanding be-

fore the class driver synchronized with the MSCP  server

in  the  exact  order  that they were originally issued.

ALGORITHMS AND USAGE RULES                              PAGE 4-25

CLASS DRIVER / MSCP SERVER SYNCHRONIZATION

However, any commands that have been aborted (i.e.,  for

which  an  ABORT  command  has been issued) shall not be

reissued.  Instead the class driver should  assume  that

the  command has been successfully aborted and is there-

fore no longer outstanding.

There  is  one exception to reissuing all commands that were out-

standing.  The class driver shall maintain a retry  limit  count,

to  ensure  that  its oldest outstanding command won't be retried

infinitely many times.  The  magnitude  of  this  limit  is  host

dependent,  although the oldest command shall be retried at least

once.  When the class driver's oldest outstanding command exceeds

the  retry  limit  count,  an error shall be returned rather than

retrying the command  again.   All  other  outstanding  commands,

however,     should    still    be    retried.     See    Section

section(command_timos), "Command Timeouts," for more information.

It may be possible that a class driver will receive messages from

an MSCP server after the class driver has initiated  synchroniza-

tion  with  the  MSCP  server but before the synchronization com-

pletes.   Whether  or  not  this   is   in   fact   possible   is

communications  mechanism  dependent,  as it depends upon the de-

tailed design of the communications mechanism  and  port  driver.

Any attention messages that the class driver receives during this

interval should be discarded.  Any error log messages  should  be

logged  in  the  normal fashion.  Any end messages that the class

driver receives during this interval may be either  handled  nor-

mally  (thus  completing  the corresponding command) or else dis-

carded.  If the class driver discards one end message,  it  shall

discard  all  subsequent  end  messages until the synchronization

completes.  It is recommended, although not required, that  class

drivers handle all such end messages normally.

4.7  CLASS DRIVER ERROR RECOVERY

The  principal  method of error recovery used by class drivers is

to resynchronize with the MSCP server, as  described  in  Section

section(drvrsrvr_sync),    "Class    Driver    /    MSCP   Server

Synchronization." All communications mechanism failures and  many

controller  failures  are  reported by terminating the connection

between the class driver and MSCP server, in  response  to  which

the  class  driver  should attempt to resynchronize with the MSCP

server.  If the class driver decides that the controller  is  in-

sane, either because the class driver received an invalid message

or  because  a  command  timed  out,   it   should   recover   by

resynchronizing  with  the  MSCP  server.  Similarly, if the MSCP

ALGORITHMS AND USAGE RULES                              PAGE 4-26

CLASS DRIVER ERROR RECOVERY

server decides that the class driver is insane, it may  terminate

the  connection  to  the class driver.  If the class driver is in

fact actually sane, it will resynchronize with  the  MSCP  server

after  the  port  driver notifies it that the connection has been

terminated.

Aside  from resynchronization as described in the preceding para-

graph, class drivers need perform  very  little  error  recovery,

since controllers handle all recoverable errors.  The only excep-

tions to this guideline are as follows:

1.  Errors  on  multiaccess  drives  should be retried using

another controller.

2.  Commands    that    fail   due   to   the   unit   being

"Unit-Available"  should  typically  be  reissued  after

bringing the unit "Unit-Online."

3.  High  availability  systems  may  wish  to  perform  the

enhanced error recovery described below.

High  availability  systems  may  wish to use the following addi-

tional error recovery strategy in order to minimize the impact of

certain types of controller problems.  Rather than reissuing out-

standing commands all at once after resynchronizing with  a  con-

troller,   such  a  system  should  instead  reissue  the  oldest

outstanding command all by itself.  The host class driver  should

reissue all other outstanding commands only after the oldest com-

mand completes.  The other outstanding commands may  be  reissued

either all at once (recommended) or one at a time.

If  the  oldest  command times out or otherwise fails again after

being reissued, then it was presumably the source of the  problem

and   normal  operation  will  resume.   If  the  oldest  command

succeeds, then the problem is most likely due to some race condi-

tion  within  the  controller and will not recur.  If the problem

does recur, then successive iterations of this algorithm will ex-

ecute commands one at a time until the offending command is found

and discarded.

ALGORITHMS AND USAGE RULES                              PAGE 4-27

SERIOUS EXCEPTIONS

4.8  SERIOUS EXCEPTIONS

A  serious  exception is an unrecoverable error or some other un-

usual condition that requires host  acknowledgment  before  addi-

tional  commands are processed.  Serious exceptions occur only on

tape class devices.   See  Tape  Mass  Storage  Control  Protocol

Specification (TMSCP) for complete details.

4.9  HOST ACCESS TIMEOUTS

MSCP  servers provide host access timeouts to guarantee that mul-

tiaccess drives (described  in  Section  section(multiaccesdrvs),

"Multiaccess Drives") will be accessible in spite of certain com-

munications  mechanism  failures.   If  the  communications  path

between  hosts and a controller fails when one or more drives are

"Unit-Online" or under "Exclusive Access"  via  that  controller,

the  drives would normally become inaccessible.  The drives can't

be  accessed  via  the  controller   through   which   they   are

"Unit-Online"  or under "Exclusive Access," since that controller

can't be accessed, and they can't be accessed via a  second  con-

troller,  since  they are already accessed through the first con-

troller.  The host access timeout, if  enabled,  eliminates  this

problem  by causing the first controller to automatically release

all drives if it doesn't receive a  command  within  a  specified

time interval.

The  exact mechanism used for host access timeouts is to have the

controller's MSCP server become  "Controller-Available"  relative

to  any host class driver whose host access timeout expires.  The

MSCP server automatically resets the timeout whenever it receives

a command from the class driver or has a command outstanding from

that class driver.  Therefore the timeout will  never  expire  if

the  class driver maintains a reasonably constant dialog with the

MSCP server.  Note that implementing host access  timeouts  on  a

per  host basis has the benefit that the MSCP server will automa-

tically release any resources allocated to a host  if  that  host

goes down.

If  communication  with all hosts ceases, the host access timeout

of each class driver will ultimately  expire,  causing  the  MSCP

server  to  become  "Controller-Available" relative to all hosts.

Each unit will be released, allowing it to  be  accessed  via  an

alternate  controller,  as  soon  as  all  class drivers that are

ALGORITHMS AND USAGE RULES                              PAGE 4-28

HOST ACCESS TIMEOUTS

"Unit-Online" or under "Exclusive Access"  with  respect  to  the

unit  (or any other unit that shares its access path) cease to be

"Unit-Online" or under "Exclusive Access" by virtue  of  becoming

"Controller-Available." Ultimately  all class drivers will become

"Controller-Available," thus releasing all units.

Class  drivers specify the time interval that an MSCP server will

use for the host access timeout in the SET CONTROLLER  CHARACTER-

ISTICS  command.   A  host access timeout value of zero specifies

that host access timeouts should be disabled.  A default host ac-

cess  timeout  of 60 seconds is used from the time a class driver

becomes "Controller-Online" until it issues its  first  SET  CON-

TROLLER CHARACTERISTICS command.

Each class driver may specify a separate time interval.  This al-

lows class drivers to vary the host access  timeout  interval  in

accordance  with host policy and availability requirements.  High

availability systems should typically use a fairly short host ac-

cess  timeout,  on the order of 10 to 30 seconds, together with a

background process that verifies the  continued  availability  of

the  host to controller communications path.  The background pro-

cess would have the desirable side effect of  ensuring  that  the

host  access  timeout never expired.  Normal systems should use a

larger timeout, on the order of 1 to 5 minutes, to  avoid  exces-

sive  resynchronizations, yet still allow failure recovery.  Sin-

gle user and other specialized systems may  disable  host  access

timeouts.   Note  that  host  access timeouts should typically be

enabled even on systems that do not  seem  to  require  them,  as

drives  may be multiaccessed to a totally separate host used as a

backup system.

MSCP  servers shall implement host access timeouts subject to the

following constraints:

1.  The  host access timeout expires, for a particular class

driver, at the amount of the host access timeout  inter-

val  after  the  controller or MSCP server completes all

outstanding commands from that  class  driver,  provided

that no new commands are received from that class driver

during this interval.

2.  The  MSCP  server shall enter the "Controller-Available"

state (as a result of host  access  timeout  expiration)

relative  to a class driver no sooner than the moment of

host access timeout expiration defined in item 1 above.

ALGORITHMS AND USAGE RULES                              PAGE 4-29

HOST ACCESS TIMEOUTS

3.  The  MSCP server should enter the "Controller-Available"

state (as a result of host access timeout expiration) as

soon as possible after the moment of host access timeout

expiration defined in item 1  above.   Except  when  the

controller  is  saturated  with  work  for  other  class

drivers, this shall be no later than one second plus the

amount of the host access timeout interval after the mo-

ment of host access timeout expiration.

In other words, the host access timeout is measured from the time

that the last command is completed.  The MSCP  server  may  defer

recognition  of  host  access  timeout  expiration  for up to one

second plus the amount of  the  host  access  timeout  after  the

formal  expiration  of  the  timeout.   That  is, the host access

timeout shall be  implemented  with  an  accuracy  range  of  -0%

through  +100%+1 second.  Furthermore, this accuracy range may be

extended on the high side if the  controller  is  saturated  with

work for other class drivers.

This  definition  of  host access timeouts has been structured to

allow at least two alternative implementations.  In the first im-

plementation,  the  controller or MSCP server starts a timer when

it goes "idle" -- that is, when it completes the last outstanding

command.   The  duration  of the timer is the host access timeout

interval.  The timer is designed to not err on the low side  (too

short  an interval) and to err by no more than a factor of two on

the high side (too long an interval).  If the timer  expires  be-

fore the MSCP server receives another command, declare a host ac-

cess  timeout  and  become   "Controller-Available." The   second

implementation  uses a continuously running timer that "ticks" at

the host access timeout interval.  If  there  is  no  outstanding

command  and  the  timer  "ticks" twice in succession without the

MSCP server having received a command from the host, then declare

a host access timeout and become "Controller-Available."

Each  controller  or MSCP server has a minimum and a maximum host

access timeout interval that it implements.  If  a  class  driver

specifies  a  host  access timeout interval that is less than the

minimum, then the MSCP server  uses  its  minimum.   If  a  class

driver  specifies  a host access timeout interval that is greater

than the controller's maximum, then the controller uses its  max-

imum.   A  controller's  or  MSCP  server's  minimum  host access

timeout interval shall be 10 seconds or less.  A controller's  or

MSCP  server's  maximum  host access timeout interval shall be at

least 255 seconds.  That is, all controllers shall  fully  imple-

ment  host  access  timeout intervals in the range 10 through 255

seconds.  Support of intervals outside this range  is  controller

dependent.   The  range  of  host access timeout intervals that a

controller supports shall be described in the controller's  Func-

tional Specification;  it cannot be obtained via MSCP.

ALGORITHMS AND USAGE RULES                              PAGE 4-30

HOST ACCESS TIMEOUTS

Note  that  all controllers and MSCP servers shall also implement

the host access timeout value zero, which  disables  host  access

timeouts.

4.10  COMMAND TIMEOUTS

Host  class  drivers  use  command timeouts to guarantee that all

controller or communications mechanism failures will be detected.

The  failures detected by command timeouts include partially sane

or deadlocked controllers, which may continue to process new com-

mands  even  though  one  or more old commands have been lost and

will never complete.  The use of command timeouts centers  around

the  GET  COMMAND STATUS command.  Indeed, the primary purpose of

the GET COMMAND STATUS command is for command timeouts.

A  controller  is sane if and only if it will ultimately complete

all commands submitted to it.  For practical purposes,  the  term

"ultimately"  must be replaced with the phrase "within reasonable

time." What constitutes a "reasonable time" varies with the  com-

plexity  of the command and the performance of the controller and

drives.  We can eliminate the difficulty of the host class driver

having  to derive this "reasonable time" by restating the defini-

tion of a sane controller as follows:  A controller  is  sane  if

and  only  if  it  will always complete useful work on its oldest

outstanding request within reasonable time.  This definition lets

us  set  the  "reasonable time" to some fixed value, and vary the

units in which we measure "useful work"  according  to  the  com-

plexity  of  the  command.   Command  timeouts  are based on this

second definition.

A  class  driver implements the command timeout mechanism as fol-

lows.  For each MSCP server to which it  is  "Controller-Online,"

the  class driver keeps track of which command is its oldest out-

standing command plus the previous "command status" value for its

oldest  outstanding command.  The previous "command status" value

should be set to all ones whenever the oldest  command  completes

and  a  new  command becomes the oldest.  The class driver should

issue GET COMMAND STATUS commands for its oldest outstanding com-

mand  at  intervals of the controller timeout interval or longer.

When each GET COMMAND STATUS command completes, the class  driver

checks  if the command for which it was issued is still outstand-

ing and, if it is, verifies that the "command status" returned by

the  GET  COMMAND STATUS command is lower than the previous "com-

mand status." This value measures the amount  of  work  remaining

before completion of the command.  If the value ever increases or

stays the same, or if a GET COMMAND  STATUS  command  ever  takes

ALGORITHMS AND USAGE RULES                              PAGE 4-31

COMMAND TIMEOUTS

longer than the controller timeout interval to complete, then the

class driver should assume that the  controller  has  failed  and

resynchronize with the controller.

In  addition  to guaranteeing that they will complete useful work

on their  oldest  outstanding  command,  controllers  shall  also

guarantee that they will complete all aborted commands within the

controller timeout interval.  That is, an aborted  command's  end

message  shall be sent no later than the amount of the controller

timeout interval after the ABORT  command's  end  message.   Host

class  drivers  can  check  this by setting the previous "command

status" value to one whenever the oldest outstanding command  has

been  aborted  (i.e.,  when  the  class driver receives the ABORT

command's end message indicating that its oldest outstanding com-

mand has been aborted).

Single  host  controllers  (i.e., controllers that do not provide

Multihost Support) need not guarantee that all  aborted  commands

will  complete  within  the controller timeout interval if exces-

sively pathological situations arise (e.g., compound or  multiple

errors,  causing  error  recovery sequences to stretch out to un-

realistic lengths).

If the command timeout for an aborted command expires due to such

a situation, the class driver will resynchronize  with  the  MSCP

server and reissue all outstanding commands except for those that

had been aborted.  Since the aborted commands will not  be  reis-

sued,  the  timeout will not recur.  Thus this exception is tran-

sparent to host class drivers.

The  above algorithm applies to non-Immediate commands.  With one

exception the same algorithm may also be used for Immediate  com-

mands,  although a simpler one (using the fact that all Immediate

commands complete within the controller timeout interval) is also

appropriate.  The one exception is the GET COMMAND STATUS command

used by the timeout algorithm  itself.   This  command  shall  be

timed  out  as  follows.  If the class driver's credit balance is

zero when it attempts to issue the GET COMMAND STATUS command, it

shall   queue  the  GET  COMMAND  STATUS  command  for  immediate

transmission (before any other commands that may be  outstanding)

when  its  credit  balance  becomes  nonzero.   If the controller

timeout interval expires again before the GET COMMAND STATUS com-

mand  has  both  been  transmitted  and completed, then the class

driver should assume that the controller has failed.   Note  that

the  class  driver's  credit  balance is guaranteed to be nonzero

when all outstanding Immediate  commands  have  completed.   Note

also  that  controllers  or  MSCP servers guarantee that all out-

standing Immediate  commands  plus  one  additional  GET  COMMAND

STATUS  command  will  complete  within  the  controller  timeout

interval.

ALGORITHMS AND USAGE RULES                              PAGE 4-32

COMMAND TIMEOUTS

Upon  concluding  that  a controller has failed, the class driver

shall resynchronize with the controller's MSCP server and reissue

all  commands  (except  commands that it has tried to abort) that

were outstanding to that MSCP server in the same order that  they

were  originally  issued.   In particular, the oldest outstanding

command (the one that timed out) shall be issued first (after in-

itial  SET  CONTROLLER CHARACTERISTICS and ONLINE commands).  The

only exception to this is if the command's retry  count  expires.

The  class  driver should maintain a retry count of the number of

times the oldest command has timed out and been retried.  If  the

retry  count  exceeds  a  host  dependent  retry limit, the class

driver should return an error rather than  retrying  the  command

again.  The size of the retry limit is determined by host policy,

except that all commands shall be retried at  least  once.   Note

that  subcode zero of the "Controller Error" status code has been

reserved for commands that exceed their retry count.   This  sub-

code  shall  never be generated by MSCP servers;  it is generated

by class drivers or non-MSCP entities (e.g., a controller's  port

driver) as a standard means of reporting command timeout errors.

In  order  to  implement  command timeouts, the host class driver

shall first obtain the controller timeout interval  via  the  SET

CONTROLLER  CHARACTERISTICS  command.  Therefore the class driver

should issue a SET  CONTROLLER  CHARACTERISTICS  command  as  the

first   command  after  becoming  "Controller-Online." The  class

driver should use a controller timeout interval of 10 seconds for

this  initial  command.   The class driver shall never use a time

interval that is shorter than the controller specified controller

timeout  interval for its command timeout determination, although

the class driver may use a time interval that is longer than  the

one specified by the controller.  The controller timeout interval

specified by the controller shall not be larger  than  4  minutes

and 15 seconds (255 seconds).

One  characteristic  of  this command timeout algorithm that MSCP

servers need not implement, and indeed most will  not  implement,

is  the  GET COMMAND STATUS command for any command that the MSCP

server can guarantee will complete within the controller  timeout

interval.   The  GET  COMMAND STATUS command should always return

the value zero as the "command status" of such a command.  It  is

acceptable  if,  due  to  vagaries of controller optimization al-

gorithms, such a command will occasionally timeout.  This is  ac-

ceptable,  so long as the frequency of such timeouts is extremely

small and the controller immediately begins processing the  first

command  it  receives  after  resynchronization.  Since the class

driver reissues  commands  in  the  same  order  that  they  were

originally  issued,  the  oldest or timed out command is reissued

first, effectively guaranteeing that it will not be delayed again

by  the  controller's optimization algorithms.  (The simplest way

for most controllers to implement this  is  to  treat  the  first

ALGORITHMS AND USAGE RULES                              PAGE 4-33

COMMAND TIMEOUTS

Non-Sequential  command  received across a newly established con-

nection as if the Express Request modifier were set.  See Section

section(cmd_modifiers),  "Command Modifiers" for a description of

the affects of that modifier.)

4.11  HOST BUFFER ACCESS

The   block   data   communications  service  (see  Section  sec-

tion(commconnection), "Connection") transfers the contents of one

named  buffer to another.  Hosts describe buffers (name and size)

with a buffer descriptor and byte count in the command message of

a transfer command.

In  a  read  operation, the host provides a buffer to receive the

data.  To the host, that  buffer  (the  portion  of  host  memory

described  by  the buffer descriptor and specified byte count) is

exactly equivalent to an undefined field in an MSCP control  mes-

sage  from the time that the class driver issues the READ command

to its port driver to the time that it receives the  end  message

for  that  READ  command from its port driver.  That is, the con-

tents are controller implementation dependent and cannot be  used

in  any  meaningful  way  by  the class driver.  The class driver

shall ignore the contents of the buffer and  may  not  alter  the

contents  in  any way.  The server may alter the contents of that

memory in any way it chooses during that time.

Note  that this holds true regardless of any sequencing rules im-

posed by command categories on execution order.  For example, the

host  queues  an  ONLINE command followed by a READ command.  The

contents of the READ buffer are undefined  from  the  moment  the

READ  command  is  queued,  despite  the  ONLINE  command being a

Sequential command.  The contents of the buffer remain  undefined

until  the  end  message  for the READ command is received by the

class driver, even before receipt of the ONLINE end  message  and

regardless of the success or failure of the ONLINE command.

Note  also  that  once  the  end message has been received by the

class driver, only that portion of the buffer  described  by  the

"byte  count"  in  the  end message is defined as containing data

from the unit.  The contents of the remainder of the  buffer  are

undefined.

ALGORITHMS AND USAGE RULES                              PAGE 4-34

HOST BUFFER ACCESS

In  a write type operation, the host loads the data to be written

into a buffer in host memory.  The server shall not  access  that

buffer  before  the  server receives the command message or after

the server queues the end message for transmission to the host.

4.12  DISK GEOMETRY AND FORMAT

The  host  accessible portion of a disk unit consists of a vector

of fixed length blocks, called logical  blocks.   Logical  blocks

are  identified  by logical block numbers (LBNs) which range from

zero through N-1, where "N" is the total number of logical blocks

on  the  unit.  The logical blocks on a unit are divided into two

mutually exclusive regions:

1.  The host area consists of those logical blocks available

for host data storage.  Logical blocks in the host  area

have  LBNs in the range zero through US-1, where "US" is

the "unit size," or number of logical blocks in the host

area.  The host obtains "US" or the "unit size" from the

ONLINE or SET UNIT CHARACTERISTICS command end messages.

2.  The  unit's  Replacement  Control  Table  (RCT), used to

record bad block replacement information.  This informa-

tion  is  further  described  below.  The RCT (actually,

multiple copies of the RCT) occupies the logical  blocks

numbered US through N-1.

The presence of the RCT is optional;  see the description below.

All  of  the logical blocks in the host area are guaranteed to be

"good" -- that is, to be free of permanent or hard errors  (media

defects).  Most controllers implement this via bad block replace-

ment.  A pool of replacement blocks,  identified  by  replacement

block  numbers  (RBNs),  is  provided  on  the disk.  Replacement

blocks are not directly accessible to hosts.  All host area logi-

cal blocks that are bad (i.e., that contain media defects leading

to hard errors  or  large  numbers  of  correctable  errors)  are

replaced  by  a replacement block.  The mechanism used to perform

ALGORITHMS AND USAGE RULES                              PAGE 4-35

DISK GEOMETRY AND FORMAT

this replacement, and to revector accesses to bad logical  blocks

to  the  proper  replacement  blocks, is described in the Digital

Storage Architecture Disk Format Specification (DSDF) and/or  the

controller's  functional  specification.  The pool of replacement

blocks is typically distributed  throughout  the  disk,  so  that

revectoring  of  logical block accesses to replacement blocks has

negligible    impact     on     performance.      See     Section

section(bbr__overview),  "Bad  Block  Replacement,"  and DSDF for

more information on bad block replacement.

4.12.1  Replacement Control Table (RCT) Overview

The  logical blocks that contain the RCT are not guaranteed to be

"good." Since the RCT describes the bad logical block to replace-

ment  block mapping, mapping bad RCT blocks to replacement blocks

would present an insoluble recursion problem.  Instead  of  using

replacement  blocks  for  bad RCT blocks, the RCT region actually

contains multiple copies of the RCT.  Hosts obtain  the  size  of

each  RCT  copy  and  the  number  of RCT copies via the GET UNIT

STATUS command.  The last copy of the RCT may be truncated.   See

the   Digital  Storage  Architecture  Disk  Format  Specification

(DSDF).  Note that the disk  is  unusable  if  the  corresponding

block is bad in every copy of the RCT.

The  detailed  format  and  access  algorithms  for  the  RCT are

described in DSDF.  In general, however, each  copy  of  the  RCT

contains the following information:

1.  One entry per replacement block, identifying the logical

block, if any, that has been replaced by the replacement

block.

2.  Context  information  identifying the bad block replace-

ment operation, if any, that is currently in progress on

this unit.  This information is used to complete the bad

block replacement operation  if  the  host  and/or  con-

troller should crash in the middle of bad block replace-

ment.

Alternatively,  a  controller  may  use a nonstandard replacement

scheme or  some  scheme  other  than  bad  block  replacement  to

guarantee   that   the  logical  blocks  in  the  host  area  are

The mechanisms used by such a controller to  guarantee  that  the

ALGORITHMS AND USAGE RULES                              PAGE 4-36

DISK GEOMETRY AND FORMAT

host area logical blocks are "good" shall be totally invisible to

hosts, and shall not require host cooperation for their initiali-

zation,  use, or maintenance.  Such a controller or unit will not

provide a host accessible RCT.

4.12.2  Logical Block Contents

The  host  visible  portion  of  each logical block consists of a

fixed number of data bytes.  The number of data bytes  is  either

512  or  576,  determined when the disk volume is formatted.  All

logical blocks on a disk volume have  the  same  number  of  data

bytes.

The  data  bytes  are the only portion of logical blocks that are

directly host visible.  Certain other items, however, must be  in

each  logical  block as their presence is implied by various MSCP

functions:

1.  Each block shall contain a forced error indicator, so as

to properly implement and recognize  the  "Force  Error"

command modifier.

2.  Each  block shall contain a bad block indicator, so that

references to bad blocks can be  efficiently  revectored

to  the  proper replacement blocks.  This is unnecessary

if the controller does not use bad block replacement  to

provide a "perfect" host area.

Note  that  the forced error indicator is fully distinct from the

512 or 576 bytes of data in each block.  Blocks written with  the

"Force Error" command modifier shall, when read back, return both

correct data and the presence of a forced error.  Blocks  written

with  forced errors shall have similar error rate characteristics

as blocks written normally, without forced errors.  A forced  er-

ror  indicates  that  the  host or controller process writing the

block did not have fully valid data to  write  into  that  block.

The disk subsystem (controller and unit) shall, however, preserve

the data so as to avoid further data corruption.

ALGORITHMS AND USAGE RULES                              PAGE 4-37

DISK GEOMETRY AND FORMAT

4.12.3  Performance Implications

As  stated above, the host area of a disk is structured as a vec-

tor of logical blocks.  From a performance viewpoint, however, it

is  more  appropriate to view the host area as a four dimensional

hyper-cube.  The four dimensions are cylinder, group, track,  and

sector.  Thus, it is possible to decompose a logical block number

into a unique quadruple of numbers;  namely the block's  cylinder

number,  group number, track number, and sector number.  Cylinder

number is most significant and sector number  is  least  signifi-

cant.

Alternatively,  we  can  define  a track as consisting of a fixed

number of blocks, a group as consisting  of  a  fixed  number  of

tracks, and a cylinder as consisting of a fixed number of groups.

The position of a block within a track is the block's sector.

"Spindles" are physical entities across which both seek and rota-

tional latencies are unpredictable.  A unit consists  of  one  or

more spindles or spindle "groups."

The  terms sector, track, cylinder, and spindle all come from the

geometry of classical disk drives.  Groups can be  viewed  as  an

optimization  for  short seeks whose seek time is easily predict-

able.

At any particular instant, the set of logical blocks that are in-

stantaneously accessible is those blocks in all tracks  that  are

in  a  particular sector, group, and cylinder.  In the absence of

transfers to a different group or cylinder, this set  of  instan-

taneously  accessible  blocks  changes  over  time by keeping the

group and cylinder constant while incrementing the sector  modulo

the  number  of  blocks/sectors  in  a  track.   Referring to our

hyper-cube analogy, the set of instantaneously accessible  blocks

form a line parallel to the track axis.  This line moves parallel

to the sector axis, wrapping around when it reaches the  edge  of

the  hyper-cube.   A disk therefore provides cyclic access to the

blocks in a particular group and cylinder.

This  access structure to logical blocks implies that, to a close

approximation, the track to which a block belongs has  no  effect

upon  performance.   That  is,  switching  tracks within the same

group and cylinder effectively requires zero time.  Two  separate

transfers  in  the  same  group and cylinder have similar perfor-

mance, regardless of what tracks they are on.  To a  first  order

approximation,  two  separate  transfers on successive sectors of

different tracks in the same group and  cylinder  have  the  same

performance as a single, two sector (block) transfer.

ALGORITHMS AND USAGE RULES                              PAGE 4-38

DISK GEOMETRY AND FORMAT

Changing  cylinders,  however,  does  require a certain amount of

time.   The  amount  of  time  required  to  switch  between  two

cylinders  is approximated by a monotonically increasing function

of the difference between the two cylinder numbers.  The time  to

switch  between cylinders is typically not affected by whether or

not groups are also being changed.   After  switching  cylinders,

the  sector position of the disk (i.e., which sector's blocks are

instantaneously accessible) is unpredictable.

Changing   groups   also  requires  a  certain  amount  of  time.

Generally, the time to switch between groups in the same cylinder

is  no more than, and often less than, the time to switch between

one cylinder and the next.  If  cylinders  and  groups  are  both

changed  at  the  same  time, the time to switch groups is effec-

tively zero, as it is included in the time to switch cylinders.

When  changing  from  one  group  to  the next (successive) group

within the same cylinder, the time required to switch  groups  is

predictable.   Therefore the sector position following completion

of the group switch is also predictable, and transfers can be op-

timized  across  group  boundaries.  If a transfer to sector S in

group G is followed by a  transfer  in  group  G+1  of  the  same

cylinder, then sector S+1 (modulo the number of sectors/blocks in

a track) is the optimal sector for the new transfer and sector  S

is  the  least desirable.  The main effect of this is to optimize

continuous (spiral) transfers that cross group boundaries.

Note  that  what  has been described herein is the model for disk

logical geometry, which may have  a  tenuous  relationship  to  a

disk's  actual physical geometry.  Disk designers should devise a

logical to physical geometry mapping which optimizes the accuracy

of  the  model  herein described.  This will generally be done as

follows.  Head or track switches that  effectively  require  zero

time  (i.e., that require less than the intersector time) will be

reported as logical MSCP tracks.  Head or track switches that re-

quire  significant  amounts  of  time  will  be  divided into two

classes:  those that require only a little time  (typically  less

than  one rotation) and whose time is predictable, and those that

require somewhat more time and/or a lot of  time.   The  switches

that  require only a little time will be reported as logical MSCP

groups.  The switches that require more time will be reported  as

logical  MSCP  cylinders,  where the switches should be mapped to

cylinders in such a manner as to minimize the amount of time  re-

quired to switch between cylinders that are numerically close to-

gether.

The  effect  of  this geometry on multiblock transfers is as fol-

lows.  A multiblock transfer requires  a  certain  minimum  time,

which  is  the  time  to  transfer  one block or sector times the

number of blocks in  the  transfer.   Crossing  track  boundaries

ALGORITHMS AND USAGE RULES                              PAGE 4-39

DISK GEOMETRY AND FORMAT

requires  no additional time.  Crossing a group boundary requires

a small, relatively fixed additional amount of time.   This  time

is  typically  less  than  the  time  to transfer an entire track

(i.e., less than one rotation).  Crossing a cylinder boundary re-

quires a somewhat larger additional amount of time.  This time is

typically at least the time to transfer an entire track (i.e., at

least one rotation).

The  effect  of  this  geometry  on  host allocation policies for

random-access  files  is  as  follows.   Whenever   possible,   a

random-access file should be allocated within a single group.  If

this is not possible, the host should try to allocate it within a

single  cylinder.   If this is also not possible, the host should

allocate it in the minimum number of adjacent cylinders.

When a block has a high probability of being accessed immediately

after another block, hosts should attempt to allocate both blocks

in  the  same  group  or,  if  that  is not possible, in the same

cylinder.  If both blocks cannot be  allocated  within  the  same

cylinder,  then they should be in cylinders that are as close to-

gether as possible.

Hosts  obtain  the size of a unit's tracks, groups, and cylinders

from the GET UNIT STATUS command's end message.   Some  of  these

disk  geometry  concepts  may not apply to all disk units.  Units

report the fact that a concept doesn't apply  by  specifying  its

size  to  be the same as the next larger concept.  For example, a

disk unit that doesn't have groups would specify  one  group  per

cylinder,  implying  that groups and cylinders are the same size.

Note that the value of zero is reserved  and  illegal  for  track

size, group size, and cylinder size.

Track  size,  in addition to being useful for performance predic-

tion, is also used in the bad block replacement algorithm  speci-

fied   in   the   Digital   Storage   Architecture   Disk  Format

Specification  (DSDF).   It  is  possible  that  the  track  size

appropriate for performance considerations will be different from

the track size required by bad block replacement.   If  this  oc-

curs,  the  track size required by bad block replacement shall be

specified, as the performance effects are a secondary  considera-

tion.

Note that all of this performance oriented disk geometry only ap-

plies to the host area of a disk unit.  The concepts need not ap-

ply to the Replacement Control Table (RCT) and any other areas of

a disk, as access to those areas  is  not  performance  sensitive

and/or not host visible.

ALGORITHMS AND USAGE RULES                              PAGE 4-40

DISK GEOMETRY AND FORMAT

4.13  BAD BLOCK REPLACEMENT

Bad block replacement is a technique used with disk class devices

to present each unit as a single logically contiguous set of  us-

able blocks.

A unit may operate in one of two modes:  Controller Initiated Bad

Block Replacement or Host Initiated  Bad  Block  Replacement.   A

unit's  mode  is  determined by whether or not the controller re-

quires host assistance in performing dynamic bad  block  replace-

ment  for  that  unit.   Controllers report this to hosts via the

"Controller Initiated Bad Block Replacement" unit flag (see  Sec-

tion section(unit___flags), "Unit Flags" for more detail).

For  a  complete  description  of  Controller Initiated Bad Block

Replacement, see Chapter chapter(cntrlrinitbbr), "Controller Ini-

tiated Bad Block Replacement."

For  a  complete description of Host Initiated Bad Block Replace-

ment, see the Digital Storage  Architecture  Disk  Format  (DSDF)

specification section on "Host Initiated Bad Block Replacement."

4.14  WRITE PROTECTION

There  are  three  kinds of write protection that can apply to an

MSCP device:  Hardware Write Protection, Software  Write  Protec-

tion,  and  Data  Safety  Write  Protection.   Each kind of write

protection is controlled from a separate source as follows:

Hardware Write Protection

Hardware  Write Protection is controlled by the user

or operator manipulating the  unit's  write  protect

mechanism.   The status of Hardware Write Protection

may change at unpredictable times in response to hu-

man intervention.

When Hardware Write Protection is established (i.e.,

when a  user  activates  the  unit's  write  protect

mechanism),  the  controller  shall provide a smooth

transition to the write protect state.  That is, the

controller   shall  complete  all  write  operations

ALGORITHMS AND USAGE RULES                              PAGE 4-41

WRITE PROTECTION

(commands) that it has already initiated on the unit

before  actually prohibiting writes.  Note, however,

that the controller should  immediately  reject  any

new write operations that it receives after the user

activates the unit's write protect mechanism.  Write

operations  that  the controller received before the

write protect  mechanism  was  activated,  but  that

haven't  been  initiated yet, may either be rejected

or completed at the controller's  option.   The  end

result of this is that each individual write command

or operation is either completed in its entirety  or

else  rejected  before any of its data is written to

the unit.  Note that it is not possible  for  either

the  controller  or  the  host  to perform bad block

replacement when a unit is Hardware Write Protected,

since  it requires writing to the unit's Replacement

Control Table.

Software Write Protection

Software  Write Protection is controlled by the host

or hosts.  Host(s)  set  or  change  the  status  of

Software Write Protection via the ONLINE or SET UNIT

CHARACTERISTICS commands ("Enable Set Write Protect"

modifier  and  "Software  Write Protect" unit flag).

In general, the detailed causes  of  Software  Write

Protection  are  matters  of  host policy.  However,

Host Initiated Bad Block Replacement,  as  described

in  the  Digital  Storage  Architecture  Disk Format

Specification  (DSDF),  requires  that   hosts   use

Software  Write Protection to emulate certain causes

of Data Safety Write Protection.

Unlike  the transition to Hardware Write Protect the

transition to Software Write Protect is a  naturally

smooth process since it is established by Sequential

commands (i.e., ONLINE  and  SET  UNIT  CHARACTERIS-

TICS),  which implies that all write operations will

have been completed prior to Software Write  Protec-

tion being established.

Data Safety Write Protection

Data  Safety  Write  Protection is controlled by the

MSCP server.  It is set whenever some  condition  in

the unit or volume prevents reliable modification of

ALGORITHMS AND USAGE RULES                              PAGE 4-42

WRITE PROTECTION

data on the volume.  Possible causes include:

o  An incomplete bad block replacement.

o  An invalid RCT.

o  The  unit  is  only  capable  of reading the

volume's  format  (e.g.,  a   single-density

volume in a double-density drive).

Note  that there can be multiple causes for a unit

being Data Safety Write  Protected.   Furthermore,

some   causes   represent   errors   while  others

represent normal occurrences.

Each individual cause can only be established when

a unit is  brought  "Unit-Online." Causes  can  be

eliminated  -- possibly allowing the unit to cease

being Data Safety Write  Protected  --  while  the

unit  remains  "Unit-Online." However,  the server

can only establish a new cause for the unit to  be

Data Safety Write Protected by forcing the unit to

be "Unit-Available" to all  class  drivers.   Thus

class  drivers are guaranteed that a unit will not

spontaneously become Data Safety  Write  Protected

while the unit is "Unit-Online."

A  unit's  Write  Protect Status Display Mechanism shall indicate

the "inclusive or" of all forms of write  protection.   That  is,

the  display shall indicate that the unit is write protected when

it is Hardware or Software or Data Safety Write Protected.

4.15  COMPARE OPERATIONS

MSCP includes the following kinds of compare operations:

1.  The COMPARE HOST DATA command.

2.  Read-compare  operations, invoked by the "Compare Reads"

unit flag or by the "compare" modifier on  a  READ  com-

mand.

ALGORITHMS AND USAGE RULES                              PAGE 4-43

COMPARE OPERATIONS

3.  Write-compare   operations,   invoked  by  the  "Compare

Writes" unit flag or by  the  "compare"  modifier  on  a

WRITE command.

The  operation  of these different types of compare operations is

described below.  Note that all of the compare operations  report

the  first  difference or other error starting from the beginning

of the transfer.  Therefore the compare operation at the  end  of

the  transfer may be aborted if a difference is discovered at the

beginning.

The COMPARE HOST DATA command is used to verify that data in host

memory matches data on a unit.  The data  is  obtained  from  the

unit  in  the manner that is most convenient or efficient for the

controller.  In  this  respect  the  COMPARE  HOST  DATA  command

operates identically to a READ command.  Unlike the READ command,

the data is not transferred to host memory.  Instead, data is ob-

tained  from  host  memory and compared against the data obtained

from the unit.  Upon completion of  the  command  the  controller

reports  whether  the  data was identical or different.  The data

being different is reported as a "Compare Error" in the command's

end  message.   No  error log message is generated as this is not

considered to be a "significant" error (since it can be delibera-

tely  caused  by  user programs).  The controller may attempt re-

tries or not at its option.  As  the  retries  will  always  fail

(assuming the host is not modifying the buffer while the transfer

is in progress), retries will have a deleterious effect  on  per-

formance  but  will  in  no  way  effect the final outcome of the

operation.

Read-compare  and  write-compare  operations are performed at the

conclusion of the appropriate transfer commands  to  verify  that

the  data  was correctly transferred and that the data can now be

obtained from its destination.  The general algorithm used is  to

obtain  the  data from its destination and compare it against the

data reobtained from its source.

In  particular,  during  a  read-compare operation the controller

reobtains the data from the unit and compares it against the data

obtained  from  host  memory.   Conversely, for the write-compare

operation the controller obtains the data from the unit and  com-

pares it against the data reobtained from host memory.

An   additional   requirement  for  performing  read-compare  and

write-compare operations is that the controller  should,  to  the

maximum  extent  allowed  by  its  architecture, operate all data

transfer paths  in  the  opposite  direction  from  that  of  the

ALGORITHMS AND USAGE RULES                              PAGE 4-44

COMPARE OPERATIONS

original  transfer,  in  order  to  maximize  the  probability of

detecting a transfer error.

Ideally, to operate all transfer paths in the opposite direction,

during a read-compare operation, the controller would obtain  the

data  from the host and send it to the drive, and the drive would

actually compare the data against the original.  Conversely, dur-

ing a write-compare operation, the data would be sent to the host

port, which would actually perform the comparison.  However, nei-

ther implementation is currently feasible since no drive nor com-

munications mechanism provides such comparison capabilities.   So

for now controllers must perform the actual compare functions.

If  a  read-compare  or  write-compare  operation fails, the con-

troller shall  interpret  this  as  implying  that  the  original

transfer  failed  and retry the original transfer if appropriate.

If the controller successfully obtains the data from  its  source

and  destination,  but the data is different, then the controller

shall retry the original transfer and report the compare error in

an  error log message.  If the controller cannot successfully ob-

tain the data from its destination, but the error is one that may

be  eliminated by rewriting the data to the destination, then the

controller shall also retry the original transfer and report  the

error  (from the attempt to obtain the data from its destination)

in an error log message.  All other errors need not  be  retried,

but  shall  be reported in an error log message.  The only excep-

tion to the above is  commands  that  have  the  "Suppress  Error

Recovery"  modifier  set.   The controller may or may not, at the

controller's option, retry the original transfer if a compare er-

ror occurs in such a command.

For  example,  "Data Errors," such as an uncorrectable ECC error,

shall be retried on write-compare operations.  They need  not  be

retried  on read-compare operations, since an unrecoverable "Data

Error" implies that the READ itself will fail.  "Compare  Errors"

shall  always be retried.  Note that the controller need not dis-

criminate among types of errors -- it may always retry all errors

during  read-compare  or  write-compare operations, regardless of

whether or not the error will inhibit the original transfer.

The number of retries required for read-compare and write-compare

operations is controller  dependent.   However,  all  controllers

shall  retry  such operations at least once.  The exact number of

retries that a controller implements should be  chosen  based  on

undetected error rate characteristics.  The controller may either

retry the entire transfer, or else only retry  the  portion  that

includes the error.

ALGORITHMS AND USAGE RULES                              PAGE 4-45

COMPARE OPERATIONS

4.16  SPECIAL DRIVE TOPOLOGIES

4.16.1  Multiaccess Drives

A  multiaccess  drive is a drive that may be physically connected

to more than one controller simultaneously via separate  physical

paths  (ports).  The primary purpose of the multiaccess drive ca-

pability is to enhance the availability of data by providing  al-

ternate  access  paths  to the data, thereby safeguarding against

individual host, controller, or communications mechanism failure.

High  availability  systems  typically utilize multiaccess drives

and the replication of some or all of the other major system com-

ponents  in order to recover from such failures without human in-

tervention.

Although  a multiaccess drive may be physically connected to more

than one controller, only one controller  at  any  one  time  may

establish  the  logical connection (i.e., bring it online) neces-

sary for any unit of the drive to become "Unit-Online"  or  under

the "Exclusive Access" of a host via that controller (see Section

section(unit__states), "Unit States").  Note  that  the  detailed

definition  of  the logical connection between a controller and a

drive is device dependent, and therefore not the concern of  this

specification.

Once  the  logical  connection  between a multiaccess drive and a

controller is established, the unit of a drive or the units of  a

multiunit drive or formatter (see Section section(multidrvsfmts),

"Multiunit Drives and Formatters") can only be accessed via  that

same controller and MSCP server.  The unit or units therefore be-

come inaccessible (i.e., "Unit-Offline") via all other  controll-

ers to which the multiaccess drive is physically connected.

A  controller may only retain the logical connection with a drive

as long as one or more units of that drive are  "Unit-Online"  or

under  "Exclusive  Access"  via that controller.  That is, a con-

troller shall release its logical connection with a drive as soon

as  all  units of the drive cease to be "Unit-Online" to any host

or under the "Exclusive Access" of a particular host.  Host class

drivers  may therefore switch access paths to a multiaccess drive

from one controller to another under program  control  by  making

all units of the drive "Unit-Available" through the controller to

ALGORITHMS AND USAGE RULES                              PAGE 4-46

SPECIAL DRIVE TOPOLOGIES

which the drive is currently logically connected,  then  bringing

one  of  the  units "Unit-Online" or under "Exclusive Access" via

the other controller.

A  controller  shall  also release the logical connections to all

units of a multiaccess drive whenever  the  host  access  timeout

(specified  via  the  SET CONTROLLER CHARACTERISTICS command) ex-

pires, in order to permit access to the drive via another  access

path.  See Section section(hostaccestimo), "Host Access Timeouts"

for complete details.

The  logical  connection  between  a controller and a multiaccess

drive must exist in order  for  any  unit  of  the  drive  to  be

"Unit-Online"  or  under  "Exclusive  Access" with respect to any

host.  However, certain status information must be  available  to

all  controllers  without  the existence of a logical connection.

The specific cases are:

1.  When no units of the drive are "Unit-Online" to or under

the "Exclusive Access" of any  class  driver  (that  is,

when  they  are all "Unit-Available" or "Unit-Offline"),

the multiaccess drive shall report its  existence,  unit

number(s),  and certain characteristics to all controll-

ers to which it is attached.  The  actual  mechanism  by

which  this information is reported is device dependent,

and therefore may use such schemes as  time-multiplexing

or  polling  of  a  single communication path.  However,

controllers shall present the conceptual view  to  hosts

that  this  information  is  reported  continuously  and

simultaneously to all controllers.  The drive character-

istics  that shall be reported are those returned in the

AVAILABLE attention message and in the GET  UNIT  STATUS

and  SET  UNIT  CHARACTERISTICS command end messages for

the current state of the drive's units ("Unit-Available"

or "Unit-Offline").

2.  When  a drive is logically connected to some controller,

and that controller receives a  DETERMINE  ACCESS  PATHS

command  for one of the units that is "Unit-Online," the

drive shall report its existence, unit number, and  cer-

tain  characteristics  to all other controllers to which

it is  attached.   The  characteristics  that  it  shall

report  are  those returned in the ACCESS PATH attention

message.  As detailed in Section section(determineades),

"DETERMINE ACCESS PATHS Command, Description" successful

communication of the ACCESS PATH attention  message  may

be  probabilistic rather than guaranteed.  Note that the

drive need only make this report when instructed  to  do

so by the controller.

ALGORITHMS AND USAGE RULES                              PAGE 4-47

SPECIAL DRIVE TOPOLOGIES

Note  that  those  two cases are only of concern with multiaccess

drives, as a single access drive cannot have the problem of  try-

ing  to  communicate with two or more controllers at once.  Also,

if a multiaccess drive is logically connected  to  a  controller,

and  that controller never receives a DETERMINE ACCESS PATHS com-

mand for a "Unit-Online" unit of that drive, then that drive need

not  communicate in any way with any other controller.  The drive

need not even report its existence until the  logical  connection

is  broken  (i.e.,  it  ceases  to be "Unit-Online" or under "Ex-

clusive Access") or the controller receives  a  DETERMINE  ACCESS

PATHS command.

4.16.2  Multiunit Drives and Formatters

A multiunit drive is a single physical disk drive that appears as

several independent units to hosts.  So-called fixed plus  remov-

able  disk  drives,  providing  one  removable  disk unit and one

nonremovable disk unit, are the most common example of  multiunit

drives.   A  multiunit  formatter is a single set of interface or

read/write electronics that connects several  otherwise  indepen-

dent  units  to  controllers.  Multiunit formatters are most com-

monly used with tape  drives,  although  there  is  nothing  that

prevents their use with disk drives as well.

All  the  units  of a multiunit drive or formatter share a single

access path to controllers.  This implies that all of  the  units

must  be  "connected"  to the same controller.  In particular, if

one unit of a multiunit drive or formatter  is  "Unit-Online"  or

under  "Exclusive  Access"  via  a controller, then all the other

units of the multiunit drive or formatter may only be accessed by

that   same   controller.    That   is,   the   other  units  are

"Unit-Offline" to  all  other  controllers.   Awareness  of  this

characteristic  is critical for high availability systems -- if a

failed operation on a multiaccess  unit  is  to  be  retried  via

another  controller, and the unit is part of a multiunit drive or

formatter, then all units of  the  drive  or  formatter  must  be

switched to the other controller.

Some  units  of  a multiunit disk drive may share mechanical com-

ponents as well as interface electronics.  Such units are said to

share  a spindle.  That is, the units must either be all spinning

or all not spinning, just as if the units shared a drive motor or

spindle  (which they typically will).  Such units must also share

a single Run/Stop switch,  since  they  are  always  spun-up  and

spun-down  together.   Hosts  must  also  be aware of units which

ALGORITHMS AND USAGE RULES                              PAGE 4-48

SPECIAL DRIVE TOPOLOGIES

share a spindle, as dismounting one such unit requires  that  all

units sharing the same spindle be spun-down.

The units of a multiunit drive or formatter are identified by the

"multiunit code" unit characteristic field.   Hosts  obtain  this

two  byte field via the AVAILABLE attention message or in the end

message of a GET UNIT STATUS, ONLINE, or SET UNIT CHARACTERISTICS

command.  The low byte of this field contains a controller depen-

dent encoding of the access path between the controller  and  the

drive.   The high byte of this field contains a controller depen-

dent encoding of the spindle, on a particular access  path,  that

the  unit  uses.   Controllers  may  use any encoding whatsoever,

provided that each access path and each spindle (within an access

path)  has a unique value.  Note that the access path byte is im-

plicitly qualified by the controller's or MSCP server's identity,

and  that  the spindle byte is implicitly qualified by the access

path.

Hosts  use  the  "multiunit  code" field as follows.  When a host

decides to spin-down a unit, it scans all other  units  that  are

"Unit-Online"  via the same MSCP server for those units whose en-

tire "multiunit code" field (both bytes) matches the  unit  being

spun-down.  Such units, if any, share a spindle or other mechani-

cal components with the unit being spun-down, so that  they  must

be  spun-down together.  When a host decides to access a unit via

a different  controller,  it  scans  all  other  units  that  are

"Unit-Online"  via the same MSCP server for those units whose low

byte of  the  "multiunit  code"  field  matches  the  unit  being

switched.  Such units, if any, share an access path with the unit

being switched, so that they must also be  switched  to  the  new

controller.

Note  that the low byte of the "multiunit code" field (the access

path) is meaningless for units that are inherently restricted  to

a  single  controller.  Controllers may return any fixed value as

the access path encoding for such units, provided that it doesn't

duplicate the value returned for any units on the same controller

that are not inherently restricted to a single controller.

This  use  and  format  of the "multiunit code" field implies the

following architectural restrictions on all controllers:

1.  All  units that share a spindle or other mechanical com-

ponents shall also share an access path.  Note that this

is  an  essential  restriction for multiunit drives, re-

gardless of how shared components are communicated.   If

units that share a spindle did not share an access path,

then they could be simultaneously "Unit-Online" via dif-

ferent controllers, making it impossible to coordinate a

simultaneous spin-down.

ALGORITHMS AND USAGE RULES                              PAGE 4-49

SPECIAL DRIVE TOPOLOGIES

2.  There  is  a maximum of 256 access paths per controller.

In the absence of multiunit drives or  formatters,  this

implies a maximum of 256 units per controller.

3.  There  is a maximum of 256 spindles per access path.  In

the absence of shared spindles, this implies  a  maximum

of 256 units per access path or formatter.

4.17  CONTROLLER AND UNIT IDENTIFIERS

MSCP requires that all controllers and drives have unique identi-

fiers, called controller identifiers and unit  identifiers.   The

structure of these identifiers is as follows:

31 0

+-------------------------------+

| unique device number |

+-------+-------+ +

| class | model | |

+-------+-------+---------------+

The fields in a controller or unit identifier are as follows:

unique device number

Uniquely  identifies the device among all devices of that

same class and model.  The device serial number could  be

used as the "unique device number," though that isn't re-

quired.

model

Identifies  the  exact  model of the subsystem within its

class.  All valid class and model codes are nonzero,  im-

plying  that  all  valid  identifiers are nonzero.  Table

table(controllrmodel), "Mass Storage  Controller  'Model'

Values"contains the "model" values for both disk and tape

controllers.     Tables    table(diskdriveiden),    "Disk

Drive/Media   Identifier  Values",  table(tapedriveiden),

"Tape     Drive/Media     Identifier     Values",     and

table(medloaderiden),  "Media  Loader  Identifier Values"

contain the "model" values  for  disk  and  tape  drives,

respectively.

class

ALGORITHMS AND USAGE RULES                              PAGE 4-50

CONTROLLER AND UNIT IDENTIFIERS

Identifies  the type of the subsystem -- controller, disk

drive, etc.  Table table(contrlunclass), "Controller  and

Unit 'Class' Values" contains the "class" values for both

controllers and drives.

The Storage Systems Architecture Group is responsible for assign-

ing new values.  Requests for additions to the tables  should  be

addressed  to SSAG::SSAG.  Note that different units of multiunit

drives are distinguished by having different "model" bytes;   the

"class"  and  "unique device number" fields are typically identi-

cal.  Note also that all MSCP servers for the same  device  class

within the same controller shall return the same controller iden-

tifier.

As  previously  stated,  MSCP  requires  that controller and unit

identifiers be unique across all  devices  accessible  via  MSCP.

This  clearly cannot be checked by controllers.  Controllers can,

however, enforce unique unit identifiers across  the  units  that

are  attached  to  themselves.   This is done using the following

algorithms:

1.  Controllers  should detect and respond to duplicate unit

identifiers across all units whose unit identifiers  the

controller  can  obtain,  including all units that would

otherwise          be          "Unit-Online"          or

Detection  of a duplicate unit identifier on one unit of

a multiunit drive is treated as a duplicate unit identi-

fier condition on all other units that share one or more

of the following components with the unit having the du-

plicate unit identifier:

a.  A unit number select mechanism.

b.  A Run/Stop or Load/Unload switch.

c.  A spindle or other mechanical components.

Note  that  duplicate  unit identifiers are detected re-

gardless of the state of a unit's Run/Stop  or  Load/Un-

load switch.

2.  Whenever  a controller becomes aware of a duplicate unit

identifier, it immediately spins-down all units with the

duplicate   identifier   and   forces   them  to  remain

spun-down.  The controller spins-down the units  regard-

less  of  their current state.  The controller typically

forces them to remain spun-down by  spinning  them  down

again whenever an operator spins them up.

ALGORITHMS AND USAGE RULES                              PAGE 4-51

CONTROLLER AND UNIT IDENTIFIERS

3.  The    controller    reports    "Unit-Offline   (subcode

'Inoperative')" status for all units with duplicate unit

identifiers.  In addition, if the unit might potentially

be  connected  to  another  controller,  the  controller

should  flag  the presence of the duplicate unit identi-

fier in the drive.  Other controllers,  if  any,  should

check   this   flag   and   also   treat   the  unit  as

"Unit-Offline" if the flag is set.

Whether or not a controller does in fact check for duplicate unit

identifiers is controller dependent.  Note that a duplicate  unit

identifier is a drastic failure, indicative of some otherwise un-

diagnosed hardware malfunction.

4.18  MEDIA TYPE IDENTIFIERS

Controllers return a media type identifier for each unit accessi-

ble via that controller.  This identifier encodes two  pieces  of

information:

1.  The  preferred  device  type name for use with the unit.

These two alphabetic characters are conventionally  used

with  the unit number and a controller designator as the

fully qualified  operating  system  identifier  for  the

unit.

2.  The  name  (product name) of the media used on the unit.

This name should be printed on the  unit's  front  panel

and  on  all  removable  media that may be used with the

unit.

The  primary reason for returning this information is to simplify

operating system support for generic device allocation.

The  media type identifier returned via MSCP is a 32 bit quantity

encoded as follows:

31 26 21 16 11 7 6 0

+----+----+----+----+----+------+

| D0 | D1 | A0 | A1 | A2 | N |

+----+----+----+----+----+------+

where the fields are as follows:

ALGORITHMS AND USAGE RULES                              PAGE 4-52

MEDIA TYPE IDENTIFIERS

D0

D1  The  preferred  device type name for the unit.  D0 and D1 are

five bit fields, each encoding one alphabetic character.  "A"

is  encoded  with the value 1, "B" with the value 2, etc.  D0

encodes the left character of the device type  name,  D1  the

right character.

A0

A1

A2

N   The  name  of  the media used on the unit.  A0 through A2 are

five bit fields, each encoding  an  alphabetic  character  or

null.  "A" is encoded with the value 1, "B" with the value 2,

etc.  Zero represents a null or the absence of  a  character.

One  to  three characters of the media name are encoded, left

justified, in A0 through A2.  N is a seven bit field contain-

ing the value of two decimal digits.

Note  that  the  encoding of the media name assumes that the name

consists of one, two, or three alphabetic characters followed  by

exactly  two  digits  (i.e., ann, aann, or aaann).  MSCP requires

that the product names for all mass  storage  devices  adhere  to

this format.

Currently  defined  device  type and media names (i.e., currently

defined media type identifier values) are listed in Appendix  ap-

pendix(ctruntmedids),   "Controller,   Unit,   and   Media   Type

Identifiers." Tables  table(diskdriveiden),   "Disk   Drive/Media

Identifier  Values"  and  table(tapedriveiden), "Tape Drive/Media

Identifier Values" contain the "media identifier" values for disk

and  tape drives, respectively.  The Storage Systems Architecture

Group is responsible for assigning new values.  Requests for  ad-

ditions to the tables should be addressed to SSAG::SSAG.

ALGORITHMS AND USAGE RULES                              PAGE 4-53

CONTROLLER-BASED CACHE

4.19  CONTROLLER-BASED CACHE

The  purpose  of a controller-based cache is to minimize the mean

response time for the aggregate set of I/O requests  serviced  by

the  controller.   Controller-based caches are intended to be in-

visible to the user except for this performance improvement.  Im-

plementations  shall  ensure  the  data  integrity with the cache

employed is comparable to the backing-store  even  through  power

failures,  so  that  the only reason to use or avoid the cache is

performance.  Implementing controller-based cache does not  waive

any  normal  storage  device  responsibilities as defined in this

specification.

This  implies that a cached device has the same responsibility as

any other device in terms of setting the forced error flag in  an

LBN  if  the  data  integrity  of that LBN is in question.  If it

knows corrupt data exists on the device but does not  know  which

data,  it  shall  assume  the  Offline/Unit inoperative state and

maintain this state even through power failure.  In this case, it

must  provide a diagnostic or DUP path for recovery of user data.

No single point of failure may allow the unreported return of the

wrong or corrupt data.

The  following  are  implementation examples that demonstrate the

implications of these responsibilities.

o  If  storage  units  are  dual ported to two or more con-

trollers, the controllers will write data  to  the  unit

backing-store  as well as the cache before returning the

command complete response so that the failure of one  of

the  controllers  cannot  cause loss of user data or its

consistency.

o  Controllers  whose  cache may lose user data or its con-

sistency in the event of power failure  will  write  the

data  to  the  backing-store as well as the cache before

returning the command complete response.   For  commands

with the compare modifier, the command complete will not

be sent until the compare has been made with the backing

store.

ALGORITHMS AND USAGE RULES                              PAGE 4-54

CONTROLLER-BASED CACHE

o  Controllers  whose  cache  is capable of maintaining the

user data integrity and its consistency at a level  com-

parable   to   the  backing-store,  even  through  power

failure, can return the  command  complete  response  as

soon  as the data is in the cache, before writing to the

backing-store.  For commands with the compare  modifier,

the  command  complete  response may also be returned as

soon as the user data is compared to the cache data.

o  A  controller-based cache implementation can degrade its

cache actions in the event of component failure as  long

as  the specified data integrity is maintained.  For ex-

ample, if a controller implementing  a  cache  that  can

maintain  its integrity through power failure recognizes

its cache memory integrity is degrading,  it  can  flush

the  cache  and  revert to a mode where it writes to the

backing-store as well as the cache before returning  the

command complete response.

The manner in which users access their data varies greatly.  Some

access it totally randomly, others  walk  through  files  sequen-

tially.  Some access the same data many times, others access each

piece of data only once.  These differing access patterns have  a

great effect on the performance of a cache algorithm.

Transfer  commands  are provided with a "Cache Access Attributes"

field so that the cache action can be optimized  for  these  pat-

terns.   This  is an optional use field that allows users to pro-

vide  "advice"  as  to  the  access  patterns  they  expect.    A

particular cache implementation, knowing its specific caching al-

gorithm and its current state, may make use of this advice to op-

timize  its  performance.   No  specific  action  is mandatory or

architecturally defined.  Each implementation shall  specify  its

actions.

Hosts  may  provide "Cache Access Attributes" only to controllers

which have returned the "Cache Support" controller flag set in  a

SET  CONTROLLER  CHARACTERISTICS  end  message.   Controllers not

providing this information to the host will  receive  the  "Cache

Access Attributes" default value.