CHAPTER 5

MSCP CONTROL MESSAGES

5.1  GENERIC CONTROL MESSAGE FORMAT

All  MSCP  control messages -- i.e., command, end, error log, and

attention messages -- consist of a 12 byte header and a  variable

length  parameter area.  The generic control message format is as

follows:

31 0

+--------------------------------+

| |

+------- control message --------+

| type specific |

+------- header --------+

| |

+--------------------------------+

| |

/ parameters /

/ /

| |

+--------------------------------+

The  device  class  (disk or tape) does not appear in the control

message.  It is implied by the connection or MSCP server  to/from

which the control message is sent/received.

Multibyte  numbers are stored least significant byte first (i.e.,

using the standard VAX number formats).

The  communications  mechanism conveys both the text of a message

and its length.  The receiver of a message  uses  the  length  to

verify that all required parameters are in fact present.

MSCP CONTROL MESSAGES                                    PAGE 5-2

GENERIC CONTROL MESSAGE FORMAT

The  communications  mechanism may restrict the allowable message

lengths.  For example, it might require that all messages have  a

fixed  length  (typically equal to the longest message supported)

or that the length be an even multiple of bytes.  For this reason

the message lengths defined by MSCP are minimum lengths.  Senders

may pad messages as necessary to  meet  communications  mechanism

length  restrictions.   The contents of the padding (that is, the

contents of any data past the end of the message formats shown in

this  document)  are  reserved  and follow the rules for reserved

fields defined in Section section(rsvdundefflds),  "Reserved  and

Undefined Fields" (i.e., such padding contains zeros).

MSCP CONTROL MESSAGES                                    PAGE 5-3

GENERIC CONTROL MESSAGE FORMAT

5.2  RESERVED AND UNDEFINED FIELDS

Reserved  fields  are those fields that are intended for possible

future extensions to MSCP.  The use of such fields  shall  follow

certain rules, in order to ensure that such future extensions can

be upwards compatible with  the  current  version  of  MSCP.   In

general,  the  sender of a message shall supply the value zero in

all reserved fields.  The action for a message  receiver  varies,

and is discussed below.

An  undefined  field  is just that -- its contents are controller

implementation dependent, and therefore cannot  be  used  in  any

meaningful  way  by class drivers.  Undefined fields are provided

in order to simplify controller  implementation.   Class  drivers

shall ignore the contents of undefined fields.

A  field,  as  used  in this discussion, may have any length.  In

particular, it may be an individual bit of a flags word  or  byte

as well as an entire byte, word, or whatever.

Class  drivers shall supply the value zero in the reserved fields

of all messages (commands) that they send to  a  controller,  and

shall also ignore the contents of reserved fields in all the mes-

sages (end messages, attention messages, and error log  messages)

that they receive from an MSCP server.  MSCP servers shall supply

the value zero in the reserved fields of all messages  (end  mes-

sages, attention messages, and error log messages) that they send

to class drivers.  MSCP servers shall either ignore the  contents

of  reserved  fields in the messages (commands) that they receive

from class drivers or verify that the  contents  are  zero.   The

command is treated as an "Invalid Command" ("reserved field" pro-

tocol  error;   see   "Invalid   Command"   status   in   Section

section(status__codes),  "Status  and  Event  Codes") if the con-

troller chooses to verify  and  the  contents  are  found  to  be

nonzero.   Whether  an  MSCP server verifies that reserved fields

are zero is controller dependent, and need not be consistent  for

all reserved fields.

Many  controllers generate command end messages by simply modify-

ing the commands' command  messages.   That  is,  the  controller

copies  a command message into an internal buffer, modifies it in

place during execution of the command, then sends  the  resulting

contents of the internal buffer as the command's end message.  To

simplify such an implementation, controllers  may  merely  "echo"

command  message  reserved fields when the corresponding field in

the end message should be zero.  More precisely, if some field in

the   end   message   of  a  command  should  be  zero,  and  the

MSCP CONTROL MESSAGES                                    PAGE 5-4

RESERVED AND UNDEFINED FIELDS

corresponding (same position) field in the command's command mes-

sage  is  a  reserved field, the controller may copy the reserved

field from the command message to the end message rather than ex-

plicitly zeroing the field in the end message.

The  above paragraphs have listed all of the allowable controller

actions when a controller  receives  a  command  message  with  a

nonzero  value  in  a reserved field.  That is, when a controller

receives a command message with a nonzero  value  in  a  reserved

field it shall do one of the following:

1.  Reject  the  command  as an "Invalid Command" ("reserved

field" protocol error;  see "Invalid Command" status  in

Section   section(status__codes),   "Status   and  Event

Codes").

2.  Totally  ignore  the  nonzero  contents  of the reserved

field.  That is, the command's execution,  results,  and

end  message  contents  are  totally  unaffected  by the

nonzero value.

3.  If  the  corresponding  field  in the end message should

have a zero value, echo the  contents  of  the  reserved

field  in  the command message as the value of the field

in the end message.  In all other  ways  totally  ignore

the  nonzero  contents  of the reserved field.  That is,

the command's execution, results, and  the  contents  of

all  other  fields  in the end message are totally unaf-

fected by the nonzero value.

Note  that  option 3 is primarily of use when a field is reserved

in both command and end messages.

It is recommended, although not required, that which of these op-

tions  a  controller  implements  be   based   on   whether   the

controller's   code   is  hard  (ROM  based)  or  soft  (loadable

microcode, implying it is easily altered).  The purpose  of  this

is  to  provide  the  maximum degree of error checking consistent

with ease of future extensions to MSCP.  Controllers  whose  code

is  soft  or  relatively easy to alter should implement option 1,

verifying that reserved fields are  in  fact  zero.   Controllers

whose  code is hard or difficult to alter should implement option

MSCP CONTROL MESSAGES                                    PAGE 5-5

RESERVED AND UNDEFINED FIELDS

3 when the corresponding fields in command and end  messages  are

both reserved and option 2 in all other cases.

Note  that  controllers shall implement option 1 whenever the in-

ternal functioning of the controller may be altered if a reserved

field contains a nonzero value.

MSCP CONTROL MESSAGES                                    PAGE 5-6

RESERVED AND UNDEFINED FIELDS

5.3  COMMAND MESSAGES

A  host class driver sends a command message to an MSCP server to

have a certain operation performed.  Note that  command  messages

are     flow     controlled     as     described    in    Section

section(flow__control), "Flow  Control"  and  associated  subsec-

tions.

The generic command message format is as follows:

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

| reserved | unit number |

+---------------+-------+--------+

| modifiers | caa | opcode |

+---------------+-------+--------+

| |

/ parameters /

/ /

| |

+--------------------------------+

The fields in the generic command message are as follows:

command reference number

A  32  bit,  unique, nonzero number used to identify host

commands.  Class drivers should supply a unique reference

number  in each command that they send to an MSCP server.

Command reference numbers are not interpreted in any  way

by  MSCP  servers.   Their purpose is to provide a unique

identifier by which  class  drivers  can  name  commands.

They  are used by class drivers to match end messages and

error log messages with the corresponding command message

and  to  identify  the  object of an ABORT or GET COMMAND

STATUS command.  A class driver may supply a zero  refer-

ence  number  if  it does not need to associate a command

with its end message.

Command reference numbers shall be unique across all com-

mands that are outstanding on the same connection.  I.e.,

they  shall be unique across all outstanding commands is-

sued by a single class driver (host)  to  a  single  MSCP

server.  The class driver may reuse a command's reference

number when the command is no longer outstanding -- i.e.,

after  receiving  the  command's  end  message  or  after

resynchronizing with the MSCP server.  Command  reference

MSCP CONTROL MESSAGES                                    PAGE 5-7

COMMAND MESSAGES

numbers  need  not  be unique for commands issued by dif-

ferent class drivers -- i.e.,  commands  issued  by  dif-

ferent  hosts or commands for different MSCP servers from

the same host.  Therefore  controllers  shall  internally

use the combination of a command reference number and the

connection on which  the  command  was  received  as  the

unique identifier of an outstanding command.

unit number

Identifies  the  specific unit within the device class to

which the message applies.   This  value  is  the  binary

equivalent  of  the  decimal unit number displayed by the

unit select mechanism.  Note that the unit  number  field

appears in all command messages with the exception of the

ACCESS NONVOLATILE MEMORY and SET CONTROLLER CHARACTERIS-

TICS commands.  It is reserved in those messages.

opcode

Identifies  the  operation  to  be  performed by the con-

troller.  The opcode implicitly specifies the length  and

format  of  the  message, including the interpretation of

any parameters that are present.

Appendix   appendix(opcdsflgsoffs),  "Opcode,  Flag,  and

Offset Definitions," Table table(cntrlmessopcd), "Control

Message Opcodes," lists the valid opcodes and their mean-

ings.

A  command  message  is  rejected as an "Invalid Command"

("invalid opcode" protocol error;  see "Invalid  Command"

status  in  Section  section(status__codes),  "Status and

Event Codes") if its opcode  is:   zero,  any  value  not

listed as a valid command message opcode in the "Standard

Command Messages" portion of Table  table(cntrlmessopcd),

or  an  optional  command listed in the "Optional Command

Messages" portion of Table table(cntrlmessopcd)  that  is

not supported by the controller.

The "Controller Implementation Specific Command Messages"

portion of Table table(cntrlmessopcd) shows  a  range  of

opcodes   reserved   for   implementation  of  controller

specific special purpose  functions.   Implementation  of

any or all of those opcodes is a controller dependent op-

tion.  Those opcodes will typically be  used  by  control

layers  internal  to a controller to invoke diagnostic or

maintenance type functions.  Controllers may  also  honor

MSCP CONTROL MESSAGES                                    PAGE 5-8

COMMAND MESSAGES

those  opcodes  from the host for use by manufacturing or

other external diagnostics.  In cases where such  an  op-

code  is  available  for  host  use,  the command message

format and operation associated with the opcode shall  be

described  in  the controller's functional specification.

Although not an  absolute  requirement  of  MSCP,  it  is

recommended  that  controllers  implement  a manually ac-

tuated control mechanism (e.g.,  the  installation  of  a

jumper)  that  explicitly  enables and disables those op-

codes available for host use in order to prevent uninten-

tional  execution  of  a  special purpose function during

normal operations.

A  command  message  received  from the host containing a

"Controller Implementation Specific Command Message"  op-

code  is  rejected  as an "Invalid Command" ("invalid op-

code" protocol error;  see "Invalid  Command"  status  in

Section section(status__codes), "Status and Event Codes")

if the controller:

1.  does not implement the opcode at all.

2.  implements the opcode for internal use only.

3.  implements a manually actuated control mechanism

that enables and disables host use of the opcode

and host use has not been explicitly enabled via

that mechanism.

caa  (cache  access  attributes)    -- for  transfer commands

only

The  cache  access  attributes  field may contain a value

that advises the controller-based cache  manager  of  the

user's  expected future access of data.  See Section sec-

tion(xfrcmdmesfmt), "Transfer Command Message Format."

modifiers

The  modifiers  field  contains bit flags that modify the

operation identified by the command's opcode, or zero  if

no     modifiers     are    specified.     See    Section

section(cmd_modifiers), "Command Modifiers," for more de-

tails.

MSCP CONTROL MESSAGES                                    PAGE 5-9

COMMAND MESSAGES

parameters

The  parameter  area  is  typically divided into distinct

fields that convey the information necessary  to  perform

the  desired  operation  in a defined manner.  The length

and content of the parameter area varies  depending  upon

the command's opcode.

Chapter   chapter(minimalcmdset),   "Minimal  MSCP  Command  Set"

describes the operation and command message format of the various

commands required to be supported by an MSCP disk controller.

The following section describes the command message format common

to many data transfer commands.

5.3.1  Transfer Command Message Format

Although  the  parameters  and  their layout are command (opcode)

dependent, many commands perform  data  transfers  and  thus  use

similar sets of parameters.  Therefore data transfer related com-

mand messages are typically laid out as follows:

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

| reserved | unit number |

+---------------+-------+--------+

| modifiers | caa | opcode |

+---------------+-------+--------+

| byte count |

+--------------------------------+

| |

+--- buffer ---+

| |

+--- descriptor ---+

| |

+--------------------------------+

| logical block number |

+--------------------------------+

The fields in the transfer command message are as follows:

command reference number

MSCP CONTROL MESSAGES                                   PAGE 5-10

COMMAND MESSAGES

unit number

opcode

caa (cache access attributes)

The Cache Access Attributes (caa) field is only used with

specified  transfer  commands.   The  individual  command

descriptions contained in Chapter chapter(minimalcmdset),

"Minimal MSCP Command Set" show the  commands  for  which

Cache Access Attributes are applicable.

The  Cache  Access Attributes field contains a value that

describes the access attributes of the data.  The content

of  this  field  is  not  intended to be a command to the

cache, but rather it is advice to the cache as to the ac-

cess  attributes  of the data which may affect the manner

in which the cache manages itself.  Since this  field  is

only  advisory  in nature, any device may ignore it.  The

implementor of the controller-based cache is  responsible

for  defining  the  action  an  implementation  takes  in

response to each Cache Access Attribute value.

Two  generic  attributes  are defined and four values are

assigned to the four combinations of these generic attri-

butes.  The assigned values are described in Appendix ap-

pendix(opcdsflgsoffs),  "Opcodes,   Flags,   and   Offset

Definitions,"  Table  table(cacheatribval), "Cache Access

Attribute Values." The attributes are as follows:

Random  -  A storage element implements prefetch

with the assumption that it is  worthwhile  pre-

fetching  the  data logically following user re-

quested data.

-  Random  should  be specified if the user be-

lieves this will not be  true  for  his  re-

quest.

-  Otherwise, Random should not be specified.

MSCP CONTROL MESSAGES                                   PAGE 5-11

COMMAND MESSAGES

Single  -  A  storage element implements a cache

with the assumption that it is worthwhile  cach-

ing data that has been previously accessed.

-  Single  should  be specified if the user be-

lieves this will not be  true  for  his  re-

quest.

-  Otherwise, Single should not be specified.

The zero value is defined as the default value.  Each im-

plementation shall define its  action  in  terms  of  the

generic  attributes in response to this value.  All unas-

signed values shall be acted upon as though they were the

default value, unless modified by the System Manager.

Some  users  may have unique access patterns that require

unique definition.  In these cases, a  user  may  be  as-

signed  one or more Cache Access Attribute values for the

application.  These  values  shall  be  assigned  by  the

Responsible  Architect.   Its  access  pattern definition

will be negotiated between the user  and  the  cache  im-

plementors.

A  system  manager  may modify a device action to a Cache

Access Attribute value at configuration  or  reconfigura-

tion.   This modification consists of changing the action

of a given value to that defined by another  value.   For

example,  the  system  manager  could  make the action in

response to value 85 hex be the same  as  the  action  in

response  to  value  80  hex.  Implementations that allow

such modification should allow all 256 Cache  Access  At-

tribute  Values to be modified, including those that have

not yet been assigned in this specification.

modifiers

See Section section(cmd_modifiers), "Command Modifiers."

MSCP CONTROL MESSAGES                                   PAGE 5-12

COMMAND MESSAGES

byte count

The total requested length of the data transfer in bytes.

This length shall meet both architectural constraints and

a server specified maximum.

An  MSCP  server  may,  at its option, return the maximum

byte count size that it supports in  the  SET  CONTROLLER

CHARACTERISTICS  end message (see Section section(setcon-

trolemf), "SET CONTROLLER  CHARACTERISTICS,  End  Message

Format"  for  default  values  used  if  the  size is not

provided by the server).  Class drivers should not  issue

commands with larger byte counts.  The server response to

a command whose byte count is larger  than  the  server's

maximum  is undefined.  If the server does not reject the

command, it is subject to the following constraints:

1.  The  server  may  not destroy hardware or do

any other action that  would  require  field

service attention.

2.  The  server  may  alter  host memory only in

response to a READ command,  and  then  only

that  portion  of host memory implied by the

command's buffer  descriptor  and  specified

byte count.  See Section section(hostbuffac-

ces), "Host Buffer Access."

3.  The  server  may alter data on the unit only

in response to a WRITE or ERASE command, and

then  only  that portion of the unit implied

by the logical block  number  and  specified

byte count.

4.  Any error reported in the command's end mes-

sage or in an error log  message  must  have

actually occurred at the position reported.

5.  With  one exception, any other commands that

may be outstanding at the same time  as  the

command  whose  byte  count is too large may

not be affected.  The one exception is  that

the  server  need not properly implement the

command timeout algorithm for any such  com-

mand,  not just the command whose byte count

is too large.

MSCP CONTROL MESSAGES                                   PAGE 5-13

COMMAND MESSAGES

Byte  counts  shall also meet the following architectural

constraints, which vary depending upon the device class.

For  tape class devices, if the "byte count" specified is

larger than a format dependent maximum block size or  the

byte  count  is  zero, the command is rejected as an "In-

valid Command" subcode  "invalid  byte  count"  parameter

error;   (see  "Invalid Command" status in TMSCP, Chapter

4, Section "Status Codes").

For  disk  class  devices, the requirements depend on the

contents of the "logical block number" field.

If  the "logical block number" field identifies a logical

block in the host area of  the  disk  volume  (i.e.,  the

"logical  block  number" is less than the "unit size" re-

turned in the ONLINE and  SET  UNIT  CHARACTERISTICS  end

messages),  then  the  "byte count" shall be less than or

equal to the following maximum byte count:

( unit size - logical block number ) * block size

where  "unit size" is the unit's host area size (returned

in the ONLINE and SET UNIT CHARACTERISTICS end messages),

"logical  block  number"  is the contents of the "logical

block number" field in the command  message,  and  "block

size"  is  the  volume's  block  size,  either 512 or 576

bytes.  The controller or MSCP server  shall  check  that

the  "byte count" is less than or equal to the above max-

imum.  That is, the controller or MSCP server  shall  re-

ject or terminate any transfer command that begins in the

host area of a disk volume and attempts to continue  into

the  volume's  Replacement Control Table (RCT).  The com-

mand is rejected as an "Invalid  Command"  ("byte  count"

parameter error) if this restriction is violated.

If  the "logical block number" field identifies a logical

block in the disk volume's RCT (i.e., the "logical  block

number"  is  greater than or equal to the "unit size" re-

turned in the ONLINE and  SET  UNIT  CHARACTERISTICS  end

messages),  then  the  "byte  count" shall be exactly the

block size (either 512 or 576 bytes).  The treatment of a

"byte  count"  that does not match the block size differs

depending on whether or not the controller  performs  bad

block   replacement.    Section   section(cibbrrctacces),

"Replacement Control Table Access" describes the action a

controller that performs bad block replacement will take.

If the controller does not perform bad block  replacement

and  a  different (than block size) "byte count" value is

provided, the controller may either attempt the  transfer

MSCP CONTROL MESSAGES                                   PAGE 5-14

COMMAND MESSAGES

with  the  specified "byte count" or else reject the com-

mand as an "Invalid Command" ("byte count" parameter  er-

ror).   If  the controller attempts the transfer, and the

"byte count" causes the  transfer  to  extend  past  some

boundary within the RCT, then the controller performs the

transfer up to that boundary and terminates  the  command

as  an  "Invalid Command" ("byte count" parameter error).

The location of the boundary or boundaries at  which  the

controller  terminates the transfer are controller depen-

dent.

For  all  disk  and  tape  transfer commands that contain

"buffer descriptors" (i.e., all transfer commands  except

ACCESS  and  ERASE),  the "byte count" shall also be less

than or equal to the size of  the  buffer  identified  by

"buffer  descriptor." Note  that "buffer descriptor," and

thus the size of the buffer, is inherently communications

mechanism  dependent.  The size of a buffer is not neces-

sarily available to the MSCP server until it attempts  to

transfer  past the end of the buffer.  A "Host Buffer Ac-

cess Error" status code is returned if the  "byte  count"

exceeds  the length of the buffer.  Note that such errors

are not necessarily distinguishable from other causes  of

"Host Buffer Access Errors."

For  disk  transfer  commands  only,  some communications

mechanisms may prohibit odd  "byte  count"  values.   Odd

"byte  count"  values  shall  be allowed in tape transfer

commands.  A "Host Buffer Access Error"  status  code  is

returned  if  the  "byte  count"  is  an illegal odd byte

count.  For disk transfer commands that do not use a host

buffer  (ACCESS  and ERASE), a controller may return this

status code, or it may round the byte  count  up  to  the

nearest whole sector size and perform the operation.

"Byte  count" values that exceed any of the architectural

maximum values described above may be detected either be-

fore  the  transfer is initiated or when the transfer at-

tempts to cross the boundary from legal to  invalid  byte

counts.   If  detected  before  the transfer is initiated

(the transfer is rejected), the  MSCP  server  shall  not

transfer  any  data  and  shall  return zero in the "byte

count" field of the end message.  If  detected  when  the

transfer  attempts to cross the boundary (the transfer is

terminated), the MSCP server shall transfer all  data  up

to  the  maximum  legal byte count and return the maximum

legal byte count in the "byte count"  field  of  the  end

message.   Data shall not be transferred past the maximum

legal byte count.  In either case the command is  treated

as  an  "Invalid Command" ("byte count" parameter error).

MSCP CONTROL MESSAGES                                   PAGE 5-15

COMMAND MESSAGES

Which algorithm an MSCP server uses  for  detecting  byte

counts that are too large is controller dependent.

buffer descriptor

Communication  mechanism  dependent identification of the

host buffer to use for the data transfer.   The  informa-

tion encoded in this 12 byte (96 bit) field includes:

o  A host identifier (port or node identification).

o  The name of a buffer on the host.

Note  that  the inclusion of a host identifier allows for

third party transfers.   The  buffer  descriptor  formats

used  by  various  communication mechanisms are listed in

Appendix  appendix(bufferdescfmt),   "Buffer   Descriptor

Formats."

logical block number

Not  present in tape transfer commands.  In disk transfer

commands, the logical block number (position) on the disk

volume  at  which to start the data transfer.  This value

shall not identify a block past the end of  the  volume's

Replacement  Control  Table  (RCT).  Section section(dis-

kgeomformt), "Disk Geometry and  Format,"  describes  the

mapping  of logical block numbers to disk volume regions.

If the command is a write operation and the controller is

performing  bad  block replacement, this value also shall

not identify a block within the volume's RCT (see Section

section(cibbrrctacces),  "Replacement  Control  Table Ac-

cess").  Any of these errors cause the command to be  re-

jected  as  an  "Invalid  Command" "logical block number"

parameter error.

5.3.2  Command Modifiers

The  allowable modifiers on a command are command (opcode) depen-

dent.  The individual command descriptions contained in  Chapters

chapter(minimalcmdset),    "Minimal   MSCP   Command   Set"   and

chapter(multihostfunct), "Multihost Support" list  the  allowable

modifiers  for  each command.  Modifiers that are only allowed on

MSCP CONTROL MESSAGES                                   PAGE 5-16

COMMAND MESSAGES

one  command  are  described  in  that   command's   description.

Modifiers that are common to many commands are described below:

Clear Serious Exception

Clears the "serious exception" condition on the specified

unit.  Ignored if there is no serious exception condition

in  effect.   Serious exceptions only occur on tape class

devices.  See Tape Mass Storage Control Protocol Specifi-

cation (TMSCP) for complete details.

Compare

Applicable   to   data   transfer  commands.   After  the

transfer, the data will be read back  from  the  transfer

destination  and verified against the original data reob-

tained from the  source.   Specifying  this  modifier  is

similar,  but  not  identical,  to following the transfer

command with a COMPARE HOST DATA command.  In particular,

if  the  compare operation fails, an error log message is

generated (if enabled) and the original  transfer  opera-

tion  retried.   (With  the COMPARE HOST DATA command, an

error log message is not  generated  on  compare  errors.

Retries are unnecessary, although innocuous and therefore

allowable.) See Section section(compare_opers),  "Compare

Operations,"  for  a  more  detailed  description of this

modifier's effects.

Express Request

Applicable to Non-Sequential commands.  This modifier re-

quests that the controller ignore its normal optimization

policies  in order to complete this command as quickly as

possible.  The exact implementation of  express  requests

is controller dependent -- in general the controller will

complete some or all of its outstanding  commands  before

completing an express request.

Use  of  express  requests disables the normal controller

guarantees that ensure that all commands are serviced  in

a  timely manner.  If express requests are repeatedly is-

sued, some or all other outstanding commands may time out

(i.e., never be completed).

MSCP CONTROL MESSAGES                                   PAGE 5-17

COMMAND MESSAGES

Express  requests do NOT override sequential command exe-

cution guarantees.  Some controllers may  completely  ig-

nore  the  express request modifier.  The exact treatment

of express requests should be described in a controller's

functional specification.

Force Error

Applicable  only to write commands directed to disk class

devices.  Causes the data to be written with  the  forced

error  indicator  set,  so  that all attempts to read the

data will fail.  The error will be  preserved  until  the

next  time  the block is written.  The forced errors pro-

duced with this modifier shall be recognized by the  con-

troller  as  deliberate  and  never reported to the error

log.  Note that the data shall be written to the block so

as  to be obtainable by subsequent READ commands, exactly

as if this modifier were not specified.  The only differ-

ence is whether subsequent READ commands will return suc-

cess or the forced error status code.

Suppress Error Correction

Suppresses  error correction mechanisms, such as ECC cor-

rection.  Error recovery mechanisms, such as retries, are

not affected by this modifier.  This modifier, in effect,

lowers the threshold at which an error is  considered  to

be  uncorrectable.   It typically lowers the threshold to

zero, although that is not required.  Since error correc-

tion  is  drive type dependent, the lowered error correc-

tion threshold is also drive type dependent.  If a  drive

has  several error correction mechanisms, it is permissi-

ble for this modifier to suppress  some  and  not  affect

others.

Suppress Error Recovery

Suppresses  most  error recovery mechanisms, such as read

retries.  Error correction mechanisms, such as  ECC  cor-

rection, and some error recovery mechanisms, such as seek

retry, are not affected by this modifier.  The exact  de-

finition   of   which   error   recovery  mechanisms  are

suppressed and which  are  not  affected  is  drive  type

dependent.

MSCP CONTROL MESSAGES                                   PAGE 5-18

COMMAND MESSAGES

Refer  to  Appendix  appendix(opcdsflgsoffs),  "Opcode, Flag, and

Offset Definitions," Table table(commandmodifer), "Command  Modi-

fiers" for the values assigned to all command modifiers.

All  modifiers  that are not explicitly allowed for a command are

reserved, and shall be treated in accordance  with  the  require-

ments  for  reserved  fields described in Section section(rsvdun-

defflds), "Reserved and Undefined Fields." The only exception  is

that certain modifiers (allowable on certain commands) are either

ignored or treated as reserved depending on whether or not a con-

troller  supports  certain  optional  MSCP  functions.  See Table

table(commandmodifer) for complete details.  Note that those mod-

ifiers are not listed in the individual command descriptions.

5.4  END MESSAGES

An  MSCP  server sends an end message to a class driver to report

completion of a command.  Note that end messages  are  flow  con-

trolled  as  described  in  Section section(flow__control), "Flow

Control" and associated subsections.

The generic end message format is as follows:

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number| unit number |

+---------------+-------+--------+

| status | flags | endcode|

+---------------+-------+--------+

| |

/ parameters /

/ /

| |

+--------------------------------+

The fields in the generic end message are as follows:

command reference number

unit number

MSCP CONTROL MESSAGES                                   PAGE 5-19

END MESSAGES

Both of these fields are copied from the command message.

See Section section(command__mess), "Command Messages."

sequence number

This  field  depends  on  the  state  of  the  "error log

generated" flag in the "flags" field.  If  that  flag  is

set,  this  field  contains  the "sequence number" of the

LAST error log message that refers  to  this  command  --

i.e.,  that  contains  this  command's  command reference

number.  If that flag is clear, this field will be  zero.

(Note that MSCP servers meeting certain requirements need

not implement error log sequence  numbers.   See  Section

section(errorlog_mess),   "Error  Log  Messages." Servers

that meet those requirements shall always return  a  zero

in the sequence number field.) This field allows the host

to determine when all error log messages for a particular

command have been received and when the command's context

saved for inclusion in the error  log,  if  any,  can  be

released.   Section  section(errorlog_mess),  "Error  Log

Messages" provides complete details  regarding  host  and

MSCP server error logging operation.

endcode

Identifies this message as an end message and the type of

command (opcode) that this is an end message  for.   This

field implicitly specifies the length, format, and inter-

pretation of the parameters.

Normally  the  endcode  is  formed by adding the constant

(OP.END) to the command opcode (see Table table(cntrlmes-

sopcd),  "Control Message Opcodes").  An exception is the

Invalid Command End Message (see "Invalid Command" status

in  Section  section(status__codes),  "Status  and  Event

Codes").

flags

Bit  flags, collectively called end flags, used to report

various conditions detected due to this command  but  not

directly  related  to  success or failure.  The following

flags are defined:

Allocation Failure

Set  if  the  server was unable to allocate a new

"write history entry" in a History  Log  modified

MSCP CONTROL MESSAGES                                   PAGE 5-20

END MESSAGES

write  type  command.   Indicates  that no "write

history entry" was generated  for  this  command.

This  condition  is  recorded  in  the Allocation

Failure Table in the controller.   For  more  de-

tails,  see  Section  section(wrihismdes), "WRITE

HISTORY MANAGEMENT Command, Description," and the

descriptions  of  the History Log and Reuse Entry

modifiers  in  any  write  type  command  (WRITE,

ERASE, and DISK COPY DATA).

Bad Block Reported

Set  if  one or more bad blocks were detected and

the host is  performing  bad  block  replacement.

Indicates  that  the  host should replace the bad

block identified in the "first bad block"  field.

Note  that  this  end  flag is applicable only to

disk class devices.

Bad Blocks Unreported

Set  if  one or more bad blocks were detected and

not reported in the "first bad block" field.   If

the  "Bad  Block Reported" flag is also set, this

indicates  that  two  or  more  bad  blocks  were

detected,   the  host  is  performing  bad  block

replacement, and the "first bad block" field only

reports  the first bad block in the transfer.  If

the "Bad Block Reported" flag is clear, this  in-

dicates that one or more bad blocks were detected

and that the controller  either  already  has  or

soon  will replace them.  Note that this end flag

is applicable only to disk class devices.

Error Log Generated

Set  if  one  or  more  error  log  messages were

generated that refer to  this  command  --  i.e.,

that  contain  this  command's  command reference

number.  When set this flag signals the  host  to

save  any  command  context that it wishes to in-

clude in its error log until all error  log  mes-

sages  associated  with  the command have arrived

(see  also  "sequence  number"  field   described

above).

History Logged

Set  if  the "write history log" contains a valid

"write history  entry"  for  this  command.   See

MSCP CONTROL MESSAGES                                   PAGE 5-21

END MESSAGES

Section   section(wrihismdes),   "WRITE   HISTORY

MANAGEMENT Command, Description," for details  of

write history logging.

Serious Exception

Set  if  an unrecoverable error or some other un-

usual exception condition requires that the  host

recognize  the  exception  before additional com-

mands are processed.  The host  shall  clear  the

error before subsequent commands can be executed.

Note that this end flag  is  applicable  only  to

tape  class  devices.  See Tape Mass Storage Con-

trol Protocol Specification (TMSCP) for  complete

details.

Refer to Appendix appendix(opcdsflgsoffs), "Opcode, Flag,

and Offset Definitions," Table table(endmessageflg), "End

Message  Flags"  for  the  values assigned to the defined

flags.  All other bits in the "flags" field are reserved,

and  shall be treated in accordance with the requirements

for reserved fields described in Section  section(rsvdun-

defflds), "Reserved and Undefined Fields."

status

The  status  field  contains a status code that indicates

whether the operation was successfully completed  or,  if

it  wasn't  successful,  what type of error occurred (see

also Section section(status__codes),  "Status  and  Event

Codes").   Note  that  recoverable errors are reported as

successful  completion  of  the  command.   All   errors,

whether  recoverable  or  not, are reported in a separate

error log message if they should be logged  (see  Section

section(errorlog_mess), "Error Log Messages" for more de-

tail).

Note that if several errors occur in a nontransfer opera-

tion, the error that is reported is controller  dependent

unless  the  individual command description states other-

wise.  See the "status" field description in Section sec-

tion(xferendmessfmt),   "Transfer   Command  End  Message

Format" for specifics  of  multiple  error  handling  for

transfer commands.

Chapter   chapter(minimalcmdset),   "Minimal  MSCP  Command  Set"

describes the operation, command message format, and end  message

format  of  the  various  commands required to be supported by an

MSCP CONTROL MESSAGES                                   PAGE 5-22

END MESSAGES

MSCP disk controller.  The following section  describes  the  end

message format common to many data transfer commands.

5.4.1  Transfer Command End Message Format

Although  the  parameters  and their layout are command (endcode)

dependent, many commands perform data transfers and  thus  return

similar  sets of parameters.  Therefore data transfer related end

messages are typically laid out as follows:

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number| unit number |

+---------------+-------+--------+

| status | flags |endcode |

+---------------+-------+--------+

| byte count |

+--------------------------------+

| |

+--- ---+

| undefined |

+--- ---+

| |

+--------------------------------+

| first bad block |

+--------------------------------+

The fields in the transfer command end message are as follows:

command reference number

unit number

sequence number

endcode

flags

See Section section(end____mess), "End Messages."

status

If  several  errors  occur  in  a transfer operation, the

status code reports the first  error  starting  from  the

beginning of the transfer (i.e., the lowest byte count or

lowest logical block  number).   The  only  exception  is

transfer commands that include a compare operation (i.e.,

MSCP CONTROL MESSAGES                                   PAGE 5-23

END MESSAGES

read-compare and write-compare  operations);   errors  in

the  original  transfer  may  take precedence over errors

during the compare operation.

If  a  "Forced  Error"  and some other error occur at the

same point (same byte  count  or  logical  block  number)

within  a  transfer  operation,  the other error shall be

reported.  If a "Compare Error" and some other error that

is  not a "Forced Error" occur at the same point within a

transfer operation, the other error  shall  be  reported.

If  a  "Forced Error" and a "Compare Error" both occur at

the same point, and no other error occurs at that  point,

the  "Compare Error" shall be reported.  Otherwise, which

error of multiple errors  occurring  at  the  same  point

should  be reported is controller dependent.  An alterna-

tive way of stating this is that "Forced Errors" are  the

error  of  last  resort and that "Compare Errors" are the

error of second to last resort.

See  also  "Section  section(status__codes),  "Status and

Event Codes."

byte count

In  transfer  command  end  messages, the number of bytes

successfully transferred, counting from the start of  the

transfer  to the first error (i.e., the lowest byte count

or lowest logical block number with an error).  Data that

follows   the   first  error  is  not  counted,  even  if

transferred successfully.  The only exception is transfer

commands   that   include   a  compare  operation  (i.e.,

read-compare and write-compare  operations);   errors  in

the  original  transfer  may  take precedence over errors

during the compare operation.

The  error  identified by "status" must have actually oc-

curred at the position identified  by  "byte  count." The

controller must have successfully transferred all data up

to that point.  If Data Error  (subcode  "Forced  Error")

status  is  returned  for  a READ command (addressed to a

disk  class  device),  the  controller  must  have   also

transferred  all  the  data contained in the block marked

with forced error.  Beyond this point the  state  of  the

transfer  is  undefined.   None  of the transfer may have

been performed or attempted, all of it may have been  at-

tempted  (with  unknown  success), or some parts may have

been attempted and others not.

MSCP CONTROL MESSAGES                                   PAGE 5-24

END MESSAGES

Again,  the  only  exception to this is transfer commands

that include a compare operation.  Such a  command  makes

two  passes over the data;  one for the original transfer

and another for the compare operation.  For most  errors,

there  is no way to determine in which pass the error was

detected.  Therefore  the  only  guarantee  is  that  the

original  transfer  was performed up to the point identi-

fied by "byte count" without detecting any  errors.   The

compare  operation  may or may not have been performed up

to that point.  If a "Compare Error"  is  reported,  then

both the original transfer and the compare operation have

been successfully performed up to the point identified by

"byte count." The state of both the original transfer and

the compare operation after that point, however, is unde-

fined.   This implies that the compare pass of a transfer

command that includes a compare operation may be done for

the  entire  transfer  as a unit, block by block, or any-

where in between.

For disk class devices, the granularity of the byte count

on errors (i.e., the resolution with which the  point  of

error  is  identified)  need  not be any smaller than the

volume's block size.  That is, the byte count  need  only

identify  the  block  in which the error occurred, rather

than the exact word or byte.   In  particular,  the  byte

count  returned  with such errors as "Compare Errors" and

"Host Buffer Access Errors" need only identify the  block

in  which  the error occurred, rather than the exact word

or byte.  Note that a block is identified by  the  number

of  the  first  byte  in  the block.  Controllers may op-

tionally provide finer granularity  for  the  byte  count

field  on errors.  If an error is not reported, controll-

ers shall return the exact byte count  that  was  in  the

command message.

For tape class devices, the granularity of the byte count

on errors shall be either one word  (two  bytes)  or  one

byte.  If an error is not reported, controllers shall re-

turn the exact byte count that was transferred.  For host

buffer access details see Section section(hostbuffacces),

"Host Buffer Access."

first bad block

In  disk transfer command end messages, the logical block

number of the first bad block (i.e., the bad  block  with

the  lowest  logical  block  number)  detected during the

transfer that the host should replace.  Only valid if the

"Bad Block Reported" flag is set.  Undefined (garbage) if

the "Bad Block Reported" flag is clear.

MSCP CONTROL MESSAGES                                   PAGE 5-25

END MESSAGES

5.5  STATUS AND EVENT CODES

Status  and  Event codes are essentially identical except for the

manner in which they are returned.  The "status  code"  field  in

returned  in end messages and is used to report whether an opera-

tion was successfully completed or, if it  was  not  successfully

completed,  what  type of error occurred.  The "event code" field

is returned in error log messages and has the identical structure

and  encoding of a status code, but identifies the specific error

or event being reported by the error log  message.   Errors  that

are  reported in both an end message and an error log message use

identical values for the "status code" and "event  code"  fields.

The  same  value may not be used to report one type of event as a

"status code" and a different type of event as an "event code".

There  are, however, certain codes that are used as "status only"

codes and others used as "event only" codes.  For example,  if  a

command  is  issued to a unit that is Unit-Offline, the result is

reported as a "status only" code.  It is not necessary to  report

this  information  in an error log;  it is up to the program that

initiated the command to take the appropriate action.  If a WRITE

command  is  issued  and  a  bad block is encountered and is suc-

cessfully replaced, the end message for the command  will  return

the  appropriate status outcome, in addition to setting the flags

for bad block reported and error log message generated.  The  Bad

Block  Replacement Attempted event code will never be returned as

a status code;  it will only be returned as an event code  in  an

error log message.

The  "status code" and the "event code" fields are divided into a

5-bit major status code and an 11-bit status and/or event subcode

arranged as follows:

15 0

+-----------------+---------+

| subcode | code |

+-----------------+---------+

The  5-bit  major status code conveys the status information that

hosts need for normal  operation.   Therefore  the  major  status

codes  are  a  formal part of MSCP.  All controllers shall return

the same major status codes for similar situations.

The  11-bit  subcode exists to specify the exact error or unusual

MSCP CONTROL MESSAGES                                   PAGE 5-26

STATUS AND EVENT CODES

situation encountered with very  fine  detail.   As  such  it  is

primarily used for diagnostic purposes, and hosts should not need

to examine it during normal operation.

Subcodes related to protocol or state errors are a formal part of

MSCP.  All controllers shall return the same  subcodes  for  pro-

tocol  or  state errors.  These subcodes are generally bit flags,

allowing several causes of the major status code to be reported.

Subcodes  related  to  controller  and/or  drive errors, however,

shall be allowed to vary from one controller or drive to another.

There  is  no  requirement that the same subcodes be returned for

similar drive or controller errors.  These subcodes are generally

specific  values,  corresponding  to one specific event or error.

Each subcode shall have the same meaning whenever it is used.  It

is  the  use  of  a subcode that may vary (i.e., whether or not a

specific controller returns that subcode), not its meaning.   The

defined  subcodes are listed in Appendix appendix(statusevntdef),

"Status and Event Code Definitions." This list may expand (via an

ECO  to  MSCP)  whenever a new drive or controller type is intro-

duced.

The  major  status  codes  that  may  be  returned in end message

"status code" fields are listed below along with the general  use

made  of  subcodes.   The  actual subcodes used are listed in Ap-

pendix   appendix(statusevntdef),   "Status   and   Event    Code

Definitions." Those  subcodes  that are a formal part of MSCP are

also listed in the descriptions of the commands that  may  return

them.

Success

The command was successfully completed.  This status code

may also be returned, for some commands, if the  intended

effect  of  the  command  has  already  been accomplished

(e.g.,  requesting  a  drive  that  isn't   spinning   to

spin-down).   The  subcode  consists of bit flags used to

report various "alternate" forms of success.  See the in-

dividual command descriptions for details.

The  status  code  value associated with "Success" is, by

definition, zero.  Subcode value zero (i.e.,  no  subcode

bits set) is "Normal" success, and implies normal comple-

tion of a command.  One subcode bit, the "Duplicate  Unit

Number"  bit, is common to many commands.  This bit, when

set, implies that  the  unit  is  "Unit-Online"  and  the

MSCP CONTROL MESSAGES                                   PAGE 5-27

STATUS AND EVENT CODES

command succeeded, but that the unit has a duplicate unit

number.  The unit will become "Unit-Offline," due to  the

duplicate  unit  number,  as  soon  as  it  ceases  to be

"Unit-Online." Other "Success" subcode bits are unique to

a particular command, and are described under the indivi-

dual command descriptions.

Invalid Command

Controllers  detect and report two different types of in-

valid command errors.

The  first  type  is  called  a protocol error.  The con-

troller reports a protocol error if during  initial  com-

mand  message  validation  it finds:  1) a message format

violation, 2) an MSCP version number mismatch, or 3) that

a  field  contains  a  value  that specifies an undefined

operation.

The following are MSCP protocol errors:

Invalid Message Length

The  command  message is too short to contain the

parameters required by the command's opcode.

Invalid Modifiers

An  undefined  modifier  bit is set or a modifier

that is defined but not supported  by  that  con-

troller is set.  See the descriptions of the His-

tory Log and Reuse Entry modifiers  in  the  DISK

COPY  DATA,  ERASE,  and WRITE Commands, Sections

section(dskcopydatacmf),    section(erase___cmf),

and section(write___cmf).

Invalid MSCP Version

The  "MSCP  version"  field  of  a SET CONTROLLER

CHARACTERISTICS command is nonzero  (see  Section

section(setcontrolcmf),  "SET  CONTROLLER CHARAC-

TERISTICS Command, Command Message Format").

Invalid Opcode

See      "opcode"      field      in      Section

section(command__mess),  "Command  Messages"  for

possible causes.

Invalid Operation

MSCP CONTROL MESSAGES                                   PAGE 5-28

STATUS AND EVENT CODES

The  "operation"  field  of an ACCESS NONVOLATILE

MEMORY command or WRITE HISTORY  MANAGEMENT  com-

mand  contains an illegal value (see Section sec-

tion(accessnonvmcmf),      "Multihost      ACCESS

NONVOLATILE   MEMORY   Command,  Command  Message

Format" and Section  section(wrihismcmf),  "WRITE

HISTORY   MANAGEMENT   Command,  Command  Message

Format").

{reserved field violations}

A  "reserved"  field  in the command message con-

tains a  nonzero  value.   Note  that  "reserved"

field  checking  is  optional  (see  Section sec-

tion(rsvdundefflds),  "Reserved   and   Undefined

Fields").

Note  also  that modifiers not explicitly allowed

for a command received by the server and any  un-

defined  controller  flag  and unit flag bits are

considered to be "reserved" fields.

The  second type is a parameter error.  After determining

through initial command message validation that the  com-

mand  message  has a well defined meaning, the controller

reports a parameter error if some parameter is  found  to

be  outside  the range allowed.  Note that some parameter

errors are  detected  during  follow-on  command  message

validation  before command execution begins, while others

may be detected afterwards.

The following are parameter errors:

Invalid Byte Count

See   "byte   count"   field   in   Section  sec-

tion(xfrcmdmesfmt),  "Transfer  Command   Message

Format" for possible causes.  Log modifier in the

ERASE    and     WRITE     Commands,     Sections

section(erase___cmf) and section(write___cmf).

Invalid CRN

A  value of zero was supplied in the crn (connec-

tion reference number) field of a SET  CONTROLLER

CHARACTERISTICS  or TERMINATE CLASS DRIVER/SERVER

CONNECTION command message.

Invalid Display Text

MSCP CONTROL MESSAGES                                   PAGE 5-29

STATUS AND EVENT CODES

The  specified display text is invalid or not im-

plemented for  the  specified  display  item  and

mode.    For   more   information,   see  Section

section(disp___des), "DISPLAY  Command,  Descrip-

tion."

Invalid Entry Locator

The  value  specified in the "entloc (entry loca-

tor)" field of a WRITE HISTORY MANAGEMENT, ERASE,

WRITE,  or  DISK  COPY DATA command is not within

the limits of the server's "write history  entry"

set.   See  Section  section(wrihismdes),  "WRITE

HISTORY MANAGEMENT Command, Description" for more

detail.

Invalid Entry Id

The  value specified in the "entry id" field of a

WRITE HISTORY MANAGEMENT, ERASE, WRITE,  or  DISK

COPY DATA command does not match the value in the

"write history entry" identified by the entry lo-

cator  or the host supplied a value of FFFF (hex)

in the "entry id" field of a History Log modified

write    type    command.    See   Section   sec-

tion(wrihismdes), "WRITE HISTORY MANAGEMENT  Com-

mand, Description" for more detail.

Invalid Item

The  specified  display  item  code  is  not  im-

plemented and the "Must Be Implemented"  modifier

was  set.   For  more  information,  see  Section

section(disp___des), "DISPLAY  Command,  Descrip-

tion."

Invalid Length

The  "length"  field  of  an  ACCESS  NONVOLATILE

MEMORY command contains  an  odd  value  and  the

"operation"  field  requested  a  "Test  and Set"

operation (see  Section  section(accessnonvmcmf),

"Multihost  ACCESS  NONVOLATILE  MEMORY  Command,

Command Message Format").

MSCP CONTROL MESSAGES                                   PAGE 5-30

STATUS AND EVENT CODES

Invalid Logical Block Count

Invalid Src Unit Number

Invalid Source Unit Identifier

Invalid Destination LBN

Invalid Source LBN

Invalid Source Unit's Storage Subsystem Port Address

Invalid Source Unit's Storage Subsystem System

Address

See  Section  section(dskcopydatacmf), "DISK COPY

DATA  Command,  Command   Message   Format"   for

descriptions of these command message fields.

Invalid Logical Block Number

See  "logical block number" field in Section sec-

tion(xfrcmdmesfmt),  "Transfer  Command   Message

Format" for possible causes.

Invalid Mode

The specified display mode is not implemented for

the specified display item and the "Must  Be  Im-

plemented"  modifier  was set.  For more informa-

tion, see Section  section(disp___des),  "DISPLAY

Command, Description."

Invalid Replacement Block Number

See  Section section(replace__emf), "REPLACE Com-

mand, End Message Format" for possible causes.

Invalid Unit Flags

The the host-settable unit flags specified in the

"unit flags" field of an ONLINE  command  message

issued  to  a  unit that is already "Unit-Online"

are different from those already in effect on the

unit (see Sections section(online___cmd), "ONLINE

Command"  and  section(online__mcmd),  "Multihost

ONLINE Command").

Note  that  an unknown unit number does not constitute an

invalid parameter or command.  Unknown unit  numbers  are

treated as if the unit were "Unit-Offline."

In  order  for hosts to clearly distinguish the two types

of invalid commands controllers  use  two  different  end

message formats.

MSCP CONTROL MESSAGES                                   PAGE 5-31

STATUS AND EVENT CODES

Protocol  errors are reported via the Invalid Command End

Message.  The Invalid Command End Message is  largely  an

image  of  the  invalid command message that was received

from the host.  In order to construct the Invalid Command

End  Message,  the controller makes the following changes

to the invalid command message that it received:

1.  The controller stores the constant (OP.END),

defined in Appendix appendix(opcdsflgsoffs),

"Opcode,   Flag,  and  Offset  Definitions,"

Table table(cntrlmessopcd), "Control Message

Opcodes,"  in  the "endcode" field, in order

to flag this as an Invalid Command End  Mes-

sage.   (Note that the command opcode is not

added to OP.END.)

2.  The  controller  may,  at its option, either

copy the end message "flags" field from  the

corresponding  field  in the invalid command

or else clear the end message "flags" field.

3.  The  controller stores the "Invalid Command"

status code with an appropriate  subcode  in

the end message "status" field.  The subcode

is set equal to the byte  offset,  from  the

start  of  the command message, of the field

in error (any one field, at the controller's

option, if several are in error).  Multibyte

fields are identified by the offset to their

lowest  byte.  Single byte fields positioned

at an odd offset may be identified,  at  the

controller's  option, by either their actual

offset or their offset minus one.  That  is,

offsets  may  be truncated to an even value.

Subcode zero is used to report that the com-

mand  message  was  too short to contain the

parameters required by the command's opcode.

Note  that  any value is valid for the field

at  offset  zero,  the  "command   reference

number."

4.  The  controller  may optionally truncate the

parameter fields  ("echoed  command  parame-

ters")  to  some  length  convenient for the

controller.  Controllers should try to  echo

as much of the parameters as possible.

MSCP CONTROL MESSAGES                                   PAGE 5-32

STATUS AND EVENT CODES

The contents of any field copied from the invalid command

are undefined if the command message  was  too  short  to

contain  that field.  This primarily applies to the "com-

mand reference number" and "unit number" fields.

Note  that  the  controller may, at its option, enter the

"Controller-Available" state relative to the issuing host

class  driver  after  it  returns the Invalid Command End

Message (see Section section(cntrolerstates), "Controller

States").

Parameter  errors  are reported via the invalid command's

normal (i.e., opcode dependent, see  "endcode"  field  in

Section  section(end____mess),  "End  Messages") end mes-

sage.  The "status" field is set equal  to  the  "Invalid

Command"  status  code, with the subcode set equal to the

byte offset, relative to the start of  the  command  mes-

sage,  of the parameter field in error.  Multibyte fields

are identified by the offset to their lowest  byte.   All

other fields (byte count, unit characteristics, etc.) re-

flect the state of the execution of the command up to the

point the invalid parameter was detected.

Command Aborted

The  command  was  aborted  by an ABORT command.  The end

message for the aborted command (i.e.,  the  end  message

containing  the  "Command  Aborted"  status code) has the

normal format for the command and all fields  are  valid.

In  particular, the "byte count" field identifies how far

a transfer command was completed before it  was  aborted.

The status of the transfer beyond the returned byte count

is undefined.  Subcodes are not used.

Unit-Offline

The unit identified by the "unit number" field of the end

message is in the "Unit-Offline" state.  The subcode con-

sists  of  bit  flags  that  indicate  why  the  unit  is

"Unit-Offline." Note that there may  be  several  reasons

for  the  unit  being  "Unit-Offline." If  the subcode is

zero, it implies that the unit is unknown  --  i.e.,  the

controller  knows  of  no  unit  with  the specified unit

number.

MSCP CONTROL MESSAGES                                   PAGE 5-33

STATUS AND EVENT CODES

Unit-Available

The unit identified by the "unit number" field of the end

message is in the "Unit-Available" state.  The subcode is

zero,  except  when  "Exclusive  Access"  of the unit was

denied.  In that case the "Already In Use" subcode is re-

turned  (see  the  descriptions of the AVAILABLE, ONLINE,

and  SET  UNIT  CHARACTERISTICS   commands   in   Chapter

chapter(multihostfunct), "Multihost Support" for more de-

tail).

Media Format Error

Only  returned  by  the  ONLINE  command  for  disk class

devices.  The volume mounted on the unit  appears  to  be

formatted  incorrectly,  so  that  it must be reformatted

(and all data lost) before it may be used.  This error is

also  returned  if  the volume is formatted with 576 byte

sectors and the controller only supports  512  byte  sec-

tors.   Note that the volume may only "appear" to be for-

matted incorrectly;  the typical cause of this error is a

fault  in the drive's read/write electronics.  If this is

the case, the volume can usually be successfully accessed

on another drive.

Whenever  they  return  this error code controllers shall

treat the unit  as  if  an  AVAILABLE  command  with  the

"Spin-down"  modifier  set  had  been issued for it.  The

unit is therefore always in  the  "Unit-Available"  state

with  AVAILABLE attention messages suppressed until a hu-

man operator changes the volume  or  spins-up  the  unit.

The  subcode  reports  which  integrity  check the volume

failed;  it is volume format, and therefore  drive  type,

dependent.

Write Protected

The unit identified by the "unit number" field of the end

message is write protected and the command required  that

data  be written onto the drive.  The subcode consists of

bit flags indicating the reasons why the  unit  is  write

protected.

Compare Error

MSCP CONTROL MESSAGES                                   PAGE 5-34

STATUS AND EVENT CODES

A COMPARE HOST DATA command, a read compare operation, or

a write compare operation found  different  data  in  the

host  buffer and the unit identified by the "unit number"

field of the end message.  The subcode is always zero.

Data Error

Invalid  or uncorrectable data was obtained from a drive,

as determined by internal error detecting  or  correcting

codes.   The  subcode  is  used to report the exact error

detected.  Subcode zero is used for "Forced  Errors." All

errors  caused  by  the  "Force  Error" modifier shall be

reported with subcode zero.

Host Buffer Access Error

The  controller  encountered  an error when attempting to

access a buffer in host memory.  The subcode is  used  to

report  the exact error encountered.  This status code is

also returned whenever the command's buffer descriptor or

byte count violate any communications mechanism dependent

restrictions.

Note  that  this status code is NOT used to report errors

encountered when transferring command, end, attention, or

error  log  messages  between  the controller and a host.

Such errors are reported by  terminating  the  connection

between  the class driver and MSCP server.  The mechanism

for reporting such errors to the host's error log is com-

munications mechanism dependent.

Controller Error

The  controller encountered an internal controller error.

The subcode is used to report  the  exact  error  encoun-

tered.   An  internal  controller  error is reported as a

"Controller Error" if and  only  if  the  controller  has

reasonable  grounds  to  trust  its sanity and expects to

complete, either successfully or with an appropriate  er-

ror  status  code,  all of its outstanding commands.  All

more severe controller errors are reported by terminating

the  connection  between the controller's MSCP server and

the host class driver.  This is in addition,  of  course,

to attempting to generate an error log message.

MSCP CONTROL MESSAGES                                   PAGE 5-35

STATUS AND EVENT CODES

Subcode  zero of this status code is reserved for host or

non-MSCP  entity  (e.g.,  a  controller's  port   driver)

detected  command  timeouts.  It is never generated by an

MSCP server.  All other subcodes  are  controller  depen-

dent.

Note  that  some  controller errors may be reported using

other error codes if an internal controller error  causes

the controller to misdiagnose the error.

Drive Error

The  controller discovered an error within a drive.  Such

errors are typically, but not always, mechanical  in  na-

ture,  since  most  nonmechanical  errors are reported as

"Data Errors." The subcode is used to  report  the  exact

error encountered.

In many cases a "Drive Error" will indicate that the unit

is broken or inoperative.  If this occurs, the "Drive Er-

ror"  should  be  reported  once and the unit should sub-

sequently be reported  as  being  "Unit-Offline"  due  to

being inoperative.

Invalid Parameter

The controller encountered an invalid parameter value for

a parameter that is not passed directly  in  the  command

message.   Invalid  values in fields of a command message

are reported with  the  "Invalid  Command"  status  code.

"Invalid Parameter" typically reports an invalid value in

a field in a host-supplied buffer.

The  status codes that may be returned for a specific command are

command (opcode) dependent.  The status codes  that  may  be  re-

turned  for  each  command and any special meaning that they have

specific to the command are listed in  the  command  descriptions

contained  in  Chapter chapter(minimalcmdset), "Minimal MSCP Com-

mand  Set"  and   Chapter   chapter(multihostfunct),   "Multihost

Support." Note  that  the  format  of  a command's end message is

solely determined by its opcode.  The status code returned in the

end  message  does not affect the end message's format.  The only

exception is the Invalid Command End Message (see  "Invalid  Com-

mand" status above), which contains a special endcode.

"Event Only" event codes and subcodes shall only be used in error

log or informational messages and shall never be used  as  status

MSCP CONTROL MESSAGES                                   PAGE 5-36

STATUS AND EVENT CODES

codes  reported  in  end  messages.  The currently defined "Event

Only" event codes are:

Bad Block Replacement Attempted

This  event code is used after every attempt (by the con-

troller or host) to replace a bad  block,  regardless  of

the  success  or  failure  of  that  attempt (see Section

5.8.7, "BAD BLOCK REPLACEMENT ATTEMPT Error Log Format").

For  details  of  bad  block  replacement, see Chapter 8,

"Controller Initiated  Bad  Block  Replacement"  and  the

Digital  Storage Architecture Disk Format (DSDF) specifi-

cation.

Informational Log

Informational  Log  messages  are  used  by the server to

report such things as statistics or error log summary in-

formation  that may have been compiled in the device.  In

all error log messages with this event code, the  "Infor-

mational"  error  log  message  flag  (see  Section  sec-

tion(generrlogfmt), "Generic Error Log  Message  Format")

should normally be set.

5.6  CONTROLLER FLAGS

Controllers  maintain  a  set of bit flags, collectively known as

the "controller flags," which represent the controller's  operat-

ing characteristics.  Two different types of controller flags are

used to accommodate two different types of controller  character-

istics.   Host-settable controller flags allow hosts to enable or

disable the host selectable operating characteristics (e.g.,  En-

able Attention Messages) supported by all controllers.  Controll-

ers  use  non-host-settable  controller  flags  to  indicate  the

presence  or  absence  of implementation dependent optional func-

tions (e.g., Multihost  Support).   The  currently  defined  con-

troller flags are described later in this section.

The  SET CONTROLLER CHARACTERISTICS command is used to modify the

host-settable controller flags according to the value supplied in

the  "controller  flags"  field  of  the  command  message.  If a

host-settable flag is set the controller enables the  correspond-

ing  host  selectable  operating  characteristic.   A  clear flag

causes the controller to disable it.   Controllers  shall  always

ignore  the  values supplied for the non-host-settable controller

flags.  All host-settable controller flags are, by default, clear

(i.e.,   all   host   selectable  operating  characteristics  are

MSCP CONTROL MESSAGES                                   PAGE 5-37

CONTROLLER FLAGS

disabled)    whenever    a    class    driver    first    becomes

"Controller-Online" to an MSCP server.

The  controller  dependent  values  of the non-host-settable con-

troller flags, as well as the current values of the host-settable

controller flags, are returned in the "controller flags" field of

the SET CONTROLLER CHARACTERISTICS command's end message.  A con-

troller  shall  set only those non-host-settable controller flags

for the corresponding optional functions it supports, if any.

Controllers  that  provide  Multihost Support shall maintain, and

operate  according  to  the  setting  of,  a  separate   set   of

host-settable  controller  flags  for each host class driver/MSCP

server connection established.  Each class  driver  is  expressly

permitted  to  specify  different  settings for the host-settable

controller flags and as a  result  realize  different  controller

operation.   The non-host-settable controller flags remain fixed,

and are therefore common to all class drivers of the same  device

class.

The host-settable controller flags are as follows:

Enable Attention Messages

When this flag is set the controller shall send attention

messages to this host.  If this flag is  clear  the  con-

troller discards attention messages.  Note that this flag

is applicable to all attention  messages,  regardless  of

type.

Enable Miscellaneous Error Log Messages

When this flag is set the controller shall send error log

messages that do not relate to a specific command to this

host.   If this flag is clear the controller discards any

such error logs.

Enable Other Hosts' Error Log Messages

When  this  flag is set error log messages that relate to

commands issued by other hosts should  be  sent  to  this

host.   Controllers  that provide Multihost Support shall

implement this characteristic separately  for  each  host

class  driver  connected.   Even though this operation is

MSCP CONTROL MESSAGES                                   PAGE 5-38

CONTROLLER FLAGS

only useful in a multihost environment, controllers  that

do  not  provide Multihost Support shall still honor this

flag.  That is, the flag may not be treated  as  reserved

and  the  current  state  as set by the host shall be re-

turned in the end message field.

Enable This Host's Error Log Messages

When  this  flag is set error log messages that relate to

commands issued by this host should be sent to this host.

If this flag is clear the controller discards those error

logs, except in a  multihost  environment  where  another

host has set the "Enable Other Hosts' Error Log Messages"

flag.

The non-host-settable controller flags are as follows:

Cache Support

This flag shall be set if and only if the controller sup-

ports disk caching and can accept non-default  values  of

"Cache Access Attributes."

Controller Initiated Bad Block Replacement

This flag shall be set if and only if the controller sup-

ports Controller Initiated Bad Block Replacement  on  all

disk  drives that can be connected to the controller.  If

this flag is set the host will never have to perform  bad

block replacement for disks accessed via this controller.

Note that controllers may also  support  Controller  Ini-

tiated  Bad  Block Replacement on certain individual disk

drives and not others.  In that case this flag  shall  be

returned  clear;  the setting of the Controller Initiated

Bad Block Replacement unit  flag  (described  in  Section

section(unit___flags),  "Unit  Flags") determines whether

or not the host must perform bad block replacement  on  a

particular  disk  drive.   Note  also that that unit flag

shall always be returned set if  this  flag  is  returned

set.

Support  of  Controller  Initiated Bad Block Replacement,

either on a single disk drive or all disk  drives,  is  a

controller  implementation dependent option.  See Chapter

chapter(cntrlrinitbbr), "Controller Initiated  Bad  Block

Replacement" for complete details.

MSCP CONTROL MESSAGES                                   PAGE 5-39

CONTROLLER FLAGS

Local Disk Copy Data

This  flag  shall be returned set if and only if the con-

troller supports interdevice transfer of  data,  via  the

DISK  COPY  DATA  command (see Section section(dskcopyda-

tacmd)), between all the disk  drives  connected  to  the

controller.

Support  of  Local  Disk  Copy  Data is an implementation

dependent option.   Local  Disk  Copy  Data  may  be  im-

plemented  by  controllers that provide Multihost Support

as well as those that do not.  However,  for  controllers

that  provide  Multihost  Support, support of Remote Disk

Copy Data (see the Remote Disk Copy Data flag  below)  is

preferred.

Controllers  that  implement  Local  Disk Copy Data shall

also implement Controller Initiated Bad Block Replacement

on all disk drives connected to them (see Controller Ini-

tiated Bad Block Replacement flag above).

Multihost Support

This  flag  shall be returned set if and only if the con-

troller supports simultaneous access, by multiple  hosts,

to  all  drives  (disk  and  tape)  connected to the con-

troller.  See Chapter chapter(multihostfunct), "Multihost

Support."

Remote Disk Copy Data

This  flag  shall be returned set if and only if the con-

troller supports interdevice transfer of  data,  via  the

DISK  COPY  DATA  command (see Section section(dskcopyda-

tacmd)), between disk drives connected to itself and disk

drives  connected  to  another  storage subsystem that is

connected to the same communications mechanism.

Support  of  Remote  Disk  Copy Data is an implementation

dependent option.  Remote Disk Copy Data can only be  im-

plemented by controllers that also provide Multihost Sup-

port.

Controllers  that  implement  Remote Disk Copy Data shall

also implement Local Disk Copy Data (see Local Disk  Copy

Data  flag  above)  and  Controller  Initiated  Bad Block

Replacement   (see   Controller   Initiated   Bad   Block

MSCP CONTROL MESSAGES                                   PAGE 5-40

CONTROLLER FLAGS

Replacement  flag  above) on all disk drives connected to

them.

Restricted DISK COPY DATA Operations

This  flag  shall be returned set if and only if the con-

troller does not support any of:

o  Local  DISK  COPY DATA between units with unlike

geometries

o  Local  DISK  COPY  DATA  where  the  source  and

destination LBNs differ

o  DISK  COPY DATA where the source and destination

are the same unit

Note  that  controllers  either  support all of the above

operations and return this flag clear or support none  of

these options and return the flag set.

Write History Logging Support

This  flag  shall be returned set if and only if the con-

troller supports "write history logging" as described  in

Section  section(wrihismdes),  "WRITE  HISTORY MANAGEMENT

Command, Description." "Write history  logging"  was  de-

fined  primarily  for  the  purpose  of  assisting in the

operation of host based volume shadowing implementations.

Note  that Write History Logging support includes support

of the TERMINATE CLASS DRIVER/SERVER CONNECTION command.

576 Byte Sectors

This flag shall be set if and only if the controller sup-

ports disks formatted with 576 byte sectors  as  well  as

disks  formatted  with  512  byte sectors.  Note that all

controllers that support disk class devices must  support

disks formatted with 512 byte sectors.

Refer  to  Appendix  appendix(opcdsflgsoffs),  "Opcode, Flag, and

Offset  Definitions,"  Table  table(controllerflag),  "Controller

Flags"   for   the  values  assigned  to  the  currently  defined

host-settable and non-host-settable controller flags.  Those bits

in  the  "controller  flags"  field  that are not defined as con-

troller flags are reserved, and shall be  treated  in  accordance

with  the  requirements  for reserved fields described in Section

section(rsvdundefflds), "Reserved and Undefined Fields."

MSCP CONTROL MESSAGES                                   PAGE 5-41

CONTROLLER FLAGS

5.7  UNIT FLAGS

For each unit connected to a controller, the controller maintains

a set of bit flags, collectively known as the "unit flags," which

represent  the  unit's  operating characteristics.  Two different

types of unit flags are used to accommodate two  different  types

of unit characteristics.  Host-settable unit flags allow hosts to

enable or disable certain host selectable operating  characteris-

tics (e.g., Compare Reads).  The non-host-settable unit flags are

used to indicate  certain  physical  or  logical  characteristics

(e.g.,  576  Byte Sectors, Write Protect) as well as the presence

or absence of certain  controller  implementation  dependent  op-

tional  functions  (e.g., Controller Initiated Bad Block Replace-

ment).  The currently defined unit flags are described  later  in

this  section.   Except  as  noted in those descriptions all con-

trollers shall fully support all non-host-settable unit flags  on

all units.

The ONLINE and SET UNIT CHARACTERISTICS commands are used to mod-

ify the host-settable unit flags for the unit identified  in  the

"unit"  field of the command message, according to the value sup-

plied in the "unit flags" field of the  command  message.   If  a

host-settable  unit  flag  is set the controller enables the cor-

responding host selectable operating characteristic for the unit.

A  clear flag causes the controller to disable it.  Note that the

state of the unit (Offline, Available, or Online) when  the  com-

mand  is  received  affects  the  setting or clearing of the unit

flags.  See the descriptions of the ONLINE and SET  UNIT  CHARAC-

TERISTICS  commands  in Chapters chapter(minimalcmdset), "Minimal

MSCP Command Set" and  chapter(multihostfunct),  "Multihost  Sup-

port"  for  complete  details.  Except as noted in the individual

flag descriptions below, controllers shall ignore the values sup-

plied for the non-host-settable unit flags.

The  non-host-settable  unit flags, as well as the current values

of the host-settable unit flags, are returned in the "unit flags"

field  of  the ONLINE, GET UNIT STATUS, and SET UNIT CHARACTERIS-

TICS command end messages and the  AVAILABLE  Attention  message.

Many unit flags, including all host-settable unit flags, are only

valid when the unit is  "Unit-Online." The  values  returned  for

such  unit  flags  are undefined if the unit is "Unit-Offline" or

"Unit-Available." A few non-host-settable unit  flags  are  valid

when   the   unit   is   "Unit-Available"   and   during  certain

MSCP CONTROL MESSAGES                                   PAGE 5-42

UNIT FLAGS

"Unit-Offline" substates.  These  flags  are  identified  in  the

individual flag descriptions below.

Controllers  that provide Multihost Support maintain only one set

of host-settable unit flags for a particular unit  regardless  of

the  number  of  hosts (class drivers) that have brought the unit

into the "Unit-Online" state.  The unit will operate in an ident-

ical  manner  (according to the setting of the host-settable unit

flags) for each host that has brought the unit "Unit-Online." Any

host  may  alter the setting of the host-settable unit flags (for

itself and all others) only via the SET UNIT CHARACTERISTICS com-

mand  and only after bringing the unit "Unit-Online." Attempts to

change the host-settable unit flags from  their  current  setting

via an ONLINE command (regardless of whether the unit was already

"Unit-Online" to the issuing host) will cause the ONLINE  command

to  be  rejected  as  an  "Invalid  Command" ("invalid unit flag"

parameter  error.   See  "Invalid  Command"  status  in   Section

section(status__codes), "Status and Event Codes").

The unit characteristics flags are as follows:

Compare Reads

A   host-settable   characteristic;    set  if  all  read

transfers should be verified with  a  compare  operation.

Equivalent  to  specifying  the "Compare" modifier on all

READ commands.  See Section section(compare_opers), "Com-

pare Operations" for complete details.

Compare Writes

A   host-settable   characteristic;   set  if  all  write

transfers should be verified with  a  compare  operation.

Equivalent  to  specifying  the "Compare" modifier on all

WRITE  commands.   See  Section   section(compare_opers),

"Compare Operations" for complete details.

Controller Initiated Bad Block Replacement

A  non-host-settable  characteristic;   set  if  the con-

troller will perform bad block replacement for this  disk

drive  (unit),  clear  if the host must perform bad block

replacement for this disk drive.  Note  that  controllers

may support Controller Initiated Bad Block Replacement on

all or some disk drives connected to it.   If  Controller

Initiated  Bad Block Replacement is provided for all disk

drives this flag and the Controller Initiated  Bad  Block

Replacement  controller  flag  (described in Section sec-

tion(controlerflags), "Controller Flags") shall  both  be

set.

MSCP CONTROL MESSAGES                                   PAGE 5-43

UNIT FLAGS

Note  that  a  host  disk class driver shall check if the

"Controller Initiated Bad Block Replacement" unit flag is

set  or  clear after bringing a unit "Unit-Online." If it

is clear (implying that the host is performing bad  block

replacement),  then  the class driver shall invoke a pro-

cess that will  access  the  unit's  Replacement  Control

Table  to  determine if a bad block replacement operation

has been partially performed or if the unit must be write

protected.   The  details  of  this  check and its conse-

quences are described in the Digital Storage Architecture

Disk  Format specification (DSDF) in the section on "Host

Initiated Bad Block Replacement." Controllers  that  sup-

port  Controller  Initiated Bad Block Replacement perform

these checks themselves.

See Chapter chapter(cntrlrinitbbr), "Controller Initiated

Bad Block Replacement" for complete details.

Exclusive Access

A  non-host-settable characteristic;  set if this unit is

under the "Exclusive Access" of a particular host.   That

is, the unit although in a multihost environment and nor-

mally accessible by multiple hosts is now only accessible

by  the one host that requested and was granted exclusive

access of the unit via an AVAILABLE, ONLINE or  SET  UNIT

CHARACTERISTICS  command.   See the descriptions of those

commands in Chapter  chapter(multihostfunct),  "Multihost

Support."

This characteristic shall be fully supported by controll-

ers that provide Multihost Support.

Removable Media

A non-host-settable characteristic;  set if this unit has

removable media.

This  characteristic shall be fully supported by all con-

trollers that support units with removable media.

Valid  whenever  the  controller can determine the unit's

characteristics.  See the descriptions of  the  GET  UNIT

STATUS,  ONLINE, and SET UNIT CHARACTERISTICS command end

messages contained  in  Chapters  chapter(minimalcmdset),

"Minimal  MSCP  Command Set" and chapter(multihostfunct),

"Multihost Support" for complete details.

MSCP CONTROL MESSAGES                                   PAGE 5-44

UNIT FLAGS

Write History Logging Support

A  non-host-settable  characteristic;   shall be returned

set if and only if the controller supports "write history

logging"  for  this  unit  as  described  in Section sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description."  See  the  description of the Write History

Logging Support controller flag in  Section  section(con-

trolerflags), "Controller Flags."

Write Protect (data safety)

A  non-host-settable  characteristic;   set  by  the con-

troller whenever some condition in  the  unit  or  volume

prevents  reliable  modification  of  data on the volume.

Possible causes include:

o  An incomplete bad block replacement.

o  An invalid Replacement Control Table (RCT).

o  The  unit  is  only  capable  of reading the

volume's  format  (e.g.,  a   single-density

volume in a double-density drive).

The  consequences  of  Write  Protection are described in

Section section(write_protect), "Write Protection."

This  characteristic shall be fully supported if the con-

troller provides Controller Initiated Bad Block  Replace-

ment  support or supports units capable of only reading a

volume's format  (e.g.,  a  single-density  volume  in  a

double-density drive).

Write Protect (hardware)

A  non-host-settable  characteristic;   set if the unit's

write protect mechanism is activated, causing the unit to

be  Hardware  Write Protected.  The consequences of Write

Protection      are      described       in       Section

section(write_protect), "Write Protection."

Write Protect (software)

Normally  a non-host-settable characteristic;  set if the

unit is  "Software"  Write  Protected.   A  host-settable

characteristic  if the "Enable Set Write Protect" command

modifier is set in the ONLINE or SET UNIT CHARACTERISTICS

command.   If  the  "Enable  Set  Write  Protect" command

MSCP CONTROL MESSAGES                                   PAGE 5-45

UNIT FLAGS

modifier  is  set  the  controller  enables  or  disables

"Software"  Write  Protection according to the state (set

or clear, respectively) of the "Write Protect (software)"

flag in the "unit flags" field.  If the "Enable Set Write

Protect" command modifier is clear the controller ignores

the  "Write  Protect (software)" flag in the "unit flags"

field;  the state of "Software" Write Protection  remains

unchanged.   The  consequences  of  Write  Protection are

described  in  Section   section(write_protect),   "Write

Protection."

576 Byte Sectors

Normally  a non-host-settable characteristic;  set if the

disk volume mounted on the unit has 576 byte sectors.   A

host-settable  characteristic  if the controller supports

576 byte sectors and the "Ignore Media Format Error" com-

mand  modifier  is set in the ONLINE command (see Section

section(online___cmf), "ONLINE Command,  Command  Message

Format" for complete details).

This  characteristic shall be fully supported if the con-

troller supports disks formatted with either 512  or  576

byte sectors.  If the controller only supports disks for-

matted with 512 byte sectors, the controller shall ignore

any value the host specifies for this flag and always re-

turn it clear.  Note that all  controllers  that  support

disk class devices shall support disks formatted with 512

byte sectors.

Refer  to  Appendix  appendix(opcdsflgsoffs),  "Opcode, Flag, and

Offset Definitions," Table table(unitflagsdefs), "Unit Flags" for

the  values  assigned  to the currently defined host-settable and

non-host-settable unit flags.  Those bits  in  the  "unit  flags"

field  that are not defined as unit flags are reserved, and shall

be treated in  accordance  with  the  requirements  for  reserved

fields described in Section section(rsvdundefflds), "Reserved and

Undefined Fields."

MSCP CONTROL MESSAGES                                   PAGE 5-46

ATTENTION MESSAGES

5.8  ATTENTION MESSAGES

MSCP  servers  report  duplicate  unit  number conditions and the

availability of units spontaneously to hosts via  attention  mes-

sages.   Hosts  may  enable or disable attention messages via the

setting of the "Enable Attention  Messages"  flag  in  the  "con-

troller  flags"  field of the SET CONTROLLER CHARACTERISTICS com-

mand (see Section section(controlerflags),  "Controller  Flags").

Note  that attention messages are flow controlled as described in

Section section(flow__control),  "Flow  Control"  and  associated

subsections.   Hosts  shall  discard  any attention messages that

they do not recognize.

The generic attention message format is as follows:

31 0

+--------------------------------+

| reserved |

+---------------+----------------+

| reserved | unit number |

+---------------+-------+--------+

| reserved | rsvd |atncode |

+---------------+-------+--------+

| |

/ parameters /

/ /

| |

+--------------------------------+

The fields in the generic attention message are as follows:

unit number

Identifies  the  specific unit within the device class to

which the attention message applies.  This value  is  the

binary equivalent of the decimal unit number displayed by

the unit select mechanism.

atncode

Identifies  the  attention message type.  The atncode im-

plicitly specifies the length and format of the  message,

including  the  interpretation of any parameters that are

present.

Appendix   appendix(opcdsflgsoffs),  "Opcode,  Flag,  and

Offset Definitions," Table table(cntrlmessopcd), "Control

Message  Opcodes,"  lists  the  valid  atncodes and their

meanings.

MSCP CONTROL MESSAGES                                   PAGE 5-47

ATTENTION MESSAGES

Appendix  appendix(opcdsflgsoffs),  "Opcode, Flag, and Offset De-

finitions," Table table(endattmesoff), "End and Attention Message

Offsets"  defines the preferred mnemonic and numeric value of the

offset within the attention message of the various attention mes-

sage parameter fields, as well as the size of each field.

The  following  sections  describe the specific attention message

formats.

MSCP CONTROL MESSAGES                                   PAGE 5-48

ATTENTION MESSAGES

5.8.1  ACCESS PATH Attention Message

5.8.1.1  Attention Message Format  -

31 0

+--------------------------------+

| reserved |

+---------------+----------------+

| reserved | unit number |

+---------------+-------+--------+

| reserved | rsvd |atncode |

+---------------+-------+--------+

The fields in the ACCESS PATH attention message are as follows:

unit number

Identifies the unit for which an alternate access path is

being reported.

atncode

See Section section(atten___mess), "Attention Messages."

5.8.1.2  Description  -

MSCP  servers  use this attention message to report alternate ac-

cess paths to multiaccess units.  This message reports  that  the

specified  unit  is  potentially  accessible via the sending MSCP

server -- i.e., it would be "Unit-Available" if it and all  units

that  share  its  access path ceased being "Unit-Online" or under

"Exclusive Access" via another controller.   The  specific  event

that  causes an MSCP server to send this attention message is the

receipt, by the controller to which the unit is "Unit-Online"  or

is under "Exclusive Access," of a DETERMINE ACCESS PATHS command.

This attention message is sent to  all  class  drivers  that  are

"Controller-Online" to the MSCP server and have enabled attention

messages.   See  Section  section(multiaccesdrvs),   "Multiaccess

Drives," for more information.

MSCP CONTROL MESSAGES                                   PAGE 5-49

ATTENTION MESSAGES

MSCP CONTROL MESSAGES                                   PAGE 5-50

ATTENTION MESSAGES

5.8.2  AVAILABLE Attention Message

5.8.2.1  Attention Message Format  -

31 0

+--------------------------------+

| reserved |

+---------------+----------------+

| reserved | unit number |

+---------------+-------+--------+

| reserved | rsvd |atncode |

+---------------+-------+--------+

| unit flags | multiunit code |

+---------------+----------------+

| undefined |

+--------------------------------+

| |

+--- unit identifier ---+

| |

+--------------------------------+

| media type identifier |

+---------------+----------------+

The fields in the AVAILABLE attention message are as follows:

unit number

Identifies the unit that just became "Unit-Available."

atncode

See Section section(atten___mess), "Attention Messages."

multiunit code

unit flags

unit identifier

media type identifier

Identical  to  the  corresponding  fields in the GET UNIT

STATUS or SET UNIT CHARACTERISTICS command end  messages.

These fields shall be valid as defined for the unit being

"Unit-Available," regardless of the actual state  of  the

unit  when  the message is sent.  That is, the "multiunit

code," "unit identifier,"  and  "media  type  identifier"

shall  all  be  valid and the "Removable Media" unit flag

MSCP CONTROL MESSAGES                                   PAGE 5-51

ATTENTION MESSAGES

shall be valid.

5.8.2.2  Description  -

An  MSCP  server  sends  an  AVAILABLE  attention  message  to  a

"Controller-Online" class driver when a unit  asynchronously  be-

comes "Unit-Available" to that class driver, unless AVAILABLE at-

tention messages  have  been  suppressed  for  that  unit  by  an

AVAILABLE command with the "Spin-down" modifier or by an error or

other condition  with  similar  side  effects.   Changes  to  the

"Unit-Available"  state due to the class driver itself issuing an

AVAILABLE  command  are  synchronous.   All  other   changes   to

"Unit-Available"      are      asynchronous.      See     Section

section(unit__states), "Unit States."

The  actual  sending  of  an  AVAILABLE  attention message may be

delayed for an  arbitrarily  long  time,  due  to  communications

mechanism  flow control, from the time that the unit actually be-

comes "Unit-Available." The message shall  not  be  sent  if  the

class  driver ceases to be "Controller-Online" during this delay.

The message shall be sent anyway if the unit, or  any  unit  with

which  it  shares  an access path, becomes "Unit-Online" or under

"Exclusive Access" via another controller during this delay.  The

message  may  or  may not be sent, at the controller's option, if

the unit ceases to be "Unit-Available" for any other reason  dur-

ing this delay.

Note  that,  due to these delays, it is possible for an AVAILABLE

attention message to be  received  after  the  class  driver  has

already  brought  the  unit  "Unit-Online"  or  under  "Exclusive

Access." Therefore class drivers shall not use  AVAILABLE  atten-

tion   messages   to   flag   such   units   as   having   become

"Unit-Available." The proper procedure is  to  issue  a  command,

such  as  a GET UNIT STATUS, to a "Unit-Online" unit for which an

AVAILABLE attention message has been received, and only flag  the

unit  as  having  become  "Unit-Available" if the command returns

that status code.

AVAILABLE  attention  messages  are  not  sent for units that are

already "Unit-Available" when a class  driver  enables  attention

messages.    Class   drivers   that  need  to  be  aware  of  all

"Unit-Available" units shall enable attention messages, then scan

all  units  via  the GET UNIT STATUS command with the "Next Unit"

modifier   set   to   locate   all   units   that   are   already

"Unit-Available." All     units    that    subsequently    become

"Unit-Available" will be reported  with  an  AVAILABLE  attention

message.

MSCP CONTROL MESSAGES                                   PAGE 5-52

ATTENTION MESSAGES

An  MSCP  server may send redundant or erroneous AVAILABLE atten-

tion messages at any time.  The frequency of such messages  shall

be  low  enough that they do not represent a significant overhead

for either hosts or the communications mechanism.   The  informa-

tion  contained  in  such messages (unit number, unit identifier,

media type identifier, etc.) shall correspond to an actual,  phy-

sical  unit  that  is potentially accessible via that MSCP server

(i.e., connected to the controller), although the unit  need  not

be "Unit-Available." Note that hosts must be able to handle seem-

ingly erroneous AVAILABLE attention messages in any  case,  since

the  unit's state may change before the host can act on an other-

wise correct message.

MSCP CONTROL MESSAGES                                   PAGE 5-53

ATTENTION MESSAGES

5.8.3  DUPLICATE UNIT NUMBER Attention Message

5.8.3.1  Attention Message Format  -

31 0

+--------------------------------+

| reserved |

+---------------+----------------+

| reserved | unit number |

+---------------+-------+--------+

| reserved | rsvd |atncode |

+---------------+-------+--------+

The  fields in the DUPLICATE UNIT NUMBER attention message are as

follows:

unit number

Identifies  the  unit number that is duplicated on two or

more units.

atncode

See Section section(atten___mess), "Attention Messages."

5.8.3.2  Description  -

An  MSCP server sends DUPLICATE UNIT NUMBER attention messages to

notify hosts that two or more units of the same device class have

the  same  unit  number.  This allows the hosts to complain to an

operator, who can correct  the  condition.   The  DUPLICATE  UNIT

NUMBER  attention  messages  are  sent  to  all  hosts  that  are

"Controller-Online" and have  enabled  attention  messages.   See

Section  section(unit__numbers),  "Unit  Numbers," for a detailed

discussion of the handling of duplicate unit numbers.  Note  that

a  DUPLICATE  UNIT NUMBER attention message is sent regardless of

whether or not one of the units is "Unit-Online"  or  under  "Ex-

clusive Access."

MSCP CONTROL MESSAGES                                   PAGE 5-54

ATTENTION MESSAGES

The  actual  sending of a DUPLICATE UNIT NUMBER attention message

may be delayed for an arbitrarily long time,  due  to  communica-

tions  mechanism  flow control, from the time that the controller

first detects the duplicate unit number.  The message  shall  not

be sent if the class driver ceases to be "Controller-Online" dur-

ing this delay.  The message may or  may  not  be  sent,  at  the

controller's  option, if the duplicate unit number condition dis-

appears during this delay.

DUPLICATE  UNIT NUMBER attention messages are not sent for dupli-

cate unit number conditions  that  already  exist  when  a  class

driver enables attention messages.  Class drivers that need to be

aware of all duplicate unit number conditions shall enable atten-

tion  messages,  then scan all units via the GET UNIT STATUS com-

mand with the "Next Unit" modifier set to  locate  all  duplicate

unit  numbers.   All  duplicate  unit numbers that the controller

subsequently detects will  be  reported  with  a  DUPLICATE  UNIT

NUMBER attention message.

An MSCP server may send redundant DUPLICATE UNIT NUMBER attention

messages.  The frequency of such messages  shall  be  low  enough

that  they  do  not  represent  a significant overhead for either

hosts or the communications mechanism.  Furthermore,  the  dupli-

cate  unit number condition being reported must actually exist at

the time the MSCP server decides to generate the  DUPLICATE  UNIT

NUMBER attention message.  Note, however, that the duplicate unit

number condition may have disappeared by the time  the  host  re-

ceives or acts upon the message.

5.9  ERROR LOG MESSAGES

MSCP  controllers  report  errors  and unusual occurrences in two

ways:  end messages and error log messages.  Unrecoverable errors

are  reported  in the end message of the command that encountered

the error.  Such errors should be reported back  to  the  program

that  initiated  the  command so that it can take appropriate ac-

tion.  Additionally, all "significant" errors are reported in er-

ror log messages so that they can be recorded in the host's error

log for eventual use by Field Service.  The  definition  of  what

constitutes  a  "significant"  error  is controller and/or device

specific.  In general, anything that  would  be  of  interest  to

Field  Service  is a "significant" error.  The errors reported by

error log messages may be either  recoverable  or  unrecoverable.

Note that hosts should not record errors reported in end messages

MSCP CONTROL MESSAGES                                   PAGE 5-55

ERROR LOG MESSAGES

in the error log.  If the error is "significant," a separate  er-

ror log message will be generated.

When  a  host receives an error log message, the host port driver

passes the class driver the error log message text and the length

of  the  error log message.  In addition, the device class (i.e.,

disk or tape) of the error log message is implicit in the connec-

tion  on  which the message was received.  Note that the order of

receipt of error log messages relative to end or  attention  mes-

sages  is  expressly  undefined.  Therefore, an error log message

may be received either  before  or  after  an  end  message  that

reports the same error or that has the "Error Log Generated" flag

set.

MSCP  servers assign a sequence number to every error log message

they generate.  The sequence number assigned is reported  to  the

host in the "Sequence Number" field in each error log message and

in each command end message that has the  "Error  Log  Generated"

flag set.  This sequence number serves the following purposes:

1.  It  permits the host to determine when all error log

messages associated with a particular  command  have

been received.  The host can then release any of the

command's context it may have saved for inclusion in

its  error  log.  An example of how a host might use

the error log sequence number for this  purpose  ap-

pears later in this section.

2.  If  the  same  error log message is sent to multiple

hosts, which then record it in a  common  error  log

file, it allows the multiple copies of the same mes-

sage to be recognized as describing the same error.

3.  If  a  host requests that it receive every error log

message that an MSCP  server  generates,  it  allows

that  host  to detect missing or lost error log mes-

sages by detecting gaps in the sequence numbers.  In

this  case the host is required to set all three er-

ror log enable flags in the "controller flags" field

of  the  SET CONTROLLER CHARACTERISTICS command (see

Section     section(controlerflags),     "Controller

Flags").

MSCP CONTROL MESSAGES                                   PAGE 5-56

ERROR LOG MESSAGES

Some  controllers can achieve these purposes via other means, and

therefore need not implement error log sequence numbers.  The re-

quirements such controllers must meet are described later in this

section.

Each  MSCP  server implements a single error log sequence number,

which it uses for all error log messages for all  class  drivers.

The  server  increments its sequence number each time it attempts

to generate an error log message.  (Note  that  sequence  numbers

may  wrap  around, so that sequence number 0 comes after sequence

number 65535.)

Multiple  copies of the same error log message shall all have the

same sequence number.  On each individual  connection,  the  MSCP

server  shall  send error log messages in the same order as their

sequence numbers.  The relative order of error log messages  sent

on different connections is undefined (and therefore a controller

option).

Whenever an MSCP server has pending error log messages for a con-

nection, it shall send at least one error  log  message  on  that

connection within the controller timeout interval.

An  MSCP  server  has pending error log messages for a connection

whenever it has notified the class driver on that connection that

one  or  more  error  log messages are forthcoming (i.e., set the

"Error Log Generated" flag in an end message), but  has  not  yet

sent  or  dropped  all error log messages up to the one whose se-

quence  number  was  reported  in  the  end  message.   The  term

"dropped"  refers to the MSCP server determining that it will not

send a message due to lack of some internal resource.

The  sequence number is reset whenever the MSCP server loses con-

text.  The first error log message after such a loss  of  context

has sequence number zero.  The sequence number shall not be reset

as   a   normal   result   of   the    MSCP    server    becoming

"Controller-Online"  or "Controller-Available" to a class driver.

Note that each MSCP server (i.e., each  device  class)  within  a

controller has its own error log sequence number.

As stated above, the error log sequence number is only reset when

the MSCP server loses context.  The MSCP server reports the  fact

that  it  has lost context by setting the "Sequence Number Reset"

flag (see Section section(generrlogfmt), "Generic Error Log  Mes-

sage  Format")  in  the  first error log message it sends to each

class driver after said loss of context.  This means, in  effect,

that  the  MSCP  server  must  keep track of all class drivers to

MSCP CONTROL MESSAGES                                   PAGE 5-57

ERROR LOG MESSAGES

which it has sent an error log message since  its  last  loss  of

context,   regardless   of   whether   or  not  it  is  currently

"Controller-Online" to those class drivers.

An  example of how a host might use the error log sequence number

to correlate error log messages with outstanding command  context

follows.

The  class  driver  keeps track of the sequence number of

the most recent error log message it has received.   This

sequence  number  should  be initialized to zero when the

connection is established.  Furthermore, in  use  command

slots  (containing  command context) come in two forms --

active commands and commands waiting for error  log  mes-

sages.   The  class driver also keeps track of whether or

not it has received an error log message within each con-

troller  timeout  interval  for  its  command timeout al-

gorithm.

When  the  class driver receives an end message, it first

checks the "Error Log Generated" flag.  If that  flag  is

clear,  it releases the command slot as no longer in use.

If that flag is set, it calls the RELEASE_SLOT  procedure

(see  pseudo-code  below) with the "sequence number" from

the command's end message and the sequence number of  the

most  recent  error  log message the class driver has re-

ceived.  If RELEASE_SLOT returns TRUE, the  class  driver

releases  the  command slot as no longer in use.  If that

procedure returns FALSE, the class driver marks the  slot

as waiting for error log messages.

When  the  class driver receives an error log message, it

calls the RELEASE_SLOT procedure, with the error log  se-

quence  number  from  the  command  slot (copied from the

command's end message) and the sequence number of the new

error  log  message,  once  for each command slot that is

waiting for error log messages.  If RELEASE_SLOT  returns

TRUE,  the  class  driver releases the command slot as no

longer in use.  If RELEASE_SLOT returns FALSE,  the  slot

remains marked as waiting for error log messages.

Command  slots  that  are  waiting for error log messages

participate in the class  driver's  command  timeout  al-

gorithm  (see  Section  section(command_timos),  "Command

Timeouts") just the same as slots  for  outstanding  com-

mands.  If the oldest outstanding command slot is waiting

for error log messages, the class driver  checks  if  any

error   log   messages  have  been  received  within  the

MSCP CONTROL MESSAGES                                   PAGE 5-58

ERROR LOG MESSAGES

controller timeout interval.  If one or  more  error  log

messages have been received, the slot remains in use.  If

no error log messages have  been  received,  the  command

slot  is  released -- the error log message for which the

slot was waiting has been lost.

The  RELEASE_SLOT procedure accepts an error log sequence

number from an end message and a sequence number from  an

error  log  message.   It  returns  TRUE if the error log

number comes after  the  end  message  number  and  FALSE

otherwise.   This  comparison takes into account sequence

number wrap-around.  The procedure (pseudo-code) is:

function RELEASE_SLOT (

end_msg_number : 0..65535 ;

err_log_number : 0..65535 ) : boolean ;

const

FUDGE = 1 ;

var

i : -65535..65535 ;

begin ;

{ following three lines are just a 16 bit }

{ signed   subtract  operation,  ignoring }

{ integer  overflow (i.e., the PDP-11 SUB }

{ instruction). }

i := err_log_number - end_msg_number ;

if i < -32768 then i := i + 65536 ;

if i > 32767 then i := i - 65536 ;

RELEASE_SLOT := (i >= FUDGE-1) ;

end ;

The  constant  FUDGE  is the maximum degree of communica-

tions mechanism nonsequentiality for  datagrams.   Assume

that  datagrams n, n+1, n+2, etc.  are sent in that order

across a connection.  FUDGE is  the  smallest  value  for

which  the  communications  mechanism guarantees that, if

both datagram n and datagram n+FUDGE are  received,  then

datagram n is received before datagram n+FUDGE.  All cur-

rent and proposed communications mechanisms have  sequen-

tial datagrams, for which FUDGE is one.  The only type of

communications mechanism for which  FUDGE  might  have  a

different  value  is  one that has multiple paths between

nodes, and allows datagrams for a  single  connection  to

follow different paths.

As  mentioned  earlier,  controllers need not implement error log

sequence numbers, since their purposes can be achieved via  other

means.   Controllers  that meet all of the following requirements

need not implement error log sequence numbers:

MSCP CONTROL MESSAGES                                   PAGE 5-59

ERROR LOG MESSAGES

1.  The  MSCP  server  never drops error log messages.  That

is, whenever it has an error log message to generate, it

blocks  or  deadlocks  until  it is able to generate the

message.

2.  The communications mechanism between the MSCP server and

the class driver guarantees all error log messages  will

be  delivered  without  loss or duplication in the order

that they were generated.  That is,  the  communications

mechanism  provides  the  same  guarantees for datagrams

(error log messages) that  it  provides  for  sequential

messages (control messages).

3.  The  MSCP  server and communications mechanism guarantee

that the class driver will receive all  error  log  mes-

sages  that  reference  a particular command's reference

number before receiving that command's end message.

4.  The  MSCP  server  and  communications mechanism are in-

herently incapable of communicating with more  than  one

class driver.

MSCP  servers  that meet the above four requirements may generate

all error log messages with a sequence number of  zero  and  with

the  "Sequence  Number  Reset" flag set, rather than actually im-

plementing sequence numbers.  All other MSCP servers shall imple-

ment  an  actual  error  log  sequence  number.   Such other MSCP

servers may not lose context solely to reset the  error  log  se-

quence  number.  All losses of context must be caused by some ex-

ternal event such as a power failure.

Since  error  log  messages  are not subject to flow control (see

Section section(flow__control), "Flow Control"), it  is  possible

for  a  controller  to  generate error log messages faster than a

host can record them in its error log.  In order to minimize  the

probability  of this happening, and thus minimize the probability

of losing error log information, controllers  shall  generate  no

more than three error log messages in response to a single error.

Note that the definition of what constitutes an error  is  neces-

sarily  controller  and/or  drive  dependent.   The three message

limit encompasses an original error and all error recovery / cor-

rection  / retry sequences associated with that error.  Seemingly

unrelated errors that occur in a recovery sequence are  generally

considered  to be different errors, and are therefore not covered

by the three message limit.

For example, consider an uncorrectable ECC error on a read.  Rer-

eads using offset head positioning and the like are part  of  the

retry  sequence,  and thus fall under the three message limit for

the original error.  However, failure of  the  command  directing

MSCP CONTROL MESSAGES                                   PAGE 5-60

ERROR LOG MESSAGES

the  drive  to use offset head positioning (i.e., the command it-

self fails, indicating that the heads could not be offset)  would

be  considered  a  separate  error,  even  though both it and the

original read error might have a common cause (such as a bad  ca-

ble between the controller and the drive).

In order to achieve this three message limit, most error log mes-

sages summarize the results of an entire  retry  sequence.   This

approach  generally  results in exactly one error log message per

error.  The other approach is to generate a  separate  error  log

message  for each attempt or retry that fails.  This approach can

only be used with errors for which the number of retries is small

(i.e.,  three  or  fewer  attempts total), as otherwise the three

message limit will be exceeded.

5.9.1  Maximum Error Log Message Size

MSCP  and  TMSCP  do  not place any explicit limit on the maximum

size of an error log message.  Rather, they simply  require  that

in  any supported or operational configuration, the maximum error

log message size generated by a device's  MSCP  or  TMSCP  server

must  be  less than or equal to the maximum supported by the host

class driver and any intervening adapters.  In essence, error log

message size limits are outside the scope of this specification.

In  order  to  assist in determining if maximum error log message

sizes are compatible in a particular configuration, all MSCP  and

TMSCP entities shall register their error log message size limits

with the SSAG Architectural Compliance Registry.  Information  on

procedures for using the registry may be obtained by sending mail

to SSAG::SSAG.

Each  MSCP or TMSCP class driver shall register the maximum error

log message size they support.  They shall  list  multiple  sizes

with appropriate qualification if they support different sizes in

different circumstances, such as with different port  drivers  or

adapters.   Any  limits  imposed by adapters shall be included in

the class driver's registered limits, since such limitations  are

extremely few in number.

Each  MSCP or TMSCP server shall register the maximum size of any

error log message it may generate.  It is reasonable to list mul-

tiple sizes if class drivers and/or system configurations can af-

fect what sizes are used.  For example, if the  longest  messages

will  only  be  generated  if  a particular option is used, it is

reasonable to list limits both with and without that option being

MSCP CONTROL MESSAGES                                   PAGE 5-61

ERROR LOG MESSAGES

used.   An  MSCP  or TMSCP server shall not generate an error log

message longer than what it has registered, under whatever condi-

tions   or  qualifications  are  included  in  its  registration.

Furthermore, an MSCP or TMSCP server may not register  a  maximum

error  log  message  size  unless a corresponding class driver is

already registered as supporting at least that maximum.

The purpose of this last restriction is to make it clear that the

burden of obtaining support for a new, longer maximum  error  log

message  size  lies  upon  the device wishing to use those longer

messages.  The device development group must first obtain  agree-

ment  from  some  host  OS  group to support the longer messages,

leading to the host OS group changing its  Architectural  Compli-

ance  Registry  entry to indicate the longer size.  Only then may

the device group register its device and begin  to  generate  the

longer messages.

Notwithstanding  the above requirements, the following guidelines

should be followed by all new or significantly modified products.

Adherence  to these guidelines will minimize support and compati-

bility problems with regard to error log message sizes.

1.  For SSP devices, all class drivers should be designed to

support a maximum error log message size of 124 bytes or

larger.   This  corresponds  to  a  128 byte SSP message

buffer less 4 bytes overhead.  Note,  however,  that  at

the  time  this  was written the KFQSA adapter imposed a

limit of 88 bytes on MSCP or TMSCP error log messages.

2.  For  all  other  devices,  all  class  drivers should be

designed to support a maximum error log message size  of

512 bytes or larger.

3.  For SSP devices, servers should be designed to use error

log messages no longer than 60 bytes.   Longer  messages

may   encounter   compatibility   problems   with  older

software.

4.  For all other devices, servers should be designed to use

error log messages no longer  than  128  bytes.   Longer

messages may encounter compatibility problems with older

software.

These  are  only  recommended  guidelines,  and  may  be violated

without affecting MSCP or TMSCP  compliance.   But  special  care

should  be taken to consider the risk of incompatibility whenever

these recommendations are not followed.

MSCP CONTROL MESSAGES                                   PAGE 5-62

ERROR LOG MESSAGES

As  has already been stated, in any supported or operational con-

figuration, the maximum error log message  size  generated  by  a

device's  MSCP  or TMSCP server must be less than or equal to the

maximum supported by the host class driver  and  any  intervening

adapters.   The  converse  of  this  is  that  if  a device might

generate an error log message longer  than  an  adapter  or  host

class  driver supports, then their combination is not a supported

configuration and need not be operational.  All  the  same,  such

mis-configured  systems will occur in practice, so defensive pro-

gramming techniques should be  used  to  simplify  diagnosis  and

minimize  problems  with  mis-configurations.   A system crash or

weird behavior that only occurred when  a  particular  error  log

message arrived would be extremely difficult to diagnose.

To  this  end it is strongly recommended that all entities parti-

cipating in an MSCP or TMSCP connection  explicitly  check  error

log  message  sizes,  rather  than  implicitly  assuming they are

valid.  Ideally an explicit message would be displayed to  report

the  mis-configuration, although this may be impractical for most

devices.  If the communications mechanism allows  it,  error  log

message  sizes should be checked when a connection is established

rather than waiting for an overly long error log  message  to  be

generated.   If  a  mis-configuration  is present, the connection

should be rejected, rather than establishing the connection  only

to have a problem at the unpredictable time a long error log mes-

sage is generated.

If  an  error  log message longer than that which is supported is

nonetheless generated, it should be truncated  to  the  supported

maximum size on that connection.  The actual truncation may occur

in the server, class driver, an adapter, or  anywhere  else  con-

venient.   With  most  communications  mechanisms, any truncation

should be performed by the sender before the error log message is

sent.   For  example, truncation to the SCA maximum datagram size

should occur in the server sending the error log message,  before

it is passed to SCA.  Similarly, truncation to the size of an SSP

message buffer should occur in the SSP  adapter.   Truncation  to

the  maximum size supported by the host's error logging subsystem

would normally be done in the host.

The truncated size should be recorded with the error log message,

so that undefined data is not mistakenly interpreted as belonging

to  the message.  Error log analysis tools might subsequently use

device dependent knowledge to recognize that the message has been

truncated  and  report the mis-configuration.  Connections should

not be broken if an overly long error log message  is  generated,

nor should such messages be discarded due to their size.

MSCP CONTROL MESSAGES                                   PAGE 5-63

ERROR LOG MESSAGES

5.9.2  Generic Error Log Message Format

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number| unit number |

+---------------+-------+--------+

| event code | flags | format |

+---------------+-------+--------+

| |

+--- controller identifier ---+

| |

+---------------+-------+--------+

| multiunit code|chvrsn | csvrsn |

+---------------+-------+--------+

| |

+--- unit identifier ---+

| |

+---------------+-------+--------+

| fmt dependent |uhvrsn | usvrsn |

+---------------+-------+--------+

| volume serial number |

+--------------------------------+

| |

/ format dependent /

/ information /

| |

+--------------------------------+

The fields in the generic error log message are as follows:

command reference number

The  command  reference  number  of the MSCP command that

caused the error reported by this error log  message,  or

zero  if the error does not correspond to a specific out-

standing command.   If  this  field  contains  a  command

reference  number,  then  the  command's end message will

also have the "error log generated" end message flag set.

Note  that  the  error log message may be received either

before or shortly after the command's end message.

MSCP CONTROL MESSAGES                                   PAGE 5-64

ERROR LOG MESSAGES

unit number

The  unit  number of the unit to which the error log mes-

sage relates, or zero if the message does not relate to a

specific unit.  This field may contain the unit number of

any unit of the drive or formatter if the  error  relates

to  an entire multiunit drive or formatter.  The validity

of this field is determined by the value in the  "format"

field.

sequence number

The  sequence  number of this error log message since the

last time the MSCP server lost context, or  zero  if  the

MSCP   server  does  not  implement  error  log  sequence

numbers.  Note that error log sequence numbers are common

to  all  class drivers, and are not reset by class driver

resynchronization.  Note also that the class  driver  may

receive error log messages out of sequence.

format

The value in this field identifies the detailed format of

the error log message, as defined in the  following  sec-

tions.

Refer to Appendix appendix(opcdsflgsoffs), "Opcode, Flag,

and Offset Definitions," Table table(errlogmesfmt),  "Er-

ror  Log Message Format Codes" for the values assigned to

the currently defined formats.

flags

Bit  flags,  collectively called error log message flags,

used to report various attributes of the error.  The fol-

lowing flags are defined:

Bad Block Replacement Request

If  set,  the  block  identified by the error log

message is a bad block, and the  Controller  Ini-

tiated  Bad Block Replacement layer will soon at-

tempt to replace it.   Note  that  this  flag  is

reported   only   by   controllers  that  support

MSCP CONTROL MESSAGES                                   PAGE 5-65

ERROR LOG MESSAGES

Controller Initiated Bad  Block  Replacement  for

disk class devices.

Disk Copy Data Correlate

If  set,  the  reported error occurred during the

execution of a DISK COPY DATA command  where  the

source  unit  is located in a remote subsystem as

described  in  Section   section(dskcopydatades),

"DISK  COPY DATA Command, Description." Note that

the server sends a DISK COPY DATA CORRELATION er-

ror    log   as   described   in   Section   sec-

tion(dcderrlogfmt),  prior  to  sending  the  end

message  for a particular DISK COPY DATA command.

See that section for related information.

Error During Replacement

If set, the reported error occurred during an ac-

cess initiated by the bad block replacement  pro-

cess.   Note  that  this flag is reported only by

controllers that support Controller Initiated Bad

Block Replacement for disk class devices.

Operation Successful

If set, the operation causing this error log mes-

sage has been successfully completed.  The  error

log  message  summarizes  the retry sequence that

was necessary to successfully complete the opera-

tion.   If  clear, the operation has not yet been

successfully completed.

Operation Continuing

If  set,  the  retry  sequence for this operation

will  be  continued.   This  error  log   message

reports  the  unsuccessful  completion  of one or

more retries.  If clear, the retry  sequence  for

this  operation has terminated.  Provided "Opera-

tion Successful" is also clear, the  retry  limit

for  this  operation  has been reached and an un-

recoverable error will  be  reported.   Undefined

(meaningless) if "Operation Successful" is set.

Sequence Number Reset

If  set, then the error log sequence number ("se-

quence number" field) has been reset by the  MSCP

server  since  the last error log message sent to

MSCP CONTROL MESSAGES                                   PAGE 5-66

ERROR LOG MESSAGES

the receiving class driver.  If  clear,  the  se-

quence  number  has not been reset, implying that

the "sequence number" field may be used to detect

missing  error  log  messages.  Always set if the

MSCP server does not implement error log sequence

numbers.

Informational

If set, the data contained in this error log mes-

sage is informational only.  The  information  is

not  necessarily related to any single command or

event.  Note that the error log message does  NOT

signify  the occurrence of an error and shall not

be included in device error counts.

If  "Operation Successful" and "Operation Continuing" are

both clear, then the error log  message  reports  a  hard

(unrecoverable)  error.   If  "Operation  Successful"  is

clear and "Operation Continuing" is set, then  the  error

log  message reports an intermediate step within an error

recovery operation.  It is not yet  certain  whether  the

error is hard or soft.  If "Operation Successful" is set,

then the error log message summarizes the retry  sequence

used to recover from a soft error.

Refer to Appendix appendix(opcdsflgsoffs), "Opcode, Flag,

and Offset Definitions," Table table(errlogmesflg),  "Er-

ror  Log  Message  Flags"  for the values assigned to the

currently defined flags.

event code

Identifies  the specific error or event being reported by

this     error     log     message.      See      Section

section(status__codes), "Status and Event Codes."

The subcode portion of the event code is potentially con-

troller and/or device specific.  However, the same  major

code  /  subcode  combination,  whenever it is used, must

always have the same meaning.   Therefore,  new  subcodes

may  be  defined  as  new devices are introduced, but the

meaning of old subcodes should not  change.   Event  code

values  are  listed  in Appendix appendix(statusevntdef),

"Status and Event Code Definitions."

controller identifier

MSCP CONTROL MESSAGES                                   PAGE 5-67

ERROR LOG MESSAGES

Uniquely  identifies the controller among all devices ac-

cessible via MSCP.  See  Section  section(cntrlrunitids),

"Controller and Unit Identifiers."

csvrsn

The  controller's  software, firmware, or microcode revi-

sion number.  Always zero if  the  product's  development

and  maintainability  groups  determine  that there is no

benefit  from  supporting   machine   readable   revision

numbers.

chvrsn

The  controller's  hardware revision number.  Always zero

if the product's development and  maintainability  groups

determine  that  there  is  no  benefit  from  supporting

machine readable revision numbers.

multiunit code

The  multiunit  code,  as defined in Section section(mul-

tidrvsfmts), "Multiunit Drives and  Formatters,"  of  the

unit  to which the error log message relates.  This field

may contain the multiunit code of any unit of  the  drive

or  formatter if the error relates to an entire multiunit

drive or formatter.

unit identifier

Uniquely identifies the unit among all devices accessible

via  MSCP.   See  Section  section(cntrlrunitids),  "Con-

troller and Unit Identifiers." This field is only present

for errors that relate to a specific unit.

usvrsn

The  unit's  software,  firmware,  or  microcode revision

number.  This field is only present for errors  that  re-

late  to  a  specific unit.  Always zero if the product's

development and  maintainability  groups  determine  that

there  is  no  benefit  from  supporting machine readable

MSCP CONTROL MESSAGES                                   PAGE 5-68

ERROR LOG MESSAGES

revision numbers.

uhvrsn

The  unit's hardware revision number.  This field is only

present for  errors  that  relate  to  a  specific  unit.

Always  zero  if the product's development and maintaina-

bility groups determine that there  is  no  benefit  from

supporting machine readable revision numbers.

volume serial number

The  low order 32 bits of the serial number of the volume

that is mounted on the unit.  Zero if the  unit's  format

does  not  provide for a volume serial number.  Undefined

(garbage) if there is no volume mounted in the unit,  the

area of the volume that contains the serial number cannot

be read  successfully,  the  error  occurred  before  the

volume  serial  number could be determined while bringing

the unit "Unit-Online," the unit is not "Unit-Online"  to

any  host, or if the "Ignore Media Format Error" modifier

was specified in the ONLINE command  that  first  brought

the  unit  "Unit-Online." This  field is only present for

errors that relate to a specific disk unit.

fmt dependent

format dependent information

The  format  of  the  remainder  of the error log message

depends upon the value of the "format" field.  Note  that

the  fields  "unit  number"  and "multiunit code" through

"volume serial number" also depend on the  value  of  the

"format"  field,  as they are only present for those for-

mats used to report errors  that  relate  to  a  specific

unit.

Appendix  appendix(opcdsflgsoffs),  "Opcode, Flag, and Offset De-

finitions,"  Table  table(errlogmesoff),   "Error   Log   Message

Offsets"  defines the preferred mnemonic and numeric value of the

offset within the error log message  of  the  various  error  log

parameter fields, as well as the size of each field.

MSCP CONTROL MESSAGES                                   PAGE 5-69

ERROR LOG MESSAGES

The  following  sections  describe the specific error log message

formats.

MSCP CONTROL MESSAGES                                   PAGE 5-70

ERROR LOG MESSAGES

5.9.3  CONTROLLER ERRORS Error Log Format

The  following  error  log  message format is used to report con-

troller errors:

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number| reserved |

+---------------+-------+--------+

| event code | flags | format |

+---------------+-------+--------+

| |

+--- controller identifier ---+

| |

+---------------+-------+--------+

| |chvrsn | csvrsn |

| +-------+--------+

| controller |

/ dependent /

/ information /

| |

+--------------------------------+

The  fields  in  the  CONTROLLER  ERRORS error log message are as

follows:

command reference number

sequence number

format

flags

event code

controller identifier

csvrsn

chvrsn

See  Section  section(generrlogfmt),  "Generic  Error Log

Message Format."

MSCP CONTROL MESSAGES                                   PAGE 5-71

ERROR LOG MESSAGES

controller dependent information

A  variable (controller dependent) amount of information.

Often no controller dependent  information  is  provided.

The  length  of  this information is implied by the total

length of the error log  message,  passed  to  the  class

driver  by  the port driver.  This information will typi-

cally not be interpreted by  error  log  formatting  pro-

grams, instead being printed as a series of values.

5.9.4  MEMORY ERRORS Error Log Format

The  following  error log message format is used to report memory

errors when the address of the offending location is available to

the  controller.  This format is used for either host or internal

controller memory access errors.  The format is:

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number| reserved |

+---------------+-------+--------+

| event code | flags | format |

+---------------+-------+--------+

| |

+--- controller identifier ---+

| |

+---------------+-------+--------+

| reserved |chvrsn | csvrsn |

+---------------+-------+--------+

| memory address |

+--------------------------------+

| |

/ controller /

/ dependent /

/ information /

/ (optional) /

| |

+--------------------------------+

The fields in the MEMORY ERRORS error log message are as follows:

command reference number

sequence number

format

flags

MSCP CONTROL MESSAGES                                   PAGE 5-72

ERROR LOG MESSAGES

event code

controller identifier

csvrsn

chvrsn

See  Section  section(generrlogfmt),  "Generic  Error Log

Message Format."

event code

The   event  code  identifies  whether  the  error  being

reported occurred in host or controller memory.  For  er-

rors   in   host   memory  the  event  code  contains  an

appropriate "Host Buffer Access Error" subcode (see Table

table(hostbufaerrsc),  "'Host  Buffer  Error'  Status  or

Event Code Subcode Values").  For  errors  in  controller

memory the event code contains an appropriate "Controller

Error"   subcode   (see    Table    table(controlrerrsc),

"'Controller   Error'   Status   or  Event  Code  Subcode

Values").

memory address (host memory access error)

For  errors  accessing  host  memory the "memory address"

field contains the address in the host's physical address

space at which the error occurred.

The  units  of  "memory address" are those natural to the

host -- bytes for 16 and 32 bit systems or words  for  36

bit  systems.  The resolution to which a controller iden-

tifies the address at which the error  occurred  is  con-

troller   dependent,   and  shall  be  described  in  the

controller's functional specification.  Disk  controllers

will typically provide a resolution of one disk block.

memory address (controller memory access error)

For  errors  accessing  controller memory the "memory ad-

dress" field contains the location  in  the  controller's

memory at which the error occurred.

Only  errors  that do not affect the controller's ability

to properly generate  end  and  error  log  messages  are

reported via MSCP and this error log format.  Errors that

might affect end and error log messages are reported  via

some mechanism other than MSCP.

The  units  of  "memory address" are those natural to the

memory in which the error occurred.  This necessarily im-

plies that the units are memory dependent.  Bytes are the

MSCP CONTROL MESSAGES                                   PAGE 5-73

ERROR LOG MESSAGES

preferred units whenever there is  any  ambiguity  as  to

what  are the natural units.  The resolution to which the

address of the error is reported is  also  memory  depen-

dent.   It is acceptable for some memories to have no ad-

dress resolution.  That is, errors in such  memories  are

reported  with  a fixed address of zero regardless of the

address at which the error occurred.  This will typically

be  used  for memories in which any error renders the en-

tire memory unusable.

Controllers  with  multiple  memories  may  identify  the

particular memory in which the error occurred  either  by

embedding  the  identification  (as  a  subfield)  in the

"memory address" field or by providing the identification

in   the   "controller   dependent   information"   field

(described below).  The units of  "memory  address,"  ad-

dress  resolution,  and  memory identification (where re-

quired) are controller dependent and shall  be  described

in the controller's functional specification.

controller dependent information

An  optional, variable length field containing controller

dependent information.  The length of this field  is  im-

plied  by  the  total  length  of  the error log message,

passed to the class driver  by  the  port  driver.   This

field  may be used to provide additional information when

reporting either controller  or  host  memory  errors  or

both.   The  use  and  contents shall be described in the

controller's functional specification.

MSCP CONTROL MESSAGES                                   PAGE 5-74

ERROR LOG MESSAGES

5.9.5  DISK TRANSFER ERRORS Error Log Format

The  following  error log message format is used to report errors

that occur during a disk transfer.   Note  that  this  format  is

generally used to report the results of a sequence of retries.

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number| unit number |

+---------------+-------+--------+

| event code | flags | format |

+---------------+-------+--------+

| |

+--- controller identifier ---+

| |

+---------------+-------+--------+

|multiunit code |chvrsn | csvrsn |

+---------------+-------+--------+

| |

+--- unit identifier ---+

| |

+-------+-------+-------+--------+

| retry | level |uhvrsn | usvrsn |

+-------+-------+-------+--------+

| volume serial number |

+--------------------------------+

| header code |

+--------------------------------+

| |

/ controller or disk /

/ dependent information /

| |

+--------------------------------+

The  fields  in the DISK TRANSFER ERRORS error log message are as

follows:

command reference number

unit number

sequence number

format

flags

event code

controller identifier

csvrsn

chvrsn

multiunit code

MSCP CONTROL MESSAGES                                   PAGE 5-75

ERROR LOG MESSAGES

unit identifier

usvrsn

uhvrsn

volume serial number

See  Section  section(generrlogfmt),  "Generic  Error Log

Message Format."

level

The error recovery level used for the most recent attempt

at the transfer.  The error recovery level  is  a  device

dependent  encoding  of  the  special error recovery pro-

cedures, such as offset head positioning,  used  for  the

most  recent  transfer  attempt.  The values zero and 255

(all ones) are reserved to indicate that no special error

recovery procedures were used.

retry

The retry count, within the current error recovery level,

of the most recent attempt at the transfer.   This  value

starts  at  one  for the first attempt using a particular

error recovery level and increments for  each  subsequent

attempt  at  the  same  level.  This continues up to some

drive dependent maximum, at which time the retry count is

reset  to  one and the next error recovery level (if any)

is tried.

header code

Identifies  the physical disk location at which the error

occurred.  If the high four bits are 0000 (binary),  then

the low 28 bits are the logical block number at which the

error occurred.  If the high four bits are 0110 (binary),

then  the low 28 bits are the replacement block number at

which the error occurred.  All other patterns of the high

four  bits  of "header code" are reserved, and may not be

returned without an ECO to this specification  to  define

their  interpretation.  The 28 bit logical or replacement

block number may be decomposed, using the  disk  geometry

parameters  returned  by  the GET UNIT STATUS command, to

obtain the cylinder, group, track, and sector position at

which the error occurred.

MSCP CONTROL MESSAGES                                   PAGE 5-76

ERROR LOG MESSAGES

controller or disk dependent information

A  variable  (controller or disk dependent) amount of in-

formation.  Often no controller or disk dependent  infor-

mation  is  provided.   The length of this information is

implied by the total length of  the  error  log  message,

passed  to the class driver by the port driver.  This in-

formation will typically not be interpreted by error  log

formatting programs, instead being printed as a series of

values.

MSCP CONTROL MESSAGES                                   PAGE 5-77

ERROR LOG MESSAGES

5.9.6  SDI ERRORS Error Log Format

The  following  error log message format is used by Standard Disk

Interconnect (SDI) disk controllers to report drive detected  er-

rors  and SDI communication errors.  Note that the controller re-

tries these errors only once or twice, so a  separate  error  log

message will be generated for each attempt that fails.

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number| unit number |

+---------------+-------+--------+

| event code | flags | format |

+---------------+-------+--------+

| |

+--- controller identifier ---+

| |

+---------------+-------+--------+

|multiunit code |chvrsn | csvrsn |

+---------------+-------+--------+

| |

+--- unit identifier ---+

| |

+---------------+-------+--------+

| reserved |uhvrsn | usvrsn |

+---------------+-------+--------+

| volume serial number |

+--------------------------------+

| header code |

+--------------------------------+

| |

+--- SDI status ---+

| information |

+--- (12 bytes) ---+

| |

+--------------------------------+

The fields in the SDI ERRORS error log message are as follows:

command reference number

unit number

sequence number

format

flags

event code

controller identifier

csvrsn

MSCP CONTROL MESSAGES                                   PAGE 5-78

ERROR LOG MESSAGES

chvrsn

multiunit code

unit identifier

usvrsn

uhvrsn

volume serial number

See  Section  section(generrlogfmt),  "Generic  Error Log

Message Format."

header code

Identifies  the physical disk location at which the error

occurred.  If the high four bits are 0000 (binary),  then

the low 28 bits are the logical block number at which the

error occurred.  If the high four bits are 0110 (binary),

then  the low 28 bits are the replacement block number at

which the error occurred.  All other patterns of the high

four  bits  of "header code" are reserved, and may not be

returned without an ECO to this specification  to  define

their  interpretation.  The 28 bit logical or replacement

block number may be decomposed, using the  disk  geometry

parameters  returned  by  the GET UNIT STATUS command, to

obtain the cylinder, group, track, and sector position at

which the error occurred.

SDI status information

Twelve  bytes  of  status information returned by the SDI

GET STATUS command or by the SDI  UNSUCCESSFUL  response.

The  unit  number information returned by the SDI command

or response is  not  included,  as  that  information  is

provided  elsewhere in the error log message.  Otherwise,

all of the SDI status information is included.   See  the

SDI specification for the format of this information.

MSCP CONTROL MESSAGES                                   PAGE 5-79

ERROR LOG MESSAGES

5.9.7  SMALL DISK ERRORS Error Log Format

The  following  error  log message format is used by low-end disk

controllers to report drive detected errors and other errors  not

amenable  to  the  Disk  Transfer Error error log message format.

Note that this format can only be used with disks  that  have  no

more than 65535 cylinders.

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number| unit number |

+---------------+-------+--------+

| event code | flags | format |

+---------------+-------+------- +

| |

+--- controller identifier ---+

| |

+---------------+-------+--------+

|multiunit code |chvrsn | csvrsn |

+---------------+-------+--------+

| |

+--- unit identifier ---+

| |

+---------------+-------+--------+

| cylinder |uhvrsn | usvrsn |

+---------------+-------+--------+

| volume serial number |

+--------------------------------+

| controller or |

/ drive dependent /

/ information /

| |

+--------------------------------+

The  fields  in  the  SMALL  DISK ERRORS error log message are as

follows:

command reference number

unit number

sequence number

format

flags

event code

controller identifier

csvrsn

chvrsn

multiunit code

MSCP CONTROL MESSAGES                                   PAGE 5-80

ERROR LOG MESSAGES

unit identifier

usvrsn

uhvrsn

volume serial number

See  Section  section(generrlogfmt),  "Generic  Error Log

Message Format."

cylinder

The  cylinder  to which the current transfer is directed.

This field may or may not be meaningful, depending on the

value in "event code."

controller or drive dependent information

A  variable (controller or drive dependent) amount of in-

formation.  The length of this information is implied  by

the  total length of the error log message, passed to the

class driver by the port driver.  This  information  will

typically not be interpreted by error log formatting pro-

grams, instead being printed as a series of values.

MSCP CONTROL MESSAGES                                   PAGE 5-81

ERROR LOG MESSAGES

5.9.8  BAD BLOCK REPLACEMENT ATTEMPT Error Log Format

The following error log message format is used by the (Controller

Initiated) Bad Block Replacement layer to report completion of  a

bad  block  replacement  attempt.   Note  that  a message of this

format is always generated regardless of the success  or  failure

of the replacement attempt.

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number| unit number |

+---------------+-------+--------+

| event code | flags | format |

+---------------+-------+--------+

| |

+--- controller identifier ---+

| |

+---------------+-------+--------+

|multiunit code |chvrsn | csvrsn |

+---------------+-------+--------+

| |

+--- unit identifier ---+

| |

+---------------+-------+--------+

| replace flags |uhvrsn | usvrsn |

+---------------+-------+--------+

| volume serial number |

+--------------------------------+

| Bad LBN |

+--------------------------------+

| Old RBN |

+--------------------------------+

| New RBN |

+---------------+----------------+

| cause |

+----------------+

The fields in the BAD BLOCK REPLACEMENT ATTEMPT error log message

are as follows:

unit number

sequence number

format

flags

controller identifier

csvrsn

chvrsn

MSCP CONTROL MESSAGES                                   PAGE 5-82

ERROR LOG MESSAGES

multiunit code

unit identifier

usvrsn

uhvrsn

volume serial number

See  Section  section(generrlogfmt),  "Generic  Error Log

Message Format."

command reference number

Either  zero  or the command reference number of the com-

mand that  caused  the  bad  block  to  be  detected  and

replaced.  Controllers need not save this value, in which

case this field will always be zero.

event code

Reports  the  completion status of the bad block replace-

ment attempt.   The  major  code  is  always  "Bad  Block

Completion." Subcodes  are defined in Table table(bbrcom-

plerrsc), "'Bad Block Replacement Completion' Event  Only

Subcode Values."

replace flags

Bit  flags  reporting  in  detail  the outcome of the bad

block      replacement      attempt.       See      Table

table(bbratmprpflg),   "Bad   Block  Replacement  Attempt

'Replace Flags'" for the definitions of those flags.

Bad LBN

The LBN that was the target of the replacement attempt.

Old RBN

If  the "Bad RBN" replace flag is set, the RBN with which

the Bad LBN was formerly replaced.  Zero otherwise.

New RBN

If  the  "Replacement Attempted" replace flag is set, the

RBN with which the Bad LBN is now replaced.  Zero  other-

wise.

MSCP CONTROL MESSAGES                                   PAGE 5-83

ERROR LOG MESSAGES

Cause

The  event  code  from the original error that caused the

replacement to be attempted.  Zero if that event code  is

not available.

5.9.9  DISK COPY DATA CORRELATION Error Log Format

The following error log message format is used to report specific

context related to errors detected during the execution of a DISK

COPY  DATA  command.   See Section section(dskcopydatades), "DISK

COPY DATA Command, Description."

This  error  log shall be generated if one or more error logs was

generated by a remote source.  In this case, the event  code  re-

turned  will  be  Informational (subcode "Disk Copy Data Correla-

tion").  The error logs generated by the remote source will  have

the Disk Copy Data Correlate error log flag set and will have the

same "command reference number" as this Informational error  log.

This  error log allows all error logs related to such a DISK COPY

DATA command to be correlated with that command's context.

This  error  log is also sent for certain classes of fatal errors

detected by the server during attempted  execution  of  the  DISK

COPY  DATA  command.  See the descriptions of the status codes in

Section section(dskcopydataemf), "DISK  COPY  DATA  Command,  End

Message  Format"  for  specifics.  In these cases, this error log

shall contain the appropriate event code/subcode.  Note that  for

most  of  these fatal errors, this will be the only error log re-

lated to this DISK COPY DATA command generated by the server.

Note  that  in either case this error log is only sent once for a

particular DISK COPY DATA command.

MSCP CONTROL MESSAGES                                   PAGE 5-84

ERROR LOG MESSAGES

31 0

+--------------------------------+

| command reference number |

+---------------+----------------+

|sequence number|dst unit number |

+---------------+-------+--------+

| event code | flags | format |

+---------------+-------+--------+

| |

+--- controller identifier ---+

| |

+---------------+-------+--------+

| undefined |chvrsn | csvrsn |

+---------------+-------+--------+

| destination |

+--- unit ---+

| identifier |

+---------------+----------------+

| undefined |src unit number |

+---------------+----------------+

| source |

+--- controller ---+

| identifier |

+---------------+-------+--------+

| undefined |schvrsn|scsvrsn |

+---------------+-------+--------+

| source |

+--- unit ---+

| identifier |

+--------------------------------+

|source unit's storage subsystem |

+--- port ---+

| address |

+--------------------------------+

|source unit's storage subsystem |

+--- system ---+

| address |

+--------------------------------+

| |

/ event dependent /

/ information /

| |

+--------------------------------+

The  fields  in  the DISK COPY DATA CORRELATION error log message

are as follows:

command reference number

sequence number

format

MSCP CONTROL MESSAGES                                   PAGE 5-85

ERROR LOG MESSAGES

controller identifier

csvrsn

chvrsn

See  Section  section(generrlogfmt),  "Generic  Error Log

Message Format."

The  contents  of the "command reference number" field in

this message shall be identical to the  contents  of  the

"command  reference  number"  field of the DISK COPY DATA

command for which this message is sent.

dst unit number (destination unit number)

destination unit identifier

src unit number

source unit identifier

source unit's storage subsystem port address

source unit's storage subsystem system address

The  values  of these fields are copied without modifica-

tion from the corresponding fields of the DISK COPY  DATA

command  message  identified  in  the  "command reference

number"  field  of  this  message.   See   Section   sec-

tion(dskcopydatacmf),  "DISK  COPY  DATA Command, Command

Message Format" for descriptions of these fields.

flags

See  Section  section(generrlogfmt),  "Generic  Error Log

Message Format."

The  Informational  error  log  flag shall be set in this

message if the event code is Informational.

event code

For  DISK  COPY  DATA commands that terminate with an end

message major  status  code  of  Controller  Error,  Host

Buffer  Access Error, or Subcommand Error, the event code

shall be set to the corresponding event code/subcode.

Otherwise, the event code is Informational (subcode "Disk

Copy Data Correlation").

source controller identifier

scsvrsn (source csvrsn)

schvrsn (source chvrsn)

The  values  of  these fields are from the end message of

the SET CONTROLLER CHARACTERISTICS  command  issued  when

MSCP CONTROL MESSAGES                                   PAGE 5-86

ERROR LOG MESSAGES

bringing  the  remote server "Controller-Online." Zero if

that information is not available.

event dependent information

A  variable  amount of information dependent on the event

that caused this error log message to be generated.   The

length of this information is implied by the total length

of the error log message, passed to the class  driver  by

the  port driver.  This information will typically not be

interpreted by error log formatting programs, instead be-

ing  printed  as  a  series of values.  See Sections sec-

tion(dskcopydataemf),  "DISK  COPY  DATA   Command,   End

Message  Format," and section(dskcopydatades), "DISK COPY

DATA Command, Description" for details on what,  if  any-

thing, should be placed in this field.

MSCP CONTROL MESSAGES                                   PAGE 5-87

ERROR LOG MESSAGES

5.9.10  MEDIA LOADER ERRORS Error Log Format

The  following format is used to report errors that occur

during media loader operations.

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| event code | flags | format|

+---------------+-------+-------+

| |

+--- controller identifier ---+

| |

+---------------+-------+-------+

|multiunit code |chvrsn | csvrsn|

+---------------+-------+-------+

| |

+--- unit identifier ---+

| |

+-------+-------+-------+-------+

| reserved | uhvrsn| usvrsn|

+-------+-------+-------+-------+

| |

+--- media loader identifier ---+

| |

+-------+-------+-------+-------+

| ml unit number|mlhvrsn|mlsvrsn|

+-------+-------+-------+-------+

| |

/ controller or media loader /

/ dependent information /

| |

+-------------------------------+

The  fields  in  the MEDIA LOADER ERROR error log message

are as follows (unit number, multiunit code, unit identi-

fier,  usvrsn,  and uhvrsn refer to the disk or tape unit

that is the target of the media loader operation):

command reference number

unit number

sequence number

format

flags

event code

csvrsn

MSCP CONTROL MESSAGES                                   PAGE 5-88

ERROR LOG MESSAGES

chvrsn

multiunit code

unit identifier

usvrsn

uhvrsn

See Section section(generrlogfmt), "Generic Error

Log Message Format."

controller identifier

Uniquely  identifies  the  controller  among  all

devices accessible via MSCP.   See  Section  sec-

tion(cntrlrunitids), "Controller and Unit Identi-

fiers".  If zero, the error log was generated  by

host software and not by a controller.

media loader identifier

Uniquely  identifies  the  media loader among all

devices accessible via MSCP.   See  Section  sec-

tion(cntrlrunitids), "Controller and Unit Identi-

fiers." This field is  only  present  for  errors

that relate to a specific media loader.

mlsvrsn

The   media   loader's   software,  firmware,  or

microcode revision number.  Always  zero  if  the

product's  development and maintainability groups

determine that there is no benefit from  support-

ing machine readable revision numbers.

mlhvrsn

The  media  loader's  hardware  revision  number.

Always zero  if  the  product's  development  and

maintainability groups determine that there is no

benefit from supporting machine readable revision

numbers.

ml unit number

The  unit number of the media loader to which the

error log message relates, or zero if either  the

message  does not relate to a specific unit or if

the media loader does not support unit numbers.

controller or media loader dependent information

MSCP CONTROL MESSAGES                                   PAGE 5-89

ERROR LOG MESSAGES

A variable (controller or media loader dependent)

amount of information.  The length of this infor-

mation  is implied by the total length of the er-

ror log message, passed to the  class  driver  by

the port driver.  This information will typically

not be interpreted by error log  formatting  pro-

grams,  instead  being  printed  as  a  series of

values.