CHAPTER 6

MINIMAL MSCP COMMAND SET

6.1  OVERVIEW

This  chapter  describes the minimal MSCP command set implemented

by an MSCP disk controller.  Refer to the Tape Mass Storage  Con-

trol  Protocol  Specification  (TMSCP) for details of the minimal

command set implemented by MSCP tape controllers.   A  number  of

commands operate identically on both disk and tape class devices.

TMSCP relies on the descriptions contained in  this  chapter  for

those commands.

Controllers that support only the minimal MSCP command set impli-

citly provide single host operation and rely on the host for  the

replacement of bad blocks.

General  purpose disk class drivers shall at the very minimum im-

plement both the minimal MSCP  command  set,  described  in  this

chapter,   and   Controller   Initiated  Bad  Block  Replacement,

described in Chapter chapter(cntrlrinitbbr).

Note  that  with  the  exception of the commands noted in Section

section(nooperationcmd), "No-Operation Commands" controllers that

support  only the minimal MSCP command set reject any command not

described in this chapter as an "Invalid Command"  ("invalid  op-

code"  protocol  error.   See "Invalid Command" status in Section

section(status__codes), "Status Codes").

Within this chapter a separate major section is dedicated to each

minimal command set command.  The major  section  identifies  the

command  by  name  as well as its permissible command category(s)

MINIMAL MSCP COMMAND SET                                 PAGE 6-2

OVERVIEW

(see Section section(cmdcatsexord), "Command Categories and  Exe-

cution  Order").   Separate  subordinate  sections  describe  the

command's command message format, end message format, and (opera-

tion) description.

The  command  message  format  subsection shows the layout of the

command message fields and describes their  use.   (Appendix  ap-

pendix(opcdsflgsoffs),  "Opcode,  Flag,  and Offset Definitions,"

Table table(commandmesoff), "Command Message Offsets" defines the

size  and  the preferred mnemonic and numeric value of the offset

of the command message fields.)  Note  that  the  omission  of  a

parameter   field   description  implies  that  it  functions  as

described elsewhere in this specification.   In  particular,  the

command  message  header  fields  (i.e.,  the  "command reference

number," "unit number," "opcode," and "modifiers") are  described

in  Section  section(command__mess),  "Command  Messages" and the

fields common to data  transfer  commands  (i.e.,  "byte  count,"

"buffer descriptor," and "logical block number") are described in

Section   section(xfrcmdmesfmt),   "Transfer   Command    Message

Format."

The  command  message  format subsection also lists the modifiers

allowed for the command (modifier use is  opcode  dependent)  and

describes  their  use.   Note  that  the  omission  of a modifier

description implies that it functions  as  described  in  Section

section(cmd_modifiers),  "Command  Modifiers." Note also that any

command modifiers other than the ones listed  for  an  individual

command  are reserved and treated in accordance with the require-

ments for reserved fields described  in  Section  section(rsvdun-

defflds), "Reserved and Undefined Fields."

The  end  message  format  subsection shows the layout of the end

message  fields  and  describes   their   use.    (Appendix   ap-

pendix(opcdsflgsoffs),  "Opcode,  Flag,  and Offset Definitions,"

Table table(endattmesoff), "End and  Attention  Message  Offsets"

defines  the size and the preferred mnemonic and numeric value of

the offset of the end message fields.) Note that the omission  of

a  parameter  field  description  implies  that  it  functions as

described elsewhere in this specification.   In  particular,  the

end  message header fields (i.e., the "command reference number,"

"unit  number,"  "sequence  number,"  "endcode,"   "flags,"   and

"status")  are  described  in  Section section(end____mess), "End

Messages" and the fields common to data transfer commands  (i.e.,

"status,"  "byte  count," and "first bad block") are described in

Section section(xferendmessfmt), "Transfer  Command  End  Message

Format."

The end message format subsection also lists all the status codes

(and subcodes) possible for the command and describes their mean-

ing.  Note that the omission of a status description implies that

MINIMAL MSCP COMMAND SET                                 PAGE 6-3

OVERVIEW

its meaning is defined in Section section(status__codes), "Status

Codes." Note  also that the MSCP protocol violation "Invalid Com-

mand" statuses are not listed for any of the commands but can oc-

cur   for   each.    See  "Invalid  Command"  status  in  Section

section(status__codes), "Status Codes" for more detail.

The description subsection defines the purpose of the command and

provides details about the command's operation.   In  some  cases

the  description  subsection  relies heavily on the other subsec-

tions associated with the command for  specific  operational  de-

tails.

6.1.1  No-Operation Commands

The  opcodes listed in the "Standard Command Messages" portion of

Appendix appendix(opcdsflgsoffs), "Opcode, Flag, and  Offset  De-

finitions,"  Table  table(cntrlmessopcd) as "allocated for future

use" act as place holders in anticipation of their use in  Volume

Shadowing and/or Data Caching.  Volume Shadowing and Data Caching

have not yet been formalized for disk class devices.  Data  Cach-

ing has, however, been formalized for tape class devices (see the

Tape Mass Storage Control Protocol Specification (TMSCP) for  de-

tails).   In  order to ensure compatibility with controllers that

support Volume Shadowing and/or Data Caching,  controllers  which

do  not  shall  treat  those  opcodes as no-operations.  In other

words, controllers that do not support  Volume  Shadowing  and/or

Data Caching shall recognize those opcodes and not reject them as

"Invalid Commands" and furthermore shall not perform  any  action

that alters the state of the unit, its media, or the controller.

The  particular actions such a controller may perform in treating

those opcodes as no-operations are as follows:

1.  Treat  those  opcodes as either Immediate commands or as

Non-Sequential commands at its  option.   (Host  command

timeout  algorithms  shall  treat  them as non-Immediate

commands.)

2.  Construct  the  end  message  by  making  the  following

changes to the command message:

a.  Substitute the proper "endcode" for the "opcode."

b.  Clear  the "flags" (end flags) byte, unless the con-

troller has previously verified that the correspond-

ing  "rsvd"  byte  in  the command message was zero.

MINIMAL MSCP COMMAND SET                                 PAGE 6-4

OVERVIEW

(Most controllers will verify that the corresponding

"rsvd"  byte is zero as part of checking for invalid

opcodes.)

c.  Substitute  the  "Success" status code combined with

the "Normal" status subcode for the "modifiers." The

controller  may optionally validate the "byte count"

and "logical block number" fields to see if they are

within  proper  limits  and return the "Invalid Com-

mand" status with appropriate subcode  if  they  are

not  (i.e.,  validate the command as a transfer com-

mand similar to the ACCESS and ERASE commands).

MINIMAL MSCP COMMAND SET                                 PAGE 6-5

OVERVIEW

6.2  ABORT COMMAND

Command Category:

Immediate

6.2.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| outstanding reference number |

+-------------------------------+

unit number

Shall  be the same as the "unit number" field in the out-

standing command to be aborted.  This allows  controllers

to optimize their search for the outstanding command.  If

the incorrect unit number is supplied,  some  controllers

may  erroneously  conclude  that the command is no longer

outstanding and therefore not abort the command.

outstanding reference number

Command reference number of the command to be aborted.

Allowable modifiers:

none

MINIMAL MSCP COMMAND SET                                 PAGE 6-6

ABORT COMMAND

MINIMAL MSCP COMMAND SET                                 PAGE 6-7

ABORT COMMAND

6.2.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| outstanding reference number |

+-------------------------------+

outstanding reference number

The  command  reference  number  of  the command that was

aborted.  Identical to the value supplied  in  the  ABORT

command message.

Status Codes:

Success (subcode "Normal")

MINIMAL MSCP COMMAND SET                                 PAGE 6-8

ABORT COMMAND

6.2.3  Description

The ABORT command causes a specified command to be aborted at the

earliest time convenient for the controller.  The specified  com-

mand shall, however, either be aborted or be completed within the

controller timeout interval.  See Section section(command_timos),

"Command  Timeouts,"  for a discussion of the interaction between

ABORT and command timeouts.

The  ABORT command always succeeds.  If the command to be aborted

is not known to the controller, this implies that it has  already

completed  and the ABORT command will be ignored.  The controller

always returns the Success (subcode "Normal") status code/subcode

in the ABORT command's end message.

The  controller may ignore the ABORT command if the command being

aborted will always complete within the controller timeout inter-

val.

The  class  driver  shall wait for the aborted command's end mes-

sage, or else resynchronize with the MSCP server, before  reusing

the  aborted  command's command reference number or releasing the

aborted command's context.  If the command was aborted,  its  end

message  will  contain the "Command Aborted" status code.  Other-

wise, the command was completed.  The class driver may ignore the

ABORT  command's end message.  Note that the class driver may re-

ceive the ABORT command's end message either before or after  the

aborted command's end message.

MINIMAL MSCP COMMAND SET                                 PAGE 6-9

ABORT COMMAND

6.3  ACCESS COMMAND

Command Category:

Non-Sequential

6.3.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | caa | opcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- ---+

| reserved |

+--- ---+

| |

+-------------------------------+

| logical block number |

+-------------------------------+

Allowable modifiers:

Express Request

Clear Serious Exception (ignored for disk class devices)

Suppress Error Correction

Suppress Error Recovery

MINIMAL MSCP COMMAND SET                                PAGE 6-10

ACCESS COMMAND

6.3.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- ---+

| undefined |

+--- ---+

| |

+-------------------------------+

| first bad block |

+-------------------------------+

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Invalid Command (subcode "Invalid Byte Count")

Invalid Command (subcode "Invalid Logical Block Number")

Command Aborted

Unit-Offline

Unit-Available

Data Error

Host Buffer Access Error (subcode "Odd Byte Count")

If  the  communications mechanism does not allow odd byte

transfers,  the  controller  may   return   this   status

code/subcode  despite  there  being no host buffer.  This

allows the use of common code for command  validation  of

all transfer commands.  Alternatively, the controller may

round the byte count up to the nearest whole sector  size

and perform the operation.

Controller Error

Drive Error

MINIMAL MSCP COMMAND SET                                PAGE 6-11

ACCESS COMMAND

MINIMAL MSCP COMMAND SET                                PAGE 6-12

ACCESS COMMAND

6.3.3  Description

Data  is  read  from the unit, checked for errors, and discarded.

The purpose of this command is to verify that the designated data

can be accessed (read) without error.

This  command  is exactly equivalent in function, although not in

performance, to a READ whose data is ignored by the host.

The caa field will be ignored and the backing store will be read.

MINIMAL MSCP COMMAND SET                                PAGE 6-13

ACCESS COMMAND

6.4  AVAILABLE COMMAND

Command Category:

Sequential

6.4.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

Allowable modifiers:

All Class Drivers       (ignored in single host case)

Clear Serious Exception (ignored for disk class devices)

Spin-down

Requests  that  the disk stop spinning and that the heads

be unloaded.  Note that the command completes as soon  as

the spin-down has been initiated, rather than waiting for

the disk to stop spinning.  The spin-down will not be in-

itiated  if  this  unit  belongs to a multiunit drive and

this unit shares a spindle with some other unit  that  is

"Unit-Online." See  Section section(multidrvsfmts), "Mul-

tiunit Drives and Formatters." Regardless of  whether  or

not the spin-down is actually initiated, AVAILABLE atten-

tion messages will be suppressed for this unit, both  for

this  controller  and  for any other controllers to which

the    unit    may    be    connected.     See    Section

section(unit__states), "Unit States."

MINIMAL MSCP COMMAND SET                                PAGE 6-14

AVAILABLE COMMAND

6.4.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Success (subcode "Spin-down Ignored")

The  "Spin-down  Ignored"  subcode bit flag is set if and

only if the "Spin-down" modifier was specified and one or

more other units with which this unit shares a spindle is

still "Unit-Online," preventing this unit  from  spinning

down.   See  Section  section(multidrvsfmts),  "Multiunit

Drives and Formatters,"  for  an  explanation  of  shared

spindles.

Success (subcode "Still Connected")

The "Still Connected" subcode bit flag is set if and only

if this unit may potentially be accessed via another con-

troller  and one or more other units with which this unit

shares an access path is still "Unit-Online,"  preventing

this  unit  from  being accessed via the other controller

(if any).  The "Still Connected" subcode  bit  flag  will

always be set if the "Spin-down Ignored" bit flag is set.

See   Section    section(multiaccesdrvs),    "Multiaccess

Drives," for a discussion of access paths.

Command Aborted

The  command  has  been rejected and the unit's state has

MINIMAL MSCP COMMAND SET                                PAGE 6-15

AVAILABLE COMMAND

not changed.  If the unit was "Unit-Online" prior to  is-

suing  this command, then the unit is still "Unit-Online"

and it is still spinning.

Unit-Offline

Controller Error

Drive Error

MINIMAL MSCP COMMAND SET                                PAGE 6-16

AVAILABLE COMMAND

6.4.3  Description

All  outstanding  commands  for the specified unit are completed,

then the unit becomes "Unit-Available." If the "Spin-down"  modi-

fier was not specified, the unit is not already "Unit-Available,"

and no other  units  that  share  this  unit's  access  path  are

"Unit-Online"  (i.e.,  the  "Still  Connected" status subcode bit

flag is clear), then an AVAILABLE attention message  is  sent  by

any  other  controller  to which the unit is connected.  The con-

troller to which this command was sent need not  itself  send  an

AVAILABLE attention message.

If the "Spin-down" modifier is specified, the disk spins down and

its heads are unloaded, unless some other unit  with  which  this

unit  shares  a spindle is "Unit-Online." The disk may be spun up

with  an  ONLINE  command  or  by  operator  intervention.    The

"Spin-down" modifier also suppresses AVAILABLE attention messages

for this unit, both for this controller and any other controllers

to    which   the   unit   may   be   connected.    See   Section

section(unit__states),  "Unit  States,"  for  a   discussion   of

suppressing AVAILABLE attention messages.

This  command  will  be  accepted if the unit is "Unit-Online" or

"Unit-Available." It is nugatory to issue this command to a  unit

that  is  "Unit-Available"  unless  the  "Spin-down"  modifier is

specified.  Assuming no other errors occur, the "Success"  status

code  will  be  returned  regardless of whether the unit was pre-

viously "Unit-Online" or "Unit-Available."

If  the  unit  was  "Unit-Online" but had a duplicate unit number

prior to the AVAILABLE command being issued, the  AVAILABLE  com-

mand  may  complete,  at  the  controller's option, with either a

"Success" or a "Unit-Offline" status  code.   The  "Unit-Offline"

status  code  shall have the "Duplicate Unit Number" subcode flag

set.   The  "Success"  status  code  may  or  may  not,  at   the

controller's  option,  have  the  "Duplicate Unit Number" subcode

flag set.  Subsequent attempts to access  the  unit  will  return

"Unit-Offline"  status  with  the "Duplicate Unit Number" subcode

flag set unless the duplicate unit number has been eliminated.

When  a  unit with controller-based write-back cache ceases to be

Online, it should write to the media all data  contained  in  the

cache but not yet written to the backing store.

MINIMAL MSCP COMMAND SET                                PAGE 6-17

AVAILABLE COMMAND

6.5  COMPARE HOST DATA COMMAND

Command Category:

Non-Sequential

6.5.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | caa | opcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- buffer ---+

| |

+--- descriptor ---+

| |

+-------------------------------+

| logical block number |

+-------------------------------+

Allowable modifiers:

Clear Serious Exception (ignored for disk class devices)

Express Request

Suppress Error Correction

Suppress Error Recovery

MINIMAL MSCP COMMAND SET                                PAGE 6-18

COMPARE HOST DATA COMMAND

6.5.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- ---+

| undefined |

+--- ---+

| |

+-------------------------------+

| first bad block |

+-------------------------------+

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Invalid Command (subcode "Invalid Byte Count")

Invalid Command (subcode "Invalid Logical Block Number")

Command Aborted

Unit-Offline

Unit-Available

Compare Error

Data Error

Host Buffer Access Error

Controller Error

Drive Error

MINIMAL MSCP COMMAND SET                                PAGE 6-19

COMPARE HOST DATA COMMAND

6.5.3  Description

Data is read from the unit and compared with the data in the host

buffer.  A "Compare Error" is reported unless the data is identi-

cal.   Note  that  the  occurrence  of  any other error, except a

"Forced Error," at the same point as  the  "Compare  Error"  will

override the "Compare Error." Note also that any "Compare Errors"

detected by this command shall NOT be reported to the error  log.

Controllers  may  retry  compare errors detected by this command.

Provided the host does not modify the buffer while  the  transfer

is  in  progress,  such retries will have a deleterious effect on

performance but cannot alter the final outcome of the command.

If a controller-based cache exists, the cache manager may use the

Cache Access Attribute advice in managing the caching and compar-

ing of the requested data area.

MINIMAL MSCP COMMAND SET                                PAGE 6-20

COMPARE HOST DATA COMMAND

6.6  DETERMINE ACCESS PATHS COMMAND

Command Category:

Controller   implementation   dependent.   See  Section  sec-

tion(determineades).

6.6.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

Allowable modifiers:

none

MINIMAL MSCP COMMAND SET                                PAGE 6-21

DETERMINE ACCESS PATHS COMMAND

6.6.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

Status codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Command Aborted

Unit-Offline

Unit-Available

Controller Error

Drive Error

MINIMAL MSCP COMMAND SET                                PAGE 6-22

DETERMINE ACCESS PATHS COMMAND

6.6.3  Description

Class  drivers use this command to determine the topology of mul-

tiaccess drive configurations (described in Section  section(mul-

tiaccesdrvs), "Multiaccess Drives").  When sent to a unit that is

"Unit-Online," it causes that unit and any other units that share

its access path to identify themselves to any other controller to

which they are connected.  The MSCP servers  in  the  other  con-

trollers  will, as a result, become aware that the unit is online

via the controller receiving this command.  They will  then  send

an  ACCESS  PATH  attention  message to their "Controller-Online"

class drivers, informing the class drivers  of  alternate  access

paths to the unit.  This command shall be treated as a no-op that

always succeeds if the unit is incapable of  being  connected  to

more than one controller.

The actual notification to another controller, and thus the send-

ing of an ACCESS PATH attention  message,  is  dependent  on  the

proper  functioning  of  the unit and both controllers.  Further-

more, it need not be 100% reliable.  That is, assuming  the  unit

and both controllers are functioning properly, there need only be

a high probability (better than 50%) that  the  other  controller

will  in  fact be notified and send an ACCESS PATH attention mes-

sage.  For this reason, plus  the  fact  that  the  topology  may

change  while  the unit remains "Unit-Online," hosts that need to

know multiaccess drive topology must issue DETERMINE ACCESS PATHS

commands  to all "Unit-Online" units on a periodic basis, such as

once every 5 to 15 minutes.

This  command  in  no  way  affects  the unit's actual state with

respect to any controller.  The unit  remains  "Unit-Online"  via

the  receiving  controller  and  remains "Unit-Offline" via other

controllers.  Note, however, that this  command  may  affect  the

unit's  "Unit-Offline"  substate  that is perceived by other con-

trollers.

The processing of this command may, and often will, cause a tran-

sient performance degradation for the specified unit.   Controll-

ers  shall  consider this performance degradation when specifying

their controller timeout interval.

A  controller  may,  at  its  option, treat this as either an Im-

mediate, Sequential, or  Non-Sequential  command.   Host  command

timeout algorithms shall treat this as a non-Immediate command.

MINIMAL MSCP COMMAND SET                                PAGE 6-23

DETERMINE ACCESS PATHS COMMAND

6.7  DISK COPY DATA COMMAND

Command Category:

Non-Sequential   (except   as   described   in  Section  sec-

tion(dskcopydatades))

6.7.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| logical block count |

+---------------+---------------+

| reserved |src unit number|

+---------------+---------------+

| source |

+--- unit ---+

| identifier |

+-------------------------------+

| destination lbn |

+---------------+---------------+

| entry id | hrn or entloc |

+---------------+---------------+

| reserved |

+-------------------------------+

| source lbn |

+-------------------------------+

|source unit's storage subsystem|

+--- port ---+

| address |

+-------------------------------+

|source unit's storage subsystem|

+--- system ---+

| address |

+-------------------------------+

unit number

MINIMAL MSCP COMMAND SET                                PAGE 6-24

DISK COPY DATA COMMAND

Identifies  the  destination  unit -- i.e., the device to

which data is to be copied.

Allowable modifiers:

Clear Serious Exception (ignored)

Establish Communications Paths

To perform a copy operation the server establishes a com-

munications path to the source unit in the same manner as

a    host   class   driver   --   i.e.,   establishes   a

"Controller-Online" connection  with  the  source  unit's

server  (possibly itself) and brings the source unit into

the "Unit-Online" state.  The server already has  a  com-

munications path established to the destination;  if not,

it  would  reject  the   command   with   a   status   of

"Unit-Available"  or  "Unit-Offline."  The communications

path is established on behalf of the issuing  host  class

driver.   See Section section(dskcopydatades), "DISK COPY

DATA Command, Description" for details of  the  establish

communications paths operation.

The  server  rejects  the command as an "Invalid Command"

("invalid modifier" protocol error.   See  "Invalid  Com-

mand"  status  in Section section(status__codes), "Status

Codes") if either of the following conditions exist:

1.  This  modifier is clear and communications

paths have not been established on  behalf

of the issuing host class driver.

2.  This  modifier  is  set and communications

paths previously established on behalf  of

the  issuing  host  class driver are still

intact.

History Log

Reuse Entry

The History Log and Reuse Entry modifiers are interdepen-

dent and described together here to avoid confusion.

Note that the History Log modifier, the Reuse Entry modi-

fier, the "hrn or entloc" field, and the "entry id" field

are  valid  only  for servers that provide "write history

logging" support  (see  Section  section(controlerflags),

"Controller Flags" for related information).

MINIMAL MSCP COMMAND SET                                PAGE 6-25

DISK COPY DATA COMMAND

The  setting  of  the  History  Log  modifier  determines

whether information about this command is to be stored in

a "write history entry."

The  setting  of  the  Reuse  Entry  modifier  determines

whether the server is to allocate a  new  "write  history

entry" or reuse one that was previously allocated.

If  "write history logging" is supported, the History Log

modifier is set, and the Reuse Entry modifier  is  clear,

the server allocates a new "write history entry" and then

stores the value contained in the  "hrn  (host  reference

number)"  field,  "entry id" field, and other information

related to this command into the  "write  history  entry"

just  allocated.   If  the  server  has no "write history

entry" to allocate, it shall  remember  this  fact.   See

Section  section(wrihismdes),  "WRITE  HISTORY MANAGEMENT

Command, Description" for more details.  To  report  this

allocation  failure  to  the  host,  the  server  returns

FFFF(hex) in the "entry locator" field of the end message

and  sets  the  Allocation Failure end flag.  See Section

section(end____mess), "End Messages," for  a  description

of this flag.

If  "write history logging" is supported, the History Log

modifier is set, and the Reuse Entry modifier is set, the

server uses the value contained in the "entloc (entry lo-

cator)" field to locate  the  associated  "write  history

entry"  and  then stores the other information related to

this command into that "write history entry."

See  Section  section(wrihismdes), "WRITE HISTORY MANAGE-

MENT Command,  Description"  for  details  regarding  the

maintenance and format of "write history entries."

If  "write  history logging" is supported and the History

Log modifier is clear, the server ignores the setting  of

the  Reuse Entry modifier and the contents of the "hrn or

entloc" and "entry id" fields.

If  "write history logging" is not supported and the His-

tory Log modifier or the Reuse Entry modifier is set, the

server  rejects the command as an "Invalid Command" ("in-

valid modifier" protocol error.   See  "Invalid  Command"

status   in   Section   section(status__codes),   "Status

Codes").

If  "write history logging" is supported, the History Log

modifier is set, and the "logical block count"  field  is

zero, the server reject the command as an Invalid Command

MINIMAL MSCP COMMAND SET                                PAGE 6-26

DISK COPY DATA COMMAND

("invalid logical block count" parameter error.  See "In-

valid  Command" status in Section section(status__codes),

"Status Codes").

The  server shall set the History Logged end flag in this

command's end message if it has caused the write  history

log  to  contain  a  valid "write history entry" for this

command.  See  Section  section(end____mess),  "End  Mes-

sages," for descriptions of all end message flags.

Local Source Unit

Host class drivers set this modifier to indicate that the

source unit is served by the server  that  received  this

command.  If this modifier is set, the server ignores the

contents of the "source unit's  subsystem  port  address"

and the "source unit's subsystem system address" fields.

Host  class  drivers clear this modifier to indicate that

the source unit is served by a server located in  another

storage subsystem.

This modifier is ignored by servers that only support Lo-

cal Disk Copy Data.

Retain Communications Paths

See the description of the Establish Communications Paths

modifier above for information  about  establishing  com-

munications paths.

The  setting  of  this  modifier  determines  whether the

server should retain the communications paths established

on behalf of the issuing host class driver after the copy

operation has been aborted, completed, or terminated.

If  this modifier is set, the server retains the communi-

cations paths established on behalf of the  issuing  host

class  driver.   A  host class driver may therefore issue

subsequent DISK COPY DATA commands without incurring  the

overhead  associated  with the server re-establishing the

communications paths.

If this modifier is clear, the server performs the opera-

tions necessary to disconnect  the  communications  paths

established  on  behalf of the issuing host class driver,

upon  completion  of  the  request.   See  Section   sec-

tion(dskcopydatades),  "DISK  COPY DATA Command, Descrip-

tion" for details of the disconnect operation.

MINIMAL MSCP COMMAND SET                                PAGE 6-27

DISK COPY DATA COMMAND

Note  that  under  certain  circumstances the server will

disconnect the communications paths established on behalf

of  the  issuing host class driver regardless of the set-

ting of this modifier.  The particular circumstances  are

described  in Section section(dskcopydatades), "DISK COPY

DATA Command, Description."

Note also that the Communications Paths Retained end flag

reflects the state of the communications  paths  relative

to  the  issuing  host  as described in the "flags" field

description contained in Section section(dskcopydataemf),

"DISK COPY DATA Command, End Message Format."

logical block count

Zero  or  the total number of logical blocks to be copied

from the source unit to the destination unit.

If  this  value  is  nonzero,  certain architectural con-

straints imposed by the destination unit and  the  source

unit  shall  be  met.  The constraints depend on the con-

tents of the "destination lbn" or  "source  lbn"  fields.

Those  fields  shall identify a logical block in the host

area of the associated disk volume (i.e.,  the  "lbn"  is

less than the "unit size" returned in the ONLINE end mes-

sage) and the "logical block count" shall be less than or

equal to the following maximum:

unit size - logical block number

where  "unit size" is the unit's host area size (returned

in the ONLINE end message) and "logical block number"  is

the contents of the associated "lbn" field in the command

message.  If the "logical block count"  is  greater  than

the  above maximum for either the destination unit or the

source unit, the server rejects the command  as  an  "In-

valid  Command"  ("invalid logical block count" parameter

error.   See  "Invalid   Command"   status   in   Section

section(status__codes), "Status Codes").

A  host  class driver should specify a zero value in this

field when it is necessary to manipulate  the  communica-

tions  paths established on its behalf without performing

a copy operation.  See the descriptions of the  Establish

Communications Paths and Retain Communications Paths mod-

ifiers later in this  section  for  related  information.

The  server  ignores the contents of the "source lbn" and

"destination lbn" fields when this  field  is  zero.   In

MINIMAL MSCP COMMAND SET                                PAGE 6-28

DISK COPY DATA COMMAND

addition, if this field is zero and the Local Source Unit

modifier is set or the server only  supports  Local  Disk

Copy Data, the server ignores the contents of the "source

unit's storage subsystem port address" and "source unit's

storage  subsystem  system address" fields as well as the

other fields listed above.

src unit number

Identifies the source unit -- i.e., the device from which

data is to be copied.

If the server does not support local copies between units

with unlike geometries and copies where  the  source  and

destination are the same unit, has indicated this lack of

support by setting the Restricted DISK COPY  DATA  Opera-

tions  controller  flag  (see  Section section(controler-

flags),  "Controller  Flags")  in  the   SET   CONTROLLER

CHARACTERISTICS  end  message,  and receives a local DISK

COPY DATA command in which either of these conditions ex-

ists,  it  rejects  the  command  as an "Invalid Command"

("invalid src unit number" parameter error.  See "Invalid

Command"   status   in   Section  section(status__codes),

"Status Codes").

If  the  server supports Remote Disk Copy Data the source

unit may be the same as the destination unit, one of  the

other  units served by the server that received this com-

mand, or a unit served by a (disk) MSCP server located in

another  storage  subsystem, depending on the the setting

of the Local Source Unit modifier and the  values  speci-

fied  in this field, the "source unit's storage subsystem

port address" field, and the "source unit's storage  sub-

system  system  address"  field.   Note that servers that

provide Remote Disk Copy Data support implicitly  provide

Local Disk Copy Data support.

source unit identifier

The  unique identifier assigned to the source unit as re-

turned in the "unit identifier" end message  field  of  a

GET UNIT STATUS, ONLINE, or SET UNIT CHARACTERISTICS com-

mand addressed to the source unit.

Host   class   drivers  should  obtain  this  information

directly from the source unit, via one  of  the  commands

listed above, prior to issuing this command.

MINIMAL MSCP COMMAND SET                                PAGE 6-29

DISK COPY DATA COMMAND

The  server  compares the value of this field against the

"unit identifier" it obtains  directly  from  the  source

unit.  If the values do not match exactly, the server re-

jects the  command  as  an  "Invalid  Command"  ("invalid

source  unit  identifier"  parameter error.  See "Invalid

Command"  status   in   Section   section(status__codes),

"Status Codes").

destination lbn

The  logical  block  number (position) on the destination

unit's disk volume at which  the  copy  operation  is  to

start.   Write  access  to  the  destination  unit's disk

volume Replacement Control Table  as  part  of  the  copy

operation  is  prohibited.   Therefore,  this value shall

identify a logical block in the host area of the destina-

tion  unit's  disk volume (i.e., the "destination lbn" is

less than the "unit size" returned in the ONLINE end mes-

sage).  If this value is not in the host area, the server

rejects the command as  an  "Invalid  Command"  ("invalid

destination  lbn" parameter error.  See "Invalid Command"

status   in   Section   section(status__codes),   "Status

Codes").

hrn or entloc

entry id

These fields are used in conjunction with the History Log

and Reuse Entry modifiers.  See the description of  those

modifiers below.

source lbn

The  logical block number (position) on the source unit's

disk volume at which the  copy  operation  is  to  start.

Read  access to the source unit's disk volume Replacement

Control Table as part of the  copy  operation  is  prohi-

bited.   Therefore,  this  value shall identify a logical

block in the host area of the source unit's  disk  volume

(i.e.,  the "source lbn" is less than the "unit size" re-

turned in the ONLINE end message).  If this value is  not

in  the  host  area, the server rejects the command as an

"Invalid Command" ("invalid source lbn" parameter  error.

See     "Invalid     Command"     status    in    Section

section(status__codes), "Status Codes").

If  the  server does not support local DISK COPY DATA for

transfers in which the source and  destination  LBNs  are

different  and  has  indicated  this  lack  of support by

setting  the  Restricted  DISK   COPY   DATA   Operations

MINIMAL MSCP COMMAND SET                                PAGE 6-30

DISK COPY DATA COMMAND

controller  flag  (see  Section  section(controlerflags),

"Controller Flags") in the SET CONTROLLER CHARACTERISTICS

end  message,  the  server rejects the command as an "In-

valid Command" ("invalid source lbn" parameter error).

source unit's storage subsystem port address

The  communications  mechanism  dependent port address of

the storage subsystem to which the source unit is  physi-

cally  connected.  The information encoded in this 8 byte

(64 bit) field is the port address formatted as  required

by the communications mechanism being used.  See Appendix

appendix(pasaaddrfmts), "Port Address And System  Address

Formats"  for  a description of the formats currently de-

fined.

The  server ignores this field if either of the following

conditions exists:

1.  The  server  does  not provide Remote Disk

Copy Data support -- i.e., only Local Disk

Copy Data is supported.

2.  The  server supports Remote Disk Copy Data

but the Local Source Unit modifier is set.

Note that servers that provide Remote Disk

Copy Data support implicitly provide Local

Disk Copy Data support.

If  Remote  Disk  Copy  Data  is  supported and the Local

Source Unit modifier is clear,  the  server  rejects  the

command  as  an "Invalid Command" ("source unit's storage

subsystem port address" parameter  error.   See  "Invalid

Command"   status   in   Section  section(status__codes),

"Status Codes") if the content of this field:

1.  Is  not  formatted  correctly for the com-

munications mechanism being used.

2.  Identifies  the  subsystem  in  which  the

server receiving this command is  located.

(Host  class  drivers  shall set the Local

Source Unit modifier to  request  a  Local

Disk Copy Data operation.)

MINIMAL MSCP COMMAND SET                                PAGE 6-31

DISK COPY DATA COMMAND

source unit's storage subsystem system address

The  communications  protocol dependent system address of

the storage subsystem to which the source unit is  physi-

cally  connected.  The information encoded in this 8 byte

(64 bit) field is the system address value  formatted  as

required  by the communications protocol being used.  See

Appendix appendix(pasaaddrfmts), "Port Address And System

Address  Formats"  for  a description of the formats cur-

rently defined.

The  server ignores this field if either of the following

conditions exists:

1.  The  server  does  not provide Remote Disk

Copy Data support -- i.e., only Local Disk

Copy Data is supported.

2.  The  server supports Remote Disk Copy Data

but the Local Source Unit modifier is set.

Note that servers that provide Remote Disk

Copy Data support implicitly provide Local

Disk Copy Data support.

If  Remote  Disk Copy Data is supported, the Local Source

Unit modifier is clear, and this field is  not  formatted

correctly for the communications protocol being used, the

server  rejects  the  command  as  an  "Invalid  Command"

("source unit's storage subsystem system address" parame-

ter error.   See  "Invalid  Command"  status  in  Section

section(status__codes), "Status Codes").

MINIMAL MSCP COMMAND SET                                PAGE 6-32

DISK COPY DATA COMMAND

6.7.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| logical block count |

+-------------------------------+

| |

+--- ---+

| |

+--- subcommand status ---+

| |

+--- ---+

| |

+---------------+---------------+

| entry id | entry locator |

+---------------+---------------+

flags

Communications Paths Retained

The  setting  of  this end flag indicates whether the

communications paths the server established on behalf

of  the issuing host class driver have been retained.

If this end flag is  set,  the  communications  paths

have  been  retained.  If this end flag is clear, the

communications paths have been disconnected.

Note  that  the Communications Paths Retained end flag is

only valid in this end message.   Furthermore,  only  the

Communications Paths Retained, Error Log Generated, Allo-

cation Failure, and Bad Block Unreported end flags may be

returned  set  in  this end message;  all other end flags

are reserved.

logical block count

The number of logical blocks successfully copied from the

source unit to the destination unit.

MINIMAL MSCP COMMAND SET                                PAGE 6-33

DISK COPY DATA COMMAND

If  an  unrecoverable  error occurs, the server sets this

value equal to the number of  contiguous  logical  blocks

beginning  with  the  "destination  lbn"  that  were suc-

cessfully copied to the destination unit prior to the oc-

currence of the unrecoverable error.

If  no  unrecoverable errors occur during the copy opera-

tion, the server sets this value equal to the exact "log-

ical  block  count"  that  was  specified  in the command

message.

subcommand status

This field contains information related to the failure of

a subcommand issued by the server to the source  unit  or

the destination unit.

The contents of this field are undefined unless this com-

mand was terminated with a Subcommand Error status  code.

If  this command is terminated with that status code, the

"Destination" or "Source" bit flag (contained in the sub-

code portion of the status field) identifies which device

encountered the condition or error.  In cases where  mul-

tiple  conditions  or errors have occurred, either on one

or on both devices, the server only records  and  reports

the  first condition or error detected;  all other condi-

tion or error context is discarded.

The  information supplied when this field is valid (i.e.,

defined) is a portion  of  the  command  or  end  message

(depending on the Subcommand Error subcode reported;  see

the descriptions of those subcodes for  more  detail)  of

the  subcommand  that encountered the condition or error.

The portion of the command or end message supplied begins

with  the message's "unit number" field and ends with the

highest field capable of being stored in the space allot-

ted for this field.

The  purpose  of  this  field  is  to  provide host class

drivers  with  information  to  aid  in  recovering  from

failures  that occur during the copy operation.  However,

when a failure occurs information is  only  provided  for

one  of  the devices involved in the copy operation.  The

state of the other device cannot be deduced from that in-

formation.  Host class drivers should therefore query the

other device for its current state to  determine  whether

copy operation recovery should be attempted.

entry locator

MINIMAL MSCP COMMAND SET                                PAGE 6-34

DISK COPY DATA COMMAND

If  "write  history logging" is supported and the History

Log modifier is set in the command  message,  this  field

contains  the  value assigned by the server that uniquely

identifies the internal location of the  "history  entry"

where   this   command's  information  was  stored.   If,

however, an allocation failure occurred, this field  con-

tains  the value FFFF(hex).  Otherwise, this field is un-

defined.

This  field  contains  a  valid entry locator of a "write

history entry" for this command if and only if  the  His-

tory  Logged end flag is set in the "flags" field of this

end message.

See  the  description  of the History Log and Reuse Entry

modifiers in Section section(dskcopydatacmf), "DISK  COPY

DATA  Command,  Command  Message Format" and Section sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

entry id

If  "write  history logging" is supported and the History

Log modifier is set in the command message, the  contents

of  the  corresponding  command  message  field is copied

without modification to this field.  Otherwise this field

is undefined.

See  the  description  of the History Log and Reuse Entry

modifies in Section section(dskcopydatacmf),  "DISK  COPY

DATA  Command,  Command  Message Format" and Section sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Before  returning  this  status  the  server performs the

following:

1.  Sets the "logical block count" end message

field  as  described   in   that   field's

description.

2.  If the the Retain Communication Paths mod-

ifier is clear,  performs  the  operations

necessary to disconnect the communications

paths established on behalf of the issuing

MINIMAL MSCP COMMAND SET                                PAGE 6-35

DISK COPY DATA COMMAND

host   class  driver.   See  Section  sec-

tion(dskcopydatades), "DISK COPY DATA Com-

mand,  Description"  for  details  of  the

disconnect operations.

Invalid Command (subcode "Invalid Modifier")

See the description of the Establish Communications Paths

modifier in Section section(dskcopydatacmf),  "DISK  COPY

DATA Command, Command Message Format" for specific infor-

mation about this error.  Note that if the command is re-

jected   with   this  status,  communications  paths  are

retained only if already established by a  previous  DISK

COPY DATA command.

Invalid Command (subcode "Invalid Logical Block Count")

Invalid Command (subcode "Invalid Destination LBN")

Invalid Command (subcode "Invalid Source LBN")

See  the  descriptions  of  the  "logical  block  count,"

"destination lbn,"  and  "source  lbn,"  command  message

fields  in  Section  section(dskcopydatacmf),  "DISK COPY

DATA Command, Command Message Format" for specific infor-

mation about these errors.

Note that communications paths are retained even when the

command is rejected with one of these status codes.

Invalid Command (subcode "Invalid Src Unit Number")

Invalid Command (subcode "Invalid Source Unit Identifier")

See  the description of the "src unit number" and "source

unit identifier" command message fields in  Section  sec-

tion(dskcopydatacmf),  "DISK  COPY  DATA Command, Command

Message Format" for specific information about these  er-

rors.   Note  that if the command is rejected with one of

these status codes, the communications paths are not  re-

tained,  regardless of the setting of the Retain Communi-

cations Path command modifier.

Invalid Command (subcode "Invalid  Source  Unit's  Storage

Subsystem Port Address")

Invalid Command (subcode "Invalid  Source  Unit's  Storage

Subsystem System Address")

See  the  descriptions of the "source unit's storage sub-

system port address," and  "source  unit's  storage  sub-

system  system address" command message fields in Section

section(dskcopydatacmf), "DISK COPY DATA Command, Command

MINIMAL MSCP COMMAND SET                                PAGE 6-36

DISK COPY DATA COMMAND

Message  Format" for specific information about these er-

rors.

Invalid Command (subcode "Invalid Entry Locator")

The value specified in the "entloc (entry locator)" field

is not within the limits of the server's  "write  history

entry"  set.   Note  that if the command is rejected with

this status, communications paths are  retained  only  if

already established by a previous DISK COPY DATA command.

See  the  description  of the History Log and Reuse Entry

modifiers in Section section(dskcopydatacmf), "DISK  COPY

DATA  Command,  Command  Message Format" and Section sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

Note  that  this  error  can only occur if "write history

logging" is supported and the History Log and Reuse Entry

modifiers are both set in the command message.

Invalid Command (subcode "Invalid Entry Id")

The  server  located the "write history entry" identified

in the "entloc (entry locator)" field but found that  the

"entry  id"  contained  in  that entry does not equal the

value supplied by a host class driver when that entry was

allocated.   This status may also be returned if the His-

tory Log modifier was set, the Reuse Entry  modifier  was

clear, and the host supplied a value of FFFF (hex) in the

"entry id" field.  Note that if the command  is  rejected

with  this status, communications paths are retained only

if already established by a previous DISK COPY DATA  com-

mand.

See  the  description  of the History Log and Reuse Entry

modifiers in Section section(dskcopydatacmf), "DISK  COPY

DATA  Command,  Command  Message Format" and Section sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

Note  that  this  error  can only occur if "write history

logging" is supported and the History Log modifier is set

in the command message.

Command Aborted

Before returning this status code the server performs the

following:

MINIMAL MSCP COMMAND SET                                PAGE 6-37

DISK COPY DATA COMMAND

1.  Ceases issuing transfer subcommands to the

destination unit and the source unit.

2.  Allows  all  transfer subcommands that are

currently outstanding on  the  destination

unit  to  complete.   Outstanding transfer

subcommands issued to the  source  may  be

aborted or allowed to complete.

3.  Discards  any  and  all data obtained from

the source unit and buffered prior to  the

receipt  of  the  ABORT command as well as

any data obtained from the source unit  as

a  result of allowing outstanding transfer

subcommands to complete in Step 2 above.

4.  Sets the "logical block count" end message

field as if an unrecoverable error had oc-

curred   as   described  in  that  field's

description.

5.  If the Retain Communication Paths modifier

is clear, performs the  operations  neces-

sary   to  disconnect  the  communications

paths established on behalf of the issuing

host   class  driver.   See  Section  sec-

tion(dskcopydatades), "DISK COPY DATA Com-

mand,  Description"  for  details  of  the

disconnect operations.

Unit-Available

Unit-Offline

Write Protected

If any of these conditions is detected on the destination

unit prior to initiating the copy operation,  the  server

shall  reject  the  command  with  the appropriate status

code.

If any of these conditions is detected on the destination

unit after the copy operation  has  been  initiated,  the

server reports it via the Subcommand Error status code.

In the case of the Unit-Available and Unit-Offline condi-

tions, the server shall perform the operations  necessary

to   disconnect   the   communications  paths  previously

established on behalf of the issuing host  class  driver,

if  any.  See Section section(dskcopydatades), "DISK COPY

MINIMAL MSCP COMMAND SET                                PAGE 6-38

DISK COPY DATA COMMAND

DATA Command, Description" for details of the  disconnect

operations.

In  the case of the Write Protected condition, communica-

tions paths are not retained, regardless of  the  setting

of the Retain Communications Paths command modifier.

Controller Error (subcode "Remote Connection Lost")

The "Controller-Online" connection between the server and

a remote source unit's server was spontaneously lost (for

any reason).

Note  that unlike a host class driver the server will not

attempt to recover from a remote connection  lost  condi-

tion (e.g., re-establish the communications path, reissue

previously outstanding  commands,  etc.).   Instead,  the

server  performs  the  following  before  returning  this

status:

1.  Ceases issuing transfer subcommands to the

destination unit.

2.  Allows  all  transfer subcommands that are

currently outstanding on  the  destination

unit to complete.

3.  Discards  any  and  all data obtained from

the source unit and buffered prior to  the

occurrence  of  the connection lost condi-

tion.

4.  Sets the "logical block count" end message

field as if an unrecoverable error had oc-

curred   as   described  in  that  field's

description.

5.  Performs  the operations necessary to dis-

connect the  destination  unit  communica-

tions  path  established  on behalf of the

issuing host class driver  (regardless  of

the  setting  of  the Retain Communication

Paths   modifier).    See   Section   sec-

tion(dskcopydatades), "DISK COPY DATA Com-

mand,  Description"  for  details  of  the

disconnect operations.

6.  Sends  a DISK COPY DATA CORRELATION infor-

mation log message with the Controller Er-

ror  (subcode  "Remote  Connection  Lost")

MINIMAL MSCP COMMAND SET                                PAGE 6-39

DISK COPY DATA COMMAND

event code to the appropriate  host  class

driver.   See  Section  section(dskcopyda-

tades), "DISK COPY DATA Command,  Descrip-

tion,  Error  Logs  and Correlation Logs,"

for related information.

Note  also  that  this error can only occur if the source

unit is located in a remote subsystem.

Controller Error (subcode "Remote Connection Request Failed,

Insufficient Resources to Request Remote

Connection)

Controller Error (subcode "Remote Connection Request Failed,

Subsystem Port Address Responded But

Subsystem System Address Does Not Match")

Controller Error (subcode "Remote Connection Request Failed,

Subsystem Port Address Responded But No

MSCP Server Present")

Controller Error (subcode "Remote Connection Request Failed,

Subsystem Port Address Responded But No

Resources Available")

Controller Error (subcode "Remote Connection Request Failed,

Subsystem Port Address Responded But No

Credits Available")

Controller Error (subcode "Remote Connection Request Failed,

No Response From Subsystem Port Address")

Controller Error (subcode "Remote Connection Request Failed,

Subsystem Port Address Responding But

Negative Acknowledge  Retry  Algorithm

Exhausted")

Controller Error (subcode "Remote Connection Request Failed,

Subsystem Port Request Timeout Exceeded")

Controller Error (subcode "Remote Connection Request Failed,

Subsystem Port Address Responded But

Rejected With Disconnected")

The    server's    initial   attempt   to   establish   a

"Controller-Online"  connection  with  a  remote   source

unit's server failed for the reason indicated by the sub-

code value.  New subcode values for this  status  may  be

added  as  the  need arises via an ECO to this specifica-

tion.

The  server should check for and report a "subsystem sys-

tem address" mismatch before checking for  and  reporting

the  unavailability  of  an  MSCP server.  If that is not

possible, a statement to that effect shall appear in  the

server's (controller's) Functional Specification.

MINIMAL MSCP COMMAND SET                                PAGE 6-40

DISK COPY DATA COMMAND

Before returning this status code the server shall send a

DISK COPY DATA CORRELATION information log  message  with

the appropriate Controller Error (subcode "Remote Connec-

tion Request Failed") event code to the appropriate  host

class  driver.  See Section section(dcddeslogging), "DISK

COPY DATA Command, Description, Error Logs  and  Correla-

tion Logs," for related information.

Note  that these errors can only occur if the source unit

is located in a remote subsystem.

Controller Error (subcode "Local Connection Request Failed,

Insufficient Resources to Request Local

Connection)

The    server's    initial   attempt   to   establish   a

"Controller-Online" connection with a local source unit's

server  failed  due to insufficient controller resources.

Note that this error can only occur if the source unit is

located in the same subsystem as the server.

Write History Entry Access Error (subcode "Allocation Failure

Table Full")

The server was unable to allocate a "write history entry"

and could not record this  fact  because  the  Allocation

Failure  Table  was full.  This is a transient condition;

therefore, the host should reissue the command.  See Sec-

tion  section(wrihismdes), "WRITE HISTORY MANAGEMENT Com-

mand, Description" for more details.

Note  that  this  error  can occur only if "write history

logging" is supported and the History Log modifier is set

and Reuse Entry modifier is clear in the command message.

Note also that communications paths are retained, regard-

less of the setting of the  Retain  Communications  Paths

command modifier.

Subcommand Error (subcode "Destination -- Command Timed Out")

Subcommand Error (subcode "Source -- Command Timed Out")

The  oldest  outstanding  subcommand the server issued to

the destination unit or the source unit timed  out.   The

subcode  bit  flag, "Destination" or "Source," identifies

the device on which the time out occurred.

Note  that the server shall implement the command timeout

algorithm described  in  Section  section(command_timos),

"Command Timeouts" for all outstanding subcommands in the

MINIMAL MSCP COMMAND SET                                PAGE 6-41

DISK COPY DATA COMMAND

same manner as a host class driver, with  one  exception.

The only exception is that the server makes no attempt to

recover  from  a  command   time   out   failure   (e.g.,

re-establish  the communications path, reissue previously

outstanding commands, etc.).

Note  also that the server's command timeout interval may

be shorter than  the  command  timeout  interval  of  the

remote  server.  Therefore, the server may indicate "pro-

gress" to the host while  waiting  for  the  subcommand's

longer interval to lapse.

Before  returning  this  status  the  server performs the

following:

1.  Ceases issuing transfer subcommands to the

destination unit and the source unit.

2.  Allows  all  transfer subcommands that are

currently outstanding on  the  destination

unit  and  not  affected  by  the  command

timeout to complete.  Outstanding transfer

subcommands  issued  to the source and not

affected by the  command  timeout  may  be

aborted or allowed to complete.

3.  Discards  any  and  all data obtained from

the source unit and buffered prior to  the

occurrence  of the command timeout as well

as any data obtained from the source  unit

as   a   result  of  allowing  outstanding

transfer subcommands to complete in Step 2

above.

4.  Places  the  appropriate  portion  of  the

timed out command's command message in the

"subcommand status" end message field.

5.  Sets the "logical block count" end message

field as if an unrecoverable error had oc-

curred   as   described  in  that  field's

description.

6.  Performs  the operations necessary to dis-

connect    the    communications     paths

established  on behalf of the issuing host

class driver (regardless of the setting of

the  Retain Communication Paths modifier).

See Section section(dskcopydatades), "DISK

MINIMAL MSCP COMMAND SET                                PAGE 6-42

DISK COPY DATA COMMAND

COPY   DATA   Command,   Description"  for

details of the disconnect operations.

7.  Sends  a DISK COPY DATA CORRELATION infor-

mation log message  with  the  appropriate

Subcommand Error event code/subcode to the

appropriate host class driver.  The "event

dependent  information"  field should con-

tain a copy of the  MSCP  command  message

for  the  subcommand  that timed out.  See

Section section(dcddeslogging), "DISK COPY

DATA  Command, Description, Error Logs and

Correlation Logs,"  for  related  informa-

tion.

Subcommand  Error  (subcode "Destination -- Inconsistent

State")

Subcommand Error (subcode "Source -- Inconsistent State")

The  server  detected  an  operational inconsistency as a

result of the execution of a subcommand it issued to  the

destination  unit  or  the  source unit.  The subcode bit

flag, "Destination" or "Source," identifies the device on

which the inconsistent state was detected.

This status is returned under the following conditions:

A.  The  source  unit  is  located in a remote

subsystem and the Controller Initiated Bad

Block  Replacement (CIBBR) flag is not set

in the end message of the  SET  CONTROLLER

CHARACTERISTICS command issued to a remote

source unit's server or the ONLINE command

issued to a remote source unit.  Note that

CIBBR is required on a remote source  unit

because  the server will not provide "Host

Initiated Bad Block Replacement."

B.  The  server determines via ONLINE commands

issued to the source unit and the destina-

tion  unit  that  the  sector  format (512

bytes or 576 bytes) of the source unit  is

not  identical  to that of the destination

unit.

C.  An  ONLINE  command issued to the destina-

tion unit or the source unit is  completed

with a status of Success (subcode "Already

Online").

MINIMAL MSCP COMMAND SET                                PAGE 6-43

DISK COPY DATA COMMAND

D.  An Invalid Command status was returned for

a subcommand  issued  to  the  destination

unit or the source unit.

E.  A  READ command issued to the source unit,

or a WRITE command issued to the  destina-

tion  unit  is terminated with a status of

Host Buffer Access Error (all subcodes ex-

cept   "Cause  Not  Available"  and  "Host

Memory Parity Error").

F.  An   AVAILABLE   command   issued  to  the

destination unit or  the  source  unit  is

completed  or  rejected  with  a status of

Success  (subcode  "Spin-down   Ignored"),

Success  (subcode "Still Online/Unload Ig-

nored"),   or   Unit-Available    (subcode

"Already In Use").

Before  returning  this  status  the  server performs the

following:

A.  Ceases issuing transfer subcommands to the

destination unit and the source unit.

B.  Allows  all  transfer subcommands that are

currently outstanding on  the  destination

unit  to  complete.   Outstanding transfer

subcommands issued to the  source  may  be

aborted or allowed to complete.

C.  Discards  any  and  all data obtained from

the source unit and buffered prior to  the

detection  of the inconsistency as well as

any data obtained from the source unit  as

a  result of allowing outstanding transfer

subcommands to complete in Step 2 above.

D.  Places  the  appropriate  portion  of  the

failed command's end message in the  "sub-

command status" end message field.

MINIMAL MSCP COMMAND SET                                PAGE 6-44

DISK COPY DATA COMMAND

E.  Sets the "logical block count" end message

field as if an unrecoverable error had oc-

curred   as   described  in  that  field's

description.

F.  Performs  the operations necessary to dis-

connect    the    communications     paths

established  on behalf of the issuing host

class driver (regardless of the setting of

the  Retain Communication Paths modifier).

See Section section(dskcopydatades), "DISK

COPY  DATA  Command,  Description" for de-

tails of the disconnect operations.

G.  For  the  last  four  "Inconsistent State"

conditions listed above (i.e.,  C  through

F), sends a DISK COPY DATA CORRELATION er-

ror log message with the appropriate  Sub-

command  Error  event  code/subcode to the

appropriate host class driver.  The "event

dependent  information"  field should con-

tain a copy of the subcommand's  end  mes-

sage.  See Section section(dcddeslogging),

"DISK COPY DATA Command, Description,  Er-

ror  Logs  and  Correlation Logs," for re-

lated information.

H.  The  last four "Inconsistent State" condi-

tions listed above (i.e., C through F) in-

dicate   a   serious  malfunction  in  the

server's operation.

After  attempting  to deliver the end mes-

sage for the DISK COPY DATA  Command,  the

server        shall        enter       the

"Controller-Available" state  relative  to

all hosts.

Subcommand Error (subcode "Destination -- Unrecoverable

Error")

Subcommand Error (subcode "Source -- Unrecoverable Error")

MINIMAL MSCP COMMAND SET                                PAGE 6-45

DISK COPY DATA COMMAND

An unrecoverable error occurred during the execution of a

subcommand issued by the server to the  destination  unit

or  the source unit.  The subcode bit flag, "Destination"

or "Source," identifies  the  device  on  which  the  un-

recoverable error occurred.

The following are classed as unrecoverable errors:

A.  Any  subcommand  issued to the destination

unit or the source unit is  rejected  with

any  of  the following status codes:  Con-

troller Error, Drive Error,  Media  Format

Error, Unit-Available, or Unit-Offline.

B.  A  READ command issued to the source unit,

or a WRITE command issued to the  destina-

tion  unit  is terminated with a status of

Host Buffer Access Error  (subcode  "Cause

Not  Available" or "Host Memory Parity Er-

ror").

C.  A  READ  subcommand  issued  to the source

unit is rejected with either of  the  fol-

lowing  status  codes:   Compare  Error or

Data Error.

D.  A  WRITE subcommand issued to the destina-

tion unit is rejected or  terminated  with

any  of  the following status codes:  Com-

pare   Error,   Data   Error,   or   Write

Protected.

Before  returning  this  status  the  server performs the

following:

A.  Ceases issuing transfer subcommands to the

destination unit and the source unit.

B.  Allows  all  transfer subcommands that are

currently outstanding on  the  destination

unit or the source unit to complete.

C.  Discards  any  and  all data obtained from

the source unit and buffered prior to  the

detection of the unrecoverable error.

MINIMAL MSCP COMMAND SET                                PAGE 6-46

DISK COPY DATA COMMAND

D.  Allows  all  transfer subcommands that are

currently outstanding on  the  destination

unit  to  complete.   Outstanding transfer

subcommands issued to the  source  may  be

aborted or allowed to complete.

E.  Discards  any  and  all data obtained from

the source unit and buffered prior to  the

detection  of  the  unrecoverable error as

well as any data obtained from the  source

unit  as  a result of allowing outstanding

transfer subcommands to complete in Step 2

above.

F.  Places  the  appropriate  portion  of  the

failed command's end message in the  "sub-

command status" end message field.

G.  Sets the "logical block count" end message

field  as  described   in   that   field's

description.

H.  With  the  exception  of the occurrence of

the Media  Format  Error,  Unit-Available,

and  Unit-Offline  error conditions listed

in A above, performs the operations neces-

sary   to  disconnect  the  communications

paths established on behalf of the issuing

host class driver if the Retain Communica-

tion Paths modifier is clear.

If the Media Format Error, Unit-Available,

and Unit-Offline error  conditions  listed

in  A above occur, performs the operations

necessary to disconnect the communications

paths established on behalf of the issuing

host class driver (regardless of the  set-

ting  of  the  Retain  Communication Paths

modifier).  See Section section(dskcopyda-

tades),  "DISK COPY DATA Command, Descrip-

tion"  for  details  of   the   disconnect

operations.

I.  Sends  a DISK COPY DATA CORRELATION infor-

mation log message  with  the  appropriate

Subcommand Error event code/subcode to the

appropriate host class driver.  The "event

dependent  information"  field should con-

tain  a  copy  of  the  subcommand's   end

message.             See           Section

MINIMAL MSCP COMMAND SET                                PAGE 6-47

DISK COPY DATA COMMAND

section(dcddeslogging),  "DISK  COPY  DATA

Command,  Description, Error Logs and Cor-

relation Logs," for related information.

MINIMAL MSCP COMMAND SET                                PAGE 6-48

DISK COPY DATA COMMAND

6.7.3  Description

Support  of the DISK COPY DATA command is a server implementation

dependent option.  Servers that do not provide such support shall

reject  this  command  as  an "Invalid Command" ("invalid opcode"

protocol  error.   See  "Invalid  Command"  status   in   Section

section(status__codes), "Status Codes").

For  servers  that  support only local DISK COPY DATA operations,

the minimum command length is 44 bytes.  For servers that support

remote  DISK  COPY DATA operations, the minimum command length is

60 bytes.  All DISK COPY DATA commands issued by the  host  shall

be  at  least  as  long  as  the  minimum length supported by the

server.  If a server receives a DISK COPY DATA  command  that  is

shorter  than  the minimum command length, it rejects the command

as an "Invalid Command" ("invalid message length" protocol error.

See  "Invalid  Command" status in Section section(status__codes),

"Status Codes").

Servers  that  support remote DISK COPY DATA shall support remote

copies between units with unlike  geometries  and  remote  copies

where  the  source  LBN  and destination LBN differ.  All servers

should support  copies  between  units  with  unlike  geometries,

copies  where  the  source and destination LBN differ, and copies

where the source and destination are the same unit.  Servers that

do not support local copies between units with unlike geometries,

local copies where the source and  destination  LBN  differ,  and

copies  where  the source and destination are the same unit indi-

cate this lack of support by setting  the  Restricted  DISK  COPY

DATA  Operations controller flag in the SET CONTROLLER CHARACTER-

ISTICS end message (see  Section  section(controlerflags),  "Con-

troller Flags").

This  command  allows  data  to be copied from one disk device to

another without host intervention.  A server may support two dif-

ferent  types  of copy operations:  local and remote.  In a local

copy operation, both the source and destination disk  drives  are

physically connected to the storage subsystem in which the server

that receives this command is located.  In a remote  copy  opera-

tion,  the  destination disk drive is physically connected to the

storage subsystem in which the server that received this  command

is  located  and the source disk drive is physically connected to

another storage subsystem connected to  the  same  communications

mechanism.   Local  copy  operations  may be supported by servers

that provide Multihost Support as well as those that do  not.   A

server  that  provides Multihost Support may, at its option, sup-

port both local and remote copy operations  or  only  local  copy

operations.   However,  support  of  both  local  and remote copy

operations is preferred.  A server signifies  the  type  of  copy

MINIMAL MSCP COMMAND SET                                PAGE 6-49

DISK COPY DATA COMMAND

operation  it  supports  by  setting the Local Disk Copy Data and

Remote Disk Copy Data controller flags appropriately.

6.7.3.1  Operation   - To  perform  a copy operation a host class

driver merely specifies the devices that are to act as  the  data

source  and  data destination, the starting logical block numbers

on the source and destination, and the quantity  of  data  to  be

copied  from the source to the destination.  The server then per-

forms the following operations to fulfill the  host's  DISK  COPY

DATA request:

A.  Checks  the  contents  of  the command message

modifiers and fields to ensure  that  a  valid

copy  operation  can be performed.  The checks

performed are described in (or implied by) the

various  modifier  and  field  descriptions in

Section  section(dskcopydatacmf),  "DISK  COPY

DATA  Command, Command Message Format." If any

of the checks fail, the command is rejected as

an "Invalid Command."

Some   of   the   checks  require  information

provided by operations performed in  Steps  4,

5, and 6 below.  Command message field valida-

tion is therefore  not  complete  until  those

steps have been completed.

B.  If  communications  paths  to  the destination

unit and the source  unit  have  already  been

established  on  behalf  of  the  issuing host

class driver and those  paths  remain  intact,

proceeds to Step 6.

Otherwise, proceeds to Step 3.

C.  Performs  the internal operations necessary to

establish  a  "Controller-Online"   connection

with  itself to enable communications with the

destination unit and in the case  of  a  Local

Disk  Copy  Data  (i.e., the Local Source Unit

MINIMAL MSCP COMMAND SET                                PAGE 6-50

DISK COPY DATA COMMAND

modifier is set or the  server  only  supports

Local Disk Copy Data) with the source unit.

If  the source unit is local, proceeds to Step

4.

If the source unit is remote (i.e., it is con-

nected to a storage subsystem other  than  the

one  in which the server is located), performs

the communication  protocol  dependent  opera-

tions     necessary     to     establish     a

"Controller-Online" connection with the source

unit's  server  (i.e.,  the  remote  disk MSCP

server) and then proceeds to Step 4.

D.  Issues  a  SET CONTROLLER CHARACTERISTICS com-

mands to itself and, if the source unit is lo-

cated  in  a  remote  subsystem, to the source

unit's server, to  set  an  appropriate  "host

timeout,"  to  request the generation of error

logs if required, and to determine the maximum

byte count supported.

Note that processing related to "host timeout"

and error logging is discussed later  in  this

section.

E.  Issues an ONLINE command to the source unit to

bring it into the "Unit-Online"  state  (rela-

tive  to  the  server)  and  ensures  that the

destination unit is "Unit-Online."

Note  that the server shall issue only one ON-

LINE command if the destination  unit  is  the

same as the source unit.

F.  Issues   GET   UNIT  STATUS  commands  to  the

destination unit and the source unit to obtain

device   characteristics  (i.e.,  disk  volume

geometry)  useful  for  optimizing  the   copy

operation.

MINIMAL MSCP COMMAND SET                                PAGE 6-51

DISK COPY DATA COMMAND

Note  that  during  this step the server shall

validate the "source unit identifier"  command

message  field using the value returned in the

"unit identifier" end message field of the GET

UNIT  STATUS command issued.  See the descrip-

tion of the "source unit identifier" field  in

Section  section(dskcopydatacmf)  for more de-

tail.

G.  Issues  a  READ  command to the source unit to

obtain the source data.

H.  Issues a WRITE command to the destination unit

to copy the source data.

I.  Steps  7  and  8  are  repeated until the copy

operation  is  aborted,  completed,  or   ter-

minated.

Note  that  for subsequent DISK COPY DATA com-

mands issued by the same host class driver the

server  will  not  perform  Steps  3, 4, and 5

(provided the  communications  paths  involved

remain  intact).   However,  the  server shall

perform Steps 1 and 6 (as well as Steps 7  and

8)  for  each DISK COPY DATA command received.

The server shall retain  all  information  ac-

quired  in  Steps  4 and 5 that is required to

perform Step 1 until the communications  paths

involved are no longer intact.

The  operations  described  in  the preceding steps provide a

simplified model of operation  that  is  solely  intended  to

identify  and  describe  functions  that are implied in other

sections of this command's description.  Any deviations  from

the  model  shall  be implemented in a manner that guarantees

that all host class driver visible results are  identical  to

those described in this command description.

The  actions  the  server takes after the copy operation has been

aborted, completed, or terminated are described  in  the  various

status  code  descriptions  in  Section  section(dskcopydataemf),

"DISK COPY DATA Command, End Message Format."

MINIMAL MSCP COMMAND SET                                PAGE 6-52

DISK COPY DATA COMMAND

Throughout  this command description numerous references are made

to the effect that the server issues standard MSCP  commands  (in

the same manner as a host class driver).  Note that the server is

expressly permitted to perform any equivalent internal  operation

instead of issuing a standard command provided that the following

rules are followed:

A.  The  result  of  the  internal  operation shall be

identical to that of the standard command.

B.  If the internal operation results in an error, the

"subcommand status" end message field shall be set

as if a standard command had been issued.

Naturally,  the use of internal operations is limited to transac-

tions the server can perform on itself and the unit(s) it serves.

Standard  commands shall be used in all transactions dealing with

a remote source unit and its server.

The server shall provide "Controller Initiated Bad Block Replace-

ment" (CIBBR) support on all disk drives that it serves --  i.e.,

the CIBBR controller flag shall be set.

The  server  will  not perform "Host Initiated Bad Block Replace-

ment" for a remote source unit.  Host class drivers should verify

that  CIBBR is supported on a remote source unit prior to issuing

this command.  Failure to do so will result in the  rejection  of

this  command  with a status of Subcommand Error (subcode "Source

-- Inconsistent State").  See the description of that  status  in

Section  section(dskcopydataemf),  "DISK  COPY  DATA Command, End

Message Format," for more detail.

As  mentioned in the description of the Subcommand Error (subcode

"Destination/Source -- Command Timeout) status  in  Section  sec-

tion(dskcopydataemf)  the  server  shall  implement  the  command

timeout algorithm described  in  Section  section(command_timos),

"Command  Timeouts"  for  all outstanding subcommands in the same

manner as a host class driver, with one exception.  The only  ex-

ception  is  that  the  server makes no attempt to recover from a

command time out failure (e.g.,  reissue  previously  outstanding

commands).

With regard to GET COMMAND STATUS commands that specify the "out-

standing reference number" of a DISK COPY DATA command the server

shall  provide status based on the progress of the copy operation

in the same manner required for any other command.  See  Sections

section(getcommandcmd),  "GET  COMMAND  STATUS  Command" and sec-

tion(getcommandmcmd), "Multihost GET COMMAND STATUS Command"  for

more  detail.   Note that certain communications protocols (e.g.,

SCA) do not provide a direct method  for  determining  whether  a

MINIMAL MSCP COMMAND SET                                PAGE 6-53

DISK COPY DATA COMMAND

remote subsystem exists.  If such a communications protocol is in

use, the server's request to establish a "Controller-Online" con-

nection with a remote source unit's server will never complete if

the subsystem does not exist.  To allow  host  class  drivers  to

react  to  such  an occurrence the server shall not show progress

for a DISK COPY DATA command where the source unit is located  in

a remote subsystem until after the "Controller-Online" connection

to the remote subsystem has been successfully established.   That

action will eventually cause the host class driver to timeout the

affected  DISK  COPY  DATA  command  as  described   in   Section

section(command_timos),  "Command Timeouts." In such a case it is

highly recommended that a host  class  driver  not  resynchronize

with  the  server without first checking that it can still access

the remote source unit.  If the remote source can  no  longer  be

accessed,  the  host  class  driver should issue an ABORT command

referencing the affected DISK COPY DATA command.  In response  to

such  an  ABORT  command  the  server shall cancel the connection

establishment request and terminate the  command  in  the  normal

fashion.   If  the  remote source unit can still be accessed, the

host class driver should assume that the server may be insane and

perform the normal command timeout operations as prescribed.

The  server  shall  not limit the use of a particular destination

unit or source unit to copy operations requested  by  any  single

host class driver.  Communications paths established on behalf of

multiple class drivers may be managed in any manner convenient to

the  server.   Regardless  of  the method used for communications

path management the server shall:

A.  Maintain  state  context  separately for each host

class driver to give the appearance  that  a  com-

munications  path  is established solely on an in-

dividual host class driver's behalf.

B.  Associate  the communications paths established on

a particular host class driver's behalf  with  the

"Controller-Online" connection established between

the server and that host class driver.  (This  as-

sociation  is provided to ensure that the communi-

cations paths are dissolved following the loss  of

a "Controller-Online" connection.)

The  server shall disconnect the communications paths established

on behalf of a host class  driver  under  the  following  circum-

stances  if  and only if the Retain Communications Paths modifier

is clear:

A.  This command is aborted -- i.e., status is Command

Aborted.

MINIMAL MSCP COMMAND SET                                PAGE 6-54

DISK COPY DATA COMMAND

B.  This  command  is  completed successfully -- i.e.,

status is Success (subcode "Normal").

C.  An  unrecoverable error occurred during the execu-

tion of a subcommand issued by the server  to  the

destination  unit or the source unit -- i.e., this

command is  terminated  with  a  Subcommand  Error

(subcode  "Destination/Source -- Unrecoverable Er-

ror") status, all causes except Media  Format  Er-

ror, Unit-Available, and Unit-Offline.

Regardless of the setting of the Retain Communications Paths mod-

ifier  the  server  shall  disconnect  the  communications  paths

established  on behalf of a host class driver under the following

circumstances:

A.  The  "Controller-Online"  connection  to  the host

class driver is lost -- i.e., the  server  becomes

"Controller-Available" or "Controller-Offline" re-

lative to the issuing host class driver.

B.  The  destination  unit enters the "Unit-Available"

or "Unit-Offline" state relative to the host class

driver  --  i.e.,  this command is rejected with a

Unit-Available or Unit-Offline status.

C.  An  unrecoverable error occurred during the execu-

tion of a subcommand issued by the server  to  the

destination  unit or the source unit -- i.e., this

command is  terminated  with  a  Subcommand  Error

(subcode  "Destination/Source -- Unrecoverable Er-

ror") status, only if the error  status  is  Media

Format Error, Unit-Available, or Unit-Offline.

D.  The  "Controller-Online"  connection  to  a remote

source unit's server is lost -- i.e., this command

is  terminated  with  a  Controller Error (subcode

"Remote Connection Lost") status.

E.  A  subcommand issued by the server to the destina-

tion or the source unit is timed out -- i.e., this

command  is  terminated  with  a  Subcommand Error

(subcode  "Destination/Source  --  Command   Timed

Out") status.

F.  An inconsistent state is detected by the server --

i.e., this command is terminated with a Subcommand

Error (subcode "Destination/Source -- Inconsistent

State") status.

MINIMAL MSCP COMMAND SET                                PAGE 6-55

DISK COPY DATA COMMAND

The  server  performs  the following operations to disconnect the

communications paths  established  on  behalf  of  a  host  class

driver:

A.  Discards the communications path state context be-

ing maintained on behalf of the issuing host class

driver.

B.  Issues  an  AVAILABLE  command  to the destination

unit.

Note  that the server shall not issue this command

if the disconnect is being performed as  a  result

of  a  command time out failure on the destination

unit.

C.  Issues an AVAILABLE command to the source unit.

Note  that the server shall not issue this command

if the disconnect is being performed as  a  result

of  a  command time out failure on the source unit

or if the destination unit  is  the  same  as  the

source unit.

D.  Performs the internal operations necessary to dis-

connect the internal "Controller  Online"  connec-

tion to itself.

E.  If  the  source  unit  is located on a remote sub-

system,  performs  the   communications   protocol

dependent  operations  necessary to disconnect the

"Controller Online" connection between itself  and

the source unit's server.

Note  that if a communications path is shared among multiple host

class drivers, the server shall not perform any of the disconnect

steps described above that would result in the loss of a communi-

cations path established on behalf of a host class  driver  other

than  the  issuing  host class driver.  That is, the server shall

not disconnect the communications path established on behalf of a

host  class  driver  unless  the server is processing a DISK COPY

DATA command issued by that host class driver.

To  prevent  needless  consumption  of  remote source unit server

resources in cases where the server spontaneously  loses  context

(e.g.,  crashes, power failures) after having established and re-

tained a communications path on behalf of a  host  class  driver,

the server shall perform the following:

MINIMAL MSCP COMMAND SET                                PAGE 6-56

DISK COPY DATA COMMAND

A.  Supply  an implementation dependent "host timeout"

value in the SET CONTROLLER  CHARACTERISTICS  com-

mand  issued to the remote source unit's server in

Step 4 above.

The  "host timeout" value supplied should be large

enough to not cause a timeout  on  the  communica-

tions path established and retained on behalf of a

host class driver  while  the  "Controller-Online"

connection  between that host class driver and the

server remains intact and operations appear to  be

normal.

B.  Whenever  a  DISK  COPY DATA command that involves

the remote source unit (and  the  associated  com-

munications  path)  is  not currently in progress,

periodically issue an innocuous command (e.g., GET

UNIT  STATUS)  to the remote source unit at inter-

vals no greater than the "host timeout" supplied.

MINIMAL MSCP COMMAND SET                                PAGE 6-57

DISK COPY DATA COMMAND

The  chart  below summarizes the condition under which the server

retains or terminates communications paths prior to returning the

DISK COPY DATA end message.

+-----------------------------------+------------+--------------+

| | Retain CP | Comm Paths |

| End Message Status Code/Subcode | modifier | Retained |

| |in cmd mess | end flag |

+-----------------------------------+------------+--------------+

| Success, subcodes | | |

| "Normal" | | |

| "Duplicate Unit Number" | Set | |

| Command Aborted | | |

| Subcommand Error, subcode +------------+--------------+

| "Src/Dst Unrecoverable Error" (1)| Clear | Clear |

+-----------------------------------+------------+--------------+

| Invalid Command, subcodes | | |

| "Invalid Source LBN" | | Set |

| "Invalid Destination LBN" | | (Comm Paths |

| "Invalid Logical Block Count" | | always |

| "Invalid Entry ID" | | retained) |

| "Invalid Entry Locator" | | |

| Write Hist Entry Access Error | | |

| "Allocation Failure Table Full" | | |

+-----------------------------------+ +--------------+

| Invalid Command, subcodes | | Set if Comm |

| "Invalid Modifier" | | Paths up from|

| | | previous DCD;|

| | | no otherwise.|

+-----------------------------------+ +--------------+

| Invalid Command, subcodes | | |

| "Invalid Src Unit Number" | | Clear |

| "Invalid Src Unit Identifier" | Ignored | (Comm Paths |

| Controller Error (all subcodes) | | never |

| Subcommand Error, subcodes | | retained) |

| "Src/Dst Unrecoverable Error" (2)| | |

| "Src/Dst Command Timed Out" | | |

| "Src/Dst Inconsistent State" | | |

+-----------------------------------+ +--------------+

| Invalid Command, subcodes | | |

| "Invalid Source Unit's Storage | | Clear.  Server|

| Subsystem Port Address" | | rejects the |

| "Invalid Source Unit's Storage | | DCD command.  |

| Subsystem System Address" | | Comm Paths |

| Unit Available | | do not get |

| Unit Offline | | established.  |

| Write Protected | | |

+-----------------------------------+------------+--------------+

MINIMAL MSCP COMMAND SET                                PAGE 6-58

DISK COPY DATA COMMAND

Notes:

1.  All cases except Media Format Error, Unit-Available,

Unit-

Offline.   So, errors like Data Error on a READ subcom-

mand

only  cause  the  server  to  terminate the communica-

tions

paths if the Retain CP modifier is clear.

2.  Reports  cases  where  a subcommand is rejected with

Media

Format Error, Unit-Available, or Unit-Offline status.

The  server  shall  not request attention messages (i.e., set the

Enable Attention Messages controller flag) from a  remote  source

unit's  server via the SET CONTROLLER CHARACTERISTICS command is-

sued in Step 4 above.  Therefore, host class drivers  should  set

the "Enable Attention Messages" flag via a SET CONTROLLER CHARAC-

TERISTICS command issued over a separate "Controller-Online" con-

nection  to the remote source unit's server if attention messages

related to that unit are desired.

The server shall not attempt to change the current device charac-

teristics of the source unit via the  ONLINE  command  issued  in

Step  5  above.   If  a host class driver requires certain source

unit characteristics to be set, it should set them prior to issu-

ing this command.

The   server  shall  execute  a  DISK  COPY  DATA  command  as  a

Non-Sequential command -- i.e., in relation to all other commands

addressed  to  the destination unit, the transfer subcommands the

server issues to the destination unit as part of the copy  opera-

tion may be reordered to optimize performance, may be interleaved

with other Non-Sequential commands, and all of them shall be com-

pleted  before  the execution of a Sequential command can be ini-

tiated.  Host software developers are  advised  to  give  careful

consideration  to the effects of issuing a lengthy DISK COPY DATA

command on Sequential command execution.

The only exception to the Non-Sequential execution just described

is that access to the server's "immediate target" on the destina-

tion  unit is controlled.  The server's "immediate target" is the

range of logical blocks on the destination unit that  the  server

is  about  to  act  upon or is currently acting upon.  To be more

specific the "immediate target" is the range  of  logical  blocks

specified  in  the  READ  subcommand used to obtain a fragment of

source data and the WRITE subcommand used to copy  the  resultant

data to the destination unit.

MINIMAL MSCP COMMAND SET                                PAGE 6-59

DISK COPY DATA COMMAND

Access to the server's "immediate target" on the destination unit

shall be controlled according to the following rules:

a.  If  a  command  that  specifies a range of logical

blocks that is within  or  overlaps  the  server's

"immediate  target"  is currently in progress, the

server postpones issuing any subcommands that  af-

fect  the "immediate target" until such an in pro-

gress  command  completes,   terminates,   or   is

aborted.

b.  If  a  command  that  specifies a range of logical

blocks that is within  or  overlaps  the  server's

"immediate target" is received while server action

on  the  "immediate  target"  is   postponed   (as

described  in "a" above), the server postpones the

execution of the command received until  the  sub-

commands  that  affect the "immediate target" have

been initiated and have  then  completed  or  ter-

minated.

c.  If  a  command  that  specifies a range of logical

blocks that is within  or  overlaps  the  server's

"immediate  target"  is received while the subcom-

mands that affect the "immediate target" are  cur-

rently  in  progress,  the  server  postpones  the

execution of the command received until  the  sub-

commands  that  affect the "immediate target" have

been completed or terminated.

The effect of these rules is to treat access to the server's "im-

mediate target" as Sequential  operations.   When  selecting  the

number  of logical blocks to be included in a server's "immediate

target," server developers are advised to give careful considera-

tion  to  the  effects  of  postponing host I/O to the "immediate

target."

The  reason  for providing the "immediate target" access behavior

described above is to allow host class drivers to  perform  write

type  operations  (i.e., DISK COPY DATA, ERASE, and WRITE) on the

destination unit in a manner that guarantees that  while  a  DISK

COPY  DATA  command  is  in progress, all data written to logical

blocks on the destination unit will be identical  to  those  con-

tained on the source unit.  To fulfill that guarantee, host class

drivers shall ensure that write operations that specify  a  range

of logical blocks that is within or overlaps the range of logical

blocks specified in a DISK COPY DATA command are  issued  to  the

source  unit  and completed there before the equivalent operation

is issued to the destination unit.

MINIMAL MSCP COMMAND SET                                PAGE 6-60

DISK COPY DATA COMMAND

Note that the result of a read type operation (i.e., ACCESS, COM-

PARE HOST DATA, and READ) issued to  the  destination  unit  that

specifies  a  range  of logical blocks that is within or overlaps

the range of logical blocks specified in a DISK COPY DATA command

is  unpredictable.   Host  class  drivers  should  therefore rely

solely on the contents of the source unit for  read  type  opera-

tions.

6.7.3.2  Error Logs and Correlation Logs  -

All error logs generated due to the execution of a DISK COPY DATA

command are handled by the server and returned to the appropriate

host  class drivers, irrespective of whether the error originated

from the destination unit, a  local  source  unit,  or  a  remote

source unit.

For error logs generated as a result of subcommands issued to the

destination and source units the server  performs  the  following

operations:

A.  Sets  the  "command reference number" field equal to the

value supplied in the "command reference  number"  field

of the related DISK COPY DATA command message.

B.  If  the  error log message is from the source unit, sets

the Informational error log message flag.

C.  If  the  error log message is from a remote source unit,

sets the Disk Copy Data Correlate error log message flag

and  remembers  that  at  least one error log associated

with this command has been delivered so  that  one  DISK

COPY DATA CORRELATION error log message may be delivered

to the appropriate class drivers prior to  the  delivery

of  the  end  message for this particular DISK COPY DATA

command.

D.  Sends the modified message to the appropriate host class

drivers.

MINIMAL MSCP COMMAND SET                                PAGE 6-61

DISK COPY DATA COMMAND

E.  If a source unit communications path is shared by multi-

ple host class drivers, the  server  shall  treat  error

logs  received over that path according to the following

rules (assuming that the host class driver has  properly

enabled error logging):

1.  Error logs generated by subcommands issued on behalf

of an individual host class  driver  shall  be  sent

only to that particular host class driver.

2.  Error logs generated by subcommands issued on behalf

of all the host class drivers that  share  the  same

path  shall  be  sent  to all the host class drivers

sharing that path.

Prior  to sending the end message for a particular DISK COPY DATA

command, the server generates a DISK COPY DATA CORRELATION  error

log and delivers it to the appropriate class drivers for the fol-

lowing cases:

1.  The  server  determines  that  the source or destination

unit is in an inconsistent state for any of the  reasons

described in Section section(dskcopydataemf), "DISK COPY

DATA Command, End Message Format,"  except:   Controller

Initiated  Bad Block Replacement is not supported by the

remote source unit's server or the source  and  destina-

tion units have different formats.

The  event  code is the same as the status code returned

in the DISK COPY DATA command's end message (see Section

section(dskcopydataemf),  "DISK  COPY  DATA Command, End

Message Format").   The  "event  dependent  information"

field should contain a copy of the subcommand's end mes-

sage.

MINIMAL MSCP COMMAND SET                                PAGE 6-62

DISK COPY DATA COMMAND

2.  The status code returned in the end message of a subcom-

mand issued to the source or destination unit  indicates

an  unrecoverable  error other than Unit Available, Unit

Offline, or Write Protected.

The  event  code is the same as the status code returned

in the DISK COPY DATA command's end message (see Section

section(dskcopydataemf),  "DISK  COPY  DATA Command, End

Message Format").   The  "event  dependent  information"

field should contain a copy of the subcommand's end mes-

sage.

3.  The  server  fails to establish a connection to or spon-

taneously loses an established connection to the  remote

source unit's server.

The  event  code  is  the  same  as the Controller Error

status code returned in the DISK COPY DATA command's end

message (see Section section(dskcopydataemf), "DISK COPY

DATA Command, End Message Format").

4.  The  server  times out the oldest outstanding command to

either the source or destination unit.

The  event  code is the same as the status code returned

in the DISK COPY DATA command's end message (see Section

section(dskcopydataemf),  "DISK  COPY  DATA Command, End

Message Format").   The  "event  dependent  information"

field  should contain a copy of the subcommand's command

message.

5.  At  least  one error log has been delivered as described

in item C of the previous list.

The event code is Informational (subcode "Disk Copy Data

Correlation").

Note  that  if  more than one of the above conditions exists, the

DISK COPY DATA CORRELATION error log shall be as specified by the

lowest  numbered condition.  That is, the conditions listed above

are ordered by priority.

MINIMAL MSCP COMMAND SET                                PAGE 6-63

DISK COPY DATA COMMAND

The  server  shall  send the DISK COPY DATA CORRELATION error log

and the error logs modified as described above only to host class

drivers  that  have  enabled  error logging by setting the Enable

This Host's Error Logs controller  flag  or  setting  the  Enable

Other  Host's  Error  Logs  controller  flag  in a SET CONTROLLER

CHARACTERISTICS command.

The  server performs the following special actions with regard to

the source unit communications path established on  behalf  of  a

host  class  driver  when  error logging is enabled as previously

described:

A.  If  a  host  class driver enables error logging before a

source unit communications path  has  been  established,

the  server  shall set the Enable This Host's Error Logs

controller flag via the SET  CONTROLLER  CHARACTERISTICS

command  issued  to  the  source unit's server in Step 4

above.

B.  If  a  host  class  driver enables error logging after a

source unit communications path  has  been  established,

the  server shall issue a SET CONTROLLER CHARACTERISTICS

command to the source unit's server to  set  the  Enable

This  Host's Error Logs controller flag for that connec-

tion before issuing any other subcommand to  the  source

unit.

C.  If  any  host class driver or server action occurs which

causes error logging to be disabled after the server has

issued  a  SET CONTROLLER CHARACTERISTICS command to the

source unit's server to set the Enable This Host's Error

Logs  controller  flag on a connection, the server shall

issue a SET CONTROLLER CHARACTERISTICS  command  to  the

source unit's server to clear the Enable This Host's Er-

ror Logs controller flag for that connection before  is-

suing any other subcommand to the source unit.

MINIMAL MSCP COMMAND SET                                PAGE 6-64

DISK COPY DATA COMMAND

6.8  DISPLAY COMMAND

Command Category:

Controller    implementation    dependent.     See    Section

section(disp___des)

6.8.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| mode | item |

+---------------+---------------+

| |

/ display /

/ text /

| |

+-------------------------------+

item

Identifies  the  information  meant  to be conveyed to an

operator.  Qualifies the interpretation of the  mode  and

display text fields.  See description below.

mode

Identifies  the mode or state of the information meant to

be conveyed to an operator.  Often represented as an icon

or  by  the  visual  rendition  of  the information.  See

description below.

display text

One or more text strings containing the information meant

to be conveyed to an operator.  The contents of the  item

and  mode  fields  determine  the expected number of text

strings and their interpretation;  see description below.

MINIMAL MSCP COMMAND SET                                PAGE 6-65

DISPLAY COMMAND

Each  text  string  consists  of  one byte containing the

length  followed  by  that  number  of   bytes   encoding

characters  from the ISO Latin-1 character set.  A length

byte that contains zero is followed by no characters  and

denotes  a null or zero length string.  While ISO Latin-1

is the character  set  encoding,  operator  communication

devices  need  only  implement  those characters that are

meaningful for  the  specified  item  and  mode  on  that

device.   Multiple  text strings are concatenated without

any intervening bytes.  Any bytes between the  last  text

string  and  the end of the command message shall contain

zeros, denoting null strings.

If  the  host  supplies fewer than the expected number of

text strings, then the  MSCP  server  shall  append  null

strings  (zero  bytes) to make up the difference.  If the

host supplies more  than  the  expected  number  of  text

strings,  and  the extra strings are null, then the extra

strings shall be ignored.  If any  extra  string  is  not

null,  the server shall ignore the string if the "Must Be

Implemented" modifier is clear or reject the  command  if

that  modifier  is  set.   If the length byte of a string

specifies that the string extends past  the  end  of  the

command  message, the server shall truncate the string at

the end of the command message;  this is  not  considered

an error.

The  following  example  display  text  field  shows each

byte's  value  in  hexadecimal  and  the  equivalent  ISO

Latin-1 character in parentheses.  This example specifies

"VMSRL5" as the first string followed by "Placez" as  the

second  string.   These  are  followed  by null third and

fourth strings;  these two strings would  be  ignored  if

the  item  and  mode  field  values implied that only two

strings were expected.

+-------+-------+-------+-------+

| 73(S) | 4D(M) | 56(V) | 06 |

+-------+-------+-------+-------+

| 06 | 35(5) | 4C(L) | 52(R) |

+-------+-------+-------+-------+

| 63(c) | 61(a) | 6C(l) | 50(P) |

+-------+-------+-------+-------+

| 00 | 00 | 7A(z) | 65(e) |

+-------+-------+-------+-------+

MINIMAL MSCP COMMAND SET                                PAGE 6-66

DISPLAY COMMAND

Allowable modifiers:

Must Be Implemented

If clear, the MSCP server shall ignore and return success

for an unimplemented DISPLAY operation.  If set, the MSCP

server  shall  reject and return the appropriate "Invalid

Command" status for an unimplemented DISPLAY operation.

MINIMAL MSCP COMMAND SET                                PAGE 6-67

DISPLAY COMMAND

6.8.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

Status Codes:

Success (subcode "Normal")

Either the requested DISPLAY operation is implemented and

has been successfully accomplished,  or  it  is  not  im-

plemented  and  the  "Must  Be  Implemented" modifier was

clear.

Invalid Command (subcode "Invalid Opcode")

The  unit  or  controller  does not implement the DISPLAY

command in its entirety.

Invalid Command (subcode "Invalid Item")

The  specified  display  item code is not implemented and

the "Must Be Implemented" modifier was set.

Invalid Command (subcode "Invalid Mode")

The  specified  display  mode  is not implemented for the

specified display item and the "Must Be Implemented" mod-

ifier was set.

Invalid Command (subcodes "Invalid Display Text")

The  specified display text is invalid or not implemented

for the specified display item and mode.  Note that  many

different  subcodes may be returned, as the subcode indi-

cates the location of the invalid display text.  The fol-

lowing conditions return these subcodes:

MINIMAL MSCP COMMAND SET                                PAGE 6-68

DISPLAY COMMAND

A.  The  host supplied a non-null value for an extra

or unimplemented string and  the  "Must  Be  Im-

plemented"  modifier was set.  The subcode indi-

cates the location  of  the  offending  string's

length byte.

B.  The host did not supply a value or it supplied a

null value for a string that requires a non-null

value.   The  subcode  indicates the location of

the offending string's length byte.   Note  that

the subcode may indicate a location past the end

of the command message if the host did not  sup-

ply a value for the string.

C.  The host supplied a value that is invalid due to

being either too long or too short.  The subcode

indicates the location of the offending string's

length byte.

D.  The  host  supplied a value that contains an in-

valid character.  The subcode indicates the  lo-

cation of the offending character.

Command Aborted

Unit-Offline

Unit-Available

The specified display operation is implemented but is in-

appropriate in the unit's current  state.   These  status

codes  shall  not  be  returned for unimplemented display

operations.

Controller Error

Drive Error

Media Loader Error

MINIMAL MSCP COMMAND SET                                PAGE 6-69

DISPLAY COMMAND

6.8.3  Description

A controller may, at its option, treat the DISPLAY command as ei-

ther an Immediate, Sequential, or Non-sequential  command.   Host

command timeout algorithms shall treat it as a non-Immediate com-

mand.

The  DISPLAY command conveys information meant to be communicated

to an operator.  Some devices implement device displays  to  com-

municate with human operators.  A device display provides a means

of displaying  text  and  other  information  associated  with  a

particular  device  or unit.  Other devices may pass the informa-

tion to a robotic operator, such as a media loader.  This  speci-

fication  architects  the  intent  or  meaning of the information

being conveyed.  Whether or not  particular  information  can  be

communicated,  the  means  of  communication,  the  nature of the

operator communicated to, and the result  of  such  communication

are device and operator dependent.

The  conceptual model for use of the DISPLAY command is a simpli-

fied version of the model for forms management  systems  such  as

DECforms.   Information  is  presented in a manner analogous to a

pre-printed form.  A form dictates the arrangement of fields, how

they  appear  or are separated, and can provide fixed explanatory

text.  Forms also contain variable fields whose contents are sup-

plied by an application program.

A  key feature of forms management systems is that the format and

appearance of a form is kept separate from supplying  information

contained in the form.  The application passes information to the

run-time control portion of the forms system,  called  the  Forms

Control  System.   The Forms Control System presents the informa-

tion using a form layout appropriate to  the  particular  display

device.   Form  layouts can be defined for new display devices or

be redefined for existing devices at any time, without any appli-

cation  modifications.   Also, display devices need not be visual

displays:  voice  output,  tactile  output,  or  other  nonvisual

devices might be used.

In  this  forms management system model, the DISPLAY command cor-

responds to the interface between the application and  the  Forms

Control  System.  The host class driver, together with other host

software, acts as the application.  The DISPLAY  command  is  the

means  by  which the host passes information to the Forms Control

System associated with a drive.

The  Forms  Control  System  associated with a drive has complete

control of the appearance of the device display.   It  takes  the

information  provided  by  the  host  via the DISPLAY command and

MINIMAL MSCP COMMAND SET                                PAGE 6-70

DISPLAY COMMAND

presents  it  in  whatever  manner  the  drive  designers  deemed

appropriate.  In effect, each new drive design incorporates a new

display device and the drive designers must define the layout  of

the  MSCP DISPLAY command form on that display device.  Note that

not all information need appear in the layout.  That is, the form

layout for a device might ignore and not display certain informa-

tion that is unimportant for that device.  A device with suitable

hardware capability might act on the information directly, rather

than waiting for a human operator to perform equivalent  actions.

Or the drive designers might prioritize the available information

and only display the most important, perhaps using drive state or

other  internal  information  sources.  Such techniques are typi-

cally used to fit within the constraints of a small physical dis-

play.

Whereas  general  purpose forms management systems typically deal

with many forms, the DISPLAY command only deals with  one.   That

is, there is a single form, a single set of variable fields whose

contents can be specified by the host.  The logical definition of

this  single  form,  its  variable  fields  and their meaning, is

specified  here  as  part  of  the  DISPLAY  command  definition.

Changes to this logical form definition are ECOs to this specifi-

cation.  However, the layout or appearance of the information  is

part  of  individual  device designs;  new or changed layouts for

this information need not be reflected in this specification.

The item field of the DISPLAY command designates a variable field

in the device display form.  Item field values are defined below;

those  definitions specify the meaning, interpretation, or seman-

tic content of the information in that field.  The mode and  dis-

play  text  fields  provide  the information for that field.  The

mode field provides nontextual or state information.  It is typi-

cally  used  with  items  that can be in one of several states or

modes.  The drive designer might choose to present this  informa-

tion  by  displaying a keyword describing the mode, displaying an

icon designating the mode, by changing the  visual  rendition  of

the  field,  or  by  any  other  means.   The  display text field

provides textual information that  might  be  displayed  in  that

field.   The  drive  designer may alter the text or choose to not

display it at all, so long as its meaning is  preserved.   Often,

state  information  is  conveyed  in both the mode field and as a

string in the display text field.

A  unit or controller that does not implement the DISPLAY command

at all shall return "Invalid Command" status with an "Invalid Op-

code"  subcode  for this command.  A controller may implement the

DISPLAY command for some units but not for  others.   A  unit  or

controller  that  does  implement  the DISPLAY command may selec-

tively implement the item and mode field  values  appropriate  to

that  device,  subject to the restrictions on individual item and

MINIMAL MSCP COMMAND SET                                PAGE 6-71

DISPLAY COMMAND

mode combinations described below.  Not implementing an  item  or

mode  field value is equivalent to determining that that informa-

tion is irrelevant to the device and need never be  displayed  or

utilized.  The host can use the "Must Be Implemented" modifier to

indicate that the host considers the information to be important,

so  that  the  item  and mode field value must be implemented and

presented to the operator or else an error returned.

If  the  "Must  Be Implemented" modifier is clear, a unit or con-

troller that implements the DISPLAY command shall ignore  DISPLAY

commands  that  request  unimplemented display operations.  Hosts

will typically use this for advisory  messages,  where  the  host

will continue with the same actions regardless of whether the in-

formation is actually displayed  or  otherwise  utilized  by  the

device.

If  the  "Must  Be  Implemented"  modifier is set, a unit or con-

troller that implements the DISPLAY command shall return "Invalid

Command"  status  for DISPLAY commands that request unimplemented

display operations.  Hosts will typically use this  when  absence

of  a device display or other device specific means of satisfying

the request requires that the host  communicate  the  information

via another means.

All  item and mode field values are defined below.  An ECO adding

new item and mode field values to this list must be approved  be-

fore any device can implement the new values.

Item = 0, Mode = any

Item  0  shall never be implemented by any display device.  Hosts

may use this item field value to determine if the DISPLAY command

is  implemented  with  the  assurance that no display information

will be altered.

MINIMAL MSCP COMMAND SET                                PAGE 6-72

DISPLAY COMMAND

Item = any, Mode = 0

Mode  0  shall  be implemented for every implemented item;  it is

not implemented for unimplemented items.   Mode  0  restores  the

specified  display  item  to  its  default  state  upon  reset or

power-up, typically blank or  empty.   Mode  0  expects  no  text

strings regardless of the item field value.

Item = 1   Volume Identification

Item  1  identifies  the  volume  associated with the unit or the

volume mounted on the unit.  For  all  nonzero  mode  values  the

first  text  string  is  the volume label.  Device displays shall

support at least the number of characters and character set  that

the  applicable  volume format standard allows for volume labels.

If a device display is present, any character not displayable  by

the  device display is invalid.  For most disk devices the appli-

cable standard is Files-11, which in the past has allowed  up  to

12  characters from the upper case alphabet, digits, dollar sign,

underscore, and space.  For  most  tape  devices  the  applicable

standard  is  the ANSI tape label standard, which in the past has

allowed up to 6 characters from the upper case alphabet,  digits,

dollar   sign,  underscore,  space,  and  the  following  special

characters:

! " % ' ( ) * + , - . / : ; < = > ?

Implementors are responsible for verifying that their device sup-

ports the current version of the applicable standard.  They shall

not assume that the above description is current.

Any unit that implements display item 1 shall implement the three

modes 0, 1, and 2 for that display item.  This display item shall

function whenever the unit is "Unit-Online," "Unit-Available," or

"Unit-Offline,  Known." This  display  item  shall  not  function

whenever  the  unit  is "Unit-Offline" for any other reason;  the

DISPLAY command shall be rejected with "Unit-Offline" status  and

the  appropriate  subcode.   A unit may optionally reject mode 2,

Mount  Request  Volume   Identification,   when   the   unit   is

"Unit-Online"  to  any  host.  If the unit chooses to reject this

mode when "Unit-Online", it should treat it as unimplemented  and

return  either  "Success"  status  (subcode "Normal") or "Invalid

Command" status (subcode "Invalid Mode"), depending on the  state

of the "Must Be Implemented" modifier.

MINIMAL MSCP COMMAND SET                                PAGE 6-73

DISPLAY COMMAND

The  unit or device display shall clear its volume identification

information on reset, power-up, or any other loss of context.  It

shall also clear its volume identification information whenever a

volume is inserted or removed from  the  drive.   It  should  not

otherwise  alter  its  volume  identification  information unless

directed to by a host (via this command).

Item = 1, Mode = 0   Clear Volume Identification

Clears  the  volume  identification  information.  Expects no

text strings.

Item = 1, Mode = 1   Current Volume Identification

Conveys  the  volume label currently associated with the unit

or the volume mounted on the unit.  Expects one non-null text

string containing the volume label.

Item = 1, Mode = 2   Mount Request Volume Identification

Identifies  the  volume  that an operator should mount on the

unit.  Typically the drive should highlight or emphasize  the

volume identification to notify the operator of the mount re-

quest.  Units that are associated with a media loader may sa-

tisfy mount requests directly, without passing mount requests

to a human operator.  Expects three text strings.

The  first  text  string  is  either  null  or  the requested

volume's label.  The second text string  is  optional;   this

mode  shall  function regardless of whether the second string

is null or non-null.  If the second text string is  non-null,

then  it  is the command "Mount" expressed in the local lang-

uage.  The entire ISO Latin-1 character set is valid for  the

second  text string;  the device display may ignore this text

string (treat it as if it were null), display it  unmodified,

or  translate  it to a form that can be displayed.  For exam-

ple, a device display might translate the lower case alphabet

to  upper  case.  If a device display can neither display the

second text string nor translate the string to  a  form  that

can  be  displayed,  then it shall ignore the string as if it

were null.

MINIMAL MSCP COMMAND SET                                PAGE 6-74

DISPLAY COMMAND

The  third  text  string  is  either  null  or  the requested

volume's location.  With device displays and human operators,

this would typically use the same character set as the volume

label.  Any character not displayable by the  device  display

is invalid.  The format and meaning of the location codes are

outside the scope of T/MSCP;  they will typically  be  chosen

by  the  individual site or installation.  With robotic media

loaders, the location  will  typically  be  a  decimal  digit

string  encoding  the number of the element within the loader

that contains the requested volume.  If the string  does  not

identify  a  valid  or  potentially  valid element within the

media loader, then some character in it is invalid.

If  both the first and third text strings are null, then this

is a request to mount an unidentified volume.  If  the  first

is  non-null  and  the  third null, then this is a request to

mount a volume with the specified volume label.  If the first

is  null  and  the  third non-null, then this is a request to

mount the volume that normally resides at the specified loca-

tion.   If both are non-null, then this is a request to mount

a volume that both has the specified label and that  normally

resides  at the specified location.  A unit need only use the

information that is consistent with its display  device's  or

media loader's capabilities.

The  means by which a mount request is resolved, resulting in

a specific physical volume being  located  and  mounted,  are

outside  the scope of T/MSCP.  In general, mount requests may

specify volumes that do not exist, ambiguously specify multi-

ple  volumes,  or be self-contradictory.  With a human opera-

tor,  treatment  of  these  conditions  will  depend  on  the

operator's  intelligence  and  local  site-specific policies.

With robotic operators, treatment of  these  conditions  will

depend  on  the  cleverness of the device's designers.  While

not required, it is recommended that  robotic  media  loaders

detect  any  mount  request  that  does  not specify a unique

volume and either treat it as unimplemented  or  return  some

other error.

Note  that the DISPLAY command with this mode and item merely

delivers a mount request.  Completion of the DISPLAY  command

does  not  mean  that the mount request itself can or will be

satisfied.  If the mount request is in fact eventually satis-

fied,  the  unit will become "Unit-Available" when the volume

is mounted and an attention  message  delivered  if  enabled.

However  there  is no guarantee that the correct or requested

volume was indeed mounted, since operators (whether human  or

robotic)  make mistakes.  The host must use non-T/MSCP means,

such as label verification, to ensure that the correct  or  a

suitable  volume has been mounted.  If a mount request cannot

MINIMAL MSCP COMMAND SET                                PAGE 6-75

DISPLAY COMMAND

be satisfied, there is in general no way to report that  with

T/MSCP.   Hosts might use timeouts or similar means to detect

mount request failures.

Notwithstanding  the above, if a device can determine, during

the normal execution of the DISPLAY command, that a mount re-

quest  cannot or will not be satisfied, then it should report

that failure with the DISPLAY command's status code.  This is

most  likely  to  be  possible  with  robotic  media loaders.

However T/MSCP do not preclude such  failure  reporting  with

human operators as well, if any device can implement it.  The

ability to detect mount request failures and  which  failures

are  or  are not reported as DISPLAY command status codes are

all extremely device dependent.

MINIMAL MSCP COMMAND SET                                PAGE 6-76

DISPLAY COMMAND

6.9  ERASE COMMAND

Command Category:

Non-Sequential

6.9.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | caa | opcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- ---+

| reserved |

+--- ---+

| |

+-------------------------------+

| logical block number |

+---------------+---------------+

| entry id | hrn or entloc | (optional)

+---------------+---------------+

hrn (host reference number) or entloc (entry locator)

entry id

Optional fields, used in conjunction with the History Log

and Reuse Entry modifiers.  See the descriptions of those

modifiers below.

Allowable modifiers:

Clear Serious Exception (ignored for disk class devices)

Express Request

MINIMAL MSCP COMMAND SET                                PAGE 6-77

ERASE COMMAND

Force Error

Suppress Error Recovery

History Log

Reuse Entry

The History Log and Reuse Entry modifiers are interdepen-

dent and described together here to avoid confusion.

Note that the History Log modifier, the Reuse Entry modi-

fier, the "hrn or entloc" field, and the "entry id" field

are  valid  only  for servers that provide "write history

logging" support (i.e., the server sets the Write History

Logging   Support  controller  flag;   see  Section  sec-

tion(controlerflags), "Controller Flags" for related  in-

formation).

The  setting  of  the  History  Log  modifier  determines

whether information about this command is to be stored in

a "write history entry."

The  setting  of  the  Reuse  Entry  modifier  determines

whether the server is to allocate a  new  "write  history

entry" or reuse one that was previously allocated.

If  "write  history logging" is supported and the History

Log modifier is set  and  the  Reuse  Entry  modifier  is

clear,  the  server allocates a new "write history entry"

and then stores the value contained  in  the  "hrn  (host

reference number)" field, "entry id" field, and other in-

formation related to this command into the "write history

entry"  just allocated.  If the server has no "write his-

tory entry" to allocate, it  shall  remember  this  fact.

See  Section  section(wrihismdes), "WRITE HISTORY MANAGE-

MENT Command, Description" for more details.   To  report

this  allocation  failure to the host, the server returns

FFFF(hex) in the "entry locator" field of the end message

and  sets  the  Allocation Failure end flag.  See Section

section(end____mess), "End Messages," for  a  description

of this flag.

If  "write  history logging" is supported and the History

Log modifier is set and the Reuse Entry modifier is  set,

the server uses the value contained in the "entloc (entry

locator)" field to locate the associated  "write  history

entry"  and  then stores the other information related to

this command into that "write history entry."

MINIMAL MSCP COMMAND SET                                PAGE 6-78

ERASE COMMAND

See  Section  section(wrihismdes), "WRITE HISTORY MANAGE-

MENT Command,  Description"  for  details  regarding  the

maintenance and format of "write history entries."

Note  that  when "write history logging" is supported and

the History Log modifier is set, host class drivers shall

supply  both  the  "hrn  or  entloc"  and  the "entry id"

fields.  If the History Log modifier  is  set  and  those

fields  are  not supplied, the server rejects the command

as an "Invalid Command" ("invalid  message  length"  pro-

tocol  error.   See  "Invalid  Command" status in Section

section(status__codes), "Status Codes").

If  "write  history logging" is supported and the History

Log modifier is clear, the server ignores the setting  of

the  Reuse Entry modifier and the contents of the "hrn or

entloc" and "entry id" fields if they are supplied.  Note

that host class drivers need not supply those fields when

the History Log modifier is clear.

If  "write history logging" is not supported and the His-

tory Log modifier or the Reuse Entry modifier is set, the

server  rejects the command as an "Invalid Command" ("in-

valid modifier" protocol error.   See  "Invalid  Command"

status   in   Section   section(status__codes),   "Status

Codes").

If  "write history logging" is supported, the History Log

modifier is set, and the "byte count" field is zero,  the

server  rejects  the  command as an Invalid Command ("in-

valid byte count" parameter error.  See "Invalid Command"

status   in   Section   section(status__codes),   "Status

Codes").

The  server shall set the History Logged end flag in this

command's end message if it has caused the write  history

log  to  contain  a  valid "write history entry" for this

command.  See  Section  section(end____mess),  "End  Mes-

sages," for descriptions of all end message flags.

MINIMAL MSCP COMMAND SET                                PAGE 6-79

ERASE COMMAND

6.9.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- ---+

| undefined |

+--- ---+

| |

+-------------------------------+

| first bad block |

+---------------+---------------+

| entry id | entry locator | (conditionally present)

+---------------+---------------+

entry locator

This  field  is  present if and only if the corresponding

command message field is present.   If  present  and  the

History  Log modifier is set in the command message, this

field contains the value  assigned  by  the  server  that

uniquely identifies the internal location of the "history

entry" where this command's information was stored.   If,

however,  an allocation failure occurred, this field con-

tains the value FFFF(hex).  If present  and  the  History

Log  modifier is clear in the command message, this field

is undefined.

This  field  contains  a  valid entry locator of a "write

history entry" for this command if and only if  the  His-

tory  Logged end flag is set in the "flags" field of this

end message.

See  the  description  of the History Log and Reuse Entry

modifiers in Section  section(erase___cmf),  "ERASE  Com-

mand,   Command   Message   Format"   and   Section  sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

MINIMAL MSCP COMMAND SET                                PAGE 6-80

ERASE COMMAND

entry id

This  field  is  present if and only if the corresponding

command message field is present.   If  present  and  the

History  Log  modifier is set in the command message, the

contents of the corresponding command  message  field  is

copied  without  modification  to this field.  If present

and the History Log modifier is clear in the command mes-

sage, this field is undefined.

See  the  description  of the History Log and Reuse Entry

modifies in Section section(erase___cmf), "ERASE Command,

Command  Message Format" and Section section(wrihismdes),

"WRITE HISTORY MANAGEMENT Command, Description" for  more

detail.

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Invalid Command (subcode "Invalid Byte Count")

A  byte  count of zero was specified with the History Log

modifier set.

Invalid Command (subcode "Invalid Entry Locator")

The value specified in the "entloc (entry locator)" field

is not within the limits of the server's  "write  history

entry" set.

See  the  description  of the History Log and Reuse Entry

modifiers in Section  section(erase___cmf),  "ERASE  Com-

mand,   Command   Message   Format"   and   Section  sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

Note  that  this  error  can only occur if "write history

logging" is supported and the History Log and Reuse Entry

modifiers are both set in the command message.

Invalid Command (subcode "Invalid Entry Id")

The  server  located the "write history entry" identified

in the "entloc (entry locator)" field but found that  the

"entry  id"  contained  in  that entry does not equal the

value supplied by a host class driver when that entry was

MINIMAL MSCP COMMAND SET                                PAGE 6-81

ERASE COMMAND

allocated.   This status may also be returned if the His-

tory Log modifier was set, the Reuse Entry  modifier  was

clear, and the host supplied a value of FFFF (hex) in the

"entry id" field.

See  the  description  of the History Log and Reuse Entry

modifiers in Section  section(erase___cmf),  "ERASE  Com-

mand,   Command   Message   Format"   and   Section  sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

Note  that  this  error  can only occur if "write history

logging" is supported and the History Log modifier is set

in the command message.

Invalid Command (subcode "Invalid Logical Block Number")

Command Aborted

Unit-Offline

Unit-Available

Write Protected

Host Buffer Access Error (subcode "Odd Byte Count")

If  the  communications mechanism does not allow odd byte

transfers,  the  controller  may   return   this   status

code/subcode  despite  there  being no host buffer.  This

allows the use of common code for command  validation  of

all transfer commands.  Alternatively, the controller may

round the byte count up to the nearest whole sector  size

and perform the operation.

Controller Error

Drive Error

Write History Entry Access Error (subcode "Allocation Failure

Table Full")

The server was unable to allocate a "write history entry"

and could not record this  fact  because  the  Allocation

Failure  Table  was full.  This is a transient condition;

therefore, the host should reissue the command.  See Sec-

tion  section(wrihismdes), "WRITE HISTORY MANAGEMENT Com-

mand, Description" for more details.

Note  that  this  error  can occur only if "write history

logging" is supported and the History Log modifier is set

and Reuse Entry modifier is clear in the command message.

MINIMAL MSCP COMMAND SET                                PAGE 6-82

ERASE COMMAND

MINIMAL MSCP COMMAND SET                                PAGE 6-83

ERASE COMMAND

6.9.3  Description

All  data  in the specified region of the unit is erased by over-

writing it with zero.

This  command  is exactly equivalent in function, although not in

performance, to a WRITE command which references  a  buffer  that

the host has zeroed.

If a controller-based cache exists, the cache manager may use the

Cache Access Attribute advice in managing the caching and compar-

ing of the requested data area.

MINIMAL MSCP COMMAND SET                                PAGE 6-84

ERASE COMMAND

6.10  FORMAT COMMAND

Command Category:

Sequential

6.10.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- buffer ---+

| |

+--- descriptor ---+

| |

+-------------------------------+

| format function |

+-------------------------------+

byte count

buffer descriptor

If  "byte  count" is non-zero, these two parameters iden-

tify a buffer containing additional  format  information.

If  "byte  count"  is  zero,  then "buffer descriptor" is

reserved.  See description below.

format function

A  code from Table table(frmtfunccodes), "Format Function

Codes," indicating the desired format for the volume.

MINIMAL MSCP COMMAND SET                                PAGE 6-85

FORMAT COMMAND

Allowable modifiers:

Clear Serious Exception (ignored for disk class devices)

Compare                             -- ignored

Express Request                     -- ignored

Suppress Error Correction           -- ignored

Suppress Error Recovery             -- ignored

MINIMAL MSCP COMMAND SET                                PAGE 6-86

FORMAT COMMAND

6.10.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

Status Codes:

Success (subcode "Normal")

Invalid Command

Command Aborted

Unit-Offline

Unit-Available

Media Format Error

Write Protected

Host Buffer Access Error

Controller Error

Drive Error

Invalid Parameter

MINIMAL MSCP COMMAND SET                                PAGE 6-87

FORMAT COMMAND

6.10.3  Description

This  command  formats  or  reformats  the  volume mounted on the

specified unit.  Support for this command is completely optional.

Controllers  that  do  not implement this command, or that do not

implement it for the specified unit, shall reject  it  with  "In-

valid Command -- Invalid Opcode" status.

A  controller may choose to implement this command, a DUP format-

ting program, both, or neither.  All combinations are allowed and

reasonable  for  various  devices.   However, this command is the

preferred means of formatting devices  for  which  formatting  is

considered  a  normal,  user-invoked operation.  Floppy diskettes

are common examples of such devices.

Hosts  shall  use  the  following  command sequence to format the

volume on a unit using this command.  This  command  sequence  is

the only context in which the FORMAT command shall be issued.

A.  The unit should be "Unit-Available" or "Unit-Offline" to

all hosts at the start of the  command  sequence.   That

is, the unit should not be "Unit-Online" to any host.

B.  The  host  brings the unit "Unit-Online" using an ONLINE

command with the "Ignore Media  Format  Error"  modifier

set.   The  host shall exit (not continue the formatting

sequence) if this  command  does  not  succeed.   It  is

recommended,  although  not required, that the host also

set the "Exclusive Access" modifier.

C.  The  host  may  optionally issue any number of READ com-

mands to the unit.  These  commands  might  be  used  to

verify that the correct volume is mounted.  The host may

exit or continue as it sees fit, regardless of the  suc-

cess or failure of these READ commands.  The host should

issue an AVAILABLE command to the unit if it chooses  to

exit.   Note that these READ commands will normally fail

if the volume has not been previously formatted.

D.  The host issues a FORMAT command to the unit.

The   controller   shall   verify   that  the  unit  was  brought

"Unit-Online" by an ONLINE command with the "Ignore Media  Format

Error"   modifier   set.    If  the  unit  is  "Unit-Offline"  or

"Unit-Available" the controller shall reject the  FORMAT  command

MINIMAL MSCP COMMAND SET                                PAGE 6-88

FORMAT COMMAND

with the corresponding status code and leave the unit state unal-

tered.  If the unit is "Unit-Online" but the "Ignore Media Format

Error"  modifier was not specified, then the controller shall re-

ject the FORMAT command with a "Unit-Available" status  code  and

the unit becomes "Unit-Available."

The  controller  shall  return "Invalid Command," "Unit-Offline,"

"Unit-Available,"  "Write  Protected,"  or  "Invalid   Parameter"

status  if  a FORMAT command is rejected.  These status codes all

imply that the volume on the unit has not been altered.  The unit

state  is unaltered, except for the case described in the preced-

ing paragraph.

The controller shall return "Success" status if the format opera-

tion is successful.  The unit is left "Unit-Available."

Any  other status code implies that the volume may have been par-

tially formatted.  The volume must  be  successfully  reformatted

before  it  can  be  used  reliably.   The  unit  may  be  either

"Unit-Available" or "Unit-Offline."

The  FORMAT  command  often  causes  the  unit to transition from

"Unit-Online" to "Unit-Available." This is a synchronous  transi-

tion.   Therefore  the  controller need not generate an AVAILABLE

attention message.

The  FORMAT command neither obtains nor releases Exclusive Access

to the unit.

The actual formatting operation is controlled by the format func-

tion code value and the  additional  format  information  in  the

buffer  described  by  the  "byte  count" and "buffer descriptor"

fields.   Format  function  code  values  are  listed  in   Table

table(frmtfunccodes).   The  format  function determines the use,

structure, and interpretation of any additional  format  informa-

tion.

Most  format  functions do not use additional format information.

The controller shall ignore the "byte count" and "buffer descrip-

tor"  fields,  as  well as any buffer described by them, when the

format function does not use additional format  information.   If

the  format  function does use additional format information, the

controller shall validate the additional format  information  be-

fore  it begins the formatting process.  The controller shall re-

ject the FORMAT command with an "Invalid Parameter"  status  code

if the additional format information is invalid.

Format  function code value zero specifies that a default format-

ting operation should be performed.  A  default  format  function

shall  be  supported  by  all  controller/unit  combinations that

MINIMAL MSCP COMMAND SET                                PAGE 6-89

FORMAT COMMAND

support the FORMAT command.  The exact definition of the  default

format  function  is  device specific.  It does not require addi-

tional format information.

Support  for  non-default  format  functions is device dependent.

Table table(frmtfunccodes) indicates which functions are applica-

ble  to different classes of devices, but the final determination

of supported format functions is device  dependent.   Controllers

shall  reject  FORMAT commands that specify an unsupported format

function  with  "Invalid  Command  --  Invalid  Format  Function"

status.

NOTE

At  the present time there are no format functions that

use additional format information.  The  definition  of

additional format information has been included in this

command for those host operating systems that choose to

implement  support  for it in advance of any device re-

quirements for such support.   Valid  host  implementa-

tions  need  not support additional format information.

Any device that anticipates using additional format in-

formation  must  negotiate for host support, just as if

they were proposing a totally  new  feature.   The  ECO

that  defines the first use of additional format infor-

mation should also remove this note.

MINIMAL MSCP COMMAND SET                                PAGE 6-90

FORMAT COMMAND

6.11  GET COMMAND STATUS COMMAND

Command Category:

Immediate

6.11.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| outstanding reference number |

+-------------------------------+

unit number

Shall  be the same as the "unit number" field in the out-

standing command whose status is to  be  obtained.   This

allows  controllers to optimize their search for the out-

standing command.  If the incorrect unit number  is  sup-

plied, some controllers may erroneously conclude that the

command is no longer outstanding,  leading  to  erroneous

command timeouts.

outstanding reference number

The  command reference number of the command whose status

is to be obtained.

Allowable modifiers:

none

MINIMAL MSCP COMMAND SET                                PAGE 6-91

GET COMMAND STATUS COMMAND

MINIMAL MSCP COMMAND SET                                PAGE 6-92

GET COMMAND STATUS COMMAND

6.11.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| outstanding reference number |

+---------------+---------------+

| command status |

+---------------+---------------+

outstanding reference number

The  command reference number of the command whose status

has been returned.  Identical to the  value  supplied  in

the GET COMMAND STATUS command message.

command status

The  amount  of work remaining to be done to complete the

command, expressed as an unsigned integer.  This field is

zero  if the command is not known to the controller, such

as when the command has already  completed.   This  field

should also be zero if the command has been aborted.  The

controller may also return zero in this field if  it  can

guarantee  that the command will complete within the con-

troller timeout interval.  The controller shall never re-

turn a value of all ones (2**32-1) in this field, as that

value is used  to  initialize  the  command  timeout  al-

gorithm.

The  units  in which this value is measured are arbitrary

and may be controller, device type, and/or command depen-

dent.   However,  the  units  shall remain the same for a

particular command for as long as that  command  is  out-

standing.

Status Codes:

MINIMAL MSCP COMMAND SET                                PAGE 6-93

GET COMMAND STATUS COMMAND

Success (subcode "Normal")

MINIMAL MSCP COMMAND SET                                PAGE 6-94

GET COMMAND STATUS COMMAND

6.11.3  Description

The GET COMMAND STATUS command is used to monitor the progress of

a command towards completion.  The command  status  measures  the

"doneness"  of  the  command.   The value returned in the command

status field is guaranteed to not increase over  time.   Further-

more,  the  command status of an MSCP server's oldest outstanding

command is guaranteed to decrease within the  controller  timeout

interval.   This  last feature may be used by a host class driver

to detect an insane or malfunctioning  controller.   See  Section

section(command_timos), "Command Timeouts," for more details.

The  GET  COMMAND STATUS command always succeeds.  If the command

referenced by the "outstanding reference number" is not known  to

the  MSCP server or has been aborted, then the MSCP server should

return zero for its command status.  The MSCP server may also re-

turn  zero  as the command status of any command that will always

complete within the controller timeout interval.  The MSCP server

always returns the "Normal" status code in the GET COMMAND STATUS

command's end message.

MINIMAL MSCP COMMAND SET                                PAGE 6-95

GET COMMAND STATUS COMMAND

6.12  GET UNIT STATUS COMMAND

Command Category:

Immediate

6.12.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

Allowable modifiers:

Clear Serious Exception (ignored for disk class devices)

Next Unit

Requests  that  the  controller  return the status of the

next unit (in order of ascending unit numbers)  that  the

controller  knows  to  exist  and  whose  unit  number is

greater than or equal to the unit number specified in the

command.    See   Section   section(unit__states),  "Unit

States," for a detailed definition  of  which  units  are

acknowledged by this modifier.

If  this  modifier is set, the "unit number" field in the

end message corresponds to the unit whose characteristics

are  being returned, and is typically not the same as the

"unit number" field in the command message.  If there are

no  units that are both known to the controller and whose

unit numbers are greater than or equal to the unit number

specified  in  the command message, then zero is returned

in the "unit number"  field  of  the  end  message.   The

remaining  fields of the end message are identical to the

values that would be returned for a GET UNIT STATUS  com-

mand  with  a  "unit  number" of zero and the "Next Unit"

modifier left clear.

MINIMAL MSCP COMMAND SET                                PAGE 6-96

GET UNIT STATUS COMMAND

MINIMAL MSCP COMMAND SET                                PAGE 6-97

GET UNIT STATUS COMMAND

6.12.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| unit flags |multiunit code |

+---------------+---------------+

| reserved |spndles|

+-------------------------------+

| |

+--- unit identifier ---+

| |

+-------------------------------+

| media type identifier |

+-------------------------------+

| reserved |

+-------------------------------+

| group size | track size |

+-------+-------+---------------+

|uhvrsn | usvrsn| cylinder size |

+-------+-------+---------------+

|copies | RBNs | RCT size |

+-------+-------+---------------+

status

The  validity of the unit characteristics varies with the

unit's state implied by the contents of this field.   See

the Status Codes listed later in this section and Section

section(getunitstdes), "GET UNIT STATUS Command, Descrip-

tion" for complete details.

multiunit code

The  low  byte  of  this field identifies the access path

between the controller and the unit.  The  high  byte  of

this  field  identifies  the  spindle,  within the access

path, to  which  the  unit  belongs.   See  Section  sec-

tion(multidrvsfmts),  "Multiunit  Drives and Formatters,"

MINIMAL MSCP COMMAND SET                                PAGE 6-98

GET UNIT STATUS COMMAND

for more information.

unit flags

See Section section(unit___flags), "Unit Flags."

spndles

The  number  minus one of spindles or spindle "groups" in

the unit.  The value of zero indicates one spindle.   See

Section   section(diskgeomformt),   "Disk   Geometry  and

Format."

IMPLEMENTATION NOTE

The "spndles" field was defined in February 1991.  Before

that it was a reserved field,  but  not  all  controllers

zeroed  it.   To  ensure validity of the "spndles" field,

class drivers should send  GET UNIT STATUS  command  mes-

sages of at least 20 bytes and zero all bytes beyond byte

12.

unit identifier

Uniquely identifies the unit among all devices accessible

via  MSCP.   See  Section  section(cntrlrunitids),  "Con-

troller and Unit Identifiers."

media type identifier

Identifies  the  type of media used by this unit, for use

by host generic device allocation mechanisms.   See  Sec-

tion section(mediatype_ids), "Media Type Identifiers."

track size

The  number  of  logical blocks in a track.  The value of

zero is reserved and illegal.  See  Section  section(dis-

kgeomformt), "Disk Geometry and Format."

MINIMAL MSCP COMMAND SET                                PAGE 6-99

GET UNIT STATUS COMMAND

group size

The  number  of  tracks in a group.  The value of zero is

reserved  and  illegal.   See  Section  section(diskgeom-

formt), "Disk Geometry and Format."

cylinder size

The number of groups in a cylinder.  The value of zero is

reserved  and  illegal.   See  Section  section(diskgeom-

formt), "Disk Geometry and Format."

usvrsn

The  unit's  software,  firmware,  or  microcode revision

number.  Always zero if  the  product's  development  and

maintainability groups determine that there is no benefit

from supporting machine readable revision numbers.

uhvrsn

The  unit's hardware revision number.  Always zero if the

product's development and maintainability  groups  deter-

mine  that  there  is  no benefit from supporting machine

readable revision numbers.

RCT size

The difference between the starting logical block numbers

of successive copies of the  unit's  Replacement  Control

Table (RCT).  Excepting only the last copy, this value is

also the size of each  copy  of  the  RCT.   See  Digital

Storage  Architecture  Disk  Format Specification (DSDF).

Zero if this disk has no  RCT  whatsoever;   see  Section

section(diskgeomformt), "Disk Geometry and Format."

RBNs

MINIMAL MSCP COMMAND SET                               PAGE 6-100

GET UNIT STATUS COMMAND

The number of replacement blocks per track.

See  Digital  Storage Architecture Disk Format Specifica-

tion (DSDF).  Zero if this disk has  no  RCT  whatsoever;

see  Section  section(diskgeomformt),  "Disk Geometry and

Format."

copies

The  number  of  copies  of the Replacement Control Table

that are stored on the unit.  See Digital Storage  Archi-

tecture  Disk  Format Specification (DSDF).  Zero if this

disk has no RCT  whatsoever;   see  Section  section(dis-

kgeomformt), "Disk Geometry and Format."

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Both   of   these   codes   imply   that   the   unit  is

"Unit-Online."

Unit-Offline

Unit-Available

Controller Error

Drive Error

For  both of these status codes the actual drive state is

unknown (although it will usually be  "Unit-Offline"  due

to being inoperative).  Therefore the class driver should

assume that the unit is "Unit-Offline."

MINIMAL MSCP COMMAND SET                               PAGE 6-101

GET UNIT STATUS COMMAND

6.12.3  Description

The  GET  UNIT STATUS command returns the current state of a unit

plus certain unit characteristics.  In particular, the  GET  UNIT

STATUS  command  is  used to obtain host-settable characteristics

and those fixed unit characteristics that are not normally needed

by the class driver.

Class drivers can determine which of the returned unit character-

istics are valid from the unit  state  implied  by  the  returned

"status" (see Status Codes listed above) as follows:

A.  Unit  is  "Unit-Offline" and unknown to controller.  All

characteristic fields and all unit flags are undefined.

B.  Unit is "Unit-Offline" and known to the controller or it

is "Unit-Available." The "multiunit code," "unit identi-

fier,"  "media type identifier," "usvrsn," "uhvrsn," and

"spndles" characteristic fields are valid.  The  "Remov-

able  Media" unit flag is valid.  All other characteris-

tic fields and unit flags are undefined.

C.  Unit  is  "Unit-Online." All  characteristics fields and

all unit flags are valid.

MINIMAL MSCP COMMAND SET                               PAGE 6-102

GET UNIT STATUS COMMAND

6.13  ONLINE COMMAND

Command categories:

Sequential

6.13.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| unit flags | reserved |

+---------------+---------------+

| reserved |

+-------------------------------+

| |

+--- reserved ---+

| |

+-------------------------------+

| device dependent parameters |

+-------------------------------+

| reserved |

+-------------------------------+

unit flags

Host-settable      unit      flags.       See     Section

section(unit___flags), "Unit Flags."

If the unit is already "Unit-Online" to the class driver,

then the class driver shall specify the same  values  for

controller supported host-settable unit flags as are cur-

rently in effect on the unit.  The controller may, at its

option,  check  that the flag values are the same.  If it

does check, then  it  shall  reject  the  command  as  an

MINIMAL MSCP COMMAND SET                               PAGE 6-103

ONLINE COMMAND

"Invalid  Command" ("invalid unit flags" parameter error.

See    "Invalid    Command"     status     in     Section

section(status__codes)),  if  they are different.  If the

controller does not check that they are the same, then it

shall ignore the unit flags specified by the class driver

and preserve the flag settings currently in effect on the

unit.

device dependent parameters

Device  and/or controller dependent device tuning parame-

ters.  The value zero in this field means that default or

normal  tuning parameters should be used.  Nonzero values

for this field should normally be established through the

system startup command file.  Examples of the use of this

field  include  selecting  alternative  optimization  al-

gorithms  or  enabling  and  disabling automatic (online)

diagnosis of the unit.

Allowable modifiers:

Allow Self Destruction

Some controllers and/or drives are able to predict that a

unit is in danger of imminent self destruction,  and  au-

tomatically spin-down and disable the unit to prevent its

destruction.  Such mechanisms typically sense an exponen-

tially  increasing  (correctable)  error rate, indicating

that the disk surface has been contaminated with dust  or

other foreign objects.  Units that have been disabled for

this reason appear to be "Unit-Offline," with  a  subcode

indicating  that they have been disabled by field service

or a diagnostic.  Therefore such a unit  cannot  normally

be brought "Unit-Online."

This modifier allows a host to bring a unit that has been

so disabled "Unit-Online," even though  the  consequences

for the unit may be fatal.  For this reason THIS MODIFIER

SHALL NEVER  BE  USED  UNLESS  FIELD  SERVICE  EXPLICITLY

DIRECTS  A SITE TO DO SO.  When imminent self destruction

has been predicted for a unit, it is usually possible  to

make  one  "last  ditch"  copy of the unit before it dies

completely, recovering all or most of  the  data  on  the

unit.  This modifier exists primarily to simplify support

of such a "last ditch" copy.  This modifier also provides

MINIMAL MSCP COMMAND SET                               PAGE 6-104

ONLINE COMMAND

a  means,  if necessary, to work around a diagnostic that

is erroneously disabling a unit.

This modifier shall be supported by:

A.  Any  controller  that may disable a unit for the

reason mentioned above.

B.  Any controller that may potentially be connected

to a unit that can disable itself.

C.  Any  controller  that does not disable units it-

self, but that will respond properly if a  drive

is  disabled  by  some other (more capable) con-

troller.

This  modifier  shall be ignored if the unit has not been

disabled or if the controller does not fall into  any  of

the above categories.

Clear Serious Exception (ignored for disk class devices)

Ignore Media Format Error

Suppresses  most  checking  for "Media Format Errors" and

causes  the  "576  Byte  Sectors"   unit   flag   to   be

host-settable.

If  either  the  controller  or the unit only support 512

byte  sectors,  then   setting   this   modifier   merely

suppresses  Media  Format  Error  checking.   The unit is

brought "Unit-Online" as a 512 byte formatted volume  and

the  "576  Byte Sectors" unit flag is returned clear.  If

both the controller and the unit support  576  byte  sec-

tors,  then the state of the "576 Byte Sectors" unit flag

(in the unit flags field of this ONLINE  command)  speci-

fies whether the volume is formatted with 512 or 576 byte

sectors.  The unit is brought "Unit-Online" in the speci-

fied format, rather than the format being determined from

the volume itself.

MINIMAL MSCP COMMAND SET                               PAGE 6-105

ONLINE COMMAND

Use of this modifier allows the host to set a unit to the

wrong block or sector size.  Reading a unit that  is  set

to  the wrong block or sector size may yield a mix of er-

roneous "Data Errors," "Drive  Errors,"  and  "Controller

Errors." Writing a unit that is set to the wrong block or

sector size may  permanently  corrupt  the  volume.   The

volume must be reformatted if this occurs.

Enable Set Write Protect

Causes the "Write Protect" unit flag to be host-settable.

This modifier causes the  state  of  the  "Write  Protect

(software)"  unit flag to be copied to the Software Write

Protect   flag    for    this    unit.     See    Section

section(write_protect), "Write Protection."

MINIMAL MSCP COMMAND SET                               PAGE 6-106

ONLINE COMMAND

6.13.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| unit flags |multiunit code |

+---------------+---------------+

| reserved |spndles|

+-------------------------------+

| |

+--- unit identifier ---+

| |

+-------------------------------+

| media type identifier |

+-------------------------------+

| reserved |

+-------------------------------+

| unit size |

+-------------------------------+

| volume serial number |

+-------------------------------+

status

The  validity of the unit characteristics varies with the

unit's state implied by the contents of this field.   See

Status    Codes    listed    below   and   Section   sec-

tion(setunitchdes), "SET  UNIT  CHARACTERISTICS  Command,

Description" for complete details.

All  other  fields  of  the  ONLINE command's end message are

identical to the SET UNIT CHARACTERISTICS command's end  mes-

sage.   Refer  to  Section  section(setunitchemf),  "SET UNIT

CHARACTERISTICS Command, End Message Format" for  a  descrip-

tion of the fields.

Status Codes:

Success (subcode "Normal")

MINIMAL MSCP COMMAND SET                               PAGE 6-107

ONLINE COMMAND

Implies that the unit is "Unit-Online."

Success (subcode "Already Online")

The  "Already Online" subcode bit flag is set if and only

if the unit is already "Unit-Online"  to  the  requesting

class  driver.   The unit's state and characteristics are

not altered.  When the unit is already  "Unit-Online"  to

the  requesting  class  driver, the controller merely re-

turns the unit characteristics in the  end  message  with

this  status  bit  flag set, without performing any other

actions.

Invalid Command (subcode "Invalid Unit Flags")

The  unit  is  already  "Unit-Online" and, for those unit

flags that are both host-settable and  supported  by  the

controller,  the  class  driver  has  specified different

values from those currently in effect on the  unit.   The

unit  remains "Unit-Online." The host-settable unit flags

are not changed, and the end  message  returns  all  unit

flags.   Controllers  may, at their option, omit checking

that the unit flags are the  same  and  just  ignore  the

class  driver  specified  unit  flags  for units that are

already "Unit-Online," thus returning a "Success"  status

code with "Already Online" subcode.

Command Aborted

The  command  has  been rejected and the unit's state has

not been changed.  If the unit was "Unit-Available" prior

to    issuing    this   command,   then   it   is   still

"Unit-Available." However, as the  unit's  state  is  not

reported to the host, the host should assume that the re-

turned unit characteristics are invalid as the  unit  may

be "Unit-Offline."

Unit-Offline

Note  that some causes of a unit being "Unit-Offline" may

be overridden (suppressed) by the  "Allow  Self  Destruc-

tion" command modifier.

MINIMAL MSCP COMMAND SET                               PAGE 6-108

ONLINE COMMAND

Media Format Error

The unit is and remains "Unit-Available." However, atten-

tion messages are suppressed for this unit and  the  con-

troller  attempts to spin-down this unit exactly as if an

AVAILABLE command with the "Spin-down" modifier set  were

issued.    Note,   however,   that  this  error  will  be

suppressed and the unit brought "Unit-Online"  anyway  if

the  "Ignore Media Format Error" command modifier is set.

See the modifier description above.

Controller Error

The  actual  drive state is unknown.  Therefore the class

driver should assume that the unit is "Unit-Offline."

Drive Error

The unit is "Unit-Offline" due to being inoperative.  The

controller shall suppress  AVAILABLE  attention  messages

for  the  unit and attempt to spin-down the unit, exactly

as if an AVAILABLE command with the "Spin-down"  modifier

set  were  issued  for the unit.  The controller may sub-

sequently report the unit as being either  "Unit-Offline"

or  "Unit-Available." The reported unit state will depend

upon exactly how the unit is "broken," and  the  interac-

tions  of the failure with the controller's perception of

the unit's state.

MINIMAL MSCP COMMAND SET                               PAGE 6-109

ONLINE COMMAND

6.13.3  Description

The  ONLINE  command  is  used to bring a unit "Unit-Online," set

host-settable unit characteristics, and obtain those unit charac-

teristics  that  are  essential for proper class driver operation

(see Section section(unit___flags), "Unit Flags"  for  additional

information).   The  unit is spun-up, if necessary, and its heads

are loaded prior to returning the ONLINE command's  end  message.

Host-settable  characteristics  are  set exactly as if a SET UNIT

CHARACTERISTICS   command   were   issued   (see   Section   sec-

tion(setunitchemf),    "SET   UNIT   CHARACTERISTICS   Command").

Host-settable characteristics are set after  the  unit  has  been

successfully  spun-up  and  any  other  validity checks have suc-

ceeded.  Note that the unit's host-settable  characteristics  are

NOT altered if the unit is already "Unit-Online."

For details of class driver responsibilities with respect to Host

Initiated Bad Block Replacement, see the Digital  Storage  Archi-

tecture  Disk  Format  (DSDF) Specification section on "Host Ini-

tiated Bad Block Replacement."

MINIMAL MSCP COMMAND SET                               PAGE 6-110

ONLINE COMMAND

6.14  READ COMMAND

Command Category:

Non-Sequential

6.14.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | caa | opcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- buffer ---+

| |

+--- descriptor ---+

| |

+-------------------------------+

| logical block number |

+-------------------------------+

Allowable modifiers:

Clear Serious Exception (ignored for disk class devices)

Compare

Express Request

Suppress Error Correction

Suppress Error Recovery

MINIMAL MSCP COMMAND SET                               PAGE 6-111

READ COMMAND

6.14.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- ---+

| undefined |

+--- ---+

| |

+-------------------------------+

| first bad block |

+-------------------------------+

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Invalid Command (subcode "Invalid Byte Count")

Invalid Command (subcode "Invalid Logical Block Number")

Command Aborted

Unit-Offline

Unit-Available

Compare Error

Data Error

Host Buffer Access Error

Controller Error

Drive Error

MINIMAL MSCP COMMAND SET                               PAGE 6-112

READ COMMAND

6.14.3  Description

Data  is  read  from the unit and transferred to the host buffer.

For host buffer access details  see  Section  section(hostbuffac-

ces), "Host Buffer Access."

If a controller-based cache exists, the cache manager may use the

Cache Access Attribute advice in managing the caching and compar-

ing of the requested data area.

MINIMAL MSCP COMMAND SET                               PAGE 6-113

READ COMMAND

6.15  REPLACE COMMAND

Command Category:

Non-Sequential

6.15.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| replacement block number |

+-------------------------------+

| |

+--- ---+

| reserved |

+--- ---+

| |

+-------------------------------+

| logical block number |

+-------------------------------+

replacement block number

Identifies  the replacement block that has been allocated

to replace the bad logical block.

logical block number

Identifies the bad logical block that is being replaced.

Allowable modifiers:

MINIMAL MSCP COMMAND SET                               PAGE 6-114

REPLACE COMMAND

Clear Serious Exception (ignored for disk class devices)

Express Request

Primary Replacement Block

Shall  be  set  if  and  only  if  the "replacement block

number" specifies the primary replacement block for "log-

ical  block number." I.e., set if and only if the follow-

ing expression is true:

replacement block number =

logical block number / track size * RBNs

where  "track  size"  and "RBNs" are unit characteristics

obtained via the GET UNIT CHARACTERISTICS command and "/"

denotes   integer  (truncating)  division.   See  Digital

Storage Architecture Disk Format Specification (DSDF) for

more  information.   Note that this modifier is redundant

information provided for  the  convenience  of  the  con-

troller.

MINIMAL MSCP COMMAND SET                               PAGE 6-115

REPLACE COMMAND

6.15.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Invalid Command (subcode "Invalid Replacement Block Number")

Replacement  block  number is outside the range supported

by this disk.

Invalid Command (subcode "Invalid Logical Block Number")

Command Aborted

Unit-Offline

Unit-Available

Write Protected

Controller Error

Drive Error

MINIMAL MSCP COMMAND SET                               PAGE 6-116

REPLACE COMMAND

6.15.3  Description

The  specified  logical  block is flagged to indicate that it has

been replaced with the specified replacement block.  The volume's

Replacement  Control Table (RCT) shall have been updated prior to

using this command, and the replacement block should be  initial-

ized  with a write command to the same logical block number after

using this command.   See  Section  section(bbr__overview),  "Bad

Block  Replacement," and Digital Storage Architecture Disk Format

Specification (DSDF) for more information on the use and function

of this command.

MINIMAL MSCP COMMAND SET                               PAGE 6-117

REPLACE COMMAND

6.16  SET CONTROLLER CHARACTERISTICS COMMAND

Command Category:

Immediate

6.16.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | reserved |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| cntrlr.  flags | MSCP version |

+---------------+---------------+

| reserved | host timeout |

+---------------+---------------+

| quad-word |

+--- ---+

| time and date |

+-------------------------------+

|controller dependent parameters|

+---------------+---------------+

| crn | (optional)

+---------------+

MSCP version

Host  class  drivers  shall supply the value zero in this

field.  MSCP servers shall verify this value and,  if  it

is  not  zero, reject the command as an "Invalid Command"

("invalid MSCP version"  protocol  error.   See  "Invalid

Command" status in Section section(status__codes)).  This

value will be incremented if MSCP is ever modified  in  a

way that is not upwards compatible.

cntrlr. flags

MINIMAL MSCP COMMAND SET                               PAGE 6-118

SET CONTROLLER CHARACTERISTICS COMMAND

Host-settable controller flags.  See Section section(con-

trolerflags), "Controller Flags."

host timeout

The  time interval that the controller should use for the

host access timeout with this host, or zero if  the  con-

troller  should  disable the host access timeout for this

host.  Expressed as an unsigned binary integer  in  units

of seconds.  Controllers should use a default host access

timeout of 60 seconds if they have  not  received  a  SET

CONTROLLER   CHARACTERISTICS   command   since   becoming

"Controller-Online."

Even  though this is a sixteen bit field, controllers may

treat all values greater than 255  as  if  255  had  been

specified,  and  all  values between 1 and 9 as if 10 had

been  specified.   See  Section   section(hostaccestimo),

"Host Access Timeouts."

quad-word time and date

The  current  time  and  date, expressed as the number of

clunks since 00:00 o'clock, November 17, 1858 (in the lo-

cal  time  zone), or zero if the current time and date is

not available.  A clunk is 100 nanoseconds.  This is  the

standard  VAX/VMS  time and date format.  The use that is

made of the current time and date and the action taken if

it  is  not  supplied (i.e., if zero is supplied) is con-

troller  dependent,  and  should  be  described  in  each

controller's Functional Specification.  Controllers shall

not require that a time and date be supplied  for  proper

operation.

controller dependent parameters

Controller  dependent  tuning parameters.  The value zero

in this field means that default or normal tuning parame-

ters  should  be  used.   Nonzero  values  for this field

should normally be established through the system startup

command file.

MINIMAL MSCP COMMAND SET                               PAGE 6-119

SET CONTROLLER CHARACTERISTICS COMMAND

crn (connection reference number)

The  nonzero  value  the  server is to associate with the

"Controller-Online" connection over which this command is

received   for  the  purpose  of  terminating  the  class

driver/server connection a host has established with  the

server.   See Section section(termcdsconmcmd), "Multihost

TERMINATE CLASS  DRIVER/SERVER  CONNECTION  Command"  for

more information regarding the use of this field for that

purpose.

This  field  is valid only for servers that provide Write

History Logging Support or servers that support just  the

TERMINATE  CLASS  DRIVER/SERVER  CONNECTION command.  See

Section section(controlerflags), "Controller  Flags"  for

related information.

If  a  server  provides  such  support, this field is op-

tional.  A host class driver need only supply this  field

if  the  TERMINATE CLASS DRIVER/SERVER CONNECTION command

is to be used.  If a value of zero is  supplied  in  this

field, the server rejects the command as an "Invalid Com-

mand" ("invalid crn" parameter error).

If  the  Connection  Reference Number Present modifier is

set and the "crn" field is not supplied, the  server  re-

jects the command as an Invalid Command (subcode "Invalid

Message   Length")   protocol    error    (see    Section

section(status__codes), "Status Codes").

Allowable modifiers:

Connection Reference Number Present

Shall  be  set  if  and only if the host provides the op-

tional "crn (connection reference number)"  command  mes-

sage field.  See the description of that field above.

MINIMAL MSCP COMMAND SET                               PAGE 6-120

SET CONTROLLER CHARACTERISTICS COMMAND

6.16.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| reserved |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| cntrlr.  flags | MSCP version |

+-------+-------+---------------+

|chvrsn | csvrsn|cntrlr.  timeout|

+-------+-------+---------------+

| |

+--- controller identifier ---+

| |

+-------------------------------+

| maximum byte count (optional) |

+-------------------------------+

cntrlr.  flags

See Section section(controlerflags), "Controller Flags."

cntrlr.  timeout

The  controller  timeout interval;  the minimum amount of

time that the controller needs to guarantee it  will  ac-

complish  useful  work on its oldest outstanding command.

Expressed as an  unsigned  binary  integer  in  units  of

seconds.   This value may not exceed 255 (one byte), even

though a sixteen bit field has been provided.   See  Sec-

tion section(command_timos), "Command Timeouts."

csvrsn

The  controller's  software, firmware, or microcode revi-

sion number.  Always zero if  the  product's  development

and  maintainability  groups  determine  that there is no

benefit  from  supporting   machine   readable   revision

MINIMAL MSCP COMMAND SET                               PAGE 6-121

SET CONTROLLER CHARACTERISTICS COMMAND

numbers.

chvrsn

The  controller's  hardware revision number.  Always zero

if the product's development and  maintainability  groups

determine  that  there  is  no  benefit  from  supporting

machine readable revision numbers.

controller identifier

Uniquely  identifies the controller among all devices ac-

cessible via MSCP.  See  Section  section(cntrlrunitids),

"Controller and Unit Identifiers."

maximum byte count (optional)

The  maximum  byte  count  value supported by the server.

Class drivers should not  issue  transfer  commands  with

byte counts larger than this value;  byte counts equal to

this value are supported.

This value shall be at least as large as the following:

o  For   disk  class  devices,  16,777,216  (2**24)

bytes.

o  For tape class devices, 65535 (2**16-1) bytes.

That  is, all servers shall support transfers at least as

large as the above values.  Support for longer  transfers

is optional.

This  field is optional.  Class drivers determine whether

or not this field is  included  in  the  end  message  by

checking  the  message  length supplied by the communica-

tions mechanism.  If this field is not  supplied,  or  if

its  contents  are  zero,  then  the server supports byte

counts up to the values shown in the previous paragraph.

MINIMAL MSCP COMMAND SET                               PAGE 6-122

SET CONTROLLER CHARACTERISTICS COMMAND

Status Codes:

Success (subcode "Normal")

MINIMAL MSCP COMMAND SET                               PAGE 6-123

SET CONTROLLER CHARACTERISTICS COMMAND

6.16.3  Description

The SET CONTROLLER CHARACTERISTICS command is used to set and ob-

tain  controller  characteristics.    The   default   value   for

"cntrlr. flags" is all flags clear (i.e., all messages disabled).

The default value  for  "host  timeout"  is  60  seconds.   These

default values are used from the time that the controller becomes

"Controller-Online"   to   a   host   until   it   stops    being

"Controller-Online"  or  until  the  host issues a SET CONTROLLER

CHARACTERISTICS command.

MINIMAL MSCP COMMAND SET                               PAGE 6-124

SET CONTROLLER CHARACTERISTICS COMMAND

6.17  SET UNIT CHARACTERISTICS COMMAND

Command Category:

Sequential

6.17.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| unit flags | reserved |

+---------------+---------------+

| reserved |

+-------------------------------+

| |

+--- reserved ---+

| |

+-------------------------------+

| device dependent parameters |

+-------------------------------+

| reserved |

+-------------------------------+

unit flags

Host-settable      unit      flags.       See     Section

section(unit___flags), "Unit Flags."

device dependent parameters

Device  and/or controller dependent device tuning parame-

ters.  The value zero in this field means that default or

normal  tuning parameters should be used.  Nonzero values

for this field should normally be established through the

system startup command file.  Examples of the use of this

field  include  selecting  alternative  optimization  al-

gorithms  or  enabling  and  disabling automatic (online)

MINIMAL MSCP COMMAND SET                               PAGE 6-125

SET UNIT CHARACTERISTICS COMMAND

diagnosis of the unit.

Allowable modifiers:

Clear Serious Exception (ignored for disk class devices)

Enable Set Write Protect

Causes the "Write Protect" unit flag to be host-settable.

This modifier causes the  state  of  the  "Write  Protect

(software)"  unit flag to be copied to the Software Write

Protect   flag    for    this    unit.     See    Section

section(write_protect), "Write Protection."

MINIMAL MSCP COMMAND SET                               PAGE 6-126

SET UNIT CHARACTERISTICS COMMAND

6.17.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| unit flags |multiunit code |

+---------------+---------------+

| reserved |spndles|

+-------------------------------+

| |

+--- unit identifier ---+

| |

+-------------------------------+

| media type identifier |

+-------------------------------+

| reserved |

+-------------------------------+

| unit size |

+-------------------------------+

| volume serial number |

+-------------------------------+

status

The  validity of the unit characteristics varies with the

unit's state implied by the contents of this field.   See

Status  Codes  listed  later  in this section and Section

section(setunitchdes), "SET UNIT CHARACTERISTICS Command,

Description" for complete details.

multiunit code

The  low  byte  of  this field identifies the access path

between the controller and the unit.  The  high  byte  of

this  field  identifies  the  spindle,  within the access

path, to  which  the  unit  belongs.   See  Section  sec-

tion(multidrvsfmts),  "Multiunit  Drives and Formatters,"

for more information.

MINIMAL MSCP COMMAND SET                               PAGE 6-127

SET UNIT CHARACTERISTICS COMMAND

unit flags

See Section section(unit___flags), "Unit Flags."

spndles

The  number  minus one of spindles or spindle "groups" in

the unit.  The value of zero indicates one spindle.   See

Section   section(diskgeomformt),   "Disk   Geometry  and

Format."

unit identifier

Uniquely identifies the unit among all devices accessible

via  MSCP.   See  Section  section(cntrlrunitids),  "Con-

troller and Unit Identifiers."

media type identifier

Identifies  the  type of media used by this unit, for use

by host generic device allocation mechanisms.   See  Sec-

tion section(mediatype_ids), "Media Type Identifiers."

unit size

The  number  of  logical  blocks in the host area of this

unit.  This value does  NOT  include  the  logical  block

range  occupied  by  the unit's Replacement Control Table

(RCT).  The logical block number of the  first  block  of

the unit's RCT is equal to this value.

volume serial number

The  low order 32 bits of the serial number of the volume

that is mounted on this unit.  Zero if  the  volume  does

not  have a serial number.  When displayed in human read-

able form, this number should be formatted as a ten digit

decimal  number with leading zeros printed.  Undefined if

the unit is "Unit-Offline" or "Unit-Available," or if the

"Ignore  Media  Format Error" modifier was set in the ON-

LINE command that first brought this unit "Unit-Online."

MINIMAL MSCP COMMAND SET                               PAGE 6-128

SET UNIT CHARACTERISTICS COMMAND

Hosts  may  not  assume  that  the value returned in this

field uniquely  identifies  the  volume.   Although  this

value  often  will  be  unique,  the  uniqueness  is  not

guaranteed, especially across media obtained from several

independent suppliers.  Also, media that must meet exter-

nal (industry compatible) format standards will typically

be  unable  to  implement  a  volume serial number.  This

field will always be zero for such media.

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Imply that the unit is "Unit-Online."

Command Aborted

The  command  has  been rejected and the unit's state and

context have not been changed.  That is, the unit's  unit

flags  and  device  dependent  parameters  are unchanged.

Since the unit's state is not reported to the  host,  the

host should assume that the returned unit characteristics

are invalid as the unit may be "Unit-Offline."

Unit-Offline

Unit-Available

Controller Error

Drive Error

For  both of these status codes the actual drive state is

unknown (although it will usually be  "Unit-Offline"  due

to being inoperative).  Therefore the class driver should

assume that the unit is "Unit-Offline."

MINIMAL MSCP COMMAND SET                               PAGE 6-129

SET UNIT CHARACTERISTICS COMMAND

6.17.3  Description

The SET UNIT CHARACTERISTICS command is used to set host-settable

unit characteristics and obtain those unit  characteristics  that

are  essential  for  proper class driver operation.  This command

never alters the unit's state  ("Unit-Online,"  "Unit-Available,"

"Unit-Offline").   It is meaningless to set host-settable charac-

teristics for a unit that is "Unit-Available" or "Unit-Offline."

Class drivers can determine which of the returned unit character-

istics are valid from the unit  state  implied  by  the  returned

"status" (see Status Codes listed above) as follows:

A.  Unit  is  "Unit-Offline" and unknown to controller.  All

characteristic fields and all unit flags are undefined.

B.  Unit is "Unit-Offline" and known to the controller or it

is "Unit-Available." The "multiunit code," "unit identi-

fier," and "media type identifier" characteristic fields

are valid.  The "Removable Media" unit  flag  is  valid.

All other characteristic fields and unit flags are unde-

fined.

C.  Unit  is  "Unit-Online".  All characteristics fields and

all unit flags are valid.

Note  that  the  format of the SET UNIT CHARACTERISTICS command's

end message is identical to that of the ONLINE command's end mes-

sage  because the ONLINE command performs a SET UNIT CHARACTERIS-

TICS operation after bringing a unit "Unit-Online."

MINIMAL MSCP COMMAND SET                               PAGE 6-130

SET UNIT CHARACTERISTICS COMMAND

6.18  WRITE COMMAND

Command Category:

Non-Sequential

6.18.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | caa | opcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- buffer ---+

| |

+--- descriptor ---+

| |

+-------------------------------+

| logical block number |

+---------------+---------------+

| entry id | hrn or entloc | (optional)

+---------------+---------------+

hrn (host reference number) or entloc (entry locator)

entry id

Optional fields, used in conjunction with the History Log

and Reuse Entry modifiers.  See the descriptions of those

modifiers below.

Allowable modifiers:

Clear Serious Exception (ignored for disk class devices)

Compare

Express Request

Force Error

MINIMAL MSCP COMMAND SET                               PAGE 6-131

WRITE COMMAND

History Log

Reuse Entry

The History Log and Reuse Entry modifiers are interdepen-

dent and described together here to avoid confusion.

Note that the History Log modifier, the Reuse Entry modi-

fier, the "hrn or entloc" field, and the "entry id" field

are  valid  only  for servers that provide "write history

logging" support (i.e., the server sets the Write History

Logging   Support  controller  flag;   see  Section  sec-

tion(controlerflags), "Controller Flags" for related  in-

formation).

The  setting  of  the  History  Log  modifier  determines

whether information about this command is to be stored in

a "write history entry."

The  setting  of  the  Reuse  Entry  modifier  determines

whether the server is to allocate a  new  "write  history

entry" or reuse one that was previously allocated.

If  "write  history logging" is supported and the History

Log modifier is set  and  the  Reuse  Entry  modifier  is

clear,  the  server allocates a new "write history entry"

and then stores the value contained  in  the  "hrn  (host

reference number)" field, "entry id" field, and other in-

formation related to this command into the "write history

entry"  just allocated.  If the server has no "write his-

tory entry" to allocate, it  shall  remember  this  fact.

See  Section  section(wrihismdes), "WRITE HISTORY MANAGE-

MENT Command, Description" for more details.   To  report

this  allocation  failure to the host, the server returns

FFFF(hex) in the "entry locator" field of the end message

and  sets  the  Allocation Failure end flag.  See Section

section(end____mess), "End Messages," for  a  description

of this flag.

If  "write  history logging" is supported and the History

Log modifier is set and the Reuse Entry modifier is  set,

the server uses the value contained in the "entloc (entry

locator)" field to locate the associated  "write  history

entry"  and  then stores the other information related to

this command into that "write history entry."

See  Section  section(wrihismdes), "WRITE HISTORY MANAGE-

MENT Command,  Description"  for  details  regarding  the

maintenance and format of "write history entries."

MINIMAL MSCP COMMAND SET                               PAGE 6-132

WRITE COMMAND

Note  that  when "write history logging" is supported and

the History Log modifier is set host class drivers  shall

supply  both  the  "hrn  or  entloc"  and  the "entry id"

fields.  If the History Log modifier  is  set  and  those

fields  are  not supplied, the server rejects the command

as an "Invalid Command" ("invalid  message  length"  pro-

tocol  error.   See  "Invalid  Command" status in Section

section(status__codes), "Status Codes").

If  "write  history logging" is supported and the History

Log modifier is clear, the server ignores the setting  of

the  Reuse Entry modifier and the contents of the "hrn or

entloc" and "entry id" fields if they are supplied.  Note

that host class drivers need not supply those fields when

the History Log modifier is clear.

If  "write history logging" is not supported and the His-

tory Log modifier or the Reuse Entry modifier is set, the

server  rejects the command as an "Invalid Command" ("in-

valid modifier" protocol error.   See  "Invalid  Command"

status   in   Section   section(status__codes),   "Status

Codes").

If  "write history logging" is supported, the History Log

modifier is set, and the "byte count" field is zero,  the

server  rejects  the  command as an Invalid Command ("in-

valid byte count" parameter error.  See "Invalid Command"

status   in   Section   section(status__codes),   "Status

Codes").

The  server shall set the History Logged end flag in this

command's end message if it has caused the write  history

log  to  contain  a  valid "write history entry" for this

command.  See  Section  section(end____mess),  "End  Mes-

sages," for descriptions of all end message flags.

Supplement Write Log

The  Supplement  Write  log  modifier allows the transfer

length field of an existing write log in a controller  to

be  modified.   When  this  modifier is set, the transfer

length of the current  WRITE  command  is  added  to  the

transfer  length  field of the specified write log entry.

No other fields in the write log entry are modified.   In

particular,  the  unit number field, the flags field, the

logical block number field, the entry id field,  and  the

hrn field shall remain unchanged.

MINIMAL MSCP COMMAND SET                               PAGE 6-133

WRITE COMMAND

This  modifier is valid only if the History Log and Reuse

Entry modifiers are both set, and the controller supports

Write  History  Logging;   Write  History Logging Support

flag set, otherwise it shall be ignored.

Proper  use  of  this modifier requires that hosts should

not issue more than one MSCP command at a time using  the

same  entry  locator.  Requests for a given entry locator

using this modifier  shall  be  in  exact  logical  block

number  sequence order with each successive logical block

number being higher  by  precisely  the  transfer  length

(converted to sectors) of the previous WRITE command.

Suppress Error Correction

Note  that this modifier only affects the compare pass of

a write-compare operation.  It has no affect on the write

operation itself.

Suppress Error Recovery

MINIMAL MSCP COMMAND SET                               PAGE 6-134

WRITE COMMAND

6.18.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| byte count |

+-------------------------------+

| |

+--- ---+

| undefined |

+--- ---+

| |

+-------------------------------+

| first bad block |

+---------------+---------------+

| entry id | entry locator | (conditionally present)

+---------------+---------------+

entry locator

This  field  is  present if and only if the corresponding

command message field is present.   If  present  and  the

History  Log modifier is set in the command message, this

field contains the value  assigned  by  the  server  that

uniquely identifies the internal location of the "history

entry" where this command's information was stored.   If,

however,  an allocation failure occurred, this field con-

tains the value FFFF(hex).  If present  and  the  History

Log  modifier is clear in the command message, this field

is undefined.

This  field  contains  a  valid entry locator of a "write

history entry" for this command if and only if  the  His-

tory  Logged end flag is set in the "flags" field of this

end message.

See  the  description  of the History Log and Reuse Entry

modifiers in Section  section(write___cmf),  "WRITE  Com-

mand,   Command   Message   Format"   and   Section  sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

MINIMAL MSCP COMMAND SET                               PAGE 6-135

WRITE COMMAND

entry id

This  field  is  present if and only if the corresponding

command message field is present.   If  present  and  the

History  Log  modifier is set in the command message, the

contents of the corresponding command  message  field  is

copied  without  modification  to this field.  If present

and the History Log modifier is clear in the command mes-

sage, this field is undefined.

See  the  description  of the History Log and Reuse Entry

modifiers in Section  section(write___cmf),  "WRITE  Com-

mand,   Command   Message   Format"   and   Section  sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Invalid Command (subcode "Invalid Byte Count")

A  byte  count of zero was specified with the History Log

modifier set.

Invalid Command (subcode "Invalid Entry Locator")

The value specified in the "entloc (entry locator)" field

is not within the limits of the server's  "write  history

entry" set.

See  the  description  of the History Log and Reuse Entry

modifiers in Section  section(write___cmf),  "WRITE  Com-

mand,   Command   Message   Format"   and   Section  sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

Note  that  this  error  can only occur if "write history

logging" is supported and the History Log and Reuse Entry

modifiers are both set in the command message.

Invalid Command (subcode "Invalid Entry Id")

The  server  located the "write history entry" identified

in the "entloc (entry locator)" field but found that  the

"entry  id"  contained  in  that entry does not equal the

value supplied by a host class driver when that entry was

allocated.   This status may also be returned if the His-

tory Log modifier was set, the Reuse Entry  modifier  was

MINIMAL MSCP COMMAND SET                               PAGE 6-136

WRITE COMMAND

clear, and the host supplied a value of FFFF (hex) in the

"entry id" field.

See  the  description  of the History Log and Reuse Entry

modifiers in Section  section(write___cmf),  "WRITE  Com-

mand,   Command   Message   Format"   and   Section  sec-

tion(wrihismdes),  "WRITE  HISTORY  MANAGEMENT   Command,

Description" for more detail.

Note  that  this  error  can only occur if "write history

logging" is supported and the History Log modifier is set

in the command message.

Invalid Command (subcode "Invalid Logical Block Number")

Command Aborted

Unit-Offline

Unit-Available

Write Protected

Compare Error

Data Error

Host Buffer Access Error

Controller Error

Drive Error

Write History Entry Access Error (subcode "Allocation Failure

Table Full")

The server was unable to allocate a "write history entry"

and could not record this  fact  because  the  Allocation

Failure  Table  was full.  This is a transient condition;

therefore, the host should reissue the command.  See Sec-

tion  section(wrihismdes), "WRITE HISTORY MANAGEMENT Com-

mand, Description" for more details.

Note  that  this  error  can occur only if "write history

logging" is supported and the History Log modifier is set

and Reuse Entry modifier is clear in the command message.

MINIMAL MSCP COMMAND SET                               PAGE 6-137

WRITE COMMAND

6.18.3  Description

Data  is  fetched  from  the  host data buffer and written to the

unit.  For host buffer access details see  Section  section(host-

buffacces), "Host Buffer Access."

If a controller-based cache exists, the cache manager may use the

Cache Access Attribute advice in managing the caching and compar-

ing of the requested data area.

MINIMAL MSCP COMMAND SET                               PAGE 6-138

WRITE COMMAND

6.19  WRITE HISTORY MANAGEMENT COMMAND

Command Category:

Sequential

6.19.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| byte count |

+---------------+---------------+

| "write history" |

+--- ---+

| buffer |

+--- ---+

| descriptor |

+---------------+---------------+

| offset | operation |

+---------------+---------------+

| entry id | hrn or entloc |

+---------------+---------------+

byte count

The  total  requested length of the data transfer ("write

history entries") in  bytes;   that  is,  the  number  of

"write  history  entries"  requested  times the size of a

"write history entry" (16 bytes).  If value in this field

is  not a multiple of the "write history entry" size, the

server shall reject the command  as  an  Invalid  Command

(subcode  "Invalid Byte Count") parameter error.  See the

description of the "byte count"  field  in  Section  sec-

tion(xfrcmdmesfmt),  "Transfer  Command  Message Format,"

for more details on architectural and server constraints.

The  interpretation of this field varies depending on the

MINIMAL MSCP COMMAND SET                               PAGE 6-139

WRITE HISTORY MANAGEMENT COMMAND

"write history management" operation specified.  See  the

description  of  the  READ ALL and READ BY HOST REFERENCE

NUMBER operations in the  "operation"  field  description

below for complete details.

"write history" buffer descriptor

Identification  of  the  host buffer where "write history

entries" are to be stored when a READ ALL or  a  READ  BY

HOST  REFERENCE  NUMBER  operation is specified.  See the

description of those operations in the "operation"  field

above for complete details.

The  format of the "'write history' buffer descriptor" is

identical to the "buffer descriptor" field  described  in

Section  section(xfrcmdmesfmt), "Transfer Command Message

Format."

operation

The "write history management" operation to be performed:

DEALLOCATE ALL

The  server deallocates all of the "write history en-

tries" associated with the  unit  identified  in  the

"unit number" field.

This   operation   clears  any  "previous  allocation

failure" condition associated with this unit for  all

host  reference  numbers.   This  is  accomplished by

deleting all entries with this unit number  from  the

controller's Allocation Failure Table.

The   server  ignores  the  "'write  history'  buffer

descriptor," "byte count," "offset," "hrn or entloc,"

and  "entry  id" fields when this operation is speci-

fied.

DEALLOCATE BY HOST REFERENCE NUMBER

The  server deallocates all of the "write history en-

tries" that are associated with both the  value  sup-

plied  in the "hrn (host reference number)" field and

the unit identified in the "unit number" field.

This   operation   clears  any  "previous  allocation

failure" condition associated with this unit for this

host  reference  number.  That is, the server deletes

the entry associated with this host reference  number

MINIMAL MSCP COMMAND SET                               PAGE 6-140

WRITE HISTORY MANAGEMENT COMMAND

and  unit  in its Allocation Failure Table if such an

entry exists.  This does not affect "previous alloca-

tion  failures"  associated with other units or other

host reference numbers.

The   server  ignores  the  "'write  history'  buffer

descriptor," "byte count," "offset," and  "entry  id"

fields when this operation is specified.

DEALLOCATE BY ENTRY LOCATOR

The server deallocates the "write history entry" that

is located at the internal server location  specified

in the "entloc (entry locator)" field.

If  the  "entry  locator" is not within the limits of

the server's "write history entry"  set,  the  server

rejects the command as an "Invalid Command" ("invalid

entry locator" parameter error.   See  "Invalid  Com-

mand"   status   in  Section  section(status__codes),

"Status Codes").

If  the "write history entry" is already deallocated,

the server rejects the command as  an  "Invalid  Com-

mand" ("invalid entry id" parameter error).

If the value contained in the "entry id" field of the

command message is FFFF (hex) and the  entry  is  not

already   deallocated,  the  server  deallocates  the

"write history entry" and does not compare this value

with  the "entry id" value associated with the "write

history entry."

If the value contained in the "entry id" field of the

command message is not FFFF (hex)  and  it  does  not

equal the "entry id" value associated with the "write

history entry" located via the "entry  locator,"  the

server  rejects  the  command as an "Invalid Command"

("invalid entry id" parameter  error.   See  "Invalid

Command"  status  in  Section section(status__codes),

"Status Codes").

The   server  ignores  the  "'write  history'  buffer

descriptor," "byte count," and "offset"  fields  when

this operation is specified.

MINIMAL MSCP COMMAND SET                               PAGE 6-141

WRITE HISTORY MANAGEMENT COMMAND

DECREMENT ALLOCATION FAILURE COUNT

The  server  decrements  the count field of the entry

associated with this host reference number  and  unit

in  its Allocation Failure Table if such an entry ex-

ists.  If this causes the count field to go to  zero,

the  entry  is deleted.  If no such entry exists, the

server rejects the command with  a  status  of  Write

History Entry Access Error (subcode "No Such Entry").

Otherwise, the server returns Success.

The  server may, at its option, treat a WRITE HISTORY

MANAGEMENT  command  with  the  DECREMENT  ALLOCATION

FAILURE COUNT operation as an Immediate command.

The   server  ignores  the  "'write  history'  buffer

descriptor," "byte count," "offset," and  "entry  id"

fields when this operation is specified.

READ ALL

If  the  "byte  count" field is zero, the server sets

the command's end message "count" field equal to  the

number of "write history entries" that are associated

with the unit identified in the "unit number"  field,

and  then completes the command with a status of Suc-

cess (subcode "Normal").  In this case the server ig-

nores   the   "'write  history'  buffer  descriptor,"

"offset," "hrn or entloc," and "entry id" fields.

If  the  "byte  count"  field  is nonzero, the server

transfers the number of "write history  entries"  im-

plied  by  the  "byte  count" field to the host class

driver's buffer (specified in  the  "'write  history'

buffer  descriptor"  field)  beginning  at the offset

specified in the "offset" field into the group of en-

tries  associated  with  the  unit  identified in the

"unit number" field.  There may be fewer "write  his-

tory entries" than implied by the "byte count" field.

The server indicates the number of "write history en-

tries"  actually  transferred in the "count" field of

the end message.  Note that only those "write history

entries" that are associated with the unit identified

in the  "unit  number"  field  are  included  in  the

transfer.   In  this case the server ignores the "hrn

or entloc," and "entry id" fields.   Note  also  that

the  "count"  returned  by  the server may be zero if

there are no more entries beginning at the  specified

offset  but  there  is  at least one entry associated

with the unit identified in the "unit number"  field.

MINIMAL MSCP COMMAND SET                               PAGE 6-142

WRITE HISTORY MANAGEMENT COMMAND

If  there  is no entry in the write history log asso-

ciated with the unit identified in the "unit  number"

field, the server completes the command with a status

of Write History Entry Access Error (subcode "No Such

Entry").

READ BY HOST REFERENCE NUMBER

If  the  "byte  count" field is zero, the server sets

the command's end message "count" field equal to  the

number of "write history entries" that are associated

with both the  value  contained  in  the  "hrn  (host

reference  number)"  field and the unit identified in

the "unit number" field, and then completes the  com-

mand with a status of Success (subcode "Normal").  In

this case the server  ignores  the  "'write  history'

buffer descriptor," "offset," and "entry id" fields.

If  the  "byte  count"  field  is nonzero, the server

transfers the number of "write history  entries"  im-

plied  by  the  "byte  count" field to the host class

driver's buffer (specified in  the  "'write  history'

buffer  descriptor"  field)  beginning  at the offset

specified in the "offset" filed into the group of en-

tries associated with both the value contained in the

"hrn (host reference  number)"  field  and  the  unit

identified  in the "unit number" field.  There may be

fewer "write history entries" than specified  in  the

"count"  field.   The  server indicates the number of

"write history entries" actually transferred  in  the

"count"  field  of  the  end message.  Note that only

those "write history  entries"  that  are  associated

with  both  the  value  contained  in  the "hrn (host

reference number)" field and the unit  identified  in

the "unit number" field are included in the transfer.

In this case the server ignores the "entry id" field.

Note also that the "count" returned by the server may

be zero if there are no more entries beginning at the

specified  offset but there is at least one entry as-

sociated with both the value in the "hrn (host refer-

ence  number)"  field  and the unit identified in the

"unit number" field.  If there is  no  entry  in  the

write  history log associated with the specified host

reference number and unit, the server  completes  the

command  with  a status of Write History Entry Access

Error (subcode "No Such Entry").

See  Appendix appendix(opcdsflgsoffs), "Opcode, Flag, and

Offset  Definitions,"  Table  table(whm__cmd_op),  "Write

History  Management  Command  Operation  Codes"  for  the

MINIMAL MSCP COMMAND SET                               PAGE 6-143

WRITE HISTORY MANAGEMENT COMMAND

values and mnemonics assigned to those operations.

If an operation not listed in Table table(whm__cmd_op) is

specified, the server rejects the command as an  "Invalid

Command"  ("invalid  operation" protocol error.  See "In-

valid Command" status in Section  section(status__codes),

"Status Codes").

Note  that  the server shall perform the specified opera-

tion when the unit is in  the  "Unit-Offline"  (all  sub-

states  except  "Exclusive  Use"),  "Unit-Online," or the

"Unit-Available" state relative to the issuing host class

driver.

offset

The offset (in "write history entries") into the group of

"write history entries" specified by a READ ALL  or  READ

BY  HOST REFERENCE NUMBER operation.  See the description

of these  WRITE  HISTORY  MANAGEMENT  operations  in  the

"operation" field above for complete details.

hrn (host reference number) or entloc (entry locator)

The  interpretation of this field varies depending on the

"write history management" operation specified.  See  the

descriptions  of the DEALLOCATE BY HOST REFERENCE NUMBER,

DEALLOCATE BY ENTRY LOCATOR, DECREMENT ALLOCATION FAILURE

COUNT,  and  READ  BY HOST REFERENCE NUMBER operations in

the "operation" field above for complete details.

See  Sections section(wrihismdes), "WRITE HISTORY MANAGE-

MENT  Command,   Description,"   section(dskcopydatacmd),

"DISK  COPY  DATA  Command," section(erase___cmd), "ERASE

Command," and section(write___cmd), "WRITE  Command"  for

more information regarding this field.

entry id

The  interpretation of this field varies depending on the

"write history management" operation specified.  See  the

description  of the DEALLOCATE BY ENTRY LOCATOR operation

in the "operation" field above for complete details.

See  Sections section(wrihismdes), "WRITE HISTORY MANAGE-

MENT  Command,   Description,"   section(dskcopydatacmd),

"DISK  COPY  DATA  Command," section(erase___cmd), "ERASE

Command," and section(write___cmd), "WRITE  Command"  for

more information regarding this field.

MINIMAL MSCP COMMAND SET                               PAGE 6-144

WRITE HISTORY MANAGEMENT COMMAND

Allowable modifiers:

none

MINIMAL MSCP COMMAND SET                               PAGE 6-145

WRITE HISTORY MANAGEMENT COMMAND

6.19.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| byte count |

+---------------+---------------+

| server alloc | unit alloc |

+---------------+---------------+

| |server unalloc |

+ +---------------+

| undefined |

+---------------+---------------+

| offset | count |

+---------------+---------------+

| entry id | hrn or entloc |

+---------------+---------------+

byte count

The  number  of bytes successfully transferred.  See Sec-

tion section(xfrcmdmesfmt), "Transfer Command End Message

Format," for more details.

unit alloc

The total number of "write history entries" currently al-

located and associated with the unit  identified  in  the

command message "unit number" field.  This field is unde-

fined if the operation specified in the command was DEAL-

LOCATE  BY  ENTRY LOCATOR or DECREMENT ALLOCATION FAILURE

COUNT.

server alloc

The total number of "write history entries" currently al-

located across all units  served  by  the  server.   This

field is undefined if the operation specified in the com-

mand was DEALLOCATE BY ENTRY LOCATOR or DECREMENT ALLOCA-

TION FAILURE COUNT.

MINIMAL MSCP COMMAND SET                               PAGE 6-146

WRITE HISTORY MANAGEMENT COMMAND

server unalloc

The  total  number of unallocated "write history entries"

currently available.  This  field  is  undefined  if  the

operation  specified  in  the  command  was DEALLOCATE BY

ENTRY LOCATOR or DECREMENT ALLOCATION FAILURE COUNT.

count

The  interpretation of this field varies depending on the

"operation" specified and the outcome of the  operation's

execution.   See  the  descriptions  of  those operations

above and the descriptions of the  various  status  codes

below for specific details.

offset

Copied  without  modification from the corresponding com-

mand message field.

hrn (host reference number) or entloc (entry locator)

The  interpretation of this field varies depending on the

"operation" specified and possibly  the  outcome  of  the

operation's execution.

If  a  DEALLOCATE BY HOST REFERENCE NUMBER, DEALLOCATE BY

ENTRY LOCATOR, DECREMENT  ALLOCATION  FAILURE  COUNT,  or

READ  BY  HOST  REFERENCE NUMBER operation was specified,

this field is copied without modification from  the  cor-

responding command message field.

If  a DEALLOCATE ALL or READ ALL operation was specified,

this field is undefined.

entry id

If a DEALLOCATE BY ENTRY LOCATOR operation was specified,

this field is copied without modification from  the  cor-

responding  command message field.  Otherwise, this field

is undefined.

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

If a DEALLOCATE ALL, DEALLOCATE BY HOST REFERENCE NUMBER,

or DEALLOCATE BY ENTRY LOCATOR operation  was  specified,

the  server  sets  the end message "count" field equal to

MINIMAL MSCP COMMAND SET                               PAGE 6-147

WRITE HISTORY MANAGEMENT COMMAND

the number of "write history entries" deallocated.

If  a  DECREMENT  ALLOCATION  FAILURE COUNT operation was

specified, the server sets the end message "count"  field

to  the decremented value in the count field of the entry

in the Allocation Failure Table.

If a READ ALL operation with a zero command message "byte

count" field was specified, the server sets the end  mes-

sage  "count" field equal to the number of "write history

entries" associated with the unit identified in the "unit

number" command message field.

If  a READ BY HOST REFERENCE NUMBER operation with a zero

command message "byte count"  field  was  specified,  the

server  sets  the  end message "count" field equal to the

number of "write history entries"  associated  with  both

the  value contained in the "hrn (host reference number)"

command message field and  the  unit  identified  in  the

"unit number" command message field.

If  a READ ALL or READ BY HOST REFERENCE NUMBER operation

with a nonzero command message  "byte  count"  field  was

specified,  the server sets the end message "count" field

equal to the number of "write history  entries"  actually

transferred to the host's buffer.

Invalid Command (subcode "Invalid Byte Count")

Invalid Command (subcode "Invalid Operation")

Invalid Command (subcode "Invalid Entry Locator")

Invalid Command (subcode "Invalid Entry Id")

Command Aborted

Unit-Offline (subcode "Exclusive Use")

The  unit is under the "Exclusive Access" of a host class

driver other than the host class driver that issued  this

command.

As  described in Section section(wrihismdes), "WRITE HIS-

TORY MANAGEMENT Command, Description," all of the  "write

history entries" associated with the unit are only acces-

sible by the host class  driver  that  was  granted  "Ex-

clusive  Access"  to the unit.  The server will therefore

take no action on the "write history entries"  associated

with the unit when this condition exists.

Host Buffer Access Error

Before  returning  this  status  the  server sets the end

MINIMAL MSCP COMMAND SET                               PAGE 6-148

WRITE HISTORY MANAGEMENT COMMAND

message "count" field equal to the number of "write  his-

tory entries" transferred before the error occurred.

Note  that  this  error  can  only occur if the operation

specified is either a READ ALL or READ BY HOST  REFERENCE

NUMBER with a nonzero "byte count" field.

Write History Entry Access Error (subcode "No Such Entry")

The  server  returns  this  status  under  the  following

conditions:

A.  A  READ  ALL  operation  was  specified  but the

server could not find  any  "write  history  en-

tries"  associated  with  the unit identified in

the "unit number" command message field.

B.  A  READ  BY  HOST REFERENCE NUMBER operation was

specified but the  server  could  not  find  any

"write history entries" associated with both the

value contained  in  the  "hrn  (host  reference

number)"  command  message  field  and  the unit

identified in the "unit number" command  message

field.

C.  A  DEALLOCATE  BY  ENTRY  LOCATOR  operation was

specified  but  the  specified  "write   history

entry" is not currently associated with the unit

identified in the "unit number" command  message

field.

D.  A  DECREMENT  ALLOCATION FAILURE COUNT operation

was specified but no entry associated with  this

host  reference number and unit was found in the

Allocation Failure Table.

In  all the above cases, the server zeros the end message

"count" field.

Write History Entry Access  Error  (subcode   "Previous

Allocation Failure")

A  READ  BY HOST REFERENCE NUMBER operation was specified

but a previous "write history entry"  allocation  failure

condition  exists for this host reference number and unit

or a READ ALL operation was specified and a previous  al-

location  failure  exists  for some host reference number

and this unit.  In either case,  no  "write  history  en-

tries" are transferred to the host.

MINIMAL MSCP COMMAND SET                               PAGE 6-149

WRITE HISTORY MANAGEMENT COMMAND

MINIMAL MSCP COMMAND SET                               PAGE 6-150

WRITE HISTORY MANAGEMENT COMMAND

6.19.3  Description

Support  of  "write  history  logging" is a server implementation

dependent option.  Servers that do not provide such support shall

reject  this  command  as  an "Invalid Command" ("invalid opcode"

protocol  error.   See  "Invalid  Command"  status   in   Section

section(status__codes), "Status Codes").

Servers  that  provide "write history logging" support shall also

support the TERMINATE CLASS DRIVER/SERVER CONNECTION  command  as

described  in  Section  section(termcdsconmcmd),  "Multihost TER-

MINATE CLASS DRIVER/SERVER CONNECTION Command." Note that servers

that support "write history logging" but do not provide Multihost

Support treat the TERMINATE CLASS DRIVER/SERVER  CONNECTION  com-

mand  as  a no-operation as described in Section section(noopera-

tioncmd), "No-Operation Commands."

"Write  history  logging"  and  the TERMINATE CLASS DRIVER/SERVER

CONNECTION command were defined primarily for the purpose of  as-

sisting in the operation of host based volume shadowing implemen-

tations.  All servers that support "write history  logging"  (and

therefore  the  TERMINATE CLASS DRIVER/SERVER CONNECTION command)

shall set the Write History Logging Support controller flag.

"Write  history logging" provides host class drivers with a means

for detecting and recovering from  the  incomplete  execution  of

DISK  COPY  DATA,  ERASE,  and WRITE commands addressed to a disk

unit caused by unexpected host failure (e.g.,  a  system  crash).

The  server provides a set of "write history entries" for logging

information related to write type commands issued by a host.  The

number  of  "write history entries" contained in the set provided

by a server is implementation  dependent.   The  number  provided

depends  on  an implementation's memory availability and speed of

I/O request execution.  In a multihost environment the host class

drivers  are  responsible  for  managing  the distribution of the

"write history entry" set among themselves.

The  server  also provides a table, the Allocation Failure Table,

for recording  "write  history  entry"  allocation  failures.   A

"write  history  entry" allocation failure occurs when the server

cannot allocate a new "write history  entry"  in  response  to  a

write  type  command  requesting  such  an allocation because all

"write history entries" are already  allocated.   The  Allocation

Failure Table is initially empty (i.e., contains no entries).  An

entry has three fields:  "host reference number," "unit  number,"

and  "count."  When  an  allocation  failure  occurs,  the server

searches the Allocation Failure Table for  entry  with  the  host

reference  number  and unit number associated with the write type

command.  If such an entry is found, the  server  increments  the

MINIMAL MSCP COMMAND SET                               PAGE 6-151

WRITE HISTORY MANAGEMENT COMMAND

"count" field of that entry and completes processing of the allo-

cation failure as detailed below.  If no such entry is found  and

the  table  is  not full, the server creates a new entry with the

host reference number and unit number associated with  the  write

type  command and sets the "count" field to one.  The server then

completes processing of the allocation failure as detailed below.

If  no  such entry is found and the table is full, the server re-

jects the write type command with a status of Write History Entry

Access Error (subcode "Allocation Failure Table Full").

The host can use the WRITE HISTORY MANAGEMENT command to clear an

allocation failure condition using either the  DECREMENT  ALLOCA-

TION  FAILURE  COUNT  operation  or a DEALLOCATE operation.  Nor-

mally, the host will use the former method after  being  notified

of  the  condition  by  the end message of the write type command

that caused it.  Therefore,  allocation  failure  conditions  are

usually  transitory  in  nature.  Note that an allocation failure

condition exists for a given host reference number and  unit  ex-

actly  when there exists an entry in the Allocation Failure Table

for that host reference number and unit.  Therefore, clearing  an

allocation failure condition using a DEALLOCATE operation amounts

to deleting the entry in the Allocation Failure Table  associated

with that host reference number and unit, regardless of the value

in the "count" field in that entry.

If  a  write type command is rejected with a status of Write His-

tory  Entry  Access  Error  (subcode  "Allocation  Failure  Table

Full"),  the host should reissue the command since this condition

is transitory in nature and the reissued  command  is  likely  to

succeed.   The  server may maintain the "write history entry" set

in volatile memory and shall initialize all  "write  history  en-

tries" after each power-up.

The  initialization  process the server performs consists of set-

ting the "write history entries" into the deallocated  state  and

deleting  all  entries  in  the Allocation Failure Table.  "Write

history entries" are allocated only via a  History  Log  modified

write  type command.  A "write history entry" is considered deal-

located exactly when it is not accessible to a host via the WRITE

HISTORY  MANAGEMENT  command or via a History Log and Reuse Entry

modified write type command.  The exact  mechanism  for  tracking

the  status  of  a "write history entry" is server implementation

dependent.  However, in order to provide one such mechanism,  the

host  shall  never supply a value of FFFF (hex) in the "entry id"

field.  The server may, at its option, reject a command  with  an

"entry  id" field of FFFF (hex) as an "Invalid Command" ("invalid

entry id" parameter error).

MINIMAL MSCP COMMAND SET                               PAGE 6-152

WRITE HISTORY MANAGEMENT COMMAND

For  the  purposes of retrieving and deallocating the "write his-

tory entries," when an entry is associated with a write type com-

mand it is also associated with the unit to which the command was

addressed.  (The "unit number" is stored within the entry.)

The loss of the "Controller-Online" connection between the server

and a host class driver shall have no  effect  on  any  allocated

"write  history  entry"  or Allocation Failure Table entry.  Such

entries will remain allocated until they are specifically deallo-

cated

o  by  a  host class driver request (i.e., via a WRITE HIS-

TORY MANAGEMENT command that  specifies  an  appropriate

operation)

o  by  the  occurrence  of  a  condition  that requires the

server to initialize its set of "write history  entries"

as described earlier in this section

Host  class  drivers associate a write type command with a "write

history entry"  by  setting  the  History  Log  modifier  in  the

command's message.  The class driver also supplies information in

the command message which allows the host to:

A.  Associate an entry with a group of entries -- "hrn

(host reference number)" field.

B.  Reuse an entry it has already allocated -- "entloc

(entry locator)" field.

C.  Uniquely  identify  an  entry among all of the en-

tries in a group that are available for its use --

"entry id" field.

See  Sections  section(dskcopydatacmd), "DISK COPY DATA Command,"

section(erase___cmd), "ERASE Command," and  section(write___cmd),

"WRITE  Command"  for  descriptions  of  that  modifier and those

fields.  The actions a server takes when a History  Log  modified

write type command is received are described in more detail later

in this section.

The WRITE HISTORY MANAGEMENT command allows host class drivers to

manage the "write history entries" associated with a unit via the

following operations:

o  Deallocate all entries.

MINIMAL MSCP COMMAND SET                               PAGE 6-153

WRITE HISTORY MANAGEMENT COMMAND

o  Deallocate  the  group  of  entries  associated  with  a

particular "host reference number."

o  Deallocate a specific entry.

o  Read all entries.

o  Read  the  group of entries associated with a particular

"host reference number."

o  Decrement  the  count  of  an  Allocation  Failure Table

entry.

See  the  description  of  the  "operation" field in Section sec-

tion(wrihismcmf), "WRITE HISTORY MANAGEMENT Command, Command Mes-

sage Format" for complete descriptions of those operations.

A  server  may  use  any appropriate internal format for a "write

history entry." A "write history entry" presented to  a  host  in

response  to  a  WRITE HISTORY MANAGEMENT READ ALL or READ BY HRN

operation shall have the following format:

31 0

+---------------+---------------+

| unit number | entry locator |

+---------------+---------------+

| transfer length |

+-------------------------------+

| starting logical block number |

+---------------+---------------+

| entry flags | hrn |

+---------------+---------------+

entry locator

The value assigned by the server that uniquely identifies

the internal location of the "write history entry."

unit number

Identifies  the unit to which the command associated with

the "write history entry" was addressed.

transfer length

The  format of this field varies depending on the setting

of the "Transfer Length in  Blocks"  entry  flag  in  the

"write  history  entry."  The  server may, at its option,

store the transfer length of the  associated  command  in

MINIMAL MSCP COMMAND SET                               PAGE 6-154

WRITE HISTORY MANAGEMENT COMMAND

bytes  or  blocks.   If  the byte count in the associated

command was not a multiple of the unit's block size,  the

server  may  store  the  exact byte count, the byte count

rounded up to the next multiple of the unit's block size,

or the block count computed by rounding the byte count up

to a multiple of the unit's block size.

starting logical block number

The logical block number (position) on the disk volume at

which the write operation is to start as specified in the

associated command.

hrn (host reference number)

The value assigned by a host class driver for referencing

a group of "write history entries."

entry flags

Bit flags, collectively known as entry flags, used to in-

dicate the state of a "write history entry." The  follow-

ing flags are defined:

Error

Set  if  and  only  if the write type command for

which the "write history entry"  was  created  or

updated  did  not  complete with a status of Suc-

cess.

Transfer Length in Blocks

Set if and only if the "transfer length" field of

the "write history entry" is specified in  blocks

rather  than  in  bytes.   Note that a server may

convert the transfer length  of  a  command  from

bytes  to  blocks or vice versa before storing it

in the "transfer length" field.  Such  conversion

shall not truncate partial blocks.

Refer  to  Appendix  A, "Opcode, Flag, and Offset Defini-

tions," Table table(wrihisentflg), "Write  History  Entry

Flags" for the values assigned to the defined flags.  All

other entry flag bits are reserved, and shall be  treated

in  accordance  with the requirements for reserved fields

described in  Section  section(rsvdundefflds),  "Reserved

and Undefined Fields."

entry id

MINIMAL MSCP COMMAND SET                               PAGE 6-155

WRITE HISTORY MANAGEMENT COMMAND

The  value  assigned  by  a host class driver to uniquely

identify a "write history entry" among all of the  "write

history  entries"  in  a group that are available for its

use.  This field  is  NOT  present  in  the  host-visible

"write  history  entry"  pictured  above.   However,  the

server shall internally maintain the association  between

the "entry id" specified in the History Log modified com-

mand and its corresponding "write history entry."

Note  that  the host may not supply a value of FFFF (hex)

for this field.   That  value  is  reserved  for  use  by

servers to mark a "write history entry" as unallocated in

its internal format.  Supplying an "entry  id"  value  of

FFFF  (hex) will result in unpredictable behavior, possi-

bly leading to user data corruption.

Upon  receipt  of a host class driver issued History Log modified

write type command the server performs the following operations:

A.  Validates  the  command  message fields and checks

the unit state in the normal  manner.   Note  that

this  implies having an established communications

path in the case of History Log modified DISK COPY

DATA command (see Section section(dskcopydatacmd),

"DISK COPY DATA Command").

If the field validation or unit state checks fail,

the  server   rejects   the   command   with   the

appropriate   status.    Otherwise,   the   server

proceeds to Step 2.

B.  Checks  the setting of the Reuse Entry modifier to

see whether the command is to be associated with a

newly  allocated  "write  history entry" or with a

previously allocated "write history entry."

If  the  the  Reuse  Entry  modifier is clear, the

server proceeds to Step 3 (allocate new).   Other-

wise,  the  server  proceeds to Step 4 (reuse pre-

viously allocated).

C.  Searches the set of "write history entries" for an

entry that is not currently allocated.

If  an  unallocated  "write  history entry" is not

found, the server shall record this  fact  in  the

Allocation  Failure  Table  as described above and

continue normal processing of  the  command.   The

server   shall  return  FFFF(hex)  in  the  "entry

MINIMAL MSCP COMMAND SET                               PAGE 6-156

WRITE HISTORY MANAGEMENT COMMAND

locator" field of the end message and set the  Al-

location Failure end flag in the end message.

If the server cannot record the failure in the Al-

location Failure Table, it shall reject  the  com-

mand  with  a status of Write History Entry Access

Error (subcode "Allocation Failure  Table  Full").

Due  to  the  transitory nature of this condition,

the host should reissue the failed write type com-

mand as it will likely succeed on retry.

If  an  unallocated  "write history entry" is found,

the server performs the following operations:

a.  Copies  the  contents  of  the  associated

command's command  message  "unit  number"

field  to  the "unit number" field in that

entry.

b.  Preserves    the    associated   command's

transfer length ("byte count" or  "logical

block count") in that entry as detailed in

the  description  of   "transfer   length"

above.

c.  Copies  the  contents  of  the  associated

command's command message  "logical  block

number"  (or  "destination  lbn") field to

the "starting logical block number"  field

within that entry.

d.  Copies  the  contents  of  the  associated

command's  command  message   "hrn   (host

reference number)" field to the "hrn (host

reference  number)"  field   within   that

entry.

e.  Associates  the  associated command's com-

mand message "entry id"  field  with  that

entry.

f.  Continues  normal  processing  of the com-

mand.

D.  Checks  the  value  contained in the "entry locator"

command message field to ensure that  it  is  within

the limits of the set of "write history entries."

MINIMAL MSCP COMMAND SET                               PAGE 6-157

WRITE HISTORY MANAGEMENT COMMAND

If  the  "entry locator" is not within the limits of

the set, the server rejects the command as  an  "In-

valid  Command"  ("invalid  entry locator" parameter

error.  See  "Invalid  Command"  status  in  Section

section(status__codes), "Status Codes").

If  the  "entry locator" is within the limits of the

set, the server uses the value of the  "entry  loca-

tor"  as an index into the set of "write history en-

tries" and then proceeds to Step 5.

E.  Compares  the  "entry  id" value associated with the

located "write history entry" with  the  value  con-

tained  in  the  "entry  id"  command message field.

Note that the  value  FFFF  (hex)  is  reserved  for

servers  to  mark "write history entries" as unallo-

cated.

If those values do not match, the server rejects the

command as an "Invalid Command" ("invalid entry  id"

parameter  error.   See  "Invalid Command" status in

Section  section(status__codes),  "Status   Codes").

The  server may optionally check that the value sup-

plied in the "entry id" command message field is not

FFFF  (hex)  and,  if it is, reject the command with

this same status.  If the command  is  not  rejected

and the values do match, the server proceeds to Step

6.

F.  The server performs the following operations:

a.  Copies  the  contents  of  the  associated

command's command  message  "unit  number"

field  to  the  "unit number" field in the

located entry.

b.  Preserves    the    associated   command's

transfer length ("byte count" or  "logical

block count") in that entry as detailed in

the  description  of   "transfer   length"

above.

MINIMAL MSCP COMMAND SET                               PAGE 6-158

WRITE HISTORY MANAGEMENT COMMAND

c.  Copies  the  contents  of  the  associated

command's command message  "logical  block

number"  (or  "destination  lbn") field to

the "starting logical block number"  field

in the located entry.

d.  Continues  normal  processing  of the com-

mand.

After  a  command  associated  with a "write history entry" has

aborted, terminated, or completed,  the  server  shall  set  or

clear the "Error" flag in the "entry flags" field of the "write

history entry" and continue processing.  The server clears  the

"Error"  flag  if  and only if the associated command completed

with a status of "Success" and sets the  "Error"  flag  in  all

other cases.

In  addition,  prior  to returning the end message of a command

associated with a "write history entry" the server  shall  copy

the  "entry  id" field of the command message to the "entry id"

field of the end message and copy the "entry locator" field  of

the  "write  history entry" to the "entry locator" field of the

end message.