CHAPTER 7

MULTIHOST SUPPORT

7.1  OVERVIEW

Multihost Support is an MSCP option that allows simultaneous com-

munication between multiple host class drivers and an MSCP server

across separate logical connections.

Various  aspects of Multihost Support have already been discussed

in previous chapters.  A  brief  review  of  the  more  important

aspects, along with specific section references, follows:

o  Controllers   shall   maintain   each   host   class

driver/MSCP server connection as a separate communi-

cations    entity.    See   Section   section(multi-

hostcommu),  "Multihost   Communication." Controller

state  actually  reflects  the  state  of each class

driver/MSCP server  connection.   See  Section  sec-

tion(cntrolerstates), "Controller States."

o  Controllers  shall  maintain  the  state  of  a unit

("Unit-Offline,"        "Unit-Available,"        and

"Unit-Online") separately for each class driver/MSCP

server        connection.         See        Section

section(unit__states),  "Unit  States." That section

also contains important details regarding "Exclusive

Access"  of a unit (a mandatory feature of Multihost

Support).

o  Command  execution  ordering  is performed on a unit

basis rather than a single connection  basis.   Con-

trollers  shall  therefore  enforce  the  Sequential

command    execution    guarantees    across     all

MULTIHOST SUPPORT                                        PAGE 7-2

OVERVIEW

"Controller-Online"  connections.   Controllers  are

expressly permitted to  include  all  Non-Sequential

commands received from all "Controller-Online" class

drivers, directed to the same unit, in any optimiza-

tion  algorithm  being  performed  on the unit.  See

section(cmdcatsexord), "Command Categories and  Exe-

cution Order."

o  Controllers  shall maintain and operate according to

the setting of a separate set of host-settable  con-

troller flags for each host class driver/MSCP server

connection.  Each class driver is expressly  permit-

ted   to   specify   different   settings   for  the

host-settable controller flags and as a result real-

ize  different  controller  operation.   See Section

section(controlerflags), "Controller Flags."

o  Controllers   shall   maintain   only   one  set  of

host-settable unit flags for a particular  unit  re-

gardless  of  the  number of host class drivers that

have brought the unit into the "Unit-Online"  state.

The  unit  will  operate in an identical manner (ac-

cording to the setting  of  the  host-settable  unit

flags)  for  each  host  that  has  brought the unit

"Unit-Online." Any host may alter the setting of the

host-settable unit flags (for itself and all others)

only via the SET UNIT  CHARACTERISTICS  command  and

only after bringing the unit "Unit-Online." See Sec-

tion section(unit___flags), "Unit Flags."

Controllers  that  provide  Multihost Support shall implement all

the  commands  described   in   Chapter   chapter(minimalcmdset),

"Minimal  MSCP  Command  Set." Command  operation is identical to

what is described there, with the exception  of  the  the  ABORT,

AVAILABLE,  GET COMMAND STATUS, GET UNIT STATUS, ONLINE, SET CON-

TROLLER CHARACTERISTICS, and SET UNIT  CHARACTERISTICS  commands.

The  operation  of  each of those commands within a Multihost en-

vironment is described later in this  chapter.   The  same  major

section/subsection    format    described    in    Section   sec-

tion(minicmdsetov), "[Minimal MSCP Command Set] Overview" is used

for  the  command  descriptions  in this chapter.  The references

contained in Section section(minicmdsetov) for message offset de-

finitions  and treatment of invalid and no-operation commands ap-

ply to  the  Multihost  environment  as  well.   In  cases  where

information  contained in the Chapter chapter(minimalcmdset) com-

mand descriptions is identical  for  both  the  single  host  and

multihost    operation,    the   Chapter   chapter(minimalcmdset)

MULTIHOST SUPPORT                                        PAGE 7-3

OVERVIEW

information is  referenced  rather  than  repeating  it  in  this

chapter.   In  addition,  the  ACCESS NONVOLATILE MEMORY command,

which shall be supported by all controllers that  provide  Multi-

host  Support,  is also described.  Note that controllers that do

not provide Multihost Support may optionally implement the ACCESS

NONVOLATILE MEMORY command.

MULTIHOST SUPPORT                                        PAGE 7-4

OVERVIEW

7.2  MULTIHOST ABORT COMMAND

Command Category:

Immediate

7.2.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| outstanding reference number |

+-------------------------------+

unit number

See Section section(abort___cmf), "ABORT Command, Command

Message Format."

outstanding reference number

The  "command reference number" of the command that is to

be aborted.

As  described in Section section(command__mess), "Command

Messages," "command reference numbers" need not be unique

for  commands  issued by different class drivers -- i.e.,

commands issued by different hosts or commands  for  dif-

ferent  MSCP  servers  from  the  same host.  Controllers

shall therefore use the combination of  the  "outstanding

reference  number"  and the connection on which the ABORT

command  was  received  in  order  to  uniquely  identify

commands issued by different hosts.

MULTIHOST SUPPORT                                        PAGE 7-5

MULTIHOST ABORT COMMAND

Allowable modifiers:

none

MULTIHOST SUPPORT                                        PAGE 7-6

MULTIHOST ABORT COMMAND

7.2.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| outstanding reference number |

+-------------------------------+

outstanding reference number

See  Section  section(abort___emf),  "ABORT  Command, End

Message Format."

Status Codes:

Success (subcode "Normal")

MULTIHOST SUPPORT                                        PAGE 7-7

MULTIHOST ABORT COMMAND

7.2.3  Description

See Section section(abort___des), "ABORT Command, Description."

MULTIHOST SUPPORT                                        PAGE 7-8

MULTIHOST ABORT COMMAND

7.3  MULTIHOST ACCESS NONVOLATILE MEMORY COMMAND

Command Category:

Immediate

7.3.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | reserved |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| reserved |

+-------------------------------+

| offset |

+---------------+---------------+

| length | operation |

+---------------+---------------+

| |

/ nonvolatile /

/ memory data /

| |

+-------------------------------+

offset

The byte offset within the server's nonvolatile memory at

which to perform the operation.

operation

The operation to be performed on the server's nonvolatile

memory:

MULTIHOST SUPPORT                                        PAGE 7-9

MULTIHOST ACCESS NONVOLATILE MEMORY COMMAND

Read

Exchange

Test and Set

See  Appendix appendix(opcdsflgsoffs), "Opcode, Flag, and

Offset Definitions," Table  table(acnvmemcmdop),  "Access

Nonvolatile  Memory  Command  Operation  Codes"  for  the

values and mnemonics assigned to those operations.

If  any  other  value is specified the server rejects the

command as an "Invalid Command" ("invalid operation" pro-

tocol  error.   See  "Invalid  Command" status in Section

section(status__codes), "Status Codes").

length

The  number  of bytes of nonvolatile memory data included

in this  command  message  or  to  be  returned  in  this

command's  end  message.  This should be an even value if

the operation is "Test and Set." If an  odd  "length"  is

specified  with  the  "Test and Set" operation the server

rejects the command as  an  "Invalid  Command"  ("invalid

length" parameter error.  See "Invalid Command" status in

Section section(status__codes), "Status Codes").

The maximum valid "length" is server dependent.  However,

all servers that implement  this  command  shall  support

values  of  "length"  in  the  range  0 through 32.  If a

"length" larger than the server's  maximum  is  specified

the  server  rejects  the command as an "Invalid Command"

("invalid length" parameter error.  See "Invalid Command"

status   in   Section   section(status__codes),   "Status

Codes").

nonvolatile memory data

"Length" bytes of data to be used in the operation.

Allowable modifiers:

none

MULTIHOST SUPPORT                                       PAGE 7-10

MULTIHOST ACCESS NONVOLATILE MEMORY COMMAND

MULTIHOST SUPPORT                                       PAGE 7-11

MULTIHOST ACCESS NONVOLATILE MEMORY COMMAND

7.3.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| reserved |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| nonvolatile memory size |

+-------------------------------+

| offset |

+---------------+---------------+

| length | operation |

+---------------+---------------+

| |

/ nonvolatile /

/ memory data /

| |

+-------------------------------+

nonvolatile memory size

The  number of bytes of nonvolatile memory implemented by

the server.

offset

operation

length

Copied without alteration from the command message.

nonvolatile memory data

"Length" bytes of data returned by the operation.

Status Codes:

Success (subcode "Normal")

Invalid Command (subcode "Invalid Length")

Invalid Command (subcode "Invalid Operation")

Compare Error

MULTIHOST SUPPORT                                       PAGE 7-12

MULTIHOST ACCESS NONVOLATILE MEMORY COMMAND

MULTIHOST SUPPORT                                       PAGE 7-13

MULTIHOST ACCESS NONVOLATILE MEMORY COMMAND

7.3.3  Description

The  ACCESS  NONVOLATILE MEMORY command provides class driver ac-

cess to a nonvolatile memory  region  within  the  controller  or

server.   Hosts may use this memory to store arbitrary parameters

or information.  MSCP servers merely store the data;  they do not

use it in any way.

The  presence  and  size of a nonvolatile memory is server depen-

dent.  However, whenever a server provides a nonvolatile  memory,

the  memory must be at least 16 bytes long.  Servers that provide

Multihost Support shall always implement a 16 byte or larger non-

volatile  memory.  Single host servers may optionally implement a

nonvolatile memory.  If a single host server does not implement a

nonvolatile  memory, then it need not implement this command.  In

that case the controller shall reject this command's opcode as an

"Invalid Command" ("invalid opcode" protocol error.  See "Invalid

Command"  status  in  Section   section(status__codes),   "Status

Codes").

The  nonvolatile  memory  is  addressed at offsets 0 through N-1,

where N is the size of the server's nonvolatile memory in  bytes.

Class  drivers  may access offsets N and higher, even though they

are not included in the implemented memory.   Attempts  to  write

offsets N and higher are ignored and always succeed.  Attempts to

read offsets N and higher return zero.

Three  operations  are  provided  for  accessing  the nonvolatile

memory.  The  detailed  semantics  of  these  operations  are  as

follows:

Read

The  server  reads "length" bytes beginning at offset

"offset" and returns them as the "nonvolatile  memory

data"  in  the  end message.  The "nonvolatile memory

data" in the command message is ignored.

Exchange

The  server  reads "length" bytes beginning at offset

"offset" and returns them as the "nonvolatile  memory

data"  in  the  end message.  Next, the server writes

"length" bytes beginning at offset "offset" with  the

"nonvolatile memory data" from the command message.

MULTIHOST SUPPORT                                       PAGE 7-14

MULTIHOST ACCESS NONVOLATILE MEMORY COMMAND

This  operation exchanges the nonvolatile memory data

with the memory contents stored in the  server.   The

command  message  provides  the  new memory contents,

while the old memory contents are returned in the end

message.

Test and Set

The server reads "length"/2 bytes beginning at offset

"offset" and returns them as the first  half  of  the

"nonvolatile  memory data" in the end message.  Next,

the server compares these  "length"/2  bytes  against

the  first half of the "nonvolatile memory data" from

the command message.   If  they  are  different,  the

server  returns  a  "Compare  Error" status code.  If

they are the same, the server writes "length"/2 bytes

beginning  at offset "offset" with the second half of

the "nonvolatile memory data" from the  command  mes-

sage  and  returns  a "Success" status code.  In both

cases, the second half  of  the  "nonvolatile  memory

data" in the end message is copied without alteration

from the command message.

This operation provides an atomic test-and-set opera-

tion.  The "nonvolatile memory data" is divided  into

equal  length  halves.   The  first  half is compared

against the values stored in the server to  determine

the  "Success"  or "Compare Error" status of the com-

mand.  If comparison shows  both  values  are  equal,

then  the second half of "nonvolatile memory data" is

stored into the nonvolatile memory.  In either  case,

the  values  that  were stored in the server prior to

this operation are returned in the first half of  the

end message's "nonvolatile memory data."

For  all  three  operations,  a "length" of zero is legal.  It is

treated as a no-operation and always succeeds.  This is primarily

useful  as a means for class drivers to determine the size of the

server's nonvolatile memory.

The  server shall implement all operations requested by this com-

mand as atomic operations.  If several ACCESS NONVOLATILE  MEMORY

commands  are  outstanding simultaneously, the server may perform

them in any sequence, but shall process only one  of  them  at  a

time.

The  server shall remember the contents of its nonvolatile memory

across power failures, reinitializations, losses of context,  and

other  normal  failures.   However,  the following events are not

normal and may result in loss of the nonvolatile memory contents:

MULTIHOST SUPPORT                                       PAGE 7-15

MULTIHOST ACCESS NONVOLATILE MEMORY COMMAND

1.  Field  Service  attention  or  other maintenance ac-

tions.  If the component containing the  nonvolatile

memory  is  replaced,  its contents may become unde-

fined.

2.  A  power failure, reinitialization, or other loss of

context while the nonvolatile memory is being  modi-

fied.   The  offset  being  modified, plus some con-

troller dependent number of adjacent locations,  may

become undefined.

The  nonvolatile memory should contain all zeros in newly shipped

servers.

One  characteristic of commonly available nonvolatile memories is

that they can only be reliably written a limited number of times.

Therefore  class  drivers may not treat the nonvolatile memory as

they would standard RAM or disk  memory.   Class  drivers  should

design  their  algorithms for using the nonvolatile memory to en-

sure that they do not alter it more often than an average of once

per  day.   Short  periods  of  frequent  updates are acceptable,

provided they are balanced by long periods of few updates.

MULTIHOST SUPPORT                                       PAGE 7-16

MULTIHOST ACCESS NONVOLATILE MEMORY COMMAND

7.4  MULTIHOST AVAILABLE COMMAND

Command Category:

Sequential

7.4.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

Allowable modifiers:

All Class Drivers

If  the  "All Class Drivers" modifier is set and the unit

is "Unit-Online" to the issuing host as well  as  one  or

more  other  hosts,  the unit becomes "Unit-Available" to

all hosts.

If  the  "All Class Drivers" modifier is set and the unit

is "Unit-Online" only to the issuing host, the  unit  be-

comes "Unit-Available" to the issuing host.

If the "All Class Drivers" modifier is clear and the unit

is "Unit-Online" to the issuing host as well  as  one  or

more  other  hosts,  the unit becomes "Unit-Available" to

the issuing host only.  The state of the unit relative to

all other hosts is unaffected.

If the unit is "Unit-Offline" or already "Unit-Available"

to the issuing  host,  the  setting  of  the  "All  Class

Drivers" modifier is ignored.

Clear Serious Exception (ignored for disk class devices)

MULTIHOST SUPPORT                                       PAGE 7-17

MULTIHOST AVAILABLE COMMAND

Exclusive Access

If the "Exclusive Access" modifier is set and the unit is

"Unit-Online"  to   the   issuing   host   and   is   not

"Unit-Online"   to  any  other  host,  the  unit  becomes

"Unit-Available" to and under the "Exclusive  Access"  of

the  issuing host.  This specifically includes the possi-

bility that the unit became "Unit-Available" to the issu-

ing  host  as  well  as to other hosts as a result of the

"All Class Drivers" modifier also being set in  the  com-

mand.  Note that controllers shall service the "All Class

Drivers" modifier before servicing the "Exclusive Access"

modifier.

If the "Exclusive Access" modifier is set and the unit is

"Unit-Available"  to  the  issuing  host   and   is   not

"Unit-Online" to any other host or the unit is "Unit Off-

line (substate 'No Media Loaded')", the issuing  host  is

granted "Exclusive Access" of the unit.  The unit's state

is otherwise unchanged.

If the "Exclusive Access" modifier is set and the unit is

"Unit-Online" to and under the "Exclusive Access" of  the

issuing  host,  the  unit becomes "Unit-Available" to the

issuing  host.   "Exclusive  Access"  of  the  unit   is,

however, retained.

If  the "Exclusive Access" modifier is clear and the unit

is "Unit-Online" to and under the "Exclusive  Access"  of

the  issuing  host,  the unit becomes "Unit-Available" to

the issuing host and "Exclusive Access" of  the  unit  is

released.

If  the "Exclusive Access" modifier is clear and the unit

is "Unit-Available" or "Unit Offline (substate 'No  Media

Loaded')" to and also under the "Exclusive Access" of the

issuing host, "Exclusive Access" to the unit is released.

The unit's state is otherwise unchanged.

If the unit is "Unit-Offline" to the issuing host for any

reason other than "No Media Loaded," the  "Exclusive  Ac-

cess" modifier is ignored.

Note that granting "Exclusive Access" of a unit to a host

implicitly causes the unit to become "Unit-Offline  (sub-

state  'Exclusive Use')" to all other hosts connected via

this controller and "Unit-Offline  (substate  'Online  To

Another Controller')" to hosts which may potentially have

access to the unit via another  controller.   Conversely,

MULTIHOST SUPPORT                                       PAGE 7-18

MULTIHOST AVAILABLE COMMAND

the  release  of  a unit from the "Exclusive Access" of a

host   implicitly   causes    the    unit    to    become

"Unit-Available" to all other hosts.

Note  also  that controllers ignore duplicate unit number

conditions when a host retains "Exclusive  Access"  of  a

unit.  That is, if the unit is "Unit-Online" to and under

the "Exclusive Access" of the issuing host and  in  addi-

tion  had a duplicate unit number prior to the receipt of

an AVAILABLE command with the "Exclusive Access" modifier

set,  the unit will not be become "Unit-Offline" nor will

it   be   spun-down.    Instead    the    unit    becomes

"Unit-Available" to and "Exclusive Access" is retained by

the issuing host.

Note  also  that "Exclusive Access" is released if any of

the following events occur:

1.  The  controller  loses or breaks the connec-

tion to the host that has "Exclusive Access"

of the drive.

2.  The  drive becomes unknown to the controller

(e.g., port button disabled.)

3.  The      drive      becomes     inoperative,

"Unit-Offline, Inoperative."

4.  The  controller is 'rebooted', by itself, by

an operator, or by any host.

Spin-down

Requests  that  the disk stop spinning and that the heads

be unloaded.  Note that the command completes as soon  as

the spin-down has been initiated, rather than waiting for

the disk to stop spinning.  The spin-down will not be in-

itiated  if  either  or  both of the following conditions

exist:

1.  The  unit  belongs  to a multiunit drive and

shares a spindle with some other  unit  that

is  "Unit-Online"  (See Section section(mul-

tidrvsfmts),    "Multiunit    Drives     and

Formatters").

MULTIHOST SUPPORT                                       PAGE 7-19

MULTIHOST AVAILABLE COMMAND

2.  The  unit  is  still "Unit-Online" to one or

more other hosts and the "All Class Drivers"

modifier is clear.

Note  that  a  spin-down request is honored and performed

(unless prohibited as just described) when  the  unit  is

"Unit-Available"  to  the issuing host as well as when it

is "Unit-Online" to the issuing host.

MULTIHOST SUPPORT                                       PAGE 7-20

MULTIHOST AVAILABLE COMMAND

7.4.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

Status Codes:

Success (subcode "Normal")

This  status  code (i.e., subcode zero) is returned under

the following conditions:

1.  The   unit  changes  from  "Unit-Online"  to

"Unit-Available" and the action  or  actions

(if  any)  requested  by  the setting of the

"All Class Drivers," "Exclusive Access," and

"Spin-down" modifiers were successfully com-

pleted.

2.  The unit was already "Unit-Available" to the

issuing host and the action or  actions  (if

any)  requested  by  the setting of the "Ex-

clusive Access"  and  "Spin-down"  modifiers

were successfully completed.

Separate  bits  within the "Success" status subcode field

are used to indicate that the actions  requested  by  the

setting  of the "All Class Drivers" and "Spin-down" modi-

fiers could not be performed but the command  was  other-

wise  successful.  See the "Spin-down Ignored" and "Still

Online" subcode bit flag descriptions below.

Another  "Success" status subcode bit is used to indicate

that the command was successful and that  the  unit  will

remain  connected  to the controller (see the "Still Con-

nected" bit flag description).

MULTIHOST SUPPORT                                       PAGE 7-21

MULTIHOST AVAILABLE COMMAND

Note  that  if  the unit was "Unit-Online" (and not under

the "Exclusive Access" of the issuing host) but had a du-

plicate  unit number prior to the AVAILABLE command being

issued, the controller may  optionally  return  a  status

code  of:  "Success (subcode 'Normal')" or "Success (sub-

code 'Duplicate Unit Number')" or "Unit-Offline  (subcode

'Duplicate Unit Number')."

Success (subcode "Spin-down Ignored")

The  "Spin-down  Ignored"  subcode bit flag is set if the

unit could not be spun-down due to one of  the  following

conditions:

1.  One or more other units with which this unit

shares a spindle is still "Unit-Online" (see

Section  section(multidrvsfmts),  "Multiunit

Drives and Formatters," for  an  explanation

of shared spindles).

2.  The  unit  is  still "Unit-Online" to one or

more other hosts.

Success (subcode "Still Connected")

The "Still Connected" subcode bit flag is set if the unit

may potentially be accessed via  another  controller  and

one  or  more  other units with which this unit shares an

access path is still "Unit-Online," preventing this  unit

from  being  accessed via that other controller (if any).

See   Section    section(multiaccesdrvs),    "Multiaccess

Drives," for a discussion of access paths.

The "Still Connected" subcode bit flag will always be set

if the "Spin-down Ignored" subcode bit flag is set.

Success (subcode "Still Online")

The "Still Online" subcode bit flag is set if the unit is

still "Unit-Online" to one or more other hosts  via  this

controller.   If  the "Exclusive Access" modifier was set

this status code implies that "Exclusive Access"  of  the

unit was denied.

MULTIHOST SUPPORT                                       PAGE 7-22

MULTIHOST AVAILABLE COMMAND

The "Still Online" subcode bit flag will always be set if

the "Spin-down Ignored" subcode bit flag is set.

Command Aborted

The  command  has  been rejected and the unit's state has

not changed.  If the unit was "Unit-Online" prior to  is-

suing  this command, then the unit is still "Unit-Online"

and it is still spinning.

Unit-Available (subcode "Already In Use")

The  unit  was  "Unit-Available"  to the issuing host and

"Exclusive Access" of the unit was denied because  it  is

currently "Unit-Online" to another host.

Unit-Offline

All  subcodes  may  be returned.  Note that "Unit-Offline

(subcode 'No Media Loaded')"  implies  "Success  (subcode

'Normal')"  when  "Exclusive  Access"  of  the  unit  was

granted while the unit was  "Unit-Offline  (substate  'No

Media  Loaded')." Note  also that in cases where the unit

was "Unit-Online" (and not under the  "Exclusive  Access"

of  the  issuing  host)  but  had a duplicate unit number

prior to the AVAILABLE command  being  issued,  the  con-

troller  may,  at  its  option,  return the "Unit-Offline

(subcode 'Duplicate Unit  Number')  status  code  or  two

forms  of the "Success" status code (see Success (subcode

"Normal") above).

Unit Offline (subcode "Exclusive Use")

The  unit  is  currently  under the "Exclusive Access" of

some other host connected via this controller.

Controller Error

Drive Error

MULTIHOST SUPPORT                                       PAGE 7-23

MULTIHOST AVAILABLE COMMAND

7.4.3  Description

The operation of the AVAILABLE command depends on the settings of

the "All Class Drivers," "Exclusive Access," and "Spin-down" mod-

ifiers as well as the state of the specified unit relative to the

issuing host  and  all  other  "Controller-Online"  hosts  (class

drivers)  as  described  in the preceding sections.  All combina-

tions of those modifiers are permissible.

Note  that  issuing  an  AVAILABLE  command  with  the "All Class

Drivers," "Exclusive Access," and "Spin-down" modifiers clear  to

a unit that is already "Unit-Available" to and not under the "Ex-

clusive Access" of the issuing host is essentially  a  sequential

no-operation.

Upon  completion  of the requested AVAILABLE command operation(s)

an AVAILABLE attention message is  sent  by  all  controllers  to

which  the specified unit is connected under the following condi-

tions (provided the "Spin-down" modifier was clear):

1.  The   unit's   state   changes   from  "Unit-Online"  to

"Unit-Available" relative to the  the  issuing  host  as

well  as all other "Controller-Online" hosts and the is-

suing host was not granted  "Exclusive  Access"  of  the

unit in the process.

2.  The   unit's   state   changes   from  "Unit-Online"  to

"Unit-Available" relative to the  issuing  class  driver

and "Exclusive Access" of the unit was also released.

3.  The  unit  was  already  "Unit-Available" to the issuing

host and "Exclusive Access" of the unit was released.

In  all  the  cases  just  described  the controller to which the

AVAILABLE command was sent need not send an  AVAILABLE  attention

message.

If the "Spin-down" modifier was set, AVAILABLE attention messages

will be suppressed for the specified  unit  both  for  this  con-

troller  and  for  any other controllers to which the unit may be

connected, regardless of whether the spin-down was actually  ini-

tiated.   See  Section section(unit__states), "Unit States" for a

discussion of suppressing AVAILABLE attention messages.

MULTIHOST SUPPORT                                       PAGE 7-24

MULTIHOST AVAILABLE COMMAND

7.5  MULTIHOST GET COMMAND STATUS COMMAND

Command Category:

Immediate

7.5.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| outstanding reference number |

+-------------------------------+

unit number

See  Section  section(getcommandcmf), "GET COMMAND STATUS

Command, Command Message Format."

outstanding reference number

The  "command  reference  number"  of  the  command whose

status is to be obtained.

As  described in Section section(command__mess), "Command

Messages," "command reference numbers" need not be unique

for  commands  issued by different class drivers -- i.e.,

commands issued by different hosts or commands  for  dif-

ferent  MSCP  servers  from  the  same host.  Controllers

shall therefore use the combination of  the  "outstanding

reference  number"  and  the  connection on which the GET

COMMAND STATUS command was received in order to  uniquely

identify commands issued by different hosts.

MULTIHOST SUPPORT                                       PAGE 7-25

MULTIHOST GET COMMAND STATUS COMMAND

Allowable modifiers:

none

MULTIHOST SUPPORT                                       PAGE 7-26

MULTIHOST GET COMMAND STATUS COMMAND

7.5.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| outstanding reference number |

+---------------+---------------+

| command status |

+---------------+---------------+

outstanding reference number

See  Section  section(getcommandemf), "GET COMMAND STATUS

Command, End Message Format."

command status

The  amount  of work remaining to be done to complete the

command, expressed as an unsigned integer.  This field is

zero  if the command is not known to the controller, such

as when the command has already completed or was aborted.

The  controller  may also return zero in this field if it

can guarantee that the command will complete  within  the

controller  timeout interval.  The controller shall never

return a value of all ones (2**32-1) in  this  field,  as

that  value  is  used  to  initialize  the host's command

timeout algorithm  (see  Section  section(command_timos),

"Command Timeouts").

Each  "Controller-Online"  class  driver  has a different

concept of which command is the oldest  outstanding  com-

mand.   As mentioned earlier the command for which status

is being obtained is uniquely qualified by the "outstand-

ing  reference  number"  and  the connection on which the

command GET COMMAND  STATUS  was  received.   Controllers

shall  therefore  return a status value that reflects the

MULTIHOST SUPPORT                                       PAGE 7-27

MULTIHOST GET COMMAND STATUS COMMAND

"doneness" of  the  specified  command  relative  to  the

issuing host.

An  example  of  how  a  controller  might produce status

values that meet all the requirements defined in the pre-

ceding  paragraphs  follows.   Note  that  the  algorithm

described in the example is driven solely by  the  host's

issuance of the GET COMMAND STATUS command, not an inter-

nal controller timer.   For  the  algorithm  to  function

properly  the  host  must  not issue a GET COMMAND STATUS

command for a particular command more frequently than the

value  returned  by  the  controller  in  the "controller

timeout" field  of  the  SET  CONTROLLER  CHARACTERISTICS

command's end message.

The  controller  maintains  two  distinctly different

command status values in the context of each  command

received by the controller:

o  Controller-Host Command Status (CHCS)

A   32-bit  unsigned  integer  value

which is the actual "Command Status"

value  returned  to  the  host.  The

controller  initializes  CHCS  to  a

reasonable   maximum   value  (e.g.,

2**32-2).

o  Command-Specific Command Status (CSCS)

An unsigned integer value consisting

of  two  subfields:   the  Requested

Command's Position (RCP) and the Re-

quested Command's Status (RCS).  RCP

is  the high order field.  (The size

of the CSCS as well as the  RCP  and

RCS  subfields  is controller depen-

dent, as discussed later.) The  con-

troller  initializes the CSCS to its

maximum value (all bits set).

Upon  receipt  of  a GET COMMAND STATUS command for a

particular command the controller constructs  a  cur-

rent   copy   of  CSCS  according  to  the  requested

command's current execution state as follows:

o  Unknown (e.g., no longer outstanding)

RCP and RCS are both set to zero.

MULTIHOST SUPPORT                                       PAGE 7-28

MULTIHOST GET COMMAND STATUS COMMAND

o  Already In-progress

RCP  is  set  to zero, implying that

the requested command is  at  execu-

tion position 0.

RCS  is  set  to  an encoding of the

amount of work  (including  possible

retries,  etc.) remaining before the

requested command completes, or zero

if  the  controller expects the com-

mand  to  finish  within  the  "con-

troller timeout" interval.

o  Sequential     Command    Waiting    For

In-progress Commands to Complete

RCP is set to one, implying that the

requested command will be  the  next

command initiated.

RCS  is  set  to  an encoding of the

amount  of  work  remaining  on  all

in-progress commands.

o  Non-Sequential  Command  Waiting  For An

In-progress Sequential Command  To  Com-

plete

RCP  is  set  equal to the number of

commands which are scheduled for in-

itiation ahead of the requested com-

mand, plus one to  account  for  the

in-progress Sequential command.

RCS  is set equal to the in-progress

Sequential command's RCS value.

o  Sequential   or  Non-Sequential  Command

Waiting For Other Reasons (e.g., a  con-

troller resource to become available)

RCP  is  set  equal to the number of

commands that are scheduled for ini-

tiation  ahead of the requested com-

mand, plus one to  account  for  the

requested command.

RCS  is  set  to  an encoding of the

amount  of  work  remaining  on  all

MULTIHOST SUPPORT                                       PAGE 7-29

MULTIHOST GET COMMAND STATUS COMMAND

in-progress commands.

The  controller  then  tests  the  value  of the con-

structed CSCS.  If the constructed CSCS is  zero  due

to  the  command  being unknown, the controller zeros

the "command status" field of the GET COMMAND  STATUS

command's  end message, and completes the GET COMMAND

STATUS command.  If the constructed CSCS is zero  and

the command is known, the command's CHCS is set equal

to zero.  If the constructed  CSCS  is  nonzero,  the

controller  compares  the  constructed  CSCS  to  the

command's CSCS stored previously.  If the constructed

CSCS  is less than the command's CSCS, the controller

decrements the command's CHCS by one.   If  the  con-

structed  CSCS  is  greater  than  or  equal  to  the

command's CSCS, the controller leaves  the  command's

CHCS  at  the same value.  Then the controller stores

the constructed CSCS in the requested command's  con-

text,  obtains the current value of the CHCS from the

requested command's context  and  stores  it  in  the

"command  status"  field  of  the  GET COMMAND STATUS

command's end message, and completes the GET  COMMAND

STATUS command.

As  mentioned  above  the  size of CSCS is controller

dependent.  CSCS size actually depends on the maximum

values  of  the  RCP and RCS subfields the controller

can reasonably expect  to  encounter,  determined  as

follows:

o  RCP

The  maximum  number of command mes-

sages the controller is  capable  of

queuing at any one time.

o  RCS

The maximum amount of time the long-

est command execution (including er-

ror  recovery  retries) can possibly

consume, expressed as the number  of

ticks  starting  from the initiation

of  the  longest  command  execution

through  its  completion.  A tick is

the amount of time it takes to  com-

plete  the  longest single operation

of  all  the  individual  operations

which  are  used to execute commands

within    the    controller.     The

MULTIHOST SUPPORT                                       PAGE 7-30

MULTIHOST GET COMMAND STATUS COMMAND

"controller   timeout"  interval  is

equal to one tick.

Status Codes:

Success (subcode "Normal")

MULTIHOST SUPPORT                                       PAGE 7-31

MULTIHOST GET COMMAND STATUS COMMAND

7.5.3  Description

The GET COMMAND STATUS command is used to monitor the progress of

a command towards completion.  The command  status  measures  the

"doneness"  of  the  command.   The value returned in the command

status field is guaranteed to not increase over  time.   Further-

more,  the  command status of an MSCP server's oldest outstanding

command relative to the issuing host is  guaranteed  to  decrease

within the controller timeout interval.  This last feature may be

used by a host class driver to detect an insane or malfunctioning

controller.    See   Section   section(command_timos),   "Command

Timeouts," for more details.

The  GET  COMMAND STATUS command always succeeds.  If the command

referenced by the "outstanding reference number" is not known  to

the  MSCP server or has been aborted, then the MSCP server should

return zero for its command status.  The MSCP server may also re-

turn  zero  as the command status of any command that will always

complete within the controller timeout interval.  The MSCP server

always returns the "Normal" status code in the GET COMMAND STATUS

command's end message.

MULTIHOST SUPPORT                                       PAGE 7-32

MULTIHOST GET COMMAND STATUS COMMAND

7.6  MULTIHOST GET UNIT STATUS COMMAND

Command Category:

Immediate

7.6.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

Allowable modifiers:

Clear Serious Exception (ignored for disk class devices)

Next Unit

See  Section section(getunitstcmf), "GET UNIT STATUS Com-

mand, Command Message Format."

MULTIHOST SUPPORT                                       PAGE 7-33

MULTIHOST GET UNIT STATUS COMMAND

7.6.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| unit flags |multiunit code |

+---------------+---------------+

| reserved |spndles|

+-------------------------------+

| |

+--- unit identifier ---+

| |

+-------------------------------+

| media type identifier |

+-------------------------------+

| reserved |

+-------------------------------+

| group size | track size |

+-------+-------+---------------+

|uhvrsn | usvrsn| cylinder size |

+-------+-------+---------------+

|copies | RBNs | RCT size |

+-------+-------+---------------+

status

The  validity of the unit characteristics varies with the

unit's state implied by the contents of this field.   See

the Status Codes listed later in this section and Section

section(getunitstdes), "GET UNIT STATUS Command, Descrip-

tion" for complete details.

multiunit code

unit identifier

media type identifier

track size

group size

cylinder size

usvrsn

uhvrsn

RCT size

MULTIHOST SUPPORT                                       PAGE 7-34

MULTIHOST GET UNIT STATUS COMMAND

RBNs

copies

See  Section section(getunitstemf), "GET UNIT STATUS Com-

mand, End Message Format."

unit flags

See Section section(unit___flags), "Unit Flags."

Exclusive Access

The  Exclusive Access unit flag is non-host-settable;

it is set when the unit is under the  "Exclusive  Ac-

cess"  of this or some other host via this controller

and clear when it is not.

spndles

The  number  minus one of spindles or spindle "groups" in

the unit.  The value of zero indicates one spindle.   See

Section   section(diskgeomformt),   "Disk   Geometry  and

Format."

Status Codes:

Unit-Offline

All  subcodes  may  be returned.  Note that "Unit-Offline

(subcode 'Exclusive Use')" indicates  that  the  unit  is

under the "Exclusive Access" of some other host connected

via this controller.  All characteristic fields and  unit

flags are valid when that status is returned.

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

Unit-Available

Controller Error

Drive Error

See  Section section(getunitstemf), "GET UNIT STATUS Com-

mand, End Message Format."

MULTIHOST SUPPORT                                       PAGE 7-35

MULTIHOST GET UNIT STATUS COMMAND

7.6.3  Description

See  Section  section(getunitstdes),  "GET  UNIT  STATUS Command,

Description."

MULTIHOST SUPPORT                                       PAGE 7-36

MULTIHOST GET UNIT STATUS COMMAND

7.7  MULTIHOST ONLINE COMMAND

Command categories:

Sequential

7.7.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| unit flags | reserved |

+---------------+---------------+

| reserved |

+-------------------------------+

| |

+--- reserved ---+

| |

+-------------------------------+

| device dependent parameters |

+-------------------------------+

| reserved |

+-------------------------------+

unit flags

Host-settable      unit      flags.       See     Section

section(unit___flags), "Unit Flags."

If  the  unit is already "Unit-Online" to any host (class

driver), then the class driver  shall  specify  the  same

values  for controller supported host-settable unit flags

as are currently in effect on the unit.   The  controller

shall  check  that the flag values are the same.  If they

are different then it shall reject the command as an "In-

valid  Command"  ("invalid  unit  flags" parameter error.

MULTIHOST SUPPORT                                       PAGE 7-37

MULTIHOST ONLINE COMMAND

See    "Invalid    Command"     status     in     Section

section(status__codes), "Status Codes").

device dependent parameters

See  Section section(online___cmf), "ONLINE Command, Com-

mand Message Format."

Note  that  controllers  may  either maintain the "device

dependent     parameters"     separately     for     each

"Controller-Online"  class  driver  or singularly for all

"Controller-Online" class drivers, at their  option.   In

the  latter case the value expressed in this field by the

host that issued the  most  recent  ONLINE  or  SET  UNIT

CHARACTERISTICS  command  dictates which device dependent

parameters are in effect.

Allowable modifiers:

Allow Self Destruction

Clear Serious Exception (ignored for disk class devices)

Ignore Media Format Error

See  Section section(online___cmf), "ONLINE Command, Com-

mand Message Format."

The  server  may, at its option, reject the command as an

Invalid Command (subcode "Invalid Modifiers") if the unit

is already "Unit-Online" to another host.

Enable Set Write Protect

See  "unit  flags"  field  description  above and Section

section(online___cmf), "ONLINE Command,  Command  Message

Format."

Exclusive Access

If the "Exclusive Access" modifier is set and the unit is

"Unit-Available"  to  the  issuing  host   and   is   not

MULTIHOST SUPPORT                                       PAGE 7-38

MULTIHOST ONLINE COMMAND

"Unit-Online"  to nor under the "Exclusive Access" of any

other host, the unit becomes "Unit-Online" to  and  under

the "Exclusive Access" of the issuing host.

If the "Exclusive Access" modifier is set and the unit is

"Unit-Available" to and under the "Exclusive  Access"  of

the  issuing  host, the unit becomes "Unit-Online" to the

issuing host.  "Exclusive Access"  of  the  unit  is  re-

tained.

If the "Exclusive Access" modifier is set and the unit is

already "Unit-Online" to and not under the "Exclusive Ac-

cess" of the issuing host and is not "Unit-Online" to any

other host, the issuing host is  granted  "Exclusive  Ac-

cess"  of the unit.  The unit's state relative to the is-

suing host is otherwise unchanged.

If the "Exclusive Access" modifier is set and the unit is

"Unit-Online" to and under the "Exclusive Access" of  the

issuing host, the issuing host retains "Exclusive Access"

of the unit.  The unit's state relative  to  the  issuing

host is otherwise unchanged.

If  the "Exclusive Access" modifier is clear and the unit

is "Unit-Available" to and under the  "Exclusive  Access"

of  the  issuing  host, the unit becomes "Unit-Online" to

the issuing host and "Exclusive Access" of  the  unit  is

released.

If  the "Exclusive Access" modifier is clear and the unit

is "Unit-Online" to and under the "Exclusive  Access"  of

the  issuing  host,  "Exclusive  Access"  of  the unit is

released.  The unit's state relative to the issuing  host

is otherwise unchanged.

If  the  unit  is  "Unit-Offline" to the issuing host the

setting of the "Exclusive Access" modifier is ignored.

Note that granting "Exclusive Access" of a unit to a host

implicitly causes the unit to become "Unit-Offline  (sub-

state  'Exclusive Use')" to all other hosts connected via

this controller and "Unit-Offline  (substate  'Online  To

Another  Controller')" to hosts that may potentially have

access to the unit via another  controller.   Conversely,

the  release  of  a unit from the "Exclusive Access" of a

host   implicitly   causes    the    unit    to    become

"Unit-Available" to all other hosts.

MULTIHOST SUPPORT                                       PAGE 7-39

MULTIHOST ONLINE COMMAND

Note  also  that "Exclusive Access" is released if any of

the following events occur:

1.  The  controller  loses or breaks the connec-

tion to the host that has "Exclusive Access"

of the drive.

2.  The  drive becomes unknown to the controller

(e.g., port button disabled.)

3.  The      drive      becomes     inoperative,

"Unit-Offline, Inoperative."

4.  The  controller is 'rebooted', by itself, by

an operator, or by any host.

MULTIHOST SUPPORT                                       PAGE 7-40

MULTIHOST ONLINE COMMAND

7.7.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| unit flags |multiunit code |

+---------------+---------------+

| reserved |spndles|

+-------------------------------+

| |

+--- unit identifier ---+

| |

+-------------------------------+

| media type identifier |

+-------------------------------+

| reserved |

+-------------------------------+

| unit size |

+-------------------------------+

| volume serial number |

+-------------------------------+

status

The  validity of the unit characteristics varies with the

unit's state implied by the contents of this field.   See

Status    Codes    listed    below   and   Section   sec-

tion(setunitchdes), "SET  UNIT  CHARACTERISTICS  Command,

Description" for complete details.

unit flags

See Section section(unit___flags), "Unit Flags."

Exclusive Access

The  Exclusive Access unit flag is non-host-settable;

it is set when the unit is under the  "Exclusive  Ac-

cess"  of this or some other host via this controller

MULTIHOST SUPPORT                                       PAGE 7-41

MULTIHOST ONLINE COMMAND

and clear when it is not.

All  other  fields  of  the  ONLINE command's end message are

identical to the SET UNIT CHARACTERISTICS command's end  mes-

sage.   Refer  to  Section section(setunitchmemf), "Multihost

SET UNIT CHARACTERISTICS Command, End Message Format"  for  a

description of the fields.

Status Codes:

Success (subcode "Normal")

The  unit  has  become "Unit-Online" to the issuing host.

The unit may have already been "Unit-Online"  to  one  or

more  other hosts and, for those unit flags that are both

host-settable and supported by the controller, the  class

driver  specified  values identical to those currently in

effect on the unit.

As  described earlier (see "Exclusive Access" modifier in

the preceding section), "Exclusive Access" of the unit by

the  issuing  host  may  have  been granted, retained, or

released depending on the setting of the  "Exclusive  Ac-

cess"  modifier  and  whether or not the unit was already

under the "Exclusive Access" of the issuing host.

The  unit's state relative to all other hosts remains un-

changed unless "Exclusive Access" of the unit was granted

or released.  See "Exclusive Access" modifier in the pre-

ceding section.

Success (subcode "Already Online")

The  "Already Online" subcode bit flag is set only if the

unit was already "Unit-Online" to the issuing  host  and,

for those unit flags that are both host-settable and sup-

ported by the  controller,  the  class  driver  specified

values  identical  to  those  currently  in effect on the

unit.  The unit  remains  "Unit-Online"  to  the  issuing

host.

As  described earlier (see "Exclusive Access" modifier in

the preceding section) "Exclusive Access" of the unit  by

the  issuing  host  may  have  been granted, retained, or

released depending on the setting of the  "Exclusive  Ac-

cess"  modifier  and  whether or not the unit was already

under the "Exclusive Access" of the issuing host.

MULTIHOST SUPPORT                                       PAGE 7-42

MULTIHOST ONLINE COMMAND

The  unit's state relative to all other hosts remains un-

changed,  unless  "Exclusive  Access"  of  the  unit  was

granted  or released.  See "Exclusive Access" modifier in

the preceding section.

Invalid Command (subcode "Invalid Unit Flags")

The  unit  was already "Unit-Online" to one or more hosts

and, for those unit flags that are both host-settable and

supported  by  the controller, the class driver specified

different values from those currently in  effect  on  the

unit.   The  unit  state  relative  to  the  issuing host

remains unchanged.  The host-settable unit flags are  not

changed,  and  the end message returns their current set-

tings.

As  described earlier (see "Exclusive Access" modifier in

the preceding section) "Exclusive Access" of the unit  by

the issuing host may have been denied, granted, retained,

or released depending on the setting  of  the  "Exclusive

Access"  modifier,  whether  or  not the unit was already

under the "Exclusive Access" of the issuing host, and the

state of the unit relative to all other hosts.  Note that

controllers shall service the "Exclusive Access" modifier

before  checking  for  equality of the host-settable unit

flags.

The  unit's state relative to all other hosts remains un-

changed,  unless  "Exclusive  Access"  of  the  unit  was

granted  or released.  See "Exclusive Access" modifier in

the preceding section.

Unit-Offline

All  subcodes  may  be returned.  Note that "Unit-Offline

(subcode 'Exclusive Use')" status indicates that the unit

is  under the "Exclusive Access" of another host via this

controller.  All characteristic fields and unit flags are

valid  when that status is returned.  Also note that some

causes of a unit being "Unit-Offline" may  be  overridden

(suppressed) by the "Allow Self Destruction" command mod-

ifier.

Unit-Available (subcode "Already In Use")

The unit was "Unit-Available" to the the issuing host and

"Exclusive Access" of the unit was denied because  it  is

currently  "Unit-Online"  to  another  host via this con-

troller.  All characteristic fields and  unit  flags  are

valid.

MULTIHOST SUPPORT                                       PAGE 7-43

MULTIHOST ONLINE COMMAND

Command Aborted

Media Format Error

Controller Error

Drive Error

See  Section  section(online___emf), "ONLINE Command, End

Message Format."

MULTIHOST SUPPORT                                       PAGE 7-44

MULTIHOST ONLINE COMMAND

7.7.3  Description

The  ONLINE  command  is  used to bring a unit "Unit-Online," set

host-settable unit characteristics, and obtain those unit charac-

teristics  that  are  essential for proper class driver operation

(see Section section(unit___flags), "Unit Flags"  for  additional

information).   The  unit is spun-up, if necessary, and its heads

are loaded prior to returning the ONLINE command's  end  message.

Host-settable  characteristics  are  set exactly as if a SET UNIT

CHARACTERISTICS   command   were   issued   (see   Section   sec-

tion(setunitchmemf),  "Multihost  SET  UNIT  CHARACTERISTICS Com-

mand").  Host-settable characteristics are set after the unit has

been successfully spun-up and any other validity checks have suc-

ceeded.

In  addition,  "Exclusive Access" of the unit by the issuing host

may have been denied, granted, retained, or released depending on

the  setting  of  the "Exclusive Access" modifier, whether or not

the unit was already under the "Exclusive Access" of the  issuing

host, and the state of the unit relative to all other hosts.  See

"Exclusive Access" modifier description above.

Note  that  if  the  unit is already "Unit-Online" to the issuing

host or any other host(s), the unit's host-settable  characteris-

tics  and state relative to the issuing host as well as all other

hosts are not altered.

MULTIHOST SUPPORT                                       PAGE 7-45

MULTIHOST ONLINE COMMAND

7.8  MULTIHOST SET CONTROLLER CHARACTERISTICS COMMAND

Command Category:

Immediate

7.8.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | reserved |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| cntrlr.  flags | MSCP version |

+---------------+---------------+

| reserved | host timeout |

+---------------+---------------+

| quad-word |

+--- ---+

| time and date |

+-------------------------------+

|controller dependent parameters|

+---------------+---------------+

| crn | (optional)

+---------------+

MSCP version

See   Section   section(setcontrolcmf),  "SET  CONTROLLER

CHARACTERISTICS Command, Command Message Format."

cntrlr. flags

Host-settable controller flags.  See Section section(con-

trolerflags), "Controller Flags."

MULTIHOST SUPPORT                                       PAGE 7-46

MULTIHOST SET CONTROLLER CHARACTERISTICS COMMAND

Note   that   controllers  maintain  a  separate  set  of

host-settable     "controller     flags"     for     each

"Controller-Online" class driver.

host timeout

See   Section   section(setcontrolcmf),  "SET  CONTROLLER

CHARACTERISTICS Command, Command Message Format."

Note  that controllers maintain a separate "host timeout"

for each "Controller-Online" class driver.

quad-word time and date

See   Section   section(setcontrolcmf),  "SET  CONTROLLER

CHARACTERISTICS Command, Command Message Format."

Note  that  controllers maintain a single "quad-word time

and date" for all "Controller-Online" class drivers.

controller dependent parameters

See   Section   section(setcontrolcmf),  "SET  CONTROLLER

CHARACTERISTICS Command, Command Message Format."

Note that controllers may either maintain the "controller

dependent     parameters"     separately     for     each

"Controller-Online"  class  driver  or singularly for all

"Controller-Online" class drivers, at their option.

crn (connection reference number)

See   Section   section(setcontrolcmf),  "SET  CONTROLLER

CHARACTERISTICS Command, Command Message Format."

Allowable modifiers:

none

MULTIHOST SUPPORT                                       PAGE 7-47

MULTIHOST SET CONTROLLER CHARACTERISTICS COMMAND

7.8.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| rsvd | alloc |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| cntrlr.  flags | MSCP version |

+-------+-------+---------------+

|chvrsn | csvrsn|cntrlr.  timeout|

+-------+-------+---------------+

| |

+--- controller identifier ---+

| |

+-------------------------------+

| maximum byte count (optional) |

+-------------------------------+

alloc

Allocation  Class.   Zero,  unless  manually altered in a

controller implementation  dependent  manner.   Multihost

controllers  shall provide an external method for setting

this field (via DUP commands is sufficient).  When a  new

allocation  class  value  takes  effect,  all connections

shall be broken to allow hosts to recognize the new value

upon  reconnection.   Allocation class shall be preserved

in nonvolatile controller memory to allow correct  opera-

tion across power failure.

cntrlr.  flags

See Section section(controlerflags), "Controller Flags."

For  those  controller options that a controller supports

the  corresponding  non-host-settable  flag   and   those

host-settable  flags that are currently in effect for the

issuing host will be returned set.  Note that controllers

shall set the "Multihost Support" flag.

cntrlr.  timeout

MULTIHOST SUPPORT                                       PAGE 7-48

MULTIHOST SET CONTROLLER CHARACTERISTICS COMMAND

See   Section   section(setcontrolemf),  "SET  CONTROLLER

CHARACTERISTICS Command,  End  Message  Format,"  Section

section(command_timos),  "Command  Timeouts," and Section

section(getcommandmcmd), "Multihost  GET  COMMAND  STATUS

Command."

csvrsn

chvrsn

controller identifier

maximum byte count (optional)

See   Section   section(setcontrolemf),  "SET  CONTROLLER

CHARACTERISTICS Command, End Message Format."

Status Codes:

Success (subcode "Normal")

MULTIHOST SUPPORT                                       PAGE 7-49

MULTIHOST SET CONTROLLER CHARACTERISTICS COMMAND

7.8.3  Description

See  Section section(setcontroldes), "SET CONTROLLER CHARACTERIS-

TICS Command, Description."

For those host-settable controller characteristics that are main-

tained separately, controllers shall  operate  according  to  the

value(s)  specified  by each individual "Controller-Online" class

driver.   The  operational  characteristics  requested   by   one

"Controller-Online" class driver shall not interfere with or mod-

ify  in  any   manner   those   requested   by   another.    Each

"Controller-Online"   class  driver  is  expressly  permitted  to

specify different operational characteristics  and  as  a  result

realize different controller operation.

For  those  host-settable  characteristics  that  are  maintained

singularly, controllers shall operate according to  the  value(s)

specified  by the host that issued the most recent SET CONTROLLER

CHARACTERISTICS command.

MULTIHOST SUPPORT                                       PAGE 7-50

MULTIHOST SET CONTROLLER CHARACTERISTICS COMMAND

7.9  MULTIHOST SET UNIT CHARACTERISTICS COMMAND

Command Category:

Sequential

7.9.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved | unit number |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| unit flags | reserved |

+---------------+---------------+

| reserved |

+-------------------------------+

| |

+--- reserved ---+

| |

+-------------------------------+

| device dependent parameters |

+-------------------------------+

| reserved |

+-------------------------------+

unit flags

Host-settable      unit      flags.       See     Section

section(unit___flags), "Unit Flags."

Note   that   controllers   maintain   a  single  set  of

host-settable  unit  flags  for  all  "Controller-Online"

class drivers.

device dependent parameters

MULTIHOST SUPPORT                                       PAGE 7-51

MULTIHOST SET UNIT CHARACTERISTICS COMMAND

See Section section(setunitchcmf), "SET UNIT CHARACTERIS-

TICS Command, Command Message Format."

Note  that  controllers  may  either maintain the "device

dependent     parameters"     separately     for     each

"Controller-Online"  class  driver  or singularly for all

"Controller-Online" class drivers, at their  option.   In

the  latter case the value expressed in this field by the

host that issued the  most  recent  ONLINE  or  SET  UNIT

CHARACTERISTICS  command  dictates which device dependent

parameters are in effect.

Allowable modifiers:

Clear Serious Exception (ignored for disk class devices)

Enable Set Write Protect

See Section section(setunitchcmf), "SET UNIT CHARACTERIS-

TICS Command, Command Message Format."

Exclusive Access

If the "Exclusive Access" modifier is set and the unit is

"Unit-Available"  to  the  issuing  host   and   is   not

"Unit-Online"  to  any  other host, "Exclusive Access" of

the unit is granted to the issuing host.

If the "Exclusive Access" modifier is set and the unit is

"Unit-Available" to and under the "Exclusive  Access"  of

the  issuing  host, "Exclusive Access" of the unit is re-

tained.

If the "Exclusive Access" modifier is set and the unit is

"Unit-Online" to and not under the "Exclusive Access"  of

the  issuing  host  and is not "Unit-Online" to any other

host, the issuing host is granted "Exclusive  Access"  of

the unit.

If the "Exclusive Access" modifier is set and the unit is

"Unit-Online" to and under the "Exclusive Access" of  the

issuing host, the issuing host retains "Exclusive Access"

of the unit.

MULTIHOST SUPPORT                                       PAGE 7-52

MULTIHOST SET UNIT CHARACTERISTICS COMMAND

If  the "Exclusive Access" modifier is clear and the unit

is "Unit-Available" to and under the  "Exclusive  Access"

of  the  issuing  host, "Exclusive Access" of the unit is

released.

If  the "Exclusive Access" modifier is clear and the unit

is "Unit-Online" to and under the "Exclusive  Access"  of

the  issuing  host,  "Exclusive  Access"  of  the unit is

released.

If  the  unit  is  "Unit-Offline" to the issuing host the

setting of the "Exclusive Access" modifier is ignored.

In all the cases just listed the unit's state relative to

the issuing host remains unchanged.

Note that granting "Exclusive Access" of a unit to a host

implicitly causes the unit to become "Unit-Offline  (sub-

state  'Exclusive Use')" to all other hosts connected via

this controller and "Unit-Offline  (substate  'Online  To

Another Controller')" to hosts which may potentially have

access to the unit via another  controller.   Conversely,

the  release  of  a unit from the "Exclusive Access" of a

host   implicitly   causes    the    unit    to    become

"Unit-Available" to all other hosts.

Note  also  that "Exclusive Access" is released if any of

the following events occur:

1.  The  controller  loses or breaks the connec-

tion to the host that has "Exclusive Access"

of the drive.

2.  The  drive becomes unknown to the controller

(e.g., port button disabled.)

3.  The      drive      becomes     inoperative,

"Unit-Offline, Inoperative."

4.  The  controller is 'rebooted', by itself, by

an operator, or by any host.

MULTIHOST SUPPORT                                       PAGE 7-53

MULTIHOST SET UNIT CHARACTERISTICS COMMAND

7.9.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| unit number |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| unit flags |multiunit code |

+---------------+---------------+

| reserved |spndles|

+-------------------------------+

| |

+--- unit identifier ---+

| |

+-------------------------------+

| media type identifier |

+-------------------------------+

| reserved |

+-------------------------------+

| unit size |

+-------------------------------+

| volume serial number |

+-------------------------------+

status

The  validity of the unit characteristics varies with the

unit's state implied by the contents of this field.   See

Status  Codes  listed  later  in this section and Section

section(setunitchdes), "SET UNIT CHARACTERISTICS Command,

Description" for complete details.

unit flags

See Section section(unit___flags), "Unit Flags."

Exclusive Access

The  Exclusive Access unit flag is non-host-settable;

it is set when the unit is under the  "Exclusive  Ac-

cess"  of this or some other host via this controller

MULTIHOST SUPPORT                                       PAGE 7-54

MULTIHOST SET UNIT CHARACTERISTICS COMMAND

and clear when it is not.

spndles

The  number  minus one of spindles or spindle "groups" in

the unit.  The value of zero indicates one spindle.   See

Section   section(diskgeomformt),   "Disk   Geometry  and

Format."

multiunit code

unit identifier

media type identifier

unit size

volume serial number

See Section section(setunitchemf), "SET UNIT CHARACTERIS-

TICS Command, End Message Format."

Status Codes:

Success (subcode "Normal")

Success (subcode "Duplicate Unit Number")

The  unit is "Unit-Online" to the issuing host.  The unit

may also be "Unit-Online" to one  or  more  other  hosts.

The  unit  will operate according to the setting of those

unit flags that are both host-settable and  supported  by

the controller, as specified by the issuing class driver,

for all hosts to which the unit is "Unit-Online."

As  described earlier (see "Exclusive Access" modifier in

the preceding section) "Exclusive Access" of the unit  by

the  issuing  host  may  have  been granted, retained, or

released depending on the setting of the  "Exclusive  Ac-

cess"  modifier  and  whether or not the unit was already

under the "Exclusive Access" of the issuing host.

The  unit's state relative to all other hosts remains un-

changed,  unless  "Exclusive  Access"  of  the  unit  was

granted  or released.  See "Exclusive Access" modifier in

the preceding section.

Unit-Offline

All  subcodes  may  be returned.  Note that "Unit-Offline

(subcode 'Exclusive Use')" status indicates that the unit

is  under  the  "Exclusive Access" of some other host via

MULTIHOST SUPPORT                                       PAGE 7-55

MULTIHOST SET UNIT CHARACTERISTICS COMMAND

this controller.   All  characteristic  fields  and  unit

flags are valid when that status is returned.

Unit-Available (subcode "Available")

This  status  code  implies  "Success (subcode 'Normal')"

when "Exclusive Access" of the unit was granted while the

unit was "Unit-Available."

Unit-Available (subcode "Already In Use")

The unit was "Unit-Online" or "Unit-Available" to the the

issuing host and  "Exclusive  Access"  of  the  unit  was

denied  because  it is currently "Unit-Online" to another

host via this controller.  All characteristic fields  and

unit flags are valid.

Command Aborted

Controller Error

Drive Error

See Section section(setunitchemf), "SET UNIT CHARACTERIS-

TICS Command, End Message Format."

MULTIHOST SUPPORT                                       PAGE 7-56

MULTIHOST SET UNIT CHARACTERISTICS COMMAND

7.9.3  Description

The SET UNIT CHARACTERISTICS command is used to set host-settable

unit characteristics and obtain those unit  characteristics  that

are  essential for proper class driver operation.  The setting of

the host-settable unit flags is effective only when the  unit  is

"Unit-Online"  to the issuing host and affects all hosts to which

the unit is "Unit-Online."

In  addition,  "Exclusive Access" of the unit by the issuing host

may have been denied, granted, retained, or released depending on

the  setting of the "Exclusive Access" modifier, whether the unit

was already under the "Exclusive Access" of the issuing host, and

the  state  of  the  unit  relative to all other hosts.  See "Ex-

clusive Access" modifier description above.

Note  that  the  format of the SET UNIT CHARACTERISTICS command's

end message is identical to that of the ONLINE command's end mes-

sage  because the ONLINE command performs a SET UNIT CHARACTERIS-

TICS operation after bringing a unit "Unit-Online."

MULTIHOST SUPPORT                                       PAGE 7-57

MULTIHOST SET UNIT CHARACTERISTICS COMMAND

7.10  MULTIHOST TERMINATE CLASS DRIVER/SERVER CONNECTION COMMAND

Command Category:

Immediate

7.10.1  Command Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

| reserved |

+---------------+-------+-------+

| modifiers | rsvd | opcode|

+---------------+-------+-------+

| |

+--- ---+

| |

+--- ---+

| reserved |

+--- ---+

| |

+--- ---+

| |

+---------------+---------------+

| crn |

+---------------+

crn (connection reference number)

The  identification of the host whose class driver/server

connection is to be terminated.

Note  that  host  class  drivers  associate a "connection

reference number" with the "Controller-Online" connection

they  establish  with  the  server via the SET CONTROLLER

CHARACTERISTICS command  as  described  in  Section  sec-

tion(setcontrolcmf), "SET CONTROLLER CHARACTERISTICS Com-

mand, Command Message Format."

If  the  value supplied in this field is zero, the server

rejects the command as  an  "Invalid  Command"  ("invalid

crn"       parameter       error.        See      Section

MULTIHOST SUPPORT                                       PAGE 7-58

MULTIHOST TERMINATE CLASS DRIVER/SERVER CONNECTION COMMAND

section(status__codes), "Status Codes" for  complete  de-

tails.

Allowable modifiers:

None

MULTIHOST SUPPORT                                       PAGE 7-59

MULTIHOST TERMINATE CLASS DRIVER/SERVER CONNECTION COMMAND

7.10.2  End Message Format

31 0

+-------------------------------+

| command reference number |

+---------------+---------------+

|sequence number| reserved |

+---------------+-------+-------+

| status | flags |endcode|

+---------------+-------+-------+

| |

+--- ---+

| |

+--- ---+

| undefined |

+--- ---+

| |

+--- ---+

| |

+---------------+---------------+

| crn |

+---------------+

crn (connection reference number)

Zero  or  the  value supplied in the command message "crn

(connection reference number)" field.

If  this  field  is zero, the server was unable to find a

class driver/server connection associated with  the  host

specified  in the command message "crn (connection refer-

ence number)" field.

If  this  field  is  nonzero,  the  server  found a class

driver/server connection associated with the host  speci-

fied  in  the  command message "crn (connection reference

number)" field and performed the operations necessary  to

terminate it.

Status Codes:

Success (subcode "Normal")

MULTIHOST SUPPORT                                       PAGE 7-60

MULTIHOST TERMINATE CLASS DRIVER/SERVER CONNECTION COMMAND

7.10.3  Description

Support  of  the TERMINATE CLASS DRIVER/SERVER CONNECTION command

is a Disk MSCP server implementation dependent option.  Disk MSCP

servers  that  do not provide such support shall reject this com-

mand as an "Invalid Command" ("invalid  opcode"  protocol  error.

See  "Invalid  Command" status in Section section(status__codes),

"Status Codes").  Tape MSCP servers shall not implement this com-

mand.

Servers that support "write history logging" as described in Sec-

tion  section(wrihismdes),  "WRITE  HISTORY  MANAGEMENT  Command,

Description" shall also support the TERMINATE CLASS DRIVER/SERVER

CONNECTION command.  Note that servers that support  "write  his-

tory  logging"  but  do not provide Multihost Support shall treat

the  TERMINATE  CLASS  DRIVER/SERVER  CONNECTION  command  as   a

no-operation  as  described  in  Section section(nooperationcmd),

"No-Operation Commands."

"Write  history  logging"  and  the TERMINATE CLASS DRIVER/SERVER

CONNECTION command were defined primarily for the purpose of  as-

sisting in the operation of host based volume shadowing implemen-

tations.  All servers that support "write history  logging"  (and

therefore  the  TERMINATE CLASS DRIVER/SERVER CONNECTION command)

shall set the Write History Logging Support controller  flag.   A

server  that does not support "write history logging" may, at its

option, support the TERMINATE CLASS DRIVER/SERVER CONNECTION com-

mand.

If supported and the server also provides Multihost Support, this

command permits a  host  class  driver  to  terminate  the  class

driver/server  connection  a host has established with the server

receiving this command.

After  performing  the operations necessary to terminate the con-

nection between itself and the class driver located in the speci-

fied  host  system  the  server enters the "Controller-Available"

state  relative  to  that  class  driver.    See   Section   sec-

tion(cntrolerstates),  "Controller  States,"  for information re-

garding the processing of new and outstanding  commands  received

when  a  class driver/server connection is terminated.  Note that

the server shall "notice" connection terminations resulting  from

the  execution  of this command prior to returning this command's

end message.

Note  that  in  cases where multiple class drivers located in the

specified host system have established separate connections  with

the  server  receiving this command and each of those connections

MULTIHOST SUPPORT                                       PAGE 7-61

MULTIHOST TERMINATE CLASS DRIVER/SERVER CONNECTION COMMAND

is associated with the same "connection  reference  number,"  the

server shall terminate all connections so established.