CHAPTER 8

CONTROLLER INITIATED BAD BLOCK REPLACEMENT

8.1  CONTROLLER INITIATED BAD BLOCK REPLACEMENT OVERVIEW

Controller Initiated Bad Block Replacement is an MSCP option that

relieves host class drivers from the task of performing bad block

replacement on disk drives.

In  a disk MSCP server that does not provide Controller Initiated

Bad Block Replacement the server reports bad blocks to the  class

driver,  and  the  class driver and/or an auxiliary process asso-

ciated with it actually replaces the bad block.  With  Controller

Initiated  Bad  Block  Replacement, the disk MSCP server replaces

bad blocks itself, transparent to the  class  driver.   This  re-

quires that the server also take over maintenance of the Replace-

ment Control Table  (RCT),  which  contains  replacement  context

information.

Note  that  a  controller  may implement Controller Initiated Bad

Block Replacement on some or all of the disk types  that  may  be

connected  to  it,  at  its option.  In addition, Controller Ini-

tiated Bad Block Replacement does not apply to Tape Mass  Storage

Control  Protocol,  as  bad block replacement is not used on tape

class devices.

Conceptually,  Controller  Initiated Bad Block Replacement is im-

plemented by inserting a Bad Block Replacement  layer  between  a

Gatekeeper  layer  and  the  Disk  MSCP layer, as shown in Figure

figure(cibbr__layers) below.  The Gatekeeper layer  ensures  that

Sequential,  Non-Sequential,  and  Immediate  commands  behave as

described.  It recognizes when a Sequential command has begun ex-

ecution,  and  defers  all  Non-Sequential  commands on that unit

until  the  Sequential  command   completes.    The   Bad   Block

CONTROLLER INITIATED BAD BLOCK REPLACEMENT               PAGE 8-2

CONTROLLER INITIATED BAD BLOCK REPLACEMENT OVERVIEW

Replacement  layer performs bad block replacement and RCT mainte-

nance by issuing MSCP commands to the Disk MSCP layer.

Class Driver

A

|

V

/ +----------------------------------+

/ | Gatekeeper |

/ +----------------------------------+

MSCP server | Bad Block Replacement |

\ +----------------------------------+

\ | Disk MSCP |

\ +----------------------------------+

The  remainder  of  this chapter describes the algorithms used by

the Bad Block Replacement layer, the concepts that Controller In-

itiated  Bad  Block  Replacement  adds to standard Disk MSCP, the

changes to MSCP control message  formats  (described  in  Chapter

chapter(minimalcmdset),  "Minimal  MSCP  Command Set" and, if the

controller provides Multihost Support, Chapter chapter(multihost-

funct),  "Multihost  Support").   Actual  implementations of Con-

troller Initiated Bad Block Replacement need not  be  a  distinct

layer  or  use  exactly the algorithms given here, so long as all

class driver visible results are identical.

8.2  DATA SAFETY WRITE PROTECTION

As  described  in  Section section(write_protect), "Write Protec-

tion," MSCP has three forms of write protection:  Hardware  Write

Protection,  Software  Write  Protection,  and  Data Safety Write

Protection.  Data Safety Write Protection shall be put  into  ef-

fect  whenever  the  Bad Block Replacement layer detects that the

volume has an inconsistent Replacement Control Table (RCT)  or  a

replacement has failed on the volume.

When  a  unit is brought "Unit-Online," the Bad Block Replacement

layer examines block 0 of the RCT to see if the  volume  must  be

Data Safety Write Protected.  If so, the layer write protects the

unit and disables all bad block replacement attempts.  The  layer

provides  read-only access to the unit on a "best efforts" basis,

so that a host can copy the volume's data to a backup device.

CONTROLLER INITIATED BAD BLOCK REPLACEMENT               PAGE 8-3

DATA SAFETY WRITE PROTECTION

If  a  volume has an incomplete replacement attempt, and its unit

is Hardware Write Protected when it is brought "Unit-Online," the

Bad  Block  Replacement layer cannot complete the replacement at-

tempt.  As an incomplete replacement attempt is one  form  of  an

inconsistent RCT, the unit is Data Safety Write Protected when it

is brought "Unit-Online." If the unit subsequently ceases  to  be

Hardware Write Protected, the layer shall attempt to complete the

incomplete replacement.  If  this  succeeds,  Data  Safety  Write

Protection may be cleared for the unit.

The  only way to clear Data Safety Write Protection due to an in-

valid RCT is to reformat the volume.

8.3  BAD BLOCK REPLACEMENT

With  Controller Initiated Bad Block Replacement, the MSCP server

performs all bad block  replacement.   At  a  minimum,  the  MSCP

server  replaces the first bad block encountered by each transfer

command.  That is, the MSCP server shall replace all  bad  blocks

that  would  have  been  reported in any end message's "first bad

block" field by an MSCP server that does not  provide  Controller

Initiated  Bad  Block Replacement.  Replacement of any additional

bad blocks encountered by a transfer is strongly recommended, but

not required.

Bad  blocks  can  be  detected by both read and write operations.

For read operations, the success or failure of the read cannot be

changed  by  retrying  the read after performing the replacement.

However, with writes it is definitely possible (indeed, probable)

that a write that fails before the replacement will succeed after

the replacement.  Consider that most bad blocks detected on write

operations  will  be  due  to  invalid  headers -- defects in the

header area of a sector.  A write to such a block will  fail  be-

fore the replacement, but succeed afterwards.

For  this  reason, if a write operation fails on a bad block, the

Bad Block Replacement layer shall retry the write operation after

replacing the block.  This implies that the replacement operation

must be performed before such a write  command  completes  (i.e.,

before  the  end  message  is  returned).   In  all  other cases,

however, the  replacement  operation  may  be  asynchronous  with

respect to completion of the command that detected the bad block.

That is, since the  replacement  cannot  affect  the  success  or

failure  of  the command, the replacement may be performed either

before or after the command completes.

CONTROLLER INITIATED BAD BLOCK REPLACEMENT               PAGE 8-4

BAD BLOCK REPLACEMENT

In  addition to detecting bad blocks in transfers issued by hosts

or higher layers in the MSCP server, the  Bad  Block  Replacement

layer  may  also  check for bad blocks spontaneously, without re-

ceiving any actual commands.   In  effect,  whenever  a  unit  is

"Unit-Online,"  the  Bad Block Replacement layer may periodically

generate reads to the unit.  Any bad blocks detected by such  in-

ternally generated transfers should be replaced, exactly the same

as if the read were externally generated.

Bad  block  replacement  cannot  be  performed  when  the unit is

Hardware Write Protected.  If  a  bad  block  is  detected  on  a

Hardware  Write Protected unit, replacement is not attempted.  If

the unit becomes Hardware Write Protected after the bad block  is

detected,  but before the replacement is completed, some phase of

the bad block replacement  algorithm  will  fail  with  a  "Write

Protect"  error.   This  error  is  handled the same as any other

failure of the bad block replacement algorithm;  see the  discus-

sion below.

When  both  Disk MSCP and Controller Initiated Bad Block Replace-

ment are implemented in  the  same  controller,  it  is  strongly

recommended  that  the  controller  defer recognition of Hardware

Write Protection until all pending  replacement  operations  have

completed.   That  is, when an operator actuates the unit's write

protect mechanism,  the  controller  does  not  physically  write

protect  the  unit until after all previously detected bad blocks

have been replaced.  This is similar to the requirement in  stan-

dard  Disk  MSCP  of  providing  a smooth transition to the write

protected  state  (see  Section  section(write_protect),   "Write

Protection").

Bad  block  replacement  shall  be  performed  regardless  of the

Software Write Protect status of the unit.  If  a  bad  block  is

detected when the unit is Software Write Protected, the Bad Block

Replacement layer  shall  write  enable  the  unit,  perform  the

replacement,  then  re-write  protect  the unit.  The layer shall

guarantee that all host or higher layer write commands are  still

rejected in spite of the unit being temporarily write enabled.

Bad  block  replacement  shall  not be attempted when the unit is

Data  Safety  Write  Protected  or  when  the  unit  was  brought

"Unit-Online" with the "Ignore Media Format Error" modifier.

If  the  unit  is  Hardware  Write  Protected  when it is brought

"Unit-Online" and it subsequently ceases  to  be  Hardware  Write

Protected,  the Bad Block Replacement layer shall perform certain

actions before it attempts a replacement or modifies  the  volume

in any other way.  These actions are to read and rewrite Replace-

ment Control Table (RCT) block 0 and to attempt to  complete  any

CONTROLLER INITIATED BAD BLOCK REPLACEMENT               PAGE 8-5

BAD BLOCK REPLACEMENT

incomplete      replacement      operation.       See     Section

section(cibbractsbfmd), "Actions Before First Modification."

If  a  bad  block replacement fails for any reason, the Bad Block

Replacement layer handles the failure as follows:

1.  The  status  code returned for the command that detected

the bad block is not affected by the failed  replacement

attempt.   That  is, the status code appropriate for the

original access is returned.

2.  The  replacement  failure  is reported with a "Bad Block

Replacement Attempt" error log message (see Section sec-

tion(bbratmlogfmt)).

Many  of  the  errors that cause replacement failure are

themselves reported with error log messages.  For  exam-

ple,  failure of a multiread or multiwrite access to the

RCT  produces  at  least  two  error  log  messages:   a

(usually)  "Disk  Transfer  Error" message reporting the

access failure and a separate "Bad Block Replacement At-

tempt"  message  reporting the replacement failure.  All

such error log messages  that  report  the  cause  of  a

replacement   failure  should  have  the  "Error  During

Replacement" error log flag set and should be  sent  be-

fore  the "Bad Block Replacement Attempt" error log mes-

sage.

3.  The  Bad  Block Replacement layer forces the unit to be-

come "Unit-Available" (or "Unit-Offline")  with  respect

to all class drivers, exactly as if an AVAILABLE command

with the "All Class Drivers" modifier had  been  issued.

This,  of  course, causes an AVAILABLE Attention message

to be sent to all class drivers that have enabled them.

Note that Data Safety Write Protection will be in effect

when the unit's volume  is  next  brought  "Unit-Online"

(unless the problem is no longer present at that time).

4.  Between  the time that the server discovers the replace-

ment failure and the unit completes  its  transition  to

"Unit-Available,"  the  setting of the Data Safety Write

Protected unit flag is indeterminate.  This flag may  be

either  set  or clear if it is returned to a host during

this interval.

5.  Any  pending bad block replacement attempts for the unit

are discarded, exactly as if  the  unit  had  been  Data

Safety  Write  Protected  when  those  bad  blocks  were

CONTROLLER INITIATED BAD BLOCK REPLACEMENT               PAGE 8-6

BAD BLOCK REPLACEMENT

detected.  Error log messages or other notifications are

not generated to report the dropped replacements.

6.  No  attempt is made to replace any additional bad blocks

that  may  be  detected   during   the   transition   to

"Unit-Available,"  exactly  as  if  the  unit  were Data

Safety Write Protected.

7.  Subject  to the constraint of not attempting further bad

block replacement, the server shall  complete  any  out-

standing  commands  that  it has already initiated.  All

new commands  received  after  the  replacement  failure

shall  be  rejected.  Outstanding commands that have not

yet been initiated may be completed or  rejected.   Com-

mands   are   rejected   with   a   "Unit-Available"  or

"Unit-Offline" status code.

The  success or failure of read operations is not affected by bad

block replacement or the replacement's success or  failure.   The

status   code   reported   for  read  operations  is  the  status

appropriate to the original read attempt, before any  replacement

was performed.  The success or failure of write operations is af-

fected by replacement, since the write must be retried after  the

replacement  is  performed.   If  the  replacement  succeeds, the

status code reported is that appropriate to the retry (unless the

retry also encounters a bad block, in which case another replace-

ment and another retry are performed, and  the  process  repeated

until the retry does not encounter a bad block).  If the replace-

ment fails, the status code reported is that appropriate  to  the

original  write  attempt  that  caused the bad block replacement.

The cause of a bad block replacement failure is only reported  as

an event code in an error log message;  it is never reported as a

status code in an end message.

8.4  REPLACEMENT CONTROL TABLE ACCESS

The  standard  Disk MSCP server shall allow read and write access

to the Replacement Control Table (RCT), as  such  access  is  re-

quired  by higher layers in order to be able to perform bad block

replacement.  The Bad Block Replacement layer  need  not  provide

RCT  access,  however,  since it implements bad block replacement

itself.

CONTROLLER INITIATED BAD BLOCK REPLACEMENT               PAGE 8-7

REPLACEMENT CONTROL TABLE ACCESS

Even  though  it  is  not necessary for the Bad Block Replacement

layer to provide RCT access, in many cases it is desirable to  do

so.  Providing such access allows a host to examine certain addi-

tional information contained in RCT block 0.  Also, since access-

ing  a  replacement block is usually slower than accessing a good

logical block, a host could use  the  RCT  contents  to  allocate

files with critical, real-time access requirements in portions of

the volume that do not contain bad blocks.

Therefore  a  server's  Bad Block Replacement layer shall provide

one of the following RCT access options.  The server may  provide

different options for different disk types.  The options are:

1.  No  RCT  access.  Any attempt to access any block inside

the RCT will be rejected.

2.  Read-only  access  to  the entire RCT.  Attempts to read

any block in the first copy of the RCT will be  honored.

That  is,  a READ command with "logical block number" in

the range "unit size"  through  "unit  size"  plus  "RCT

size"  minus  one  and  "byte count" equal to the unit's

block size will be honored.  Any other attempt to access

the RCT will be rejected.

When an RCT access attempt is rejected the command is rejected as

an "Invalid  Command"  parameter  error  (see  "Invalid  Command"

status  in  Section  section(status__codes), "Status Codes").  If

the command is  a  READ  command  with  a  valid  "logical  block

number,"  but  the  "byte count" is not equal to the unit's block

size, then the subcode is  "Invalid  Byte  Count." Otherwise  the

subcode  is "Invalid Logical Block Number." Attempts to write the

RCT are rejected with an "Invalid Logical Block Number" subcode.

Note  that  option  2,  entire RCT access, only allows access at-

tempts to the first copy of the RCT.  The Bad  Block  Replacement

layer  uses the multicopy read algorithm to locate the proper RCT

copy to use in satisfying the request.

The  Bad  Block  Replacement layer reports which option it imple-

ments by altering the "RCT size" and "copies" parameters  in  the

GET  UNIT  STATUS  end  message when it passes that message on to

higher layers.  These parameters are altered to describe the host

accessible  portion  of  the  RCT.  The actual alterations are as

follows:

RCT Access Option "RCT size" "copies"

----------------- ---------- --------

No RCT access 0 0

CONTROLLER INITIATED BAD BLOCK REPLACEMENT               PAGE 8-8

REPLACEMENT CONTROL TABLE ACCESS

Entire RCT actual RCT size 1

It is recommended that high-end controllers provide access to the

entire RCT.  Minimal, low-end controllers will  probably  provide

no  RCT access.  Note that any disk that uses a nonstandard means

of bad block replacement, and therefore does not have an RCT, ap-

pears identical to a disk that provides no RCT access.

The  Bad  Block Replacement layer may optionally allow additional

RCT access when a unit is brought "Unit-Online" with the  "Ignore

Media Format Error" modifier.  This may include read-write access

and/or access to individual copies of the RCT.  The layer  should

ensure  that  the values returned for "RCT size" and "copies" re-

flect the RCT geometry presented to hosts.  Note that  any  addi-

tional  access  provided  is  intended for diagnostic use, and is

inherently controller dependent.  Attempts to write to  the  unit

when  the  "Ignore  Media Format Error" modifier has been set may

have undefined results, including  permanent  corruption  of  the

volume.   The  errors  produced  by  such  corruption  may be in-

distinguishable from device hardware failures.

8.5  ATOMIC BAD BLOCK REPLACEMENT

The  Bad Block Replacement layer shall perform bad block replace-

ment operations as atomic operations.  That is, while a bad block

replacement operation is in progress, any transfer operations for

either the bad block being replaced or block 0 of the Replacement

Control Table (RCT) shall not be performed.  Such transfer opera-

tions shall be deferred until after the bad block replacement has

completed.  For a multiblock transfer that includes the block be-

ing replaced, only the transfer of the block being replaced  need

be  deferred;  the blocks before and after the one being replaced

may be transferred immediately.

A  bad  block  replacement operation begins just before the "best

guess" data is obtained for the block.  This is  not  necessarily

the  same transfer that detects the bad block and causes it to be

replaced.  The bad block replacement operation  ends  just  after

RCT  block 0 is updated to indicate that replacement is no longer

in progress.  The Controller Bad Block  Replacement  layer  shall

lock  out  all  host  and  higher layer access to the block being

replaced and RCT block 0 between these two times.  Note that read

access must be locked out as well as write access.

CONTROLLER INITIATED BAD BLOCK REPLACEMENT               PAGE 8-9

ATOMIC BAD BLOCK REPLACEMENT

8.6  ERROR LOG MESSAGES

Any  error  severe  enough to warrant replacement of the affected

block is, by definition, a  "significant"  error  that  shall  be

reported  in  an  error log message.  The error log message shall

use the "Disk Transfer Error" format or some  other  format  that

specifies  the  disk  location  at which the error occurred.  The

"Bad Block Replacement Request" error log flag is set in the  er-

ror log message to indicate that replacement of the block will be

attempted.

When the replacement attempt completes, the Bad Block Replacement

layer generates an additional  "Bad  Block  Replacement  Attempt"

format error log message to report the replacement attempt's suc-

cess or failure.

Due  to  the  different  error characteristics of the Replacement

Control Table (RCT), controllers should use an  altered  strategy

for  logging errors on RCT accesses.  In the host area of a disk,

the most common errors are eliminated by bad  block  replacement.

Since  bad block replacement is not used in the RCT, these common

errors can be expected to occur,  and  are  therefore  relatively

uninteresting for an error log.

The  goal  for  logging  RCT  errors is to not log any error that

would cause bad block replacement in the host area.   Controllers

that contain both standard Disk MSCP and Controller Initiated Bad

Block Replacement can do this precisely, since they know  exactly

which  errors they use to trigger replacement.  Layered controll-

ers, where standard Disk MSCP and Controller Initiated Bad  Block

Replacement  are  distinct entities, can only approximate this by

filtering out "Data Errors." That is, an  independent  Bad  Block

Replacement  layer should only log the non-"Data Errors" that oc-

cur on RCT accesses.

The one exception to not logging RCT "Data Errors" is when an RCT

access fails -- that is, when the  multiread  or  multiwrite  al-

gorithms  fail,  indicating  that the read or write failed to all

copies of the RCT.  Whenever  the  multiread  or  multiwrite  al-

gorithms  fail, an error log message shall be generated reporting

the error that occurred when the last RCT copy was accessed.   If

this  error  is  a "Data Error," its event code is converted to a

"Media Format Error" with the same  subcode  value  before  being

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-10

ERROR LOG MESSAGES

reported in the error log message.  All other errors are reported

without event code alteration.

Other  than  the  altered  strategy for logging RCT "Data Errors"

described above, any errors resulting from accesses to  the  unit

during  bad block replacement are logged according to normal con-

troller policies.  The only change  is  that  the  "Error  During

Replacement" error log flag should be set in such messages, indi-

cating that the operation was requested by the bad block replace-

ment process.  All such messages should be issued before the "Bad

Block Replacement Attempt" format error log message that  reports

the replacement attempt's status.

8.7  UNIT CONTEXT INFORMATION

The  Bad  Block  Replacement  layer shall separately maintain the

following unit flags for each unit it has "Unit-Online":

Write Protected (software)

Write Protected (data safety)

The  reason for this is that the Bad Block Replacement layer sets

the "Write Protected (software)" unit flag in the Disk MSCP layer

if  either  of  these flags are set.  The replacement layer shall

keep track of these flags itself, in order to remember  the  rea-

son(s) for write protecting the unit.

The  Bad  Block  Replacement  layer  also maintains internal, per

unit, flags to record:

o  The  individual  causes of Data Safety Write Protection.

It uses these flags to return  the  proper  status  sub-

codes.

o  Whether Replacement Control Table (RCT) block 0 has been

rewritten  and  any  incomplete  replacements  finished.

Normally  these  actions  are performed when the unit is

brought "Unit-Online." However, if the unit is  Hardware

Write  Protected  when it is brought "Unit-Online," then

these actions cannot be performed.  Thus  this  flag  is

set  to remember to perform these actions when or if the

unit subsequently ceases to be Hardware Write Protected.

This    flag    is   further   described   in   Sections

section(cibbractsdonl),  "Actions  During  ONLINE"   and

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-11

UNIT CONTEXT INFORMATION

section(cibbractsbfmd),     "Actions     Before    First

Modification."

o  Whether the "Ignore Media Format Error" modifier was set

when the unit was brought  "Unit-Online." It  uses  this

flag to inhibit bad block replacement.

8.8  ACTIONS DURING ONLINE

The Bad Block Replacement layer performs two different courses of

action for an ONLINE command, depending on  whether  the  "Ignore

Media  Format  Error" modifier is set.  Normally, the layer reads

block 0 of the Replacement Control Table (RCT) to perform various

volume  consistency checks.  However, if the "Ignore Media Format

Error" modifier is set, the layer does not access the RCT in  any

way.

If an ONLINE command has the "Ignore Media Format Error" modifier

set, the Bad Block Replacement layer merely passes the unmodified

command  on to the Disk MSCP layer.  Similarly, it passes the un-

modified response from the Disk MSCP layer back to  the  host  or

higher  layer  that  issued the command.  The replacement layer's

only additional action is to remember, in an internal flag,  that

the  "Ignore  Media Format Error" modifier was set.  It uses this

flag to inhibit all bad block replacement  attempts  and  to  op-

tionally allow additional access to the RCT for diagnostics.

When the "Ignore Media Format Error" modifier is not set, the Bad

Block Replacement layer performs the following steps  to  process

an ONLINE command:

1.  Issue  an  ONLINE  command  to the Disk MSCP layer.  All

fields except the "modifiers" field are copied from  the

ONLINE  command  received by the replacement layer.  The

"modifiers" field is constructed as follows:

a.  The "Allow Self Destruction" modifier is copied from

the received command.

b.  If  the  Disk MSCP layer provides Multihost Support,

the "Exclusive Access" modifier is set.

c.  All other modifiers are clear.

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-12

ACTIONS DURING ONLINE

If the Disk MSCP ONLINE fails, return its end message as

the result of this layer's ONLINE and exit.  If the Disk

MSCP   ONLINE   reports   that   the   unit  is  already

"Unit-Online," the ONLINE has succeeded.  Again,  return

its end message as the result of this layer's ONLINE and

exit.  Otherwise save its end message as  the  prototype

for this layer's ONLINE end message.

2.  Read  block  0  of  the  RCT with the multicopy read al-

gorithm.  If the multicopy read algorithm fails,  report

its result as the status of this ONLINE and exit.

3.  If the unit is Hardware Write Protected, set an internal

flag and go to step 6.  This flag is  further  described

in Section section(cibbractsbfmd), "Actions Before First

Modification."

4.  Write the data read by step 2 back out to block 0 of the

RCT with the multicopy write algorithm.  If  the  multi-

copy  write  algorithm  fails,  report its result as the

status of this ONLINE and exit.

5.  If  the  data  read by step 2 indicates that there is an

incomplete bad block replacement on the volume,  attempt

to complete it.  If the completion attempt fails, report

it with an error log message.  Continue  with  the  next

step   regardless  of  whether  the  completion  attempt

succeeds or fails.

6.  Examine  the  contents of RCT block 0, as read in step 2

and possibly modified in step 5.  If there is an  incom-

plete  bad  block  replacement attempt or the RCT is in-

valid, set the "Write Protect (data safety)"  unit  flag

in  the  unit's  context.   The checks, if any, that the

controller performs to verify  the  RCT's  validity  are

specified  by  the  Digital  Storage  Architecture  Disk

Format and/or the controller's functional specification.

7.  If  the  "Enable  Set Write Protect" modifier was set in

the original ONLINE command  received  from  the  higher

layer,  then  copy the "Write Protected (software)" unit

flag from the original command to the unit's context.

8.  If either of the "Write Protect (data safety)" or "Write

Protect (software)" unit flags are  set  in  the  unit's

context, issue a SET UNIT CHARACTERISTICS command to the

Disk MSCP layer to software write protect the unit.   If

the  command  fails,  report its status as the status of

this ONLINE and exit.

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-13

ACTIONS DURING ONLINE

9.  The  ONLINE  operation has completed successfully.  Copy

the "Write Protect (data  safety)"  and  "Write  Protect

(software)"  unit  flags from this unit's context to the

prototype end message saved in step 1.  If the  unit  is

Data  Safety Write Protected, set the appropriate status

subcodes.  Return the resulting end message to the  host

or higher layer.

If  the ONLINE operation fails, clean-up may or may not be neces-

sary, depending on where the failure occurs.  If the failure  oc-

curs  in  step 1 (the ONLINE command to the Disk MSCP layer), the

unit will not be "Unit-Online" through the Disk MSCP layer, so no

clean-up  is necessary.  If the failure occurs anywhere else, the

Bad Block Replacement layer shall issue an AVAILABLE command with

the  "Spin-down"  modifier  set to the Disk MSCP layer, since the

unit may still be "Unit-Online" through the Disk MSCP layer.

As  part of bringing a unit "Unit-Online," the Bad Block Replace-

ment layer attempts to complete any incomplete replacement on the

volume.  If the unit is not Hardware Write Protected, the attempt

is made as part of the ONLINE command.  If the unit  is  Hardware

Write  Protected, the attempt is made after the unit ceases to be

Hardware Write Protected and no later than the first  attempt  to

write  to  the unit or the first SET UNIT CHARACTERISTICS command

after the unit ceases to be Hardware Write  Protected.   This  is

discussed  in  Section  section(cibbractsbfmd),  "Actions  Before

First Modification." In either case, there is only one attempt to

complete  the  replacement.   If that one attempt fails, no other

attempt will be made until the next time the  volume  is  brought

"Unit-Online."

8.9  ACTIONS DURING SET UNIT CHARACTERISTICS

The  Bad  Block Replacement layer performs the following steps to

process a SET UNIT CHARACTERISTICS command:

1.  If the internal flag is set indicating that the unit was

Hardware   Write   Protected   when   it   was   brought

"Unit-Online"   and  it  is  no  longer  Hardware  Write

Protected, perform the actions described in Section sec-

tion(cibbractsbfmd), "Actions Before First Modification"

and clear the flag.  If those actions fail, report their

result  as  the  status of this SET UNIT CHARACTERISTICS

and exit.  Note that this  step  may  clear  the  "Write

Protect (data safety)" unit flag in the unit's context.

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-14

ACTIONS DURING SET UNIT CHARACTERISTICS

2.  If  the  "Enable  Set Write Protect" modifier was set in

the original SET UNIT CHARACTERISTICS  command  received

from  the  higher  layer, then copy the "Write Protected

(software)" unit flag from the original command  to  the

unit's context.

3.  Issue  a  SET  UNIT  CHARACTERISTICS command to the Disk

MSCP layer.  The command is identical  to  the  original

SET   UNIT   CHARACTERISTICS  command  received  by  the

replacement layer with the following changes:

a.  The  "Enable  Set  Write Protect" modifier is always

set.

b.  The  "Write  Protect (software)" unit flag is set to

the  "inclusive-or"  of  the  "Write  Protect  (data

safety)"  and  "Write Protect (software)" unit flags

in the unit's context.

If the command fails, report its status as the status of

this SET UNIT CHARACTERISTICS and exit.

4.  The  SET  UNIT  CHARACTERISTICS  operation has completed

successfully.  Copy the "Write  Protect  (data  safety)"

and  "Write  Protect  (software)"  unit  flags from this

unit's context to the end message returned in the previ-

ous  step.   If the unit is Data Safety Write Protected,

set the appropriate status subcodes.  Return the result-

ing end message to the host or higher layer.

If  the  SET UNIT CHARACTERISTICS operation fails for any reason,

the Bad Block Replacement layer shall issue an AVAILABLE  command

with  all  modifiers  clear  to  the  Disk MSCP layer and send an

AVAILABLE attention message to the host and higher layers.   This

is  necessary  to preserve the rule that the unit may not be left

"Unit-Online" if a command that might change the  unit's  context

fails.

8.10  ACTIONS BEFORE FIRST MODIFICATION

As  part of bringing a unit "Unit-Online," the Bad Block Replace-

ment layer reads and rewrites  Replacement  Control  Table  (RCT)

block  0.   The reason for this is that, when the volume was last

in use, there may have been a failure after some  copies  of  RCT

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-15

ACTIONS BEFORE FIRST MODIFICATION

block  0  were  written  but before all of them were written.  If

this situation were to occur, a transient or  inconsistent  error

could  cause  successive reads of RCT block 0 to return different

data, as data was returned from different copies.  By reading and

rewriting  block  0 the layer ensures that all copies contain the

same data, so that successive reads will always return  the  same

results.

However,  if  the  unit  is  Hardware  Write Protected when it is

brought "Unit-Online," this problem can still occur since block 0

cannot  be rewritten.  So long as the unit remains Hardware Write

Protected, no problems can occur, since the volume cannot be mod-

ified.   However,  if  the  unit  ceases  to  be  Hardware  Write

Protected, volume modification becomes possible.  If this occurs,

RCT  block 0 shall be read and rewritten prior to the first modi-

fication.

When  a  unit is brought "Unit-Online," the Bad Block Replacement

layer sets an  internal  flag  if  the  unit  is  Hardware  Write

Protected  and  RCT block 0 cannot be rewritten.  This flag shall

be checked when a host or higher layer  attempts  to  modify  the

volume  and, if it is set, the actions described below performed.

There are three ways that the volume may be modified:

1.  Bad  block replacement.  The actions below are performed

after checking that  the  unit  is  not  Hardware  Write

Protected,  but  before  checking  if  the  unit is Data

Safety Write Protected or modifying the RCT in any  way.

That is, these actions are performed after the bad block

is detected but before the  bad  block  replacement  al-

gorithm is initiated.

2.  A  SET UNIT CHARACTERISTICS command.  The point at which

the actions below are performed is described in  Section

section(cibbractsdsuc),   "Actions   During   SET   UNIT

CHARACTERISTICS."

3.  A  host  or  higher  layer write operation.  The actions

below are performed after checking that the unit is  not

Hardware or Software Write Protected but before any mod-

ification is performed on the unit.

The  Bad  Block Replacement layer need not wait until a modifica-

tion is attempted.   It  may  perform  the  actions  below  spon-

taneously  upon  the unit ceasing to be Hardware Write Protected.

From the viewpoint of a host or higher layer,  this  is  no  dif-

ferent  from  the actions below being performed in response to an

attempted bad block replacement.

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-16

ACTIONS BEFORE FIRST MODIFICATION

The actions that shall be performed before the first modification

are:

1.  Check the internal flag discussed above.  If the flag is

clear, proceed with the modification.  If  the  flag  is

set, clear it and continue with the next step.

2.  Read  block  0  of  the  RCT with the multicopy read al-

gorithm.  If the multicopy read algorithm fails, exit.

3.  Check  the internal flags recording the reasons, if any,

for the unit being Data Safety Write Protected.  If  the

incomplete replacement flag is clear, then clear the in-

complete replacement flags in RCT block 0.  If  the  in-

valid RCT flag is clear, implying that the RCT was valid

when the unit was brought "Unit-Online," yet the current

RCT image is not valid, either exit with a "Media Format

Error" or "fix" the invalid RCT.  An example of "fixing"

the  RCT  is zeroing a reserved field that was zero when

the unit was brought  "Unit-Online,"  but  that  is  now

nonzero.

4.  Write RCT block 0 with the value read in step 2 and pos-

sibly modified in step 3, using the multicopy write  al-

gorithm.  If the multicopy write algorithm fails, exit.

5.  If the value just written to RCT block 0 indicates there

is an incomplete bad block replacement  on  the  volume,

attempt  to  complete  it.   If  the  completion attempt

fails, report it with an error log message and continue.

6.  Set the "Write Protected (data safety)" unit flag to the

"inclusive-or" of all internal flags representing possi-

ble causes of Data Safety Write Protection.

7.  Issue  a  SET  UNIT  CHARACTERISTICS command to the Disk

MSCP layer to set  the  unit's  Software  Write  Protect

status to the "inclusive-or" of the "Write Protect (data

safety)" and "Write Protect (software)"  unit  flags  in

the unit's context.  If the command fails, exit.

8.  This  algorithm has completed.  Proceed with the modifi-

cation attempt that caused it to be invoked.

There  are  four  places where this algorithm can fail.  They are

the multicopy read operation in step 2, the attempt to  "fix"  an

invalid RCT in step 3, the multicopy write in step 4, and the SET

UNIT CHARACTERISTICS in step 7.  If the algorithm fails at one of

these  steps, the modification attempt fails with the status code

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-17

ACTIONS BEFORE FIRST MODIFICATION

resulting from  the  failed  step.   Also,  the  unit  is  forced

"Unit-Available"  to  all  class  drivers.  The handling of other

commands that may be outstanding is identical to  that  which  is

done for a failed replacement operation.

Note  that  execution of the above procedure may complete the in-

complete replacement that was causing a unit to  be  Data  Safety

Write  Protected.   This  yields the possibility that Data Safety

Write Protection may disappear spontaneously, as the result of  a

bad block replacement.

8.11  MSCP CONTROL MESSAGE FORMAT CHANGES

This  section  describes  the  changes in control message formats

between standard Disk MSCP and MSCP with Controller Initiated Bad

Block  Replacement.   All  messages  and fields that are not men-

tioned in this section are identical to the descriptions found in

Chapter chapter(minimalcmdset), "Minimal MSCP Command Set" and if

the controller provides Multihost Support, Chapter chapter(multi-

hostfunct), "Multihost Support."

8.11.1  Controller Flags

The Controller Initiated Bad Block Replacement controller flag is

returned set in the "controller flags" field of a SET  CONTROLLER

CHARACTERISTICS command's end message only if the server supports

Controller Initiated Bad Block Replacement on all disk types that

can be connected to it.  Note that a controller is expressly per-

mitted to implement replacement on some disk  types  but  not  on

others.   In  that  case  this flag shall be returned clear.  See

also the description of this flag in  Section  section(controler-

flags), "Controller Flags" for additional detail.

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-18

MSCP CONTROL MESSAGE FORMAT CHANGES

8.11.2  Unit Flags

The  Controller  Initiated Bad Block Replacement unit flag is re-

turned set in the "unit flags" field of the GET UNIT STATUS,  ON-

LINE, and SET UNIT CHARACTERISTICS command's end messages only if

bad block replacement for this unit will be performed by the con-

troller.   This flag is returned clear if the host is responsible

for bad block replacement for this unit.  (As noted in the previ-

ous  section, a controller may implement replacement on some disk

types but not on others.) See also the description of  this  flag

in Section section(unit___flags), "Unit Flags" for additional de-

tail.

8.11.3  Transfer Commands

The  changes  described in this section apply to the ACCESS, COM-

PARE HOST DATA, ERASE, READ, and WRITE commands.

The  meaning  of  transfers  addressed to the Replacement Control

Table (RCT) portion of the disk is  substantially  changed;   see

Section   section(cibbrrctacces),   "Replacement   Control  Table

Access." For transfers addressed to the host area  of  the  disk,

the only change is that the controller itself replaces bad blocks

rather than the host.

Since  the  controller  is  replacing  bad blocks, the "Bad Block

Reported" end flag will always be clear and the contents  of  the

"first  bad  block"  end  message field will always be undefined.

The "Bad Block Unreported" end  flag  is  set,  however,  if  the

transfer encountered one or more bad blocks.  The specific blocks

that were bad and the results of their replacement  are  reported

only via error log messages.

8.11.4  ONLINE and SET UNIT CHARACTERISTICS Commands

Modifiers:

The  "Ignore  Media  Format  Error"  modifier is changed from

standard Disk MSCP.  Note that this modifier is only  allowed

on the ONLINE command.  Its revised meaning is as follows:

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-19

MSCP CONTROL MESSAGE FORMAT CHANGES

Ignore Media Format Error

This  modifier is only allowed on the ONLINE command.  In

standard  Disk  MSCP,  this  modifier  suppresses   media

(volume)  format validity checking.  With Controller Ini-

tiated  Bad  Block  Replacement,   this   modifier   also

suppresses  the  Replacement  Control  Table (RCT) checks

normally performed when a unit is brought "Unit-Online."

Normally,  when a unit is brought "Unit-Online," the MSCP

server makes several accesses to the unit.   In  standard

Disk  MSCP,  these  accesses  merely verify volume format

validity.  With Controller Initiated Bad  Block  Replace-

ment,  the MSCP server also accesses the RCT to check the

context information stored there.  If any  of  these  ac-

cesses fail, the unit cannot be brought "Unit-Online" and

a  Media  Format  Error  is  returned.    This   modifier

suppresses  all accesses performed when bringing the unit

"Unit-Online,"   thus   allowing   it   to   be   brought

"Unit-Online"   when  these  accesses  would  fail.   The

penalty for using this  modifier  is  that  data  on  the

volume may be corrupted.

The  exact  checks  that  are suppressed by this modifier

are:

1.  The format validity checks done by standard Disk

MSCP.

2.  Checking for an incomplete bad block replacement

operation, and completing it if possible.

The unit is always brought "Unit-Online" with Data Safety

Write Protection disabled (i.e., writes allowed).

Setting  this modifier inhibits all bad block replacement

attempts and may optionally provide additional access  to

the RCT.

Note that these checks exist to prevent data loss or cor-

ruption.  Therefore this modifier should only be used  in

exceptional circumstances, such as backing up a known bad

disk.  In particular, the results of any attempt to write

to  the  unit  are  undefined when this modifier has been

set.

Status Codes:

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-20

MSCP CONTROL MESSAGE FORMAT CHANGES

The following new subcodes may be returned with the "Success"

status code for the ONLINE and SET UNIT CHARACTERISTICS  com-

mands.   Remember  that  the subcodes are bit flags, and that

the subcode field contains the "logical-or" of all applicable

subcodes,  including  the  subcodes  defined in standard Disk

MSCP.

Success (subcode "Incomplete Replacement")

The  volume  has a partially completed bad block replace-

ment operation, and the attempt to complete  it  did  not

succeed.   The cause of the attempt's failure is reported

with an error log message.  This subcode  merely  reports

that  there  is  a partially completed replacement on the

volume.  Note that the unit will  be  Data  Safety  Write

Protected whenever this subcode is returned.

Success (subcode "Invalid RCT")

The  volume  has  an  invalid  RCT or, if the disk uses a

nonstandard means of bad  block  replacement,  some  data

structure  used  to  control bad block replacement is in-

valid.  The checks, if any, that the controller  performs

to verify the RCT's validity are specified by the Digital

Storage Architecture Disk Format and/or the  controller's

functional  specification.   Note  that  the unit will be

Data Safety Write Protected whenever this subcode is  re-

turned.

"Media Format Errors" may be reported for SET UNIT CHARACTER-

ISTICS, which could not happen with standard Disk MSCP.

8.11.5  GET UNIT STATUS Command

End Message Fields:

RCT size

copies

The interpretation and meaning of these fields is identi-

cal to standard Disk MSCP.  However, the values  returned

are  altered  as described in Section section(cibbrrctac-

ces), "Replacement Control Table Access."

Status Codes:

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-21

MSCP CONTROL MESSAGE FORMAT CHANGES

The following new subcodes may be returned with the "Success"

status code for the GET UNIT STATUS commands.  These subcodes

are identical to the new subcodes returned for the ONLINE and

SET UNIT CHARACTERISTICS commands;  see the previous  section

for  their  description.   Remember that the subcodes are bit

flags, and that the subcode field contains  the  "logical-or"

of all applicable subcodes, including the subcodes defined in

standard Disk MSCP.

Success (subcode "Incomplete Replacement")

Success (subcode "Invalid RCT")

8.11.6  REPLACE Command

This  command  is  illegal  with  Controller  Initiated Bad Block

Replacement.  As its sole use is to allow hosts  to  perform  bad

block  replacement, it is not needed.  This command, if issued to

a unit on which the controller is performing bad  block  replace-

ment  is  treated  as an "Invalid Command" ("invalid opcode" pro-

tocol  error.   See   "Invalid   Command"   status   in   Section

section(status__codes), "Status Codes").

8.12  HOST SUPPORT FOR CONTROLLER INITIATED BAD BLOCK REPLACEMENT

The only specialized support that host class drivers must provide

for Controller Initiated Bad Block Replacement is at the time the

host brings a unit "Unit-Online." That is, these actions shall be

performed immediately after the ONLINE  command  completes.   The

specific actions are:

1.  Check  the  "Controller Initiated Bad Block Replacement"

unit flag.  If this flag  is  clear,  the  class  driver

shall examine the Replacement Control Table (RCT) for an

incomplete  replacement  and  complete  it  if  one   is

present.   If  this  flag is set, the class driver shall

skip this step, as the server has already performed it.

2.  If  either  of  the "Incomplete Replacement" or "Invalid

RCT" subcode flags is set, report  the  problem  to  the

user  or  take  whatever  other  action  is  dictated by

CONTROLLER INITIATED BAD BLOCK REPLACEMENT              PAGE 8-22

HOST SUPPORT FOR CONTROLLER INITIATED BAD BLOCK REPLACEMENT

operating system policy.  For the  "Incomplete  Replace-

ment"  flag, this might be a request to remount the unit

write enabled so that the replacement can be  completed.

For the "Invalid RCT" flag, this might be a warning that

the volume has been corrupted.

In  all  other  cases,  the  Bad  Block Replacement layer reports

things in a manner that is transparent to class drivers.  It will

never  set the "Bad Block Reported" end flag, so the class driver

will never attempt a replacement.  If the class  driver  attempts

to read the RCT, the reported geometry is such that the multicopy

read algorithm will work correctly.

Aside  from  the above, a class driver that only supports servers

with Controller Initiated Bad Block Replacement need  not  imple-

ment  the  Host  Initiated Bad Block Replacement, multicopy read,

and multicopy write algorithms.  It also need never check the RCT

for   incomplete  replacements.   Such  a  class  driver  should,

however, check the "Controller Initiated Bad  Block  Replacement"

unit flag whenever it brings a unit "Unit-Online." If the flag is

clear, then that unit does  not  have  Controller  Initiated  Bad

Block Replacement, so it cannot be supported by the class driver.

The class driver should immediately issue an AVAILABLE command to

the unit and refuse to access it.